# 3GPP TSG-RAN WG4 Meeting #115	R4-2507938
Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.5.2

Source: 	OPPO, Huawei, Apple, Nokia

Title: 	TP for triggering condition option1 in FS_NR_DL_Frag_Carrier

Document for:	Approval

- Introduction

In previous RAN4 meeting, the triggering condition to enable “One RF Rx chain” and how to indicate UE capability for supporting “one Rx RF chain” were discussed and some agreements were agreed. In this contribution, we draft the TP on triggering condition proposal for Option 1.

TP for TR38.755

8	Study of means for a UE to inform the network of appropriate CA configuration it can support

<Start of TP>

8.3	Triggering condition to enable ”One Rx RF chain”

When discussing a method to enable or disable using one Rx RF chain RAN4 would need a common understanding of ‘Triggering condition’. RAN4 can consider triggering a switch between using one Rx RF chain or separate Rx RF chains for reception of multiple CCs in a fragmented deployment can be performed using different but not limited in the following approaches:

- • Controlled by network based on UE assistance information.

- • Controlled by UE based on network assistance/configuration and with/without network knowledge.

- • Controlled by UE autonomously without network assistance/configuration and with/without network knowledge.

When considering triggering conditions RAN4 needs to consider possible assistance information related to such triggering type (for example measurements, thresholds etc.), and necessary UE requirements related to providing such information to the network.

8.3.1 Triggering condition proposal for option 1: Rely on existing channel quality reporting and procedures

One possible option of the triggering condition is to rely on existing channel quality reporting and procedures.

Below Figure 8.3.1-1 shows an existing channel quality reporting procedures related to CA. Before the SCell is added/activated, L3-measurement is available, including measurement quantities of RSRP/RSRQ/SINR. Measurement gap or interruption is specified depending on the measurement type, i.e. inter-frequency/intra-band CA/inter-band CA. L3-measurement is filtered and could reflect relatively long-term channel quality. After the SCell is activated, CQI measurement and report is available. The L3-measurement is also available, but no interruption is allowed.

Figure 8.3.1-1	Existing channel quality reporting and procedures

Option 1-1: Reuse the existing channel quality reporting procedure without any change.

In this option, the measurements are done with separate Rx RF chains. With this approach, the CA with “One Rx RF chain” is handled in the same way of all the other CA configurations. One of the reasons is that the observed RF performance degradation of “One Rx RF chain” is comparable with many CA configurations in existing specification. It may not be necessary to distinguish the REFSENS MSD caused by “One Rx RF chain” and caused by other factors such as harmonics and intermodulation, although the degradation due to “One Rx RF chain” and the degradation due to channel fading may require the network to take different actions. This approach may be simpler in a commercial deployment if there is no need to upgrade or no impact to an existing network deployment. This option is feasible especially when the RF performance degradation caused by “One Rx RF chain” is small, such as TDD bands, inter-operator collocated scenario, or UE locates in the cell centre. If the RF performance degradation is very large, there might be ping-pong effect of SCell adding/releasing due to inaccurate measurement results.

Option 1-2: Reuse the existing channel quality reporting procedure with “One Rx RF chain” measurement.

In this option, the measurements can be done with “One Rx RF chain” for two CCs simultaneously. Some companies suggested refining the procedures and monitoring the impact of “One Rx RF chain” to get a better decision on CA configuration and UE RF architecture. In current channel quality reporting procedure, the L3-measurement or CQI measurement on an SCell is based on separate RF chain architecture. The measurement results can’t reflect the degradation of RF performance due to strong self-interference or strong in-gap interference. To improve the CA 1 Rx RF chain configuration decision, it is useful having the UE to additionally perform the measurement based on “One Rx RF chain”. For example, before configuring SCells as intra-band CA, the network will indicate the UE to measure the potential CCs within the same band. Depending on UE’s ability to measure the channel quality for two CCs simultaneously using “One Rx RF chain”, or if the UE has more Rx chains, the UE can use either “One Rx RF chain” or separate RF chains to perform the measurement.

<End of TP>