3GPP TSG RAN WG4 Meeting #115		 R4-2507956

Malta, MT, 19th – 23th May, 2025

Agenda Item:	7.5.2

Source: 		vivo, Apple, Qualcomm, MTK

Title: 	TP for TR signaling indication for Option 2 new notations

Document for:	Approval

1. Introduction

In RAN4#114 the TR [1] has been approved. In this paper we provide our view about the indication of fragmented CA alternative 2, i.e., the new notation. And the following part would be added to clause [8.1.3 Indication of fragmented CA alternative 2: New notation for CA configurations with single Rx RF Chain] in the TR 38.755.

2. TP

<Start of TP>

8.1.3	Indication of fragmented CA alternative 2: New notation for CA configurations with single Rx RF Chain

Regarding how to indicate UE capability for supporting “one Rx RF chain” for DL fragmented carriers, a new notation FnA could be used to represent there are n NC CCs in one band sharing one RX RF chain. For example: When there are 2 NC CCs of band nX receiving in the separate mode (e.g., fully separate mode or partially shared mode), the CA combination type is CA(2A). When the receiving state switches to one RX RF chain mode, the CA combination type should change to CA(F2A), F represents there are two NC CCs in the one RX RF chain mode and in the sharing chain.

Similarly, CA_nX(F2A-A) means there are 3 NC CCs and two of them are in the sharing chain (one RX RF chain mode), CA_nX(F2A-F2A) means there are 4 NC CCs and two of them are in one sharing chain, the other two are in another sharing chain.

Any backwards compatibility issues with a network which does not understand the capability for supporting CA with “one Rx RF chain” would have to be analysed during the WI phase.

<End of TP>

3. References

- R4-2505158 draft TR 38.755_v020_r1-cl