3GPP TSG-RAN WG4 Meeting # 115											R4-2508270

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.24.1

Source:	Samsung

Title:	Ad-hoc minutes for NR_MIMO_Ph5

Document for:	Approval

Introduction

This Ad-hoc minutes is to cover the discussion in [115][217] NR_MIMO_Ph5_Part1 and [115][218] NR_MIMO_Ph5_Part2 for the following issues:

45 min discussion for [218].

45 min disucssion for [217]

Topic #1: RRM requiements for MIMO_Ph5_Part2

- Issue 2-1: For Scenario#2, how to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Background: In RAN4#114bis, RAN4 agreed to wait for RAN1 reply regarding this issue (as shown below). The corresponding RAN1 reply LS was also sent to RAN4 during the last meeting as captured below. RAN4 need to discuss this issue based on the received reply from RAN1.

RAN4#114bis agreement: - Wait for RAN1 reply to the LS on whether the existing agreement for a single downlink timing reference is extended to Scenario#2.

RAN1 Reply LS (R1-2503091): Question: Based on RAN1 LS (R1-2410870), RAN4 discussed timing requirement for two TA enhancement for asymmetric DL sTRP/ UL mTRP. RAN4 would like to check with RAN1 whether RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). Answer: The RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are NOT applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). And RAN1 made the following new agreement for further clarification: Agreement: - For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG - Above applies for the case UE is configured with SSB-MTC-additionalPCI.

- Proposals

  - Proposal 1 (Apple, MediaTek, Huawei, Ericsson [for inter-cell multi-TRP], ZTE): Define Scenario#2 2TAs requirements based on R18 MIMO for multi-DCI mTRP operation with 2 TAs.

    - Proposal 1a (Apple): Define the following for scenario #2:

      - For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState having the same TAG as the uplink signal. For each TAG the uplink transmission timing takes place (NTA + NTA offset) x Tc before the reception of the downlink reference point.

    - Proposal 1b (Nokia): the UE shall monitor two DL reference timings, one per TAG or TRP.

  - Proposal 2 (Qualcomm): For Scenario #2 the uplink transmission timing for PUCCH/PUSCH/SRS takes place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signals (s) of the reference cell associated with DL/joint TCI state and the first TAG or of the reference cell associated with joint/UL TCI state and the second TAG.

  - Proposal 3 (Xiaomi):

    - For scenario 2, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with DLorJointTCIState or UL-TCIState having the same TAG as the uplink signal.

    - For scenario 2, if there are only UL-only TRPs with SSB, any one of UL-only TRP in the same TAG can be chosen as reference cell.

    - For Scenario 2, if there are two types of cells as candidate reference cells in the same TAG (i.e., cells with DL data and UL-only TRPs with only SSB), the UE can choose either type as the reference cell based on its implementation.

  - Proposal 4 (Samsung): For Asymmetric DL sTRP/UL mTRP Scenario#2, RAN4 to define two DL reference points for uplink timing requirements.

    - For the reference point for PRACH transmission triggered by PDCCH order, reuse the same requirements in Rel-18 two TA for inter-cell operation.

    - For inter-cell operation, for the reference point for PUCCH/PUSCH/SRS

      - For first TAG, reuse the similar requirements in Rel-18 two TA for serving cell by DLorJointTCIState

      - For second TAG, use the SSB timing from additional PCI cell.

  - Proposal 5 (Apple): Define timing reference for PDCCH ordered PRACH for scenario #2 as:

    - For multi-TRP operation with two TAs if the UE is provided SSB-MTC-AdditionalPCI, the reference point for PRACH transmission triggered by PDCCH order,

      - if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

      - if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG

  - Proposal 6 (CMCC): it is proposed to follow RAN1 agreements that two downlink reference timings are maintained. And the reference point for PRACH transmission is

    - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

    - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG

- Recommended WF

  - Check whether P1 is agreeable. P2 and P3 are similar to P1 but with some update in the wording (in bold). P4, P5, and P6 are not mutually exclusive.

Adhoc Discussion:

HW: Our understanding is scenario#2, it is the same as legacy, legacy requirements can be reused.

Nokia: It is not the same as legacy. It is new scenario but with the reused requirements. we need to capture the agreement in the spec.

Xiaomi: Ask for clarification for the “reference cell” here. If the uplink TRP and the normal TRP are not in the same TAG. What is the meaning of “downlink reference signal(s) of DL TCI state(s)” here.

MTK: The wording is coming from RAN1.

CMCC: To Xiaomi:

For the TRP with SSB, it is coming from the configured DL TCI states which may associated to the SSB in SSB-MTC-AdditionalPCI.

Xiaomi: This is the wording from RAN1. For safety, we prefer to add UL TCI as well. It is clearer for what is the DL signal is coming from.

Apple: We don’t want to argue for RAN1 agreement. From RAN4 perspective, because only SSB there, can we use SSB for more clear requirements.

Xiaomi: We don’t want to limit SSB here if the reference cell can be normal SSB or normal DL signal.

Samsung: The TRP concept is for in a cc.

QC: We are talking about scenario #2. With two DL reference timings. For the DL/UL TRP, it is for Joint TCI. If it is for UL TRP, it is for UL TCI state.

CMCC: We prefer to reuse legacy requirements and the same as RAN1 Reply LS.

HW: We still have some concern. We want to further check in this meeting. We just want to check whether to define requirements for scenario#2.

CMCC: No requirements doesn’t make sense.

Nokia: We want to check intra-cell scenario.

Samsung: Our understanding is: according to RAN1 reply LS, the scenario#2, all the agreements are only for inter-cell.

Tentative Agreement:

For Scenario#2, inter-cell operation, PRACH reference point:

  - reference point for PRACH transmission is

    - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

    - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL [or UL] TCI state(s) of the reference cell associated with the second TAG

For inter-cell multi-TRP, for PUCCH/PUSCH/SRS:

    - For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [or UL TCI states] having the same TAG as the uplink signal. For each TAG the uplink transmission timing takes place (NTA + NTA offset) x Tc before the reception of the downlink reference point.

For intra-cell scenario, Nokia want to check with RAN1

- Issue 2-5: Whether to define UL TCI state activation requirement for Scenario#2?

- Proposals

  - Proposal 1 (Huawei, MediaTek): No RRM impacts on TCI state switching requirement.

  - Proposal 2 (Xiaomi, Apple, CMCC) For scenario 2, the UL TCI state activation requirement can re-use the legacy SSB based UL TCI state activation requirement.

  - Proposal 3 (Qualcomm): RAN4 should not introduce new UL TCI state switching requirements for Scenario #2.

  - Proposal 4 (Nokia): Capture in specification that Rel 18 TCI switching requirements also apply for Rel 19 UL-only TRP.

- Recommended WF

  - Proposals are not conflicting. Check whether the following can be agreed:

The existing requirements for unified TCI state with mTRP are applicable to Scenario#2 and no new requirements to be introduced.

Adhoc Discussion:

Nokia: Our preference is to add some applicable rule that the legacy requirements can be applied to scenario#2.

HW: We prefer to check current wording in current spec. Whether to need to add such rule.

CMCC: We prefer to the suggestion from moderator. But from the high level, it is common understanding the legacy requirements can be applied. Whether to have CR impact, we can further check.

HW: Want to check for tentative agreement.

Tentative Agreement:

The existing requirements for unified TCI state with mTRP are applicable to Scenario#2 and no new requirements to be introduced. FFS on spec impact.

- Issue 2-4: For Scenario#1, whether to update RACH requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

- Proposals

  - Proposal 1 (Qualcomm): The legacy timing requirements for RACH should be reused for Scenario #1.

  - Proposal 2 (Xiaomi): For PRACH, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) associated with DLorJointTCIState or UL-TCIState having the same TAG as the uplink signal.

  - Proposal 3 (Ericsson): UE to apply additional DL reference timing offset for the RACH transmission. The DL reference timing offset can be [0.5*TAE].

  - Proposal 4 (Nokia): For an asymmetric DL sTRP/UL mTRP deployment with two TAs, RAN4 to introduce a further timing offset/delta in addition to n-TimingAdvanceOffset/n-TimingAdvanceOffset2 when PRACH is transmitted by the UE.

- Recommended WF

  - FFS

Adhoc Discussion:

Agreement:

- Issue 1-5: Whether to define RRM core requirements for phase offset reporting?

- Proposals

  - Proposal 1 (Qualcomm): RAN4 should discuss whether core requirements for phase offset reporting should be defined.

  - Proposal 2 (Nokia): Define core requirements based on 1 or 3 samples.

    - RAN4 not to consider CSI-RS precoding for PO reporting for CJT calibration.

  - Proposal 3 (Ericsson): RAN4 to send LS to RAN1 to increase the number of levels for the CJTC phase offset reporting.

- Recommended WF

  - Discuss first whether to define core requirements for phase offset reporting based on the two options:

    - Option 1: Define core requirements for phase offset reporting.

    - Option 2: Descope phase offset reporting requirements in R19 MIMO

Adhoc Discussion:

Agreement:

Topic #2: RRM requirements for UE-initiated/event-driven beam management

- Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)?

Background: in RAN4#114bis meeting: - For M in L1_measurement periods: - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam - If timeRestrictionForChannelMeasurement is not configured, - Further discuss on: - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used - Option 2: No requirements even the NW can configure it. - Option 3: FFS

For M in L1_measurement periods related to timeRestrictionForChannelMeasurement

- Proposals

  - Option 1: (Qualcomm, Nokia, Xiaomi, Samsung, LGE, ZTE, MTK)

    - Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used

    - Option 1a: (Nokia)

      - Use the same number (i.e. M=3) for both current beam and new beam

    - Option 1b: (Samsung)

      - Reuse the same values of legacy, measurement period is based on 3 sample for SSB based and for periodic and semi-persistent CSI-RS resources.

  - Option 2: (Huawei)

    - Only define RRM requirements when timeRestrictionForChannelMeasurement is configured.

  - Option 3: (Apple, Ericsson)

    - Wait for RAN1 discussion on applicability of timeRestrictionForChannelMeasurement for UEIBR. If no further progress, send LS to RAN1 to ask clarification

- Recommended WF

  - Check to agree option 1. If no consensus in this meeting, send LS to RAN1.

For L1_measurement periods

  - Option 1: (Qualcomm)

    - For the case M > 1 and P > 1, the measurement period can be defined based on N*P*M*TSSB where M and P are the corresponding scaling factors for the detecting beam.

  - Option 2: (Apple)

    - Using the existing definition of sharing factor ‘P’, the sharing factors for new beam and current beam are PN and PC respectively.

    - Option 2a: (Apple)

      - The measurement period for current beam and new beam each with periodicity TRS for FR1 and non-overlapping and non-adjacent in FR2 are defined as - 
TL1-RSRP_currBeam = M*N*PC*TRS 
TL1-RSRP_newBeam = M*N*PN*TRS

      - The measurement period for current beam and new beam each with periodicity TRS for overlapping or adjacent in FR2 are defined as -
TL1-RSRP_currBeam = M*N*2*PC*TRS 
TL1-RSRP_newBeam = M*N*2*PN*TRS

  - Option 3: (MTK)

    - RAN4 to consider the legacy sharing factor (P) for L1-RSRP measurement as baseline:

      - P value is same for intra-cell SSB resources included both current and new beam

      - P value is different for inter-cell SSB resources included both current and new beam

      - P value is different for CSI-RS resources included both current and new beam

    - RAN4 not to discuss the scenario of different periodicities for the current and new beam since RAN1 has no further agreement whether to support it or not

[Background]: It is agreed in last meeting: - For P (sharing factor) in L1_measurement periods: - For SSB based or CSI-RS based, - The definition is the same as legacy - The value can be the same or different for the current beam and new beam

- Recommended WF

  - Decide from two options:

    - Option 1:

      - The measurement period can be defined based on N*P*M*TSSB where M and P are the corresponding scaling factors for the detecting beam.

    - Option 2:

      - The measurement period can be defined based on different Pc and Pn as TL1-RSRP_currBeam , TL1-RSRP_newBeam

        - As reuse the same definition in legacy {N*Pc*M*TRS } and { N* Pn *M*TSSB }

  - Further discuss whether addition Scaling factor of 2 is needed

  - No need to further discuss different periodicities of current and new beam since no further support in RAN1

Adhoc Discussion:

Moderator: If look into the latest draft CR, RAN1 only add the mechanism of how to trigger the report. Our understanding is for L1-RSRP report in such clauses showed, it is the same as legacy.

HW: RAN1 no discussion for timeRestrictionForChannelMeasurement. For technical view, we agree with moderator’s view. We can compromise to option #1. But we can send to RAN1 to explain what is the different behaviour in RAN4 measurement if different configurations. We don’t want to kick off the RAN1 discussion.

Apple: We have the assumption based on option 1.  To check whether there is no issue. If no, then, this is what we agreed.

QC: From UE perspective, it is feasible. And From NW perspective, it can be configured.

Apple: Take LTM as an example, we add the similar parameter firstly. Then checked. After that, remove such configuration and RAN4 requirements.

Nokia: What is the intention for the LS.

E///: Just check the parameter can be configured or not. If can, reuse legacy.

MTK:

HW:

Nokia: If RAN1 has different definition for current beams and new beams, how to define RAN4 requirements.

HW, Samsung: This parameter is configured in CSIReportConfig.

Samsung: The new beams, it is in new configured in RRC. Cannot happen. Can share latest RAN1 sheet.

Please companies further check whether this can be happened or not.

HW: Need to check internally in this meeting.

Tentative Agreement:

    - RAN4 define different separate for measurement period for the UE is configured or not for “timeRestrictionForChannelMeasurement”

    - To check with RAN1 whether the parameter can be configured if RAN1 identify any issue

  - Decide from two options:

    - Option 1:

      - The measurement period can be defined based on N*P*M*TSSB where M and P are the corresponding scaling factors for the detecting beam.

    - Option 2:

      - The measurement period can be defined based on different Pc and Pn as TL1-RSRP_currBeam , TL1-RSRP_newBeam

        - As reuse the same definition in legacy {N*Pc*M*TRS } and { N* Pn *M*TSSB }

  - Further discuss whether addition Scaling factor of 2 is needed

  - No need to further discuss different periodicities of current and new beam since no further support in RAN1

Adhoc Discussion:

QC: The detecting beam can be either current beam or new beam. Not fully understanding for option 2. The concern is after the start point, the current beam and new beam are involved in the measurement. How to implemented in UE side.

Apple: UE will check current beam or new beam to check the event is met.

HW: It is generic question. QC mentioned, UE is clear which can trigger the report. But we have different cases, we just define such minimum requirements.

E///: UE is expected to evaluation the event after every measurement occasion for current beam and new beams. If they are different measurement occasions, UE has to evaluate each measurement occasion, then evaluate the event.

MTK: The condition is met over the air. We are discussing the measurement reporting delay. We think the measurement before the start point doesn’t make sense. So, we need measure both current beam and new beams after the condition is met over the air.

QC: Question is option 1: whether the only concern is it is not clear for measure both of current and new beams?

Apple: Cannot understand the comment from MTK. UE usually have the buffer for measurement for both current beam and new beams.

Xiaomi: We support the maximum of the delay. If we use the maximum principle in next issue. No need to discuss the issue which beam trigger the event.

QC: Whether the measurement is happened before the event is met over the air.

- Issue 1-2: Components of Event triggered measurement reporting delay?

Agreement in last meeting WF and Chair’s report: Agreement: - The event triggered measurement reporting delay includes the Tfirst UL channel - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH Further discuss the two options and select one option in the next meeting: - The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK) - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC)

- Proposals

  - Option 1: (China Telecom, Xiaomi, Samsung, Apple, LGE, Huawei)

    - if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams.

    - Option 1a: (Apple)

      - Revert the definition of Tfirst UL channel : Defined as time to first PUCCH after L1 measurement period for UEIBR

  - Option 2: (Qualcomm, Nokia, CMCC, Ericsson, ZTE)

    - If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam..

    - Option 2a: (Ericsson)

      - Based on agreement in last meeting, add additional component: TFirst UL Channel as the first occasion of the first UL channel after the Tnew_UCI_prep time, where Tnew_UCI_prep time is the time to prepare new UCI, which may be specified in RAN1

  - Option 3: (Qualcomm)

    - It should be explicitly captured in an agreement that the UE is not required to continue evaluating the event in the time interval Tfirst UL channel once the event has already been detected.

- Recommended WF

  - First decide from two options:

    - Option 1: China Telecom, Xiaomi, Samsung, Apple, LGE, Huawei

      - if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams.

    - Option 2: (Qualcomm, Nokia, CMCC, Ericsson, ZTE)

      - If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.

  - Then further discuss more clarification on

    - Update the definition of the TFirst UL Channel based

    - It should be explicitly captured in an agreement that the UE is not required to continue evaluating the event in the time interval Tfirst UL channel once the event has already been detected.

Adhoc Discussion:

Agreement:

- Issue 1-4: CSI-RS based L1-RSRP for inter-cell BM

- Proposals

  - Option 1: (Qualcomm, China Telecom, Samsung, Apple, CMCC, Huawei, ZTE)

    - Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

  - Option 2: (Nokia, MediaTek)

    - Not to define requirements for CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not defined a clear use case.

  - Option 3: (Ericsson)

    - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2.

    - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- Recommended WF

  - Check to agree option 1

Adhoc Discussion:

Agreement:

- Issue 1-5: Whether to specify new unified TCI switching delay requirements?

- Proposals

  - Option 1: (Qualcomm, Samsung, Huawei, Ericsson (MAC-CE based))

    - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

  - Option 2: (Nokia)

    - If UE initiated DL synchronization based on UEIBM report is supported (as discussed by RAN1), RAN4 to discuss the time point at which the UE can be expected to have performed DL synchronization: before sending the UEI beam report or after the report is sent.

  - Option 3: (Xiaomi, LGE, Ericsson (DCI based))

    - RAN4 shall wait for more RAN1 progress

  - Option 4: (Apple)

    - For DCI based DL/UL TCI state switch, RAN4 shall assume the current requirements are applicable unless there is any agreement in RAN1 on update to beam application delay.

    - There is no need to specify new unified TCI state switching delay requirements unless the TCI state switch is only defined for known TCI state, and TCI state in active TCI state list.

  - Option 5: (ZTE, MTK (If RAN1 has further agreement))

    - UE is assumed to store the QCL properties of the SSB associated with new beam, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored.

- Recommended WF

[Background]

Moderator’s observation: all analysis for potential impacts is coming from RAN1#119 meeting as below,

There is no further agreement in RAN1#120 and RAN1#120bis meeting. Even these Alts disappeared in the discussion of RAN1#121 meeting.

Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception. - Other alternatives are not precluded.

- Recommended WF

  - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management if no further RAN1 agreement on reduce beam application latency

Adhoc Discussion:

Agreement: