3GPP TSG-RAN WG4 Meeting #115	R4-2508622

# Malta, Malta, 19th – 23rd May, 2025

Title: 	Simulation assumptions for SCM

Source: 	Huawei, HiSilicon, MediaTek

Agenda item:	7.16.2

Document for:	Information

Background

This contribution summarizes the simulation assumptions for spatial channel model.

- PDSCH for alignment

Common parameters are captured in Table 2-1

Table 2-1: Common simulation assumptions for PDSCH

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD Slot Configuration Pattern |  |  | 7D1S2U (PDSCH is not scheduled in S slot for Rank8) |
| Channel Bandwidth/SCS |  |  | 40MHz/30kHz |
| Antenna configuration |  |  | Rank4: 4T4R Rank8: 8T8R |
| MCS |  |  | 13 (64 QAM table) |
| Codebook configuration for PDSCH and DMRS |  | CodebookType | Specific to each channel model |
|  |  | Codebook configuration | 4T4R: (N1,N2,O1,O2) = (2,1,4,1) 8T8R: (N1,N2,O1,O2) = (4,1,4,1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 2 for rank 8 1 for rank 4 |
| Number of HARQ Processes |  |  | 8 |
| Maximum HARQ transmissions |  |  | 4 |
| UE receiver type |  |  | MMSE-IRC |
| Test metric |  |  | - Rank 4: SNR(dB)@ 30% and 70% of maximum throughput - Rank 8: SNR(dB)@30% and 70% of max throughput for each codeword |

4 clusters and 4 TDLC channel instances

Simulation assumptions for 4 clusters and 4TDLC channel instances are captured in Table 2.1-1:

Table 2.1-1: Simulation assumptions for 4 clusters and 4TDLC channel instances

| Parameters | Values |  |
| --- | --- | --- |
|  | Rank4 | Rank8 |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (2,1,2,1) | (4,1,4,1) |
| Precoder | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination | Single panel I, Fixed PMI with i1,1=i1,2=i2=0 |
| Per cluster configuration | XPL-High, TDLC300-10, refer to Table 4-1 |  |
| Test metric | - Rank 4: SNR(dB)@ 30% and 70% of maximum throughput - Rank 8: SNR(dB)@30% and 70% of max throughput for each codeword |  |

4 clusters and 1 TDLC channel instance

Simulation assumptions for 4 clusters and 1TDLC channel instance are captured in Table 2.2-1:

Table 2.2-1: Simulation assumptions for 4 clusters and 1TDLC channel instance

| Parameters | Values |  |
| --- | --- | --- |
|  | Rank4 | Rank8 |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (2,1,2,1) | (4,1,4,1) |
| Precoder | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination | Single panel I, Fixed PMI with i1,1=i1,2=i2=0 |
| Per cluster configuration | XPL-High, TDLC300-10, refer to Table 4-2 |  |
| Test metric | - Rank 4: SNR(dB)@ 30% and 70% of maximum throughput - Rank 8: SNR(dB)@30% and 70% of max throughput for each codeword |  |

4 clusters and 2 TDLC channel instances

Simulation assumptions for 4 clusters and 2 TDLC channel instances are captured in Table 2.3-1:

Table 2.3-1: Simulation assumptions for 4 clusters and 2 TDLC channel instances

| Parameters | Values |
| --- | --- |
| Rank | 8 |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1,4,1) |
| Precoder for PDSCH and DMRS | Single panel I, Fixed PMI with i1,1=i1,2=i2=0 |
| Per cluster configuration | XPL-High, TDLC300-10, refer to Table 4-3 |
| Test metric | - Rank 4: SNR(dB)@ 30% and 70% of maximum throughput - Rank 8: SNR(dB)@30% and 70% of max throughput for each codeword |

CDL channel

Simulation assumptions for CDL channel specifically are captured in Table 2.4-1:

Table 2.4-1: Simulation assumptions for CDL channel

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | Rank4 | Rank8 |
| FR / Carrier frequency |  | FR1,3.5GHz |  |
| UE speed and movement direction |  | 3km/h, (65°,90°) |  |
| AAV assumptions |  | (M,N,P,Ms,Ns) = (1,2,2,1,1) | (M,N,P,Ms,Ns) = (1,4,2,1,1) |
| Codebook for DMRS and PDSCH |  | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination |
| Channel Geometry | LCS UE | α = 180°, β=0°, γ = 0° |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |  |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |  |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |
|  | Antenna Panel Placement | YZ Plane |  |
| Test metric | - Rank 4: SNR(dB)@ 30% and 70% of maximum throughput - Rank 8: SNR(dB)@30% and 70% of max throughput for each codeword |  |  |

- PDSCH for comparison

Comparison cases and configuration under discussion. Latest state in WF from RAN4#115.

- PMI for alignment

Common parameters are captured in Table 3-1:

Table 3-1: Common simulation assumptions for PMI

| Parameter |  |  | Unit | 8TX |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |
| Subcarrier spacing |  |  | kHz | 30 |
| Duplex Mode |  |  |  | TDD |
| TDD DL-UL configurations |  |  |  | 7D1S2U S=6D+4G+4U |
| Antenna configuration |  |  |  | 8 x 4 (N1,N2) = (4,1) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 8 |
|  | CDM Type |  |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 8, (5) |
|  | CSI-RS interval and offset |  | slot | 10, 1 |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Not configured for eType II Wideband for Type I |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 11111111111111 |
| Codebook configuration | Codebook Type |  |  | - typeII-r16 - typeI-SP |
|  | eType II CB config | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  |  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,1) |
|  | CodebookSubsetRestriction |  |  | 0x FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  |  | Rank 2: 0010 Rank 4: 1000 |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 7 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |
| Test metric |  |  |  | SNR@90% and 70% of maximum throughput with follow Type I PMI Throughput ratio of follow Type 1 PMI to random PMI with SNR@90% of maximum throughput with follow Type 1 PMI SNR@90% and 70% of maximum throughput with follow eType II PMI Throughput ratio of follow eType II PMI to random PMI with SNR@90% of maximum throughput with follow eType II PMI |
| Note: Use DM-RS based FOE and compensation. |  |  |  |  |

4 clusters and 4 TDLC channel instances

Other parameters for 4 clusters and 4 TDLC channel instances please refer to Table 3.1-1:

Table 3.1-1: PMI simulation assumptions for 4 clusters and 4TDLC channel instances

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1,2,1) |
| Number of clusters | 4 |
| Per cluster configuration | XPL-High, TDLC300-10, refer to Table 4-1 |

4 clusters and 1 TDLC channel instances

Other parameters for 4 clusters and 1 TDLC channel instances please refer to Table 3.2-1:

Table 3.2-1: PMI simulation assumptions for 4 clusters and 1 TDLC channel instances

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1,2,1) |
| Per cluster configuration | XPL-High, TDLC300-10, refer to Table 4-2 |

CDL channel

Other parameters for CDL channel please refer to Table 2.4-1

- PMI for comparison

Comparison cases and configuration under discussion. Latest state in WF from RAN4#115.

- 4 clusters TDLC and CDL channels

Table 4-1: 4 clusters and 4 TDLC channel instances

| TDLC-300 XP-high 10Hz Doppler | TX-RX beam steering |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TDL channel 1 / Cluster 1 Power: 0 dB |  | TDL channel 2 / Cluster 2 Power: 0 dB |  | TDL channel 3 / Cluster 3 Power: -9 dB |  | TDL channel 4 / Cluster 4 Power: -9 dB |  |
|  | θtx1 | θrx1 | θtx1 | θrx1 | θtx1 | θrx1 | θtx1 | θrx1 |
|  | 0 | 0 | -π/2 | -π/2 | -π | -π | π/2 | π/2 |

Table 4-2: 4 clusters and 1 TDLC channel instance

| TDLC-300 / XP-high / 100Hz Doppler |  |  | TX-RX beam steering |  |  |
| --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 |
| 2 | 65 | 0 |  |  |  |
| 3 | 70 | -7.7 | 2 | -π/2 | -π/2 |
| 4 | 190 | -2.5 |  |  |  |
| 5 | 195 | -2.4 |  |  |  |
| 6 | 200 | -9.9 |  |  |  |
| 7 | 240 | -8.0 |  |  |  |
| 8 | 325 | -6.6 |  |  |  |
| 9 | 520 | -7.1 | 3 | -π | -π |
| 10 | 1045 | -13.0 |  |  |  |
| 11 | 1510 | -14.2 | 4 | π/2 | π/2 |
| 12 | 2595 | -16.0 |  |  |  |

Table 4-3: 4 clusters and 2 TDLC channel instances

| TDLC 300 / XP-high correlation |  |  | TX-RX beam steering |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | TDL channel 1 Power: 0 dB |  |  | TDL channel 2 Power: -9 dB |  |  |
|  |  |  | Cluster index | θtx1 | θrx1 | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 | 3 | -π | -π |
| 2 | 65 | 0 |  |  |  |  |  |  |
| 3 | 70 | -7.7 |  |  |  |  |  |  |
| 4 | 190 | -2.5 |  |  |  |  |  |  |
| 5 | 195 | -2.4 | 2 | -π/2 | -π/2 | 4 | π/2 | π/2 |
| 6 | 200 | -9.9 |  |  |  |  |  |  |
| 7 | 240 | -8.0 |  |  |  |  |  |  |
| 8 | 325 | -6.6 |  |  |  |  |  |  |
| 9 | 520 | -7.1 |  |  |  |  |  |  |
| 10 | 1045 | -13.0 |  |  |  |  |  |  |
| 11 | 1510 | -14.2 |  |  |  |  |  |  |
| 12 | 2595 | -16.0 |  |  |  |  |  |  |

Table 4-4: CDL channel model:

Cluster #; Cluster delay (ns); Power in [dB]; AOD in [°]; AOA in [°]; ZOD in [°]; ZOA in [°]. 1; 0; -7.4; -38.2; -96.9; 96.8; 98.1. 2; 132; -1.2; -21.8; 118.9; 98.5; 71.1. 3; 138; -5.5; -34.2; -124.4; 100.9; 67.6. 4; 402; 0.0; -5.9; 171.2; 99.2; 76.6. 5; 416; -10.4; 44.2; 51.9; 106.4; 62.8. 6; 502; -10.1; -50.6; 63.4; 94.5; 56.6. 7; 518; -13.7; 49.1; -42.1; 107.5; 69.8. 8; 776; -8.1; -44.2; 65.1; 104.6; 103.4. 9; 826; -9.8; -50.4; -63.4; 105.2; 50.1. 10; 1372; -11.7; -60.2; 79.0; 91.7; 51.7. 11; 1712; -16.2; 64.6; 26.3; 105.2; 121.0. 12; 2906; -16.9; 55.0; -1.6; 91.8; 61.6. Per-Cluster Parameters. Parameter; cASD in [°]; cASA in [°]; cZSD in [°]; cZSA in [°]; XPR in [dB]. Value; 1.4; 15.6; 3.6; 12.2; 7.