3GPP TSG-RAN WG4 Meeting #115                                                  	R4-2508640

# Malta, Malta, 19th – 23rd May, 2025

# Agenda item:			7.11.1

# Source:	Moderator (Eutelsat Group)

# Title:	Way Forward for [115][312] NR_NTN_Ku_Band_General

# Document for:	Approval

# WF for NTN Ku band General and work plan

### Sub-topic 1-1: Numbering of the Ku bands

The existing band numbering will remain. Further discussion on optimizing band numbering is FFS.

# WF for NTN Ku band System parameters

### Sub-topic 2-1: Addition of requirement that SSB SCS and data channel SCS are the same

Add “NOTE 2:    SS Block SCS is same with SCS for data channels.” for band n248 and n247 for applicable SS raster in Table 5.4.3.3-1 as below:

### Sub- topic 2-2: Definition of channel rasters

### Modify th draft CR for channel raster to align with 38.104 wording:

For FR1-NTN satellite operating bands with 15 kHz and 30 kHz channel raster above 10 GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {1,2}. Every Ith NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <I>.

For frequency bands with two ΔFRaster in FR1, the higher ΔFRaster applies to channels using only the SCS that is equal to the higher ΔFRaster and SSB SCS is equal to the higher ΔFRaster.

### Sub- topic 2-3: Tx-Rx frequency separation

Agreement:

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n248 | -3 795 to -1 255 MHz |
| n247 | -3 295 to -1 005 MHz |

Table 5.4.4-2: UE TX-RX frequency separation (FR2-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n509 | -3 750 to -1 300 MHz |
| n508 | -3 250 to -1 050 MHz |

### Sub- topic 2-4: Add requirement to preclude simultaneous operation of FR1-NTN and FR2-NTN in the UE

Agreement

Further discuss whether and how to capture the note in UE or SAN spec.

“Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE.

### Sub-topic 2-5: SAN channel bandwidth

Agree the change below to not require support of the 7.5 kHz frequency shift for bands n247 and n248.

#### 5.4.2.1	NR-ARFCN and channel raster

The global frequency channel raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements.

The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range (0…2016666) on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NREF-Offs are given in Table 5.4.2.1-1 and NREF is the NR-ARFCN.

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |
| 24250 – 30000 | 60 | 24250.08 | 2016667 | 2016667 – 2112499 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

For the uplink of FDD FR1 NTN bands defined in Table 5.2.2-1 except n247, n248.

FREF, shift = FREF + Δshift, Δshift = 0 kHz or 7.5 kHz.

where Δshift is signalled by the network in higher layer parameter frequencyShift7p5khz [7].

The mapping between the channel raster and corresponding resource element is given in clause 5.4.2.2. The applicable entries for each operating band are defined in clause 5.4.2.3.

### Sub- topic 2-6: Corrections to channel raster definition

Agree corrections to the channel raster definition: