3GPP TSG-RAN WG4 Meeting #115	R4-2508652

St Julian’s, Malta, May 19th –May 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | 0112 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.106 for introduction of expected EIRP mask core requirement for NCR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, ZTE Corporation, Ericsson, Huawei, Samsung, Qualcomm, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is formal CR for endorsed during RAN4#114bis draft CR R4-2504433. This is CR to NCR core specification with introduction of expected EIRP mask core requirement. It reflects changes in CR to BS core spec agreed in RP-250761 during RAN#107 meeting. On top of changes in RP-250761, this CR includes updates to clause 4.5 and applicability of requirements, by adding new OTA spatial emission requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of EEIRP mask. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No EIRP mask requirement in NCR specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, new clause 7.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.176-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.

[3]	3GPP TR 25.942: "RF system scenarios".

[4]	Recommendation ITU-R SM.328: "Spectra and bandwidth of emissions".

[5]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[6]	ITU-R Recommendation M.1545: “Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications – 2000”.

[7]	3GPP TS 38.115-1: “NR; Repeater conformance testing - Part 1: Conducted conformance testing”.

[8]	3GPP TS 38.115-2: “NR; Repeater conformance testing - Part 2: Radiated conformance testing”.

[9]	ERC Recommendation 74-01, "Unwanted emissions in the spurious domain".

[10]	"Title 47 of the Code of Federal Regulations (CFR)", Federal Communications Commission.

[11]	Void

[12]	Void

[13]	3GPP TS 38.101-1: “NR User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[14]	3GPP TS 38.101-2: “NR User Equipment (UE) radio transmission and reception: Part 2: Range 2 Standalone”.

[15]	Void

[16]	Void

[17]	Void

[18]	Void

[19]	Void

[20]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”

[21]	Void

[22]	3GPP TS 38.174: "NR; Integrated Access and Backhaul (IAB) radio transmission and reception".

[23]	3GPP TS 38.331: “NR; Radio Resource Control (RRC); Protocol specification”.

[24]	3GPP TS 38.213: “NR; Physical layer procedures for control”.

[25]	3GPP TS 38.321: “NR; Medium Access Control (MAC) protocol specification”.

[26]	3GPP TS 38.211: “NR;  Physical channels and modulation”.

[27]	3GPP TS 38.306: “NR; User Equipment (UE) radio access capabilities”.

[28]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[29]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz"

[30]	3GPP TR 38.101-4: " NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements"

[31]	Commission Implementing Decision (EU) 2020/590 of 24 April 2020 amending Decision (EU) 2019/784 as regards an update of relevant technical conditions applicable to the 24,25-27,5 GHz frequency band.

[32]	ITU World Radiocommunication Conference 2023 (WRC-23) Final Acts, Resolution 220

<Next of the change>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

AoA	Angle of Arrival

BFD	Beam Failure Detection

BW	Bandwidth

BWP	Bandwidth Part

CACLR	Cumulative ACLR

CBD	Candidate Beam Detection

CP-OFDM	Cyclic Prefix-OFDM

CSI-RS	Channel State Information - Reference Signal

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DL	Downlink

DMRS	Demodulation Reference Signal

EIRP	Effective Isotropic Radiated Power

EEIRP	Expected EIRP

EVM	Error Vector Magnitude

FBW	Fractional Bandwidth

FR	Frequency Range

FSS	Fixed Satellite Service

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LA	Local Area

MR	Medium Range

NCR	Network Controlled Repeater

NCR-MT	NCR Mobile Termination

NCR-Fwd	NCR Forward

NR	New Radio

OBUE	Operating Band Unwanted Emissions

OOB	Out-of-band

OTA	Over-The-Air

QAM	Quadrature Amplitude Modulation

QCL	Quasi Co-Location

RF	Radio Frequency

RFR	RF repeater

RIB	Radiated Interface Boundary

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RMSI	Remaining Minimum System Information

RSRP	Reference Signal Received Power

RX	Receiver

SA	Standalone operation mode

SCS	Sub-Carrier Spacing

SMTC	SSB-based Measurement Timing configurationSS-RSRP	Synchronization Signal based Reference Signal Received Power

SSB	Synchronization Signal Block

SSS	Secondary Synchronization Signal

TCI	Transmission Configuration Indicator

TRS	Tracking Reference Signal

TX	Transmitter

TRP	Total Radiated Power

UL	Uplink

WA	Wide Area

<Next of the change>

## 4.5	Applicability of requirements

In Table 4.5-1, the requirement applicability for each requirement set is defined. For each requirement, the applicable requirement clause in the specification is identified. Requirements not included in a requirement set is marked not applicable (NA).

Table 4.5-1: Requirement set applicability

| Requirement | Requirement set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RFR type 1-C, NCR-Fwd type 1-C | NCR-Fwd type 1-H | NCR-MT type 1-C | NCR-MT type 1-H | RFR type 2-O, NCR-Fwd type 2-O | NCR-MT type 2-O |
| Repeater output power | 6.2 | 6.2 |  |  |  |  |
| Frequency stability | 6.3 | 6.3 |  |  |  |  |
| Out of band gain | 6.4 | 6.4 |  |  |  |  |
| Unwanted emissions | 6.5 | 6.5 |  |  |  |  |
| Repeater Error Vector Magnitude | 6.6 | 6.6 | NA | NA | NA |  |
| Input intermodulation | 6.7 | 6.7 |  |  |  |  |
| Output intermodulation | 6.8 | 6.8 |  |  |  |  |
| Adjacent Channel Rejection Ratio (ACRR) | 6.9 | 6.9 |  |  |  |  |
| Transmit ON/OFF power | 6.10 | 6.10 |  |  |  |  |
| Repeater output power for NCR-MT |  |  | 6.2.3.2 | 6.2.3.2 |  |  |
| Output power dynamics for NCR-MT |  |  | 6.11 | 6.11 |  |  |
| Transmit signal quality for NCR-MT |  |  | 6.12 | 6.12 |  |  |
| Unwanted emissions for NCR-MT |  |  | 6.5 | 6.5 |  | NA |
| Transmit intermodulation for NCR-MT |  | NA | 6.13 | 6.13 |  |  |
| Diversity characteristics for NCR-MT |  |  | 6.15 | 6.15 |  |  |
| Reference sensitivity for NCR-MT |  |  | 6.16 | 6.16 |  |  |
| Maximum input level for NCR-MT |  |  | 6.17 | 6.17 |  |  |
| Adjacent channel selectivity for NCR-MT |  |  | 6.18 | 6.18 |  |  |
| Blocking characteristics for NCR-MT |  |  | 6.19 | 6.19 |  |  |
| Spurious response for NCR-MT |  |  | 6.20 | 6.20 |  |  |
| Receiver intermodulation characteristics for NCR-MT |  |  | 6.21 | 6.21 |  |  |
| Receiver spurious emissions for NCR-MT |  |  | 6.22 | 6.22 |  |  |
| Performance requirements for NCR-MT |  |  | 8 | 8 |  |  |
| OTA Repeater output power |  | 7.2 |  |  | 7.2 |  |
| OTA frequency stability |  |  |  |  | 7.3 |  |
| OTA out of band gain |  |  |  |  | 7.4 |  |
| OTA unwanted emissions |  |  |  |  | 7.5 |  |
| OTA Repeater Error Vector Magnitude | NA |  |  | NA | 7.6 |  |
| OTA input intermodulation |  |  |  |  | 7.7 |  |
| OTA Adjacent Channel Rejection Ratio (ACRR) |  |  |  |  | 7.8 |  |
| OTA transmit ON/OFF power |  |  |  |  | 7.9 |  |
| OTA repeater output power for NCR-MT |  | NA | NA | 7.2 |  | 7.2 |
| OTA output power dynamics for NCR-MT |  |  |  |  |  | 7.10 |
| OTA transmit signal quality for NCR-MT |  |  |  |  |  | 7.11 |
| OTA unwanted emissions for NCR-MT |  |  |  |  |  | 7.5 |
| OTA diversity characteristics for NCR-MT |  |  |  |  |  | 7.12 |
| OTA reference sensitivity for NCR-MT |  |  |  | NA | NA | 7.13 |
| OTA maximum input level for NCR-MT |  |  |  |  |  | 7.14 |
| OTA adjacent channel selectivity for NCR-MT |  |  |  |  |  | 7.15 |
| OTA blocking characteristics for NCR-MT |  |  |  |  |  | 7.16 |
| OTA receiver spurious emissions for NCR-MT |  |  |  |  |  | 7.17 |
| OTA spatial emission |  | 7.18 |  |  |  | NA |
| Radiated performance requirements for NCR-MT |  |  |  |  |  | 9 |

Table 4.5-1a: Void

Table 4.5-1b: Void

<Next of the change>

## 7.18	OTA spatial emission

### 7.18.1	General

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions.

### 7.18.2	Protection of FSS UL

#### 7.18.2.1	General

This requirement shall be applied to NCR operating in band n104 to protect FSS (Earth-to-space) satellite receiver.

The expected EIRP (EEIRP) is defined as the average value of the EIRP, with the averaging being performed:

‒ over azimuth angles from −180º to +180º with reference to NCR horizontal boresight and over the specified range of angles θHLi ≤ θ < θHHi , above the horizon as defined in Table 7.18.2.2-1, and

‒ over the supported beam directions.

Note: The requirement is defined based on [32].

#### 7.18.2.2	Minimum requirement for NCR type 1-H

For NCR type 1-H operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz, shall not exceed the limits specified in table 7.18.2.2-1.

Table 7.18.2.2-1: EEIRP limits as function of elevation above horizon

| Bin number i | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<<30 | 16 |
| 6 | 30≤<<60 | 15 |
| 7 | 60≤<<90 | 15 |
| NOTE: The requirement shall apply to all supported mechanical tilts. |  |  |

Figure 7.18.2.2-1: Definitions of θHLi and θHHi angles.

<End of the change>