3GPP TSG-RAN WG4 Meeting # 115	R4-2507163

# Malta, MT, 19th - 23rd May, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | draftCR | rev | <> | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft bigCR to TR 38.863 for NTN HPUE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Xiaomi, CATT, China Telecom, Ericsson, LGE, Mediatek, Nokia, OPPO, Thales, vivo, ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Release 19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce and document the Release 19 RAN4 NTN HPUE studies and discussions to TR 38.863. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To include following endorsed draft CRs: - R4-2508069, to add new section 6b to describe NTN HPUE coex study assumption, results and conclusions, and to amend new Annex F. - R4-2508070, to introduce general part and conducted MOP for NTN supporting HPUE. - R4-2508071, to add the agreed ACLR requirements - R4-2508072, to add RSD requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Release 19 NTN HPUE co-existence study and RF requirements analysis are not documented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) Section 6b, 7.4.2.1, 7.4.2.2, 7.4.2.2.7, 7.4.3.2.2, (new) Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change: Insert new section 6b after 6a>

6	Co-existence study

<Unchanged parts skipped>

6a	Co-existence study for 17/27GHz

<Unchanged parts skipped>

6b	Co-existence study for NTN HPUE in FR1

6b.1	Co-existence simulation scenario for NTN HPUE in FR1

Scenarios for coexistence study are listed in Table 6b.1-1.

Table 6b.1-1: Scenarios for NTN HPUE co-existence

| FR1: 2GHz |  | NTN1,4,5 |  |  |
| --- | --- | --- | --- | --- |
|  |  | GEO3 | LEO 600km | LEO 1200km |
| NR/ NB-IoT | Rural | X | X | X |
|  | Urban macro | X | X | X |
|  | Dense Urban6 | N/A | N/A | N/A |
| NOTE 1: Earth Fixed beam has been considered for co-existence studies. NOTE 2: Void NOTE 3: GEO and LEO only operate at adjacent channel. NOTE 4: Use GEO and LEO@600km when TN is victim. NOTE 5: The satellite-to-satellite coexistence scenarios are not in the scope of this study considering this is already addressed by ITU (ITU RR Article 9 etc.) and regional regulations (e.g. FCC rules). NOTE 6: Dense Urban is not considered as it is expected NTN UE will connect to terrestrial networks rather than satellite networks in such scenario. |  |  |  |  |

The aggressor and victim combinations are listed in Table 6b.1-2.

Table 6b.1-2: Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes |
| --- | --- | --- | --- | --- |
| 4b | TN with NTN | NTN UL | TN UL | ACLR of NTN HPUE to be defined. |

For further visualisation of coexistence studies for NTN HPUE in FR1, the interference charts with respect to scenario 4b are further represented with respect to NTN-TN coexistence scenarios in Figure 6b.1-1.

Figure 6b.1-1: NTN-TN adjacent band coexistence scenario 4b

The frequency and bandwidth are listed in Table 6b.1-3.

Table 6b.1-3: Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth |  | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- | --- |
|  |  | NR-NTN | IoT-NTN |  |  |
| TN Rural | 2 GHz | 20MHz |  | FDD, TDD | 1 |
| TN Urban macro | 2 GHz | 20MHz |  | FDD, TDD | 1 |
| GEO | 2 GHz | 20 MHz for FR1 | 0.18MHz | FDD | 1, 31 |
| LEO | 2 GHz | 20 MHz for FR1 | 0.18MHz | FDD | 1, 31 |
| NOTE: Only FRF=1 has been used in co-existence studies for simplification. |  |  |  |  |  |

6b.2	Co-existence simulation assumption for NTN HPUE in FR1

6b.2.1	Network layout model

6b.2.1.1	Co-existence between NTN satellite and TN

Cellular cell structure is considered for both NTN satellite and TN network layout.

Referring to TR 38.811[5] Section 2.3 and Annex A, a 3D global coordinate system is considered (Earth-Centred Earth Fixed) for simulating NTN satellite beams direction and location on the earth surface. It means the NTN satellite beam location, TN randomly dropping location are generated with a set of three parameters (x,y,z).

Deployment of NTN satellite and TN cells and UEs for co-existence study is listed in Table 6b.2.1.1-1.

As defined in Figure 6b.2.1.1-1, isolation distance is the distance between the blue-dotted line which represents TN cell boarder and the red line. No UEs deployed in the isolation region is assumed to reduce the calculation complexity. The baseline is to use 2 x ISD (ISD = 750 meters) as the isolation distance

Figure 6b.2.1.1-1: Isolation distance

Table 6b.2.1.1-1:	Network and UE deployment

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 4b | TN with NTN satellite | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs dropped at the edge of TN clusters | TN randomly placed in this NTN beam | Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the TN sectors which have NTN UE(s) at their edges. |

6b.2.2	System parameters

6b.2.2.1	Satellite parameters

Set-1 satellite parameters are listed in Table 6b.2.2.1-2 and Table 6b.2.2.1-3 according to TR 38.821[6].

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 6b.2.2.1-1: NRB configuration per BandWidth size and SCS

| Configuration FR1 S-band | For IoT-NTN | For NR-NTN |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | NRB (Subcarriers) | NRB (5MHz BW) | NRB (10MHz BW) | NRB (15MHz BW) | NRB (20MHz BW) |
| For IoT-NTN : SCS 3.75kHz | 1 (12) | - | - | - | - |
| For NR-NTN and IoT-NTN : SCS 15 kHz | 1 (48) | 25 | 52 | 79 | 106 |
| For NR-NTN : SCS 30 kHz | - | 11 | 24 | 38 | 51 |

Table 6b.2.2.1-2a: Satellite parameters for co-existence study

(Note: For NR-NTN)

| Satellite orbit |  |  | GEO |  |  |  | LEO-1200 |  |  |  | LEO-600 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km |  |  |  | 1200 km |  |  |  | 600 km |  |  |  |
| Payload characteristics for DL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Satellite EIRP density |  | 2GHz | 59 dBW/MHz |  |  |  | 40 dBW/MHz |  |  |  | 34 dBW/MHz |  |  |  |
| Satellite max TX power in dBm | BW (MHz) |  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 |
|  | SCS 15kHz |  | 44.53 | 47.71 | 49.53 | 50.81 | 46.53 | 49.71 | 51.53 | 52.81 | 40.53 | 43.71 | 45.53 | 46.81 |
|  | SCS 30kHz |  | 43.98 | 47.37 | 49.36 | 50.64 | 45.98 | 49.37 | 51.36 | 52.64 | 39.98 | 43.37 | 45.36 | 46.64 |
| Satellite Tx max Gain |  |  | 51 dBi |  |  |  | 30 dBi |  |  |  | 30 dBi |  |  |  |
| Channel bandwidth |  |  | 5/10/15/20MHz |  |  |  | 5/10/15/20MHz |  |  |  | 5/10/15/20MHz |  |  |  |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 0.4011 deg |  |  |  | 4.4127 deg |  |  |  | 4.4127 deg |  |  |  |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  |  | 0.3474 deg |  |  |  | 3.8206 deg |  |  |  | 3.8206 deg |  |  |  |
| Satellite beam diameter |  |  | 250 km |  |  |  | 90 km |  |  |  | 50 km |  |  |  |
| Payload characteristics for UL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G/T |  | 2 GHz | 19 dB K-1 |  |  |  | 1.1 dB K-1 |  |  |  | 1.1 dB K-1 |  |  |  |
| Satellite Rx max Gain |  |  | 51 dBi |  |  |  | 30 dBi |  |  |  | 30 dBi |  |  |  |

Table 6b.2.2.1-2b Satellite parameters for co-existence study

(Note: For IoT-NTN)

| Satellite orbit |  |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 1200km | 600 km |
|  |  |  | Payload characteristics for DL transmissions |  |  |  |
| Satellite EIRP density |  | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite max TX power in dBm | BW (MHz) |  | 0.18 (NB-IoT) | 0.18 (NB-IoT) | 0.18 (NB-IoT) |
|  | SCS 15kHz |  | 30.55 | 32.55 | 26.55 |
|  | SCS 3.75kHz |  | 30.55 | 32.55 | 26.55 |
| Satellite Tx max Gain |  |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  |  | 0.3474 deg | 3.8206 deg | 3.8206 deg |
| Satellite beam diameter |  |  | 250 km | 90 km | 50 km |
|  |  |  | Payload characteristics for UL transmissions |  |  |  |
| G/T |  | 2 GHz | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  |  | 51 dBi | 30 dBi | 30 dBi |

Table 6b.2.2.1-3: Other parameters for NTN satellite

| Parameters | NR-NTN | IoT NTN |
| --- | --- | --- |
| Carrier frequency | 2GHz | 2GHz |
| The number of active UE (UL) | 9 UEs and 2RBs per UE for GEO and LEO1 NOTE 2 | 9 for 15kHz SCS 18 and 36 for 3.75kHz SCS (1 subcarrier per UE) |
| The number of active UE (DL) | 1 | 1 |
| Traffic model | Full buffer | Full buffer |
| DL power control | NO | NO |
| UL power control | See Session 2.2.6.2 NOTE 2 | 36.942 section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB |
| UE Tx power in dBm (PC3) | -40 to 23 | -40 to 23 |
| UE Tx power in dBm (PC2) | -40 to 26 | -40 to 26 |
| UE Tx power in dBm (PC1.5) | -40 to 29 | - |
| UE Tx power in dBm (PC1) | -40 to 31 | -40 to 31 |
| NTN satellite Noise Figure in dB | See Table 2.3-3-1 | See Table 2.3-3-1 |
| Handover margin | 3dB | 3dB |
| NOTE 1: UEs are equally splitted inside the channel bandwidth into ACIR 3 regions. Scheduled PRB position for UE1 per satellite beam should be also fully aligned to simulate the worst case for co-channel interference and this is also aligned with full buffer case. NOTE 2: For NR-NTN HPUE: Option 1: 3 dB SNR target and 5MHz UL BW (One company proposed for LEO only); Option 2: 15 dB SNR target and 2RB UL BW as baseline assumption NOTE 3: Assuming all NR-NTN/IoT-NTN UEs are HPUE, i.e. 100% HPUE ratio as baseline. |  |  |

Table 6b.2.2.1-5: NTN satellite Noise Figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 19 | 1.1 | 1.1 |
| G_Rx (dBi) | 51 | 30 | 30 |
| NF (dB) | 7.4 | 4.3 | 4.3 |

6b.2.2.2	NTN UE parameters

NTN UE parameters are given in Table 6b.2.2.2-1

Table 6b.2.2.2-1: NTN UE characteristics for system level simulations

| Characteristics | For NR-NTN and IoT-NTN |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 9 dB |
| Tx antenna gain | 0 dBi per element |

6b.2.2.3	TN parameters

TN parameters for co-existence study are given in Table 6b.2.2.3-1, 6b.2.2.3-2 and 6b.2.2.3-3.

Table 6b.2.2.3-1: Simulation assumptions of TN respectively based on NR

|  | NR |
| --- | --- |
| Carrier frequency in GHz | 2 |
| Size of each nominal channel BW in MHz | 20 |
| Transmission bandwidth in MHz | N/A |
| Environment | Deployment scenario related, see Table 6b.2.2.3-2. |
| Network layout | 19-sites 57 sectors with wrap-around |
| Inter-site distance in meter | Deployment scenario related, see Table 6b.2.2.3-2 |
| System loading and activity | See Table 6b.2.1.1-1 |
| Network location | See Table 6b.2.1.1-1 |
| DL subcarrier spacing | 15kHz |
| UL | OFDMA |
| DL power control | No |
| UL power control | TR 36.942[8] |
| Frequency reuse | 1 |
| Number of scheduled UE per cell (DL) | 1 |
| Number of scheduled UE per cell (UL) | 3 |
| UE antenna height in meter | 1.5m |
| UE TX power in dBm | -40 to 23 |
| UE antenna gain in dBi | 0 |
| Building penetration loss | In pathloss model, TR 38.901[10] |
| Cell selection margin in dB | 3 |
| BS-MS min distance in meters | 35 |
| BS noise figure in dB | 5 |
| UE noise figure in dB | 9 |
| BS-UE path-loss model | TR 38.901[10] |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901[10] |
| Shadowing correlation | Inter-cell 0.5 Intra-cell 1 |
| Link-level performance model | See Section 6b.2.8 Throughtput-SINR mapping |
| UE distribution | Uniform |
| Evaluation metrics | See Section 6b.2.8 Throughtput or SNR loss criteria |

Table 6b.2.2.3-2: Deployment-related parameters of TN (2 GHz)

|  | Urban Macro | Rural Macro | Remarks |
| --- | --- | --- | --- |
| ISD in meters | 750 | 7500 | ITU-R Report M.2292[11] |
| BS Antenna height in meters | 25 | 30 |  |
| UE Outdoor/indoor | 100% Outdoor |  |  |
| UE height in meter | 1.5 | 1.5 | RP-200559 [12] 3GPP LS to ITU-R WP5D and ITU-R WP5D [IMT_Parameters] [13] |

Table 6b.2.2.3-3: ACLR/ACS for TN (2GHz)

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 45 dB |
|  | ACS | 46 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) |
|  | ACS | 33 |

6b.2.3	Antenna and beam forming pattern modelling

6b.2.3.1	NTN SAN and NTN UE

Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.4.1 in TR 38.811[5].

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered.

1                for θ=0

4J1kasinθkasinθ2        for 0<θ≤90°

where:

-	J1(x) is the Bessel function of the first kind and first order with argument ‘x’ , where x is kasinθ;

-	a, a is the radius of the antenna's circular aperture;

-	k = 2f/c is the wave number;

-	f is the frequency of operation;

-	c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam.

Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

The antenna patterns for LEO 600km, 1200km and GEO are shown in Figure 6b.2.3.1-1 and 6b.2.3.1-2.

Figure 6b.2.3.1-1:  Antenna pattern for LEO 600KM and 1200KM (4.4127 deg for 3dB beamwidth)

Figure 6b.2.3.1-2: Antenna pattern for antenna aperture of GEO (0.4011 deg for 3dB beamwidth)

The beam layout definition for a single satellite simulation in S-Band is defined in Table 6b.2.3.1-1.

Table 6b.2.3.1-1: Beam layout definition for single satellite simulation

| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |
| --- | --- |
| Number of beams | Baseline: 7-beam layout (i.e. 6 co-frequency beams surrounding the central beam) |
| UV plane illustration (extracted from [19]) |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2) with ABS [degree]=180/pi x ABS[rad] and with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern. |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point Case 2: 45° for GEO and LEO |
|  | Option 1: FRF=1 |
| Polarization re-use | Option 1: Disable Option 2: Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UE distribution | The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |
| UE orientation | Omnidirectional |
| UE attachment | RSRP |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). |  |

6b.2.3.2	TN BS and TN UE

For AAS antenna, it refers to the pattern in TR 38.921 [14].

Table 6b.2.3.2-1: AAS antenna parameters for 2GHz

|  |  | Rural | Macro urban |
| --- | --- | --- | --- |
| 1 | Base Station Antenna Characteristics |  |  |
| 1.1 | Antenna pattern | TR 38.921[14] |  |
| 1.2 | Element gain (dBi) (Note 2) | 7.1 | 6.4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V | 90º for H 65º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 × 8 elements | 8 × 8 elements |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 |
| 1.9 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 25 | 25 |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 | 120 |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-120 |
| 1.12 | Mechanical downtilt (degrees) | 3 | 10 |

For non-AAS antenna, the parameters in Table 6b.2.3.2-1 are used for 2GHz BS antenna pattern in the NTN system simulation.

Table 6b.2.3.2-1: FR1 BS Non-AAS antenna pattern for 2GHz

| Parameter for BS | Values |  |
| --- | --- | --- |
| Antenna vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=10°,SLAV=20dB |  |
| Antenna horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=20dB |  |
| Combining method for 3D antenna pattern (dB) | A“θ",φ”=-min-AE,Vθ"+AE,Hφ”, Am |  |
| Maximum directional gain of an antenna GE,max | 17 dBi |  |
| Conducted power | 46 dBm |  |
| Mechanical downtilt in degrees | Rural | 3 |
|  | Urban | 10 |

For UE antennas, an omni-directional radiation pattern with antenna gain 0dBi is assumed.

6b.2.4	ACIR model

6b.2.4.1	NR-NTN

ACLR modelling for TN and NTN co-existence study referring to clause 5.1.1.4.1 and 5.1.1.4.2 in TR 36.942[8] is used as baseline. The number of RBs refers to Table 6b.2.2.1-4 and Table 6b.2.2.3-1 respectively.

6b.2.4.2	IoT-NTN

For scenario 4b, symmetric bandwidth IoT-NTN ACLR model is assumed according to TR 36.942 section 5.1.1.4.2 with flat NTN ACLR model and FACLR = 0 dB, assuming flat ACS.

6b.2.5	Propagation model

6b.2.5.1	Propagation model between NTN SAN and UE

Propagation model between NTN SAN and UE should reference to section 6.6 in TR 38.811[5].

6b.2.5.2	Propagation model between TN BS and UE

Propagation model between TN BS and UE should reference to section 7.4 in TR 38.901[10].

6b.2.5.3	Propagation model between NTN SAN and TB BS

Propagation model between NTN SAN and TN BS should reference to TS 38.811[5] which is used for DL-UL cross link interference for S band.

6b.2.6	Transmission power control model

6b.2.6.1	TN uplink transmission power control

For uplink scenario, transmission power control model specified in Section 9.1 TR 36.942[8] is applied for TN with following parameters.

where:

-	Pmax = 23dBm,

-	Rmin = -63 dB,

-	CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

6b.2.6.2	NTN uplink transmission power control

For NTN HPUE uplink trnasmission power control model, the following parameters are assumed:

where:

-	Pmax = 26dBm for PC2, 29 dBm for PC1.5, 31 dBm for PC1.

-	Rmin = -66 dB for PC2, -69 dB for PC1.5, -71 dB, assuming -40dBm minimum output power,

-	CLx-ile and γ are set as following:

-	CLx-ile = 10*log10(Pmax) – (SNRtarget + 10*log10(kTB) + NF ),

  - Where:

  - SNRtarget and BW

For NR-NTN HPUE:

    - Option 1: 3 dB SNR target and 5MHz UL BW (One company proposed for LEO only)

    - Option 2: 15 dB SNR target and 2RB UL BW as baseline assumption

  - NF is the SAN noise figure, i.e. 4.3dB.

  - 10log10(kT) = -174dBm/Hz.

  - γ = 1 For uplink scenario.

6b.2.6.3	Downlink transmission power control

For downlink scenario, no power control scheme is applied to both NTN and TN scenarios.

6b.2.7	Received power model

See Section 6.2.7.

6b.2.8	Performance metric

See Section 6.2.8.

6b.2.9	Throughput ~ SNR mapping

See Section 6.2.9.

6b.3	Co-existence simulation methodology for NTN HPUE in FR1

6b.3.1	Simulation procedure

See section 6.3.1.

6b.3.2	Methods and principle to process co-existence simulation results

See section 6.3.2, noting that the following scenario is selected, in principle, to draw conclusion for NTN HPUE co-existence study.

Table 6b.3.2-1: Selected option for each scenario

| Scenario | Aggressor system | Victim system | Environment | Contributing |
| --- | --- | --- | --- | --- |
| 4b | NTN UL | TN UL | Urban | NTN UE ACLR |

6b.4	Co-existence simulation results for NTN HPUE in FR1

6b.4.1	NR-NTN Power Class 2: Scenario 4b

The co-existence results from all concerned options for NR-NTN power class 2 in this scenario were evaluated, and the detailed results can be found in Annex F to this TR. The numbers in below table have taken into account all the submitted results from these contributing companies for both average throughput loss and cell edge loss, and picked from the worst results from each source.

Table 6b.4.1-1 Interpolated required ACLR values for Scenario 4b with isolation distance 1.5km assumed

| Company | Interpolated required ACLR |
| --- | --- |
| Samsung | <20 |
| Vivo | <20 |
| Ericsson | <20 |
| ZTE | 30.6 |
| Mediatek | <20 |
| Thales | <20 |
| Xiaomi | <20 |
| Qualcomm | <20 |
| Average | <20 |
| Note: Some company’s results meet performance metrics at certain starting ACIR/ACLR level they used, e.g. 20dBc. These results are listed as smaller than those starting level. |  |

Table 6b.4.1-2 Interpolated required ACLR values for Scenario 4b without isolation distance assumed

| Company | Interpolated required ACLR |
| --- | --- |
| Samsung | 25.9 |
| Vivo | 27.1 |
| Ericsson | 30.1 |
| Mediatek | 33 |
| Huawei | 23.7 |
| Xiaomi | 24.7 |
| Qualcomm | 20 |
| Average | 26.4 |

6b.4.2	NR-NTN Power Class 1.5: Scenario 4b

The co-existence results from all concerned options for NR-NTN power class 1.5 in this scenario were evaluated, and the detailed results can be found in Annex F to this TR. The numbers in below table have taken into account all the submitted results from these contributing companies for both average throughput loss and cell edge loss, and picked from the worst results from each source.

Table 6b.4.2-1 Interpolated required ACLR values for Scenario 4b with isolation distance 1.5km assumed

| Company | Interpolated required ACLR |
| --- | --- |
| Samsung | <20 |
| Vivo | <20 |
| Ericsson | 14.4 |
| ZTE | 31.4 |
| Mediatek | 22.6 |
| Thales | 13 |
| Xiaomi | 23.2 |
| Qualcomm | <20 |
| Average | <20.6 |
| Note: Some company’s results meet performance metrics at certain starting ACIR/ACLR level they used, e.g. 20dBc. These results are listed as smaller than those starting level. |  |

Table 6b.4.2-2 Interpolated required ACLR values for Scenario 4b without isolation distance assumed

| Company | Interpolated required ACLR |
| --- | --- |
| Samsung | 29.3 |
| Vivo | 31.2 |
| Ericsson | 37.9 |
| Mediatek | 37.4 |
| Huawei | 26.2 |
| Xiaomi | 43.9 |
| Qualcomm | 20 |
| Average | 32.3 |

6b.4.3	NR-NTN Power Class 1: Scenario 4b

The co-existence results from all concerned options for NR-NTN power class 1 in this scenario were evaluated, and the detailed results can be found in Annex F to this TR. The numbers in below table have taken into account all the submitted results from these contributing companies for both average throughput loss and cell edge loss, and picked from the worst results from each source.

Table 6b.4.3-1 Interpolated required ACLR values for Scenario 4b with isolation distance 1.5km assumed

| Company | Interpolated required ACLR |
| --- | --- |
| Samsung | <20 |
| Vivo | 20.5 |
| Ericsson | 14.4 |
| ZTE | 33.7 |
| Mediatek | 24.6 |
| Thales | <13 |
| Xiaomi | 25.1 |
| Average | <21.6 |
| Note: Some company’s results meet performance metrics at certain starting ACIR/ACLR level they used, e.g. 20dBc. These results are listed as smaller than those starting level. |  |

Table 6b.4.3-2 Interpolated required ACLR values for Scenario 4b without isolation distance assumed

| Company | Interpolated required ACLR |
| --- | --- |
| Samsung | 30.9 |
| Vivo | 34.5 |
| Ericsson | 40.3 |
| Mediatek | 39.4 |
| Huawei | 27.3 |
| Xiaomi | 38.6 |
| Qualcomm | 20 |
| Average | 35.2 |

6b.4.4	IoT-NTN Power Class 2: Scenario 4b

The co-existence results from all concerned options for IoT-NTN power class 2 in this scenario were evaluated, and the detailed results can be found in Annex F to this TR. The numbers in below table have taken into account all the submitted results from these contributing companies for both average throughput loss and cell edge loss, and picked from the worst results from each source.

Table 6b.4.4-1 Interpolated required ACLR values for Scenario 4b with isolation distance 1.5km assumed

| Company | Interpolated required ACLR |
| --- | --- |
| Samsung | <20 |
| Vivo | <20 |
| ZTE | 39.2 |
| Mediatek | 9.2 |
| Xiaomi | 34.3 |
| Qualcomm | <20 |
| Ericsson | 13.4 |
| Average | <22.8 |
| Note: Some company’s results meet performance metrics at certain starting ACIR/ACLR level they used, e.g. 20dBc. These results are listed as smaller than those starting level. |  |

Table 6b.4.4-2 Interpolated required ACLR values for Scenario 4b without isolation distance assumed

| Company | Interpolated required ACLR |
| --- | --- |
| Samsung | 35.3 |
| Vivo | <20 |
| Mediatek | 33.6 |
| Qualcomm | <20 |
| Average | <27.2 |
| Note: Some company’s results meet performance metrics at certain starting ACIR/ACLR level they used, e.g. 20dBc. These results are listed as smaller than those starting level. |  |

6b.4.5	IoT-NTN Power Class 1: Scenario 4b

The co-existence results from all concerned options for IoT-NTN power class 1 in this scenario were evaluated, and the detailed results can be found in Annex F to this TR. The numbers in below table have taken into account all the submitted results from these contributing companies for both average throughput loss and cell edge loss, and picked from the worst results from each source.

Table 6b.4.5-1 Interpolated required ACLR values for Scenario 4b with isolation distance 1.5km assumed

| Company | Interpolated required ACLR |
| --- | --- |
| Samsung | <20 |
| Vivo | <20 |
| ZTE | 41.8 |
| Mediatek | 9.2 |
| Xiaomi | 42.4 |
| Qualcomm | <20 |
| Ericsson | 19.0 |
| Average | <24.7 |
| Note: Some company’s results meet performance metrics at certain starting ACIR/ACLR level they used, e.g. 20dBc. These results are listed as smaller than those starting level. |  |

Table 6b.4.5-2 Interpolated required ACLR values for Scenario 4b without isolation distance assumed

| Company | Interpolated required ACLR |
| --- | --- |
| Samsung | 39.8 |
| Vivo | <20 |
| Mediatek | 33.6 |
| Qualcomm | 32 |
| Average | <31.4 |
| Note: Some company’s results meet performance metrics at certain starting ACIR/ACLR level they used, e.g. 20dBc. These results are listed as smaller than those starting level. |  |

6b.5	Summary of co-existence study for NTN HPUE in FR1

The averaged interpolate ACLR values for each power class of NR-NTN and IoT-NTN in Scenario 4b are presented in Table 6b.5-1 below. It should be noted that only co-ex study results assuming 1.5km isolation distance are taken into account in this summary.

Table 6b.5-1: Average ACLR values for NTN HPUE in FR1

| Power classes |  | PC 2 | PC 1.5 | PC 1 |
| --- | --- | --- | --- | --- |
| ACLR values (dBc) | NR-NTN | <20 | <20.6 | <21.6 |
|  | IoT-NTN | <22.8 | Note 2 | <24.7 |
| Note 1: Some company’s results meet performance metrics at certain starting ACIR/ACLR level they used, e.g. 20dBc. These results are listed as smaller than those starting level. Note 2: The power class 1.5 for IoT-NTN is not studied in this version. |  |  |  |  |

With the above averaged values from co-existence study and recognizing the characteristics of NTN HPUE in FR1 to the state of art and their future developments; and the protection to the operators in adjacent channel, the agreed ACLR of NR-NTN and IoT NTN HPUE are given in Table 6b.5.1-2.

Table 6b.5.1-2: ACLR and ACS of NTN HPUE in FR1

| Power classes |  | PC 2 | PC 1.5 | PC 1 |
| --- | --- | --- | --- | --- |
| ACLR (dBc) | NR-NTN | 31 | 31 | 343 |
|  | NB-IoT based IoT-NTN | 371 | Note 2 | 371 |
|  | eMTC based IoT-NTN | 31 | Note 2 | [34]3 |
| Note 1: NB-IoT based IoT-NTN PC2 and PC1 ACLR reused from NB-IoT based IoT-NTN PC3 UTRA_ACLR. Note 2: The power class 1.5 for IoT-NTN is not studied in this version. Note 3: It is observed in some company co-existence results that the 5%-ile throughput degradation is slightly higher than 5% with zero km isolation distance assumption. It was however agreed to progress with this ACLR as it faciliates UE RF implementation. |  |  |  |  |

<Next change>

7.4.2	UE Transmission characteristics for satellite access

7.4.2.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0dBi is assumed.

7.4.2.2	Conducted transmitter characteristics

7.4.2.2.1	Maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 7.4.2.2.1-1: UE Power Class

| NTN satellite band # | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) |  | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 31 | +2/-3 | 293 | +2/-3 | 26 |  | +2/-3 | 23 | ±2 |
| n255 | 31 | +2/-3 | 293 | +2/-3 | 26 |  | +2/-3 | 23 | ±2 |
| n254 |  |  |  |  |  |  |  | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Achieved via 2Tx in this version of the specification. NOTE 4: Generally, PC1 and PC1.5 UE are not targeted for smartphone form factor in this version of the specification. |  |  |  |  |  |  |  |  |

<Next change>

7.4.2.2.7	ACLR

For ACLR, for PC3, PC2 and PC1.5 the same ACLR for TN NR UE which is specified in TS 38.101-1 [17] sub-clause 6.5.2.4.1 can be applied for NTN satellite access UEs. For PC1, a relaxed ACLR requirement of 34dB compared to that in TN NR has been specified as a trade-off between the required protection of adjacent bands, as determined by coexistence studies, and the challenges of UE implementation.

The requirement of ALCR is specified in Table 7.4.2.2.7-1.

Table 7.4.2.2.7-1: NTN satellite access UE ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NTN UE ALCR | 34 dB | 31 dB | 31 dB | 30 dB |

<Next change>

7.4.3.2.2	Reference sensitivity

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel. For below 6GHz, the REFSENS level can be calculated by the equation below in TR 38.817-01[32]:

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

It is noted that the Rx BW is identical to the transmission bandwidth configuration, which is determined by the spectrum utilization. The RB values in the analysis of this contribution are based on the agreed SU for NR.

SNR in Nagoya meeting was agreed as -1dB for NR.

The REFSENS for E-UTRA assumes that the receiver is equipped with two Rx port as a baseline. If 2Rx is considered, the diversity gain is 3dB and the Implementation Margin (IM) uses 2.5dB. In the following calculation, same assumptions are used for NR NTN satellite bands.

For band n255, 9dB noise figure is assumed which is aligned with band n24.

Both dedicate 30MHz duplexer and n65 duplexer are allowed to be used for band n256. Finally, 9dB noise figure is assumed for band n256 as well.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2 and A.3.2 from TS 38.101-1 [17] (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1 from TS 38.101-1 [17])  with parameters specified in Table 7.4.3.2.2-1 and Table 7.4.3.2.2-2.

Table 7.4.3.2.2-1: Two antenna port reference sensitivity QPSK REFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NTN satellite band # | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 | FDD |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |
| n256 | 30 |  | -97.1 | -95.1 | -94.0 | FDD |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |

Table 7.4.3.2.2-2: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NTN satellite band # | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n255 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |

For power class 2, power class 1.5 and power class 1 UEs, certain degradation of the reference sensitivity in Table 7.4.3.2.2-1 is allowed. For a power class 2 UE the maximum amount of degradation is specified in Table 7.4.3.2.2-1a, and in Table 7.4.3.2.2-1b that indicates txDiversity-r16 or txDiversity2Tx-r18. For a power class 1.5 UE the maximum amount of degradation is specified in Table 7.4.3.2.2-1c. For a power class 1 UE the maximum amount of degradation is specified in Table 7.4.3.2.2-1d [and in Table 7.4.3.2.2-1e that indicates txDiversity-4Tx-PC1].

Table 7.4.3.2.2-1a Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| NTN satellite band # | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- |
| n255 | 0.8 | 0.8 | 0.9 | 0.9 |
| n256 | 0 | 0 | 0 | 0 |

Table 7.4.3.2.2-1b Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| NTN satellite band # | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- |
| n255 | 1.6 | 1.6 | 1.7 | 1.7 |
| n256 | 0 | 0 | 0 | 0 |

Table 7.4.3.2.2-1c Reference Sensitivity Degradation from PC3 to PC1.5 for FDD bands for UE supporting Tx Diversity

| NTN satellite band # | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- |
| n255 | [0] | [0] | [0] | [0] |
| n256 | [0] | [0] | [0] | [0] |
| Note: 66~70 dB filter rejection would be needed for the non-handheld 0dB RSD. |  |  |  |  |

Table 7.4.3.2.2-1d Reference Sensitivity Degradation from PC3 to PC1 for FDD bands for UE not supporting Tx Diversity

| NTN satellite band # | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- |
| n255 | [0] | [0] | [0] | [0] |
| n256 | [0] | [0] | [0] | [0] |
| Note: 66~70 dB filter rejection would be needed for the non-handheld 0dB RSD. |  |  |  |  |

[Table 7.4.3.2.2-1e Reference Sensitivity Degradation from PC3 to PC1 for FDD bands for UE supporting Tx Diversity]

| NTN satellite band # | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- |
| n255 | [0] | [0] | [0] | [0] |
| n256 | [0] | [0] | [0] | [0] |
| Note: 66~70 dB filter rejection would be needed for the non-handheld 0dB RSD. |  |  |  |  |

<Next change: Insert new Annex F after Annex E>

Annex E:
Examples of Delay and Doppler shift variation as a function of time for different satellite orbit types

<Unchanged parts skipped>

Annex F:
Summary of NTN HPUE simulation results

All NTN HPUE co-existence study results have been captured in the list. Please see attachment Summary of NTN HPUE simulation results.

[Editor note: see attached file Annex F, Summary of NTN HPUE simulation results.]

<End of Changes>