3GPP TSG-RAN WG4 Meeting #116	R4-2509012

Bengaluru, India, 25th – 29th August 2025

3GPP TSG-RAN WG1 Meeting #121	R1-2504943

St. Julian’s, Malta, 19th – 23rd April 2025

Title:	Reply LS on LP-WUS UE RF

Response to:	R1-2501703/R4-2503003

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	vivo

To:	RAN4

Cc:	RAN2

Contact person:         Xin Qu

quxin@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN 1 would like to thank RAN4 for the LS on LP-WUS UE RF in R1-2501703/R4-2503003. Regarding the signal configuration of LP-WUS and LP-SS for both idle/inactive and connected modes, the following has been agreed by RAN1, which may facilitate deriving SNR for LP-WUR Rx requirements:

- LP-WUS for both idle/inactive and connected modes: OOK

- Chip rate:

- For FR1, M=1, 2, 4 for 15kHz and 30kHz

- For FR2, M=1 for 120kHz in both RRC idle/inactive and RRC connected, and M=1, 2 for SCS=60kHz in RRC connected only

- Number of information bits: up to 5 bits

- Coding:

- For more than 2 information bits: RM coding as in section 5.3.3.3 of 38.212 and rate matching after RM coding is applied, as in section 5.4.3 in TS 38.212

- For 3 information bits, the coding block (coded bit) length L is in 3~32

- For 4 information bits, the coding block (coded bit) length L is in 5~32

- For 5 information bits, the coding block (coded bit) length L is in 6~32

- 2 information bits: coding as in Table 5.3.3.2-1 in 38.212 with Q_m set to 1

- The coding block (coded bit) length L is in 2~32

- 1 information bit: repetition as in Table 5.3.3.1-1 in 38.212 with Q_m set to 1.

- The coding block (coded bit) length L is in 1~32

- Further, for the coding block after above listed coding schemes, Manchester coding is further applied.

- CRC: not supported.

- LP-WUS for both RRC idle/inactive and connected modes:  overlaid OFDM sequence(s)

- Number of information bits: up to 5 bits

- Number of overlaid OFDM sequences for RRC idle/inactive and connected modes for FR1:

- support maximum 16 candidates overlaid sequences for M=1

- support maximum 8 candidates overlaid sequences for M=2

- support maximum 4 candidates overlaid sequences for M=4

- Number of overlaid OFDM sequences: Nseq is 2^n, where the value range of n is 0~4, 0~3, 0~2 for M=1, 2, 4, respectively.

- No separate UE capability for different n values.

- Number of overlaid OFDM sequences for RRC idle/inactive and/or connected modes for FR2:

- Max 2 roots and max 4 sequences for M =1 for SCS 60kHz

- Max 2 roots and max 2 sequences for M=2 for SCS 60kHz

- Max 2 roots and max 2 sequences for M=1 for SCS 120kHz

- The length of overlaid OFDM sequence(s):

- Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included), and M is chip rate.

- Roots and CS(s) of ZC sequence(s) to generate overlaid OFDM sequence(s):

- Root(s): gNB can configure up to 2 roots from any value from 1 ~ (Bzc -1), where Bzc is given by the largest prime number such that BZC<LZCBZC<LZC, where  LZC where   LZC is the overlaid OFDM sequence length. Different roots can be picked for different M values.

- Nroot=2, when Nseq=16,8,4 for M=1,2,4, where Nroot is the number of root values.

    - Nroot=1 or 2 when Nseq is other value

- How to carry information by overlaid OFDM sequence:

- Raw information bits are mapped to sequence(s)

- N raw information bits are divided into K segments from MSB to LSB, where K= ceil (N/log2L), L is the number of candidate overlaid OFDM sequences for one OOK ON chip.

- In one OOK ON chip, a segment of information bits is mapped to one sequence sequentially, e.g., for a segment of 2 information bits, 00 is mapped to sequence #1, 01 is mapped to seq #2.

- In case N/log2L is not an integer, Bit 0 as MSB is used for padding

- LP-SS: OOK and overlaid OFDM sequence

- Chip rate: M=1, 2, 4 for both FR1 and FR2

- For the length of binary sequences:

- M=1, L= {6, 8}

- M=2, L= {12, 16}

- M=4, L= {16, 32}

- Set of binary sequences for LP-SS:

- For M=1, L=6, the set of LP-SS sequence is:

[1 0 1 0 1 0]

[0 1 0 1 0 1]

[1 0 0 1 0 1]

[1 0 1 0 0 1]

- For M=1, L=8, the set of LP-SS sequence is:

[1 0 1 0 0 1 0 1]

[1 0 1 0 1 0 0 1]

[1 0 0 1 0 1 0 1]

[0 1 0 1 0 1 0 1]

- For M=2, L=12, the set of LP-SS sequence is:

[1 0 0 1 1 0 0 1 1 0 0 1]

[0 1 1 0 1 0 0 1 1 0 0 1]

[0 1 1 0 0 1 1 0 1 0 0 1]

[0 1 1 0 0 1 0 1 1 0 0 1]

- For M=2, L=16, the set of LP-SS sequence is:

[1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

[1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

[1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1]

[1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

- For M=4, L=16, the set of LP-SS sequence is:

[0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0]

[0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0]

[1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1]

[1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]

- For M=4, L=32, the set of LP-SS sequence is:

[0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

[0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

[0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]

[0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1]

- Coding: no coding

- CRC: not supported

- Roots and CS(s) of ZC sequence(s) to generate overlaid OFDM sequence(s): single root is used and gNB can configure any value from 1 ~ (Bzc -1), where Bzc is given by the largest prime number such that BZC<LZCBZC<LZC, and   LZC    LZC is the overlaid OFDM sequence length.

- cyclic shift value ncs=0 ncs=0.

- EPRE between LP-WUS/LP-SS and SSB ratio:

Agreement

For the EPRE ratio between LP-WUS/LP-SS and SSB (i.e. LP-WUS/LP-SS EPRE divided by SSB EPRE), separate configurations are provided for LP-WUS and LP-SS.

- The candidate value set for the EPRE ratio is {-3dB, 0 dB, 3 dB, 6 dB}.

- If M=1 for both LP-WUS and LP-SS, or M>1 for both LP-WUS and LP-SS, the difference between the two EPRE ratios shall be no larger than 3 dB.

- If M=1 for LP-WUS and M>1 for LP-SS, the ERPE ratio for LP-SS minus the EPRE ratio for LP-WUS should be within the range of -6 dB to 0 dB.

Note: EPRE refers to EPRE in one OFDM symbol with non-zero power (from baseband perspective) LP-WUS/LP-SS transmission.

# 2	Actions

To RAN4:

RAN1 kindly request RAN4 take the above information into account during the future work.

# 3	Dates of next TSG-RAN WG1 meetings

RAN1#122                       25th – 29th August 2025		     Bengaluru, IN

RAN1#122bis                  13rd – 17th October 2025	     Prague, CZ