3GPP TSG-RAN WG4 Meeting #116	R4-2509158

Bangalore, India, 25-29 August, 2025

Source:	Nokia

Title:	On remaining FBS requirements

Agenda item:	7.14.3.1

Document for:	Discussion

Introduction

In this contribution we discuss the remaining open issues related to L3 beam sweeping factor reduction.

Discussion

Conditions to exit FBS

In the last meeting, RAN4 agreed the following about UE exiting FBS and UE preference indication to remove FBS configuration:

Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1.

Regarding the UAI, we think this should be very similar to the corresponding Rel-18 multi-Rx UAI about UE preference for single-panel or multi-panel operation. RAN2 can define the UAI based on the agreement in the last meeting. RAN4 does not need to discuss the UAI further.

No need for RAN4 to discuss the UAI indication for no FBS preference further.

Regarding the timer to exit FBS that was agreed to be introduced, RAN4 needs to agree whether this is to be a network-configured or standards specified timer. Regarding the value, this needs to cover at least one measurement period. Hence, the timer value could be set to the maximum measurement period based on UE reported BSF multiplied by X, where X may indicate a number of measurement periods during which the UE shall use FBS.

Value for timer T1 to be defined as X * L3 measurement period with reduced BSF.

If RAN4 can agree on a single value for the timer, there is no need to add the timer in the network configuration. Otherwise, if multiple values need to be supported, the timer has to be network-configured and RAN2 would need to introduce the relevant signalling. At this point of the WI, minimizing RAN2 work would be preferable, but if the group cannot agree on a single value, signaling of the timer should be added to the FBS configuration and the RAN2 work needs to be triggered by an LS in this meeting.

Aim to define a single value for the timer. If not possible, RAN2 to introduce a configurable timer; work to be triggered by an LS.

Re-evaluation of entry condition

RAN4 did not have time to discuss the re-entry to FBS mode after timer expiration, so this issue was left FFS in the last meeting’s WF:

Way forward: For re-evaluation mechanism for scenario 1/2/3/4: - Check whether the following mechanism is agreeable: - UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode.

The recommended WF in our understanding means that another timer would be introduced from the end of the FBS exit timer after which the UE would re-evaluate the FBS entry condition. In our view, defining a requirement for re-evaluating the condition does not seem needed, as the UE is anyways continuously performing measurements and knows whether the condition is met or not.

The following cases exist:

- UE exits FBS at timer expiration while the condition continues to be met

- UE exits FBS at timer expiration and the condition after some time is not met any longer

In the latter case, UE shall re-enter FBS when the condition becomes met again. However, we do not think this counts as re-entry condition as it is the same as entry condition becoming fulfilled again. Hence, this scenario is already covered.

We do not see that the re-entry condition is mandatory for the feature to work, but we are open to define some rule for the former case if companies see this as important. However, a second signal level condition may be more useful than a second timer.

If a rule for re-entry to FBS is needed when the entry condition continues to be met after timer expiration, a second signal level based threshold could be more useful than defining a second timer.

Conclusion

The following Observations and Proposals were made:

Observation 1: No need for RAN4 to discuss the UAI indication for no FBS preference further.

Proposal 1: Value for timer T1 to be defined as X * L3 measurement period with reduced BSF.

Proposal 2: Aim to define a single value for the timer. If not possible, RAN2 to introduce a configurable timer; work to be triggered by an LS.

Proposal 3: If a rule for re-entry to FBS is needed when the entry condition continues to be met after timer expiration, a second signal level based threshold could be more useful than defining a second timer.