3GPP TSG-RAN WG4 Meeting #116		                                                        R4-2509283

# Bengaluru , IN, 25th – 29th Aug, 2025

Title:	Discussion on RRM requirements for adaptation of common signal channel transmission

Source:	CATT

Agenda Item:	7.23.3.2

Document for:	Discussion

- Introduction

In last RAN4 meeting, the RRM requirements for Rel-19 NES enhancement were discussed. The conclusions for SSB adaptation were captured in the approved WF [1] and the open issues can be found in the summary [2]. In this paper, we further discuss the potential RRM impact for SSB adaptation and give our views.

- Discussion

Sub-topic 1-1 SSB adaptation – L1 Requirements impacts

In the last meeting, two options for measurement restriction impact captured in the summary are copied as folloows [2]:

Issue 1-2-4: L1 requirements – Measurement restriction impact - Proposals - Option 1: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB. (Huawei) - Option 2: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply. (CTC, Vivo)

In our view, SSB adaptation only involves the change of SSB periodicity the legacy L1 measurement restriction can be reused, that is, the UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS, longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

Proposal 1: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can also apply.

Sub-topic 1-2: SSB adaptation – L3 Requirements impacts

Issue 1-3-4: Colliding with MG

In the last meeting, the candidate option for the case of colliding with MG is copied as follows [1]:

- Proposals - Option 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

In legacy, while measurement gaps are considered, the measurements in gaps are with higher priority than the measurement outside gaps. After SSB adaptation, if the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, we believe that UE performs the measurement within gap, otherwise the UE performs the measurement outside gap.

Proposal 2: After SSB adaptation, if the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, UE performs the measurement within gap, otherwise the UE performs the measurement outside gap.

- Conclusion

In this paper, we have some further discussions on adaptation of common signal channel transmission, and the following proposals are given:

Proposal 1: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can also apply.

Proposal 2: After SSB adaptation, if the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, UE performs the measurement within gap, otherwise the UE performs the measurement outside gap.

- Reference

- R4-2508441, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei, RAN4#115

- R4-2505571, Topic summary for [115][222] Netw_Energy_NR_enh_Part2, Huawei, RAN4#115