3GPP RAN WG4 Meeting #116	R4-2509308

Bengaluru, India, August 25th – August 29th, 2025

Title: 	Discussion on SBFD RRM performance requirements

Source: 	CATT

Agenda item:	7.21.5

Document for:	Discussion

Introduction

In RAN4#115 meeting, RAN4 approved the work plan for defining the SBFD RRM performance requirements. In this contribution, we will discuss the details of performance requirements based on the way forward [1].

Discussion

Side conditions for L1-SRS-RSRP measurement performance requirement

Timing offset

The assumption of timing offset for L1 CLI measurements depends on specific scenarios and a LS was sent to RAN1 for clarification. In RAN1#119 meeting, the LS was discussed and the following agreement is sent back to RAN4 [2]:

RAN1 Agreement: Send reply LS to RAN4 with following information: When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios. For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

According to RAN1 agreement, intra-cell UE-to-UE L1 CLI measurement scenario is prioritized, and therefore the cell phase error can be removed from the Rel-16 SRS-RSRP side condition. However, one proposal was brought up to consider cell phase error for 15 kHz SCS and 30 kHz SCS because the performance degradation between the results with and without cell phase error is quite minor, which is less than 1 dB, and the requirement coverage can also be expanded. Considering that 15 kHz SCS and 30 kHz SCS are the typical configurations in the deployment and there indeed no much difference between the results with and without cell phase error, it is reasonable to define requirements with cell phase error for 15 kHz SCS and 30 kHz SCS to make the requirements applicable to inter-cell scenario. For 60 kHz SCS and 120 kHz SCS, the performance degradation is not negligible and therefore we still prefer to define the requirements without including cell phase error.

Proposal 1: RAN4 to define measurement accuracy requirements with cell phase error for or 15 kHz SCS and 30 kHz SCS. 	For SCS larger than or equal to 60 kHz, time offset condition is defined without cell phase error.

SRS Es/Iot and BW

Based on the summary of simulation results [3], we observed that the cell phase error has the most significant impact on L1 SRS-RSRP measurement results. If RAN4 agrees to remove cell phase error, which can significantly reduce the measurement error, especially for large SCS configurations. And then selection of number of measurement samples and SRS Es/Iot will have a limited effect on measurement performance. For SRS Es/Iot, it is acceptable to reuse the Rel-16 value, -1 dB.

In regards to the assumption of number of PRBs of SRS resources for L1 CLI measurements, RAN4 needs to consider the sizes of UL subband and total channel bandwidth. For most of SCS and band combinations, it is reasonable to reuse the assumption of 48 PRBs. However, when SCS equals 60 kHz in FR1 bands, there might be some issues.

According to agreements from RF groups, the channel bandwidths supporting SBFD shall be equal to or larger than 40 MHz except for n39, and the size of UL subbands will be chosen and declared from the Table 5.3.2-1 in TS 38.104 [4]. Hence, the applicable sizes of channel bandwidth for 60 kHz SCS in FR1 bands are highlighted below:

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

If SRS-RSRP measurements apply when the size of SRS BW is not smaller than 48 PRBs, which means the size of UL subband shall not be smaller than 48 PRBs. Furthermore, if the size of DL subbands is also included, assuming it is DU case and the sizes of UL subband and DL subband are the same, at least 96 PRBs are needed. And when guardband is required, larger channel bandwidths should be configured. Based on the above analysis, it is clear that up to 3 options are applicable for channel bandwidths configured with 60 kHz SCS in FR1 bands for DU case, and DUD configuration is not applicable in this case. If a UL subband smaller than 48 PRBs is configured, configuring some small bandwidths and DUD case become applicable from the network perspective, however the SRS-RSRP measurement performance requirement will not apply any more.

Observation 1: L1 SRS-RSRP performance requirements will not apply for most of channel BW configurations with 60 kHz SCS in FR1, if 48 PRBs SRS BW is assumed.

As we discussed above, if cell phase error is removed, SRS-RSRP measurement error will be effectively reduced and the benefit of using a large number of PRBs is limited. Therefore in order to make performance requirements applicable for more channel BW configurations with SCS 60 kHz, we slightly prefer 24 PRBs between two SRS BW assumptions, i.e., 24 and 48 PRBs.

Proposal 2: RAN4 to use the side conditions below for L1 CLI SRS-RSRP measurement requirements:

- SRS Es/Iot ≥ 1 dB.

- SRS BW: ≥ 24 PRBs.

L1-RSSI measurement accuracy requirement

In RAN4#113 meeting, RAN4 already agreed to reuse 1 sample for L1-CLI-RSSI measurement, which is the same as the L3 RSSI requirement. For DU case including Method #1 and resource type#1 and Method #3, we think the legacy accuracy requirement can be directly reused since there is no technical difference.

Proposal 3: For Method #1 and resource type#1 and Method #3, reuse the legacy L3 CLI-RSSI measurement accuracy requirement.

For DUD case, i.e., Method #1 and resource type#2, where one CLI-RSSI measurement resource is configured across two DL subbands, the measurement resources are not contiguous in frequency domain, which is different from the L3 CLI-RSSI measurement. But RSSI measurement only reflects the power level in a specific time and frequency range and it is not reference signals- or UE algorithms-dependent, and therefore we think the legacy measurement accuracy requirement can be reused.

Proposal 4: For L1-CLI-RSSI measurement in DUD case, the legacy measurement accuracy requirement can be reused.

Report mapping

One LS [5] was sent from RAN1 to RAN4 in RAN4#114 meeting to confirm the following RAN1 working assumption on L1 CLI measurement report mapping:

Working assumption For L1 based CLI measurement and reporting, the following is adopted - For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value. - For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value. … RAN1 would like to ask RAN4 whether the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI are reasonable. RAN1 also would like to check with RAN4 whether there is any issue with the range and resolution for differential reporting of L1 SRS-RSRP and L1 CLI-RSSI.

From RAN4 perspective, we do not identify any issues to reuse L3 CLI measurement report mapping requirements for L1 CLI measurements with absolute report.

In addition, RAN1 agreed to support differential report for L1 CLI measurements, and therefore RAN4 should also define measurement report mapping requirements for differential reporting. From our perspective, the differential SS-RSRP and CSI-RSRP measurement (for L1 reporting) report mapping requirements defined in Table 10.1.6.1-2 in TS 38.133 can be reused for L1-SRS-RSRP measurement and L1-CLI-RSSI measurement, i.e.,

Table 1: Reporting mapping of differential L1-SRS-RSRP

| Reported value | Measured quantity value (difference in measured SRS-RSRP from strongest SRS-RSRP) | Unit |
| --- | --- | --- |
| DIFF_SRS-RSRP_0 | 0≥ΔSRS-RSRP>-2 | dB |
| DIFF_SRS-RSRP_1 | -2≥ΔSRS-RSRP>-4 | dB |
| DIFF_SRS-RSRP_2 | -4≥ΔSRS-RSRP>-6 | dB |
| DIFF_SRS-RSRP_3 | -6≥ΔSRS-RSRP>-8 | dB |
| DIFF_SRS-RSRP_4 | -8≥ΔSRS-RSRP>-10 | dB |
| DIFF_SRS-RSRP_5 | -10≥ΔSRS-RSRP>-12 | dB |
| DIFF_SRS-RSRP_6 | -12≥ΔSRS-RSRP>-14 | dB |
| DIFF_SRS-RSRP_7 | -14≥ΔSRS-RSRP>-16 | dB |
| DIFF_SRS-RSRP_8 | -16≥ΔSRS-RSRP>-18 | dB |
| DIFF_SRS-RSRP_9 | -18≥ΔSRS-RSRP>-20 | dB |
| DIFF_SRS-RSRP_10 | -20≥ΔSRS-RSRP>-22 | dB |
| DIFF_SRS-RSRP_11 | -22≥ΔSRS-RSRP>-24 | dB |
| DIFF_SRS-RSRP_12 | -24≥ΔSRS-RSRP>-26 | dB |
| DIFF_SRS-RSRP_13 | -26≥ΔSRS-RSRP>-28 | dB |
| DIFF_SRS-RSRP_14 | -28≥ΔSRS-RSRP>-30 | dB |
| DIFF_SRS-RSRP_15 | -30≥ΔSRS-RSRP | dB |

Table 2: Reporting mapping of differential L1-CLI-RSSI

| Reported value | Measured quantity value (difference in measured CLI-RSSI from strongest CLI-RSSI) | Unit |
| --- | --- | --- |
| DIFF_CLI-RSSI_0 | 0≥ΔCLI-RSSI>-2 | dB |
| DIFF_CLI-RSSI_1 | -2≥ΔCLI-RSSI>-4 | dB |
| DIFF_CLI-RSSI_2 | -4≥ΔCLI-RSSI>-6 | dB |
| DIFF_CLI-RSSI_3 | -6≥ΔCLI-RSSI>-8 | dB |
| DIFF_CLI-RSSI_4 | -8≥ΔCLI-RSSI>-10 | dB |
| DIFF_CLI-RSSI_5 | -10≥ΔCLI-RSSI>-12 | dB |
| DIFF_CLI-RSSI_6 | -12≥ΔCLI-RSSI>-14 | dB |
| DIFF_CLI-RSSI_7 | -14≥ΔCLI-RSSI>-16 | dB |
| DIFF_CLI-RSSI_8 | -16≥ΔCLI-RSSI>-18 | dB |
| DIFF_CLI-RSSI_9 | -18≥ΔCLI-RSSI>-20 | dB |
| DIFF_CLI-RSSI_10 | -20≥ΔCLI-RSSI>-22 | dB |
| DIFF_CLI-RSSI_11 | -22≥ΔCLI-RSSI>-24 | dB |
| DIFF_CLI-RSSI_12 | -24≥ΔCLI-RSSI>-26 | dB |
| DIFF_CLI-RSSI_13 | -26≥ΔCLI-RSSI>-28 | dB |
| DIFF_CLI-RSSI_14 | -28≥ΔCLI-RSSI>-30 | dB |
| DIFF_CLI-RSSI_15 | -30≥ΔCLI-RSSI | dB |

Proposal 5: RAN4 to reuse L3 CLI measurement report mapping requirements for L1 CLI measurements with absolute report.

Proposal 6: The following report mapping tables based on Table 10.1.6.1-2 are used for L1 CLI measurements with differential report,

Table 1: Reporting mapping of differential L1-SRS-RSRP

| Reported value | Measured quantity value (difference in measured SRS-RSRP from strongest SRS-RSRP) | Unit |
| --- | --- | --- |
| DIFF_SRS-RSRP_0 | 0≥ΔSRS-RSRP>-2 | dB |
| DIFF_SRS-RSRP_1 | -2≥ΔSRS-RSRP>-4 | dB |
| DIFF_SRS-RSRP_2 | -4≥ΔSRS-RSRP>-6 | dB |
| DIFF_SRS-RSRP_3 | -6≥ΔSRS-RSRP>-8 | dB |
| DIFF_SRS-RSRP_4 | -8≥ΔSRS-RSRP>-10 | dB |
| DIFF_SRS-RSRP_5 | -10≥ΔSRS-RSRP>-12 | dB |
| DIFF_SRS-RSRP_6 | -12≥ΔSRS-RSRP>-14 | dB |
| DIFF_SRS-RSRP_7 | -14≥ΔSRS-RSRP>-16 | dB |
| DIFF_SRS-RSRP_8 | -16≥ΔSRS-RSRP>-18 | dB |
| DIFF_SRS-RSRP_9 | -18≥ΔSRS-RSRP>-20 | dB |
| DIFF_SRS-RSRP_10 | -20≥ΔSRS-RSRP>-22 | dB |
| DIFF_SRS-RSRP_11 | -22≥ΔSRS-RSRP>-24 | dB |
| DIFF_SRS-RSRP_12 | -24≥ΔSRS-RSRP>-26 | dB |
| DIFF_SRS-RSRP_13 | -26≥ΔSRS-RSRP>-28 | dB |
| DIFF_SRS-RSRP_14 | -28≥ΔSRS-RSRP>-30 | dB |
| DIFF_SRS-RSRP_15 | -30≥ΔSRS-RSRP | dB |

Table 2: Reporting mapping of differential L1-CLI-RSSI

| Reported value | Measured quantity value (difference in measured CLI-RSSI from strongest CLI-RSSI) | Unit |
| --- | --- | --- |
| DIFF_CLI-RSSI_0 | 0≥ΔCLI-RSSI>-2 | dB |
| DIFF_CLI-RSSI_1 | -2≥ΔCLI-RSSI>-4 | dB |
| DIFF_CLI-RSSI_2 | -4≥ΔCLI-RSSI>-6 | dB |
| DIFF_CLI-RSSI_3 | -6≥ΔCLI-RSSI>-8 | dB |
| DIFF_CLI-RSSI_4 | -8≥ΔCLI-RSSI>-10 | dB |
| DIFF_CLI-RSSI_5 | -10≥ΔCLI-RSSI>-12 | dB |
| DIFF_CLI-RSSI_6 | -12≥ΔCLI-RSSI>-14 | dB |
| DIFF_CLI-RSSI_7 | -14≥ΔCLI-RSSI>-16 | dB |
| DIFF_CLI-RSSI_8 | -16≥ΔCLI-RSSI>-18 | dB |
| DIFF_CLI-RSSI_9 | -18≥ΔCLI-RSSI>-20 | dB |
| DIFF_CLI-RSSI_10 | -20≥ΔCLI-RSSI>-22 | dB |
| DIFF_CLI-RSSI_11 | -22≥ΔCLI-RSSI>-24 | dB |
| DIFF_CLI-RSSI_12 | -24≥ΔCLI-RSSI>-26 | dB |
| DIFF_CLI-RSSI_13 | -26≥ΔCLI-RSSI>-28 | dB |
| DIFF_CLI-RSSI_14 | -28≥ΔCLI-RSSI>-30 | dB |
| DIFF_CLI-RSSI_15 | -30≥ΔCLI-RSSI | dB |

Conclusions

This contribution discussed the impacts of SBFD on RRM performance requirements and some proposals are provided:

Proposal 1: RAN4 to define measurement accuracy requirements with cell phase error for or 15 kHz SCS and 30 kHz SCS. 	For SCS larger than or equal to 60 kHz, time offset condition is defined without cell phase error.

Observation 1: L1 SRS-RSRP performance requirements will not apply for most of channel BW configurations with 60 kHz SCS in FR1, if 48 PRBs SRS BW is assumed.

Proposal 2: RAN4 to use the side conditions below for L1 CLI SRS-RSRP measurement requirements:

- SRS Es/Iot ≥ 1 dB.

- SRS BW: ≥ 24 PRBs.

Proposal 3: For Method #1 and resource type#1 and Method #3, reuse the legacy L3 CLI-RSSI measurement accuracy requirement.

Proposal 4: For L1-CLI-RSSI measurement in DUD case, the legacy measurement accuracy requirement can be reused.

Proposal 5: RAN4 to reuse L3 CLI measurement report mapping requirements for L1 CLI measurements with absolute report.

Proposal 6: The following report mapping tables based on Table 10.1.6.1-2 are used for L1 CLI measurements with differential report,

Table 1: Reporting mapping of differential L1-SRS-RSRP

| Reported value | Measured quantity value (difference in measured SRS-RSRP from strongest SRS-RSRP) | Unit |
| --- | --- | --- |
| DIFF_SRS-RSRP_0 | 0≥ΔSRS-RSRP>-2 | dB |
| DIFF_SRS-RSRP_1 | -2≥ΔSRS-RSRP>-4 | dB |
| DIFF_SRS-RSRP_2 | -4≥ΔSRS-RSRP>-6 | dB |
| DIFF_SRS-RSRP_3 | -6≥ΔSRS-RSRP>-8 | dB |
| DIFF_SRS-RSRP_4 | -8≥ΔSRS-RSRP>-10 | dB |
| DIFF_SRS-RSRP_5 | -10≥ΔSRS-RSRP>-12 | dB |
| DIFF_SRS-RSRP_6 | -12≥ΔSRS-RSRP>-14 | dB |
| DIFF_SRS-RSRP_7 | -14≥ΔSRS-RSRP>-16 | dB |
| DIFF_SRS-RSRP_8 | -16≥ΔSRS-RSRP>-18 | dB |
| DIFF_SRS-RSRP_9 | -18≥ΔSRS-RSRP>-20 | dB |
| DIFF_SRS-RSRP_10 | -20≥ΔSRS-RSRP>-22 | dB |
| DIFF_SRS-RSRP_11 | -22≥ΔSRS-RSRP>-24 | dB |
| DIFF_SRS-RSRP_12 | -24≥ΔSRS-RSRP>-26 | dB |
| DIFF_SRS-RSRP_13 | -26≥ΔSRS-RSRP>-28 | dB |
| DIFF_SRS-RSRP_14 | -28≥ΔSRS-RSRP>-30 | dB |
| DIFF_SRS-RSRP_15 | -30≥ΔSRS-RSRP | dB |

Table 2: Reporting mapping of differential L1-CLI-RSSI

| Reported value | Measured quantity value (difference in measured CLI-RSSI from strongest CLI-RSSI) | Unit |
| --- | --- | --- |
| DIFF_CLI-RSSI_0 | 0≥ΔCLI-RSSI>-2 | dB |
| DIFF_CLI-RSSI_1 | -2≥ΔCLI-RSSI>-4 | dB |
| DIFF_CLI-RSSI_2 | -4≥ΔCLI-RSSI>-6 | dB |
| DIFF_CLI-RSSI_3 | -6≥ΔCLI-RSSI>-8 | dB |
| DIFF_CLI-RSSI_4 | -8≥ΔCLI-RSSI>-10 | dB |
| DIFF_CLI-RSSI_5 | -10≥ΔCLI-RSSI>-12 | dB |
| DIFF_CLI-RSSI_6 | -12≥ΔCLI-RSSI>-14 | dB |
| DIFF_CLI-RSSI_7 | -14≥ΔCLI-RSSI>-16 | dB |
| DIFF_CLI-RSSI_8 | -16≥ΔCLI-RSSI>-18 | dB |
| DIFF_CLI-RSSI_9 | -18≥ΔCLI-RSSI>-20 | dB |
| DIFF_CLI-RSSI_10 | -20≥ΔCLI-RSSI>-22 | dB |
| DIFF_CLI-RSSI_11 | -22≥ΔCLI-RSSI>-24 | dB |
| DIFF_CLI-RSSI_12 | -24≥ΔCLI-RSSI>-26 | dB |
| DIFF_CLI-RSSI_13 | -26≥ΔCLI-RSSI>-28 | dB |
| DIFF_CLI-RSSI_14 | -28≥ΔCLI-RSSI>-30 | dB |
| DIFF_CLI-RSSI_15 | -30≥ΔCLI-RSSI | dB |

References

[1] R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN4#115.

[2] R1-2410890, Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation, RAN1#119.

[3] R4-2508307, Updated summary of simulation results for L1-SRS-RSRP, Huawei, RAN4#115.

[4] TS 38.104, Base Station (BS) radio transmission and reception, V18.8.0.

[5] R1-2501620, LS on L1 based UE-to-UE CLI measurement and reporting, RAN1#120.