3GPP TSG-RAN WG4 Meeting #116				                                                R4-2509311

Bengaluru, India, August 25th – August 29th, 2025

Title:	Discussion on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Source:	CATT

Agenda Item:	7.14.3.2

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the enhancement for FR2-1 SSB based measurement delay reduction by optimizing CSSF outside gap in CA/DC was discussed and the conclusions were captured in the approved WF [1].In this paper, we will continue the discussion on open issues of FR2-1 SSB based measurement delay reduction by optimizing CSSF outside gap in CA/DC [2] and give our views.

2 Discussion

  - Issues related with solution 1 (measure one serving CC per band)

RAN4 discussed the following revised solution 1 without any agreement reached:

- Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

Meanwhile, RAN2 discussed the signalling type and indication mechanism for a SCC that UE needs to measure in one band but did not reach any consensus. A LS [3] is sent to RAN4 for down-selection between two alternatives and feedback is expected during August meeting:

RAN2 LS: About the network indication of a specific SCC per-band for measurement, RAN2 discussed the following two alternatives: - Alternative 1: Introduce new RRC configuration to indicate which SCC to be measured. - When CSSF optimization feature is enabled, UE only performs the serving and intra-frequency neighbor measurement on the indicated SCC per band. - Alternative 2: No additional RRC signaling is needed. - When CSSF optimization feature is enabled, UE follows the existing RAN4 rule (defined in clause 9.2.3.2 of TS 38.133) below to select the SCC to perform both serving and intra-frequency neighbor measurement. - - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.. Majority’s view in RAN2 is Alternative 2, but RAN2 did not reach a consensus. Therefore, RAN2 would like to ask RAN4 to decide which alternative to adopt for SCC indication and provide feedback during August meeting so that RAN2 can prepare the RRC CR for approval in August meeting.

We notice that RRC signalling is used in both alternatives in the LS. Based on this situation, alternative 1 does not show many benefits compared to alternative 2. For example, if network wants UE to measure a specific SCC in a band, it can configure a measurement report for this SCC only in this band. Therefore we support alternative 2, i.e., no additional RRC signalling is needed, and the solution 1 needs to be revised as follows:

Solution 1 in the BigCR R4-2508457 [4]: For UE supporting [CSSF enhancement for one serving CC measurement per-band], if UE receives network indication via [TBD] to enable one serving carrier measurement for each FR2 band, - For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - PCC, when UE is configured with SA NR or NE-DC operation mode with PCC in this FR2 band; or - PSCC, when UE is configured with EN-DC with PSCC in this FR2 band; or - PSCC, when UE is configured with NR-DC with PSCC in this FR2 band; or - SCC on which UE is configured to report SSB based measurements, when neither PCC nor PSCC is in the same band and this SCC is the only SCC on which UE is configured to report SSB based measurements in this FR2 band; or - SCC indicated by network via [TBD], when - neither PCC nor PSCC is in the same band, and, - there is no SCCs on which UE is configured to report SSB based measurements in this FR2 band, or - there are multiple SCCs on which UE is configured to report SSB based measurements in this FR2 band; Editor Note: In this scenario of “SCC indicated by network via [TBD]”, the related requirements above can be revisited if RAN4 and/or RAN2 is achieving any new conclusions. - otherwise - SCC determined by UE implementation. - UE is not required to perform SS-RSRP, SS-RSRQ, and SS-SINR measurements on serving cell for each of the other intra-frequency layer(s) in the same band.

Proposal 1: For the two alternatives in RAN2 LS, alternative 2 is supported, i.e., no additional RRC signalling is needed.

Proposal 2: Revise the solution 1 as follows:

where this single intra-frequency layer shall be:

- 	PCC, when UE is configured with SA NR or NE-DC operation mode with PCC in this FR2 band; or

-	PSCC, when UE is configured with EN-DC with PSCC in this FR2 band; or

-	PSCC, when UE is configured with NR-DC with PSCC in this FR2 band; or

-	SCC on which UE is configured to report SSB based measurements, when neither PCC nor PSCC is in the same band and this SCC is the only SCC on which UE is configured to report SSB based measurements in this FR2 band; or

-	SCC indicated by network via [TBD], when

-	neither PCC nor PSCC is in the same band, and,

-	there is no SCCs on which UE is configured to report SSB based measurements in this FR2 band, or

-	there are multiple SCCs on which UE is configured to report SSB based measurements in this FR2 band;

Editor Note: In this scenario of “SCC indicated by network via [TBD]”, the related requirements above can be revisited if RAN4 and/or RAN2 is achieving any new conclusions.

-	otherwise

-	SCC determined by UE implementation.

  - Expand solution 1 to FR1 case

In the way forward [1], an open issue is whether to expand solution 1 to FR1 band:

[Wayforward]: Further discuss in next meeting: If extend for FR1: Aspect 1: co-located or non-collocated - Option 1: only limit to serving cell measurement (E///) - Option 2: cover both serving cell and neighboring cell measurements - In clause 7.6.4 of TS 38.133, it clearly sys only co-located deployment is applied for intra-band contiguous CA. - Companies are encouraged to check the spec in this meeting. Aspect 2: Network configuration on enable/disable the feature - Option 1: the configuration is the same for different FRs - Option 2: the configuration can be different for different FRs (Nokia, ZTE, China Telecom, Apple, HW, E///) - Under the condition that the UE capability is per FR [or per band]. - Option 2 will be adopted if the solution 1 will be extended to FR1.

There are two aspects under this issue. Aspect 1 is the deployment applicability of solution 1, i.e., whether the SCCs, among which only one SCC is measured, in the same FR1 band are co-located or not. In our understanding, if the SCCs are not co-located, these SCCs may experience different propagation conditions and interference levels. The results of one SCC may not be able to reflect all other SCCs in this FR1 band. Therefore, we prefer that these SCCs are co-located. Otherwise, we support to limit solution 1 to FR2 bands only. Regarding the aspect 2, as the above way forward says, the network configuration can be different for different FRs considering that the UE capability is per FR or per band.

Proposal 3: Support to expand the solution 1 to FR1 band if SCCs are co-located. Otherwise, prefer to limit the solution 1 to FR2 bands.

Proposal 4: Support that the configuration can be different for different FRs if the solution 1 is expanded to FR1 band.

3 Conclusion

In this paper, we have some discussions on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, and the following proposals are given:

Solution 1 (measure one serving CC per band)

Proposal 1: For the two alternatives in RAN2 LS, alternative 2 is supported, i.e., no additional RRC signalling is needed.

Proposal 2: Revise the solution 1 as follows:

where this single intra-frequency layer shall be:

- 	PCC, when UE is configured with SA NR or NE-DC operation mode with PCC in this FR2 band; or

-	PSCC, when UE is configured with EN-DC with PSCC in this FR2 band; or

-	PSCC, when UE is configured with NR-DC with PSCC in this FR2 band; or

-	SCC on which UE is configured to report SSB based measurements, when neither PCC nor PSCC is in the same band and this SCC is the only SCC on which UE is configured to report SSB based measurements in this FR2 band; or

-	SCC indicated by network via [TBD], when

-	neither PCC nor PSCC is in the same band, and,

-	there is no SCCs on which UE is configured to report SSB based measurements in this FR2 band, or

-	there are multiple SCCs on which UE is configured to report SSB based measurements in this FR2 band;

Editor Note: In this scenario of “SCC indicated by network via [TBD]”, the related requirements above can be revisited if RAN4 and/or RAN2 is achieving any new conclusions.

-	otherwise

-	SCC determined by UE implementation.

Expand solution 1 to FR1 case

Proposal 3: Support to expand the solution 1 to FR1 band if SCCs are co-located. Otherwise, prefer to limit the solution 1 to FR2 bands.

Proposal 4: Support that the configuration can be different for different FRs if the solution 1 is expanded to FR1 band.

4 References

- R4-2508264, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#115.

- R4-2505561, Topic summary for [115][212] NR_RRM_Ph5_Part1, Apple, RAN4#115.

- R4-2509014, Reply LS on CSSF optimization for NR RRM Phase 5, RAN2, RAN2#130.

- R4-2508457, Draft big CR for NR Radio Resource Management (RRM) Phase 5, Apple, CATT, RAN4#115.