3GPP TSG-RAN WG4 Meeting #116	R4-2509602

# Bengaluru, India, August 25 – 29, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	(Netw_Energy_NR-Core) Discussion on intra-frequency measurement with SSB-less SCell

Agenda item:	5.22.1

Document for:	Discussion

- Introduction

At RAN2#130 meeting, RAN2 concluded on the discussion of servingCellMO for SSB-less SCell with the following agreements. An reply LS R2-2504906 was also sent to RAN4 informing the latest outcome [1]. In this contribution, we are discussing the impact to the intra-frequency measurement definition in RAN4.

RAN2#130 meeting: R2-2504905 Summary of way forward on servingCellMO for SSB-less SCell ZTE Corporation, Sanechips discussion Rel-18 Netw_Energy_NR-Core Agreements 1: Introduce per UE capability to indicate whether the UE supports the configuration of servingCellMO for SCell that does not transmit SS/PBCH block. A UE supporting this feature shall also support NR intra-frequency measurements on neighbor cells based on servingCellMO associated with SCell that does not transmit SS/PBCH block. NW can optionally configure servingCellMO for SCell that does not transmit SS/PBCH block if UE indicate such capability. 2: Clarify in the procedure text that that SSB-based serving cell measurements are skipped. 3: Clarify in the field description of servingCellMO that the ssbFrequency value is same as absoluteFrequencySSB only when the serving cell is associated with SSB; if the serving cell is not associated with SSB, the carrier frequency indicated by ssbFrequnecy of the corresponding MeasObjectNR, if configured, is within the frequency range indicated by any entry of the scs-SpecificCarrierList. 4: Clarify the presence condition of ssbFrequency in servingCellMO for a SSB-less SCell that “If the measObject is associated to an SSB-less SCell, this field is optionally present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell, otherwise, this field is mandatory present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. If ssb-ConfigMobility is not configured and associatedSSB is not configured for any cell, the field is absent, Need R.” 5: Clarify in stage 2 that SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the SSB frequency configured in the measurement object indicated by servingCellMO and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same. 6: The above changes are introduced from Rel-15. R2-2504906 Reply LS on neighboring cell measurement for Rel-18 SSB-less RAN2 LS out Rel-18 Netw_Energy_NR-Core To:RAN4 The LS is approved in R2-2504927 with the CRs attached

- Discussion

According to current RAN4 specification below, a measurement is defined as intra-frequency measurement only if the centre frequency of the SSB of the serving cell and the neighbour cell are the same. However, for SSB-less SCell, there is no SSB configuration in the serving cell hence the existing definition does not apply.

TS 38.133 clause 9.2:

A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the SSB of the serving cell and the centre frequency of the SSB of the neighbour cell indicated for measurement are the same, and the subcarrier spacing of the two SSBs are also the same.

Observation #1: Existing definition of intra-frequency measurement in RAN4 does not consider SSB-less SCell.

Based on the RAN2 agreements above, a UE may indicate the capability of supporting the configuration of servingCellMO for SSB-less SCell, and such UE shall also support intra-frequency measurements on neighbour cells on the carrier indicated by servingCellMO. This allows UE to perform intra-frequency measurements if servingCellMO is configured for SSB-less SCell and the following change was captured in RAN2 stage 2 CR [2]. Accordingly, the intra-frequency measurement definition in RAN4 shall also be updated taking into account RAN2 conclusion.

Intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements are defined as follows: - SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the SSB frequency configured in the measurement object indicated by servingCellMO and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same.

Proposal 1: The intra-frequency measurement definition in RAN4 shall be updated taking into account RAN2 conclusion on servingCellMO for SSB-less SCell.

In addition, RAN2 agreed such changes are introduced from Rel-15. Hence the RAN4 change shall also start from Rel-15. Similar as the RAN2 CR, for UE supporting either intra-band SSB-less operation in R15 or inter-band SSB-less operation in R18, if servingCellMO is configured for the SSB-less SCell, a measurement shall be defined as an SSB-based intra-frequency measurement provided the SSB frequency configured in the measurement object indicated by servingCellMO and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing in the measurement object indicated by servingCellMO and the subcarrier spacing of the SSB of the neighbour cell are also the same. The companion RAN4 CRs can be found in [3].

Proposal 2: The change on the intra-frequency measurement definition in RAN4 shall start from Rel-15.

Proposal 3: For UE supporting either intra-band SSB-less operation in R15 or inter-band SSB-less operation in R18, if servingCellMO is configured for the SSB-less SCell, a measurement shall be defined as an SSB-based intra-frequency measurement provided the SSB frequency configured in the measurement object indicated by servingCellMO and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing in the measurement object indicated by servingCellMO and the subcarrier spacing of the SSB of the neighbour cell are also the same.

- Conclusion

In this contribution, we discussed the intra-frequency measurement definition based on the RAN2 LS. The following observations and proposals are made:

Observation #1: Existing definition of intra-frequency measurement in RAN4 does not consider SSB-less SCell.

Proposal 1: The intra-frequency measurement definition in RAN4 shall be updated taking into account RAN2 conclusion on servingCellMO for SSB-less SCell.

Proposal 2: The change on the intra-frequency measurement definition in RAN4 shall start from Rel-15.

Proposal 3: For UE supporting either intra-band SSB-less operation in R15 or inter-band SSB-less operation in R18, if servingCellMO is configured for the SSB-less SCell, a measurement shall be defined as an SSB-based intra-frequency measurement provided the SSB frequency configured in the measurement object indicated by servingCellMO and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing in the measurement object indicated by servingCellMO and the subcarrier spacing of the SSB of the neighbour cell are also the same.

- References

[1]	R2-2504906, Reply LS on neighboring cell measurement for Rel-18 SSB-less, RAN2 LS out To:RAN4

[2] 	R2-2504927, Reply LS on neighboring cell measurement for Rel-18 SSB-less with the CRs attached

[3] R4-2509603, (NR_newRAT-core) 38133CR on intra-frequency measurement with SSB-less SCell – R15, Nokia, Nokia Shanghai Bell