3GPP TSG-RAN WG4 Meeting #116	R4-2509660

# Bengalore, India, 25th – 29th August, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for Ku-band NTN

Agenda item:	7.7.7

Document for:	Discussion

Introduction

In this contribution, we provide our views on open issues for Ku-band NTN based on approved WF [1].

Discussion

Satellite switch requirements

In the last RAN4 meeting, RAN4 agreed to introduce satellite switching with re-sync for Ku band for both FR1 and FR2 numerologies as below.

Agreement: - Introduce RRM requirement for satellite switching with re-sync for Ku band with VSAT UE for FR1 and FR2 numerologies - Define requirement for hard satellite switch, - Tinterrupt = Tsat_beam + Tsearch + Tprocessing + T∆ + Tmargin ms - Further discuss the details on the definition of each component. - FFS whether to define requirements for soft satellite switch - FFS only switching delay requirement or both switching delay requirement and scheduling restriction requirement - Option for the delay requirement: - Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing ms

Hard satellite switching with re-sync

For hard satellite switching with re-sync, the existing FR1-NTN requirement of hard satellite switching with re-sync can be reused by adding Tsat_beam which is the time for UE to steer the downlink spatial domain reception filter to the target cell as decribed above agreement (equation of Tinterrupt). This requirements can be applicable for both electronic and mechanical beam steering UEs. However, Tsat_beam for mechanical beam steering UEs would be long in seconds. Therefore, Tsearch should be treated as in the case of unknow target cell case.

- Proposal 1: Introduce RRM requirements for hard satellite switching with re-sync by reusing existing requirement with adding Tsat_beam.

Soft satellite switching with re-sync

Per 6.1C.3.2.3 : “The requirement in this clause applies and UE is required to keep the connection (DL and UL) with the source NGSO satellite, under the following conditions: SSBs from the two satellites are spaced apart from each other at least by 1 OFDM symbol in the time domain at UE Rx side.”

For mechanical beam steering UEs in Ku band, the Rx beam steering time Tsat_beam = (Oangle / 22.5) is typically in the order of seconds for realistic steering angles. This duration is significantly longer than an SSB periodicity and far exceeds the allowable interruption window between t-serviceStart and t-service in soft satellite switching.
During this prolonged steering period, the UE cannot simultaneously receive DL from the source satellite or transmit UL, thus breaking the DL/UL connection maintenance requirement in 6.1C.3.2.3.

- Proposal 2: Do not support mechanical beam steering UEs for FR2 soft satellite switching with re-sync.

Soft satellite switching with re-sync can still be feasible for electronically steered antenna (ESA) UEs in FR2. In ESA UEs, the beam steering process is performed electronically and can be completed within a time much shorter than the interval between t-serviceStart and t-service, without causing loss of the DL/UL connection..

For such ESA UEs, scheduling restrictions during intra-frequency measurements must be enforced. This ensures that UE can properly measure target SSB without interference from UL transmissions or DL receptions.

- Proposal 3: Introduce RRM requirements for soft satellite switching with re-sync only for ESA UEs in FR2, with application of scheduling restrictions.

Inter-satellite cell reselection requirements (section 4.2C).

Candidate options in RAN4#115 - Option 1: RAN4 to create inter-satellite cell reselection requirements for VSAT UEs with electronically steered beams in Rel-19 - Option 2: Do not define inter-satellite cell reselection requirements in this WI. - Option 2a: RAN4 to consider inter-satellite measurement related discussion for Ku-band NTN in a future release (e.g., Rel-20).

RAN4 discussed whether to consider RRM requirements for inter-satellite cell reslection. Even if FR1-NTN numerology is used in Ku-band, the UE behavior is the same VSAT defined in FR2-NTN Ka-band. If RAN4 consideres to define inter-satellite measurement in Ku-band, only for VSAT type 5 which has electronic steering antenna might be feasible, and it would be good chance to introduce new VSAT UE behavior in Ku-band . However, considering Rel-19 timeline, we prefer to do this discussion in next release.

- Proposal 4: Do not define inter-satellite cell reselection requirements in Rel-19.

RLM (section 8.1C)

Candidate options in RAN4#115 - Option 1: LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology. - Option 2: LMax is equal to 8 for both FR1-NTN numerology and FR2-NTN numerology.

There were two options for LMax definition for RLM requirements. In RF session, the frequency bands for NTN were defined as FR1-NTN and FR-2 NTN including Ku-bands. Therefore, option 1 can be used for LMax definition.

- Proposal 5: For NTN RLM requirements, LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology as Option 1.

Conclusion

In this contribution, we provide our views on open issues for Ku-band NTN, and we propose

- Proposal 1: Introduce RRM requirements for hard satellite switching with re-sync by reusing existing requirement with adding Tsat_beam.

- Proposal 2: Do not support mechanical beam steering UEs for FR2 soft satellite switching with re-sync.

- Proposal 3: Introduce RRM requirements for soft satellite switching with re-sync only for ESA UEs in FR2, with application of scheduling restrictions.

- Proposal 4: Do not define inter-satellite cell reselection requirements in Rel-19.

- Proposal 5: For NTN RLM requirements, LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology as Option 1.

Reference

[1] R4-2508262, “WF on RRM requirements for NR_NTN_Ku_bands,” Apple