3GPP TSG-RAN WG4 Meeting 115	R4-2509761

Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on core requirement and testing framework for AI beam management

Agenda Item:	7.17.4

Document for:	Discussion

- Introduction

In this contribution, we will discuss core part requirement and testing framework for AI based beam management.

- Discussion

      - 2.1 Measurement window

- Issue 2-2: Measurement period for inference - Agreement: - Case 1: The starting point is to use M, N, P (reference the values from an existing clause in the same version of the spec where the measurement period is defined) - Case 2: postpone discussion until RAN1 concludes

For beam case 2:

Since UE will measure RSRP of multiple time instance to predict future RSRPs, a new scaling factor Q will be introduced, then measurement time is Q times of legacy measurement period. Different AI model may require different measurement time, and the measurement time is determined by the UE implementation. Therefore, Q is dependent on the UE capability. The UE can report either the number of measurement samples or the ratio of measurement time to prediction time.

Measurement period TL1-RSRP_Measurement_Period_SSB for FR2 for BM-case 2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N*Q)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N*Q)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N*Q)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Q is up to AI BM Case-2 UE prediction capability, reflecting the scaling factor for the measurement time. |  |

Proposal 1: For BM-case 2, introduce a scaling factor Q to adjust the measurement time for RSRP predictions, where Q is determined by UE capability, to adopt to different UE implementations.

      - 2.2 Prediction window for BM-case 2

The RAN1 agreements about BM-case are as below:

RAN1 118 Agreement For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - FFS: how to determinate reference time for the time instance(s) - FFS: duration values of the N time instance(s) that can be predicted. Agreement(RAN 1120) For inference, for BM-Case 2 of UE-side model, - The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC. - time gap is [10ms, 20ms, 40ms, 80ms, 160ms] - N = [1, 2, 4, 8] - Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report. Agreement (RAN 1 120) For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, - the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP. The time instance information of the beam with the largest RSRP are additionally indicated in the report.

For BM-case 2, UE can report predicted RSRP for future time. During the prediction window in future time, UE doesn’t need to measure this RSRP again, then the scheduling and measurement restriction can be relaxed in the prediction window. A prediction window needs to be defined.

Stating point of prediction window is:

- the most recent occasion of the CSI-RS/SSB resource in Set B for measurement + Reference time of the earliest time instance for the predicted results

Ending point of prediction window is:

- starting point + Ttime_gap *N, where Ttime_gap is the time gap between two consecutive future time instances configured by RRC, and N is the number of future time instance(s) configured by RRC.

Proposal 2: For BM-case 2, RAN4 to define prediction window:

- Stating point of prediction window is:

- The most recent occasion of the CSI-RS/SSB resource in Set B for measurement + Reference time of the earliest time instance for the predicted results

- Ending point of prediction window is:

- Starting point + Ttime_gap *N, where Ttime_gap is the time gap between two consecutive future time instances configured by RRC, and N is the number of future time instance(s) configured by RRC.

Proposal 3: For BM-case 2, the scheduling and measurement restriction can be relaxed in the prediction window.

      - 2.3 TCI state activation

- Issue 2-3: Rx beam knowledge and TCI handling - Agreement: - if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown. - Known Rx beam means no additional Rx beam sweeping is needed - Unknown Rx beam means additional Rx beam sweeping is needed. - Rx beam is known mean TCI state is known if conditions below are also met - The following conditions related to whether the TCI state is known or unknown is defined as the starting point. Companies are invited to bring analysis. - The target TCI state is known when the following is met: The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1 RSRP report of the target TCI state (clause 8.10.2 of 38.133) - Applies for case 1 - FFS for case 2 - Whether RS resource is only from set B should be clarified - FFS on whether prediction also impacts if a TCI state is known or unknown - Companies are invited to bring analysis on how prediction would impact whether a TCI state is known or unknown - FFS on the time delay if prediction is used as a condition for a known TCI state

In last meeting, a new UE capability is introduced, therefore, we propose to define a new UE capability for RX beam known.

Proposal 4: Define a new UE capability with the following table:

BM Case-1

For BM case-1, several issues are still not clear.

Fig.1 legacy valid time for measurement report

Existing Logic

- Time window starts from the last measured transmission of the target TCI state's RS .

- The 1280ms window is defined based on measurement period, i.e. assumes RX beam sweeping over periodic RS (160ms period × 8 beams = 1280ms).

Why It Fails for AI Case 1

1.	No RS transmission for Set A :

•	Set A beams are predicted (not physically transmitted) → No "last transmission" timestamp exists for target TCI state.

2.	Fundamental mismatch with prediction :

•	Predicted beams require no physical measurement → 1280ms is aimed for measurement period and can’t be directly used for this scenario.

3.	New timing paradigm needed :

•	Prediction report validity time, Tprediction_valid, depends on channel coherence time, but not measurement duration.

•	TCI commands must arrive while predictions are still valid.

•	Prediction validity depends on channel coherence time

RAN4 needs to consider the valid time for prediction.

Fig.2 valid time for prediction report

Proposal 5: For BM case-1, the TCI state switch command is received within Tprediction_valid ms after latest predicted L1 RSRP report of the target TCI state. Where Tprediction_valid ms is prediction valid time which is related to channel coherence time. FFS Tprediction_valid ms.

Detectability/SNR Condition Failure

•	Existing Logic :

Requires target TCI state/SSB to be "detectable" with SNR ≥ -3dB during switching.

•	Why It Fails :

Set A beams have no actual RS transmission → UE cannot measure detectability or SNR.

→ Conditions become unverifiable for predicted beams.

Proposal 6: For BM case-1, when target TCI state is in set A but is not QCL-D to any measured RS before, there is no detectable RS and SNR condition.

Clarifying “QCL Type-D to a Known Tx Beam”

Ambiguity:
Does “known” refer only to beams in Set B, or to any previously measured or predicted beam outside set B?

Relevant Cases:

- Measured in Set B: Predicted Set A beam is QCL-D to an RS in Set B.

- Measured outside Set B: Predicted Set A CSI-RS beam is QCL-D to an SSB measured by legacy procedures and not included in Set B.

- Predicted with Known Rx: QCL-D to another predicted beam whose Rx is already known.

We suggest to focus on Cases 1 and 2 because both rely on UE-measured beams, ensuring reliable Rx beam knowledge without additional measurements. This approach avoids dependence on uncertain prediction chains, keeping the protocol simple and backward compatible.

Proposal 7: For BM case-1, if the predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B, the corresponding Rx beam is known.

BM Case-2

Problem Statement

Existing TCI state known conditions (TS 38.133 8.10.2) fail for AI BM Case 2 because:

1.	Multi-timeline prediction : Reports cover N future time instances (configured via RRC), invalidating the single-RS timestamp anchor.

2.	Prediction validity decay : Static 1280ms window is based on measurement period

Fig.3 valid time for prediction for BM case-2

Different BM case-1, BM-case 2 will report predicted result for future multiple instances. Therefore, the length of future multiple instances needs to be considered as well. The total valid time for prediction report is:

Tprediction_time_period + Tprediction_valid

Tprediction_time_period is Prediction time period, which is dependent on NW configuration. There are two options:

- Option 2-1 Dynamic value: Calculated according to RRC configuration, which is configured Time gap * configured number of future time instance(s) N, or

- Option 2-2: Fixed value, max(configured Time gap) * max(configured number of future time instance(s) N) = 160*8 = 1280ms

Besides, similar as BM case-1, there is no detectable RS and SNR condition.

Proposal 8: For BM case-2:

- The target TCI state is known when the following is met: The TCI state switch command is received within Tprediction_time_period + Tprediction_valid ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L1 RSRP report of the target TCI state

- there is no detectable RS and SNR condition.

      - 2.4 Test Motivation

Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

There are lots of discussions about channel model, e.g. single AOA or multiple AoAs.

Some company mentioned that without multiple AoA, UE RX beam sweeping functionality can’t be verified. On the other hand, some company mentioned that the main test motivation is to verify the AI algorithm and single AOA channel is enough to generate the emulated received signal.

If AI model can achieve final prediction accuracy, it involves two steps:

- Input data of AI model achieve accuracy requirement

- AI model algorithm achieve prediction requirement

From our understanding, it’s better to split the two parts in testing, otherwise, it’s hard to distinguish UE fails in which step. The total test includes two parts:

- Part 1: Verify input data accuracy of model AI based on RX beam sweeping

- Part 2: Verify AI model algorithm

For Part 1, it’s related to measured RSRP accuracy based on RX beam sweeping. This is not a new issue for AI. While for legacy beam management, it also has the similar issue. However, in legacy, the RX and TX beamforming gain has been transformed to equivalent SNR. RSRP accuracy is tested under certain SNR level. No RSRP accuracy requirement for RX beam sweeping is defined. If company really thinks it’s important, the RX beam sweeping performance can be tested separately. For this part, multi-AOA model can be used.

For Part 2, the emulated data can be used as input for AI model, then AI model prediction accuracy can be tested by single AOA model.

Proposal 9: For OTA test, prefer to split the test into two parts.

- Part 1: Verify AI model prediction algorithm, using single AOA model.

- Part 2: Verify input data accuracy of model AI based on RX beam sweeping, using multi-AOA model.

      - 2.5 Channel model

### 2.5.1 Single AOA

For test part 1, single AOA can be used.

There are several possible options to generate single AOA:

  - Option 1: Emulated signal considers both TX and RX beamforming gain.

Since the emulated signal is after UE RX beamforming, single AOA is assumed, then the signal shall consider both TX and RX beamforming gain:

i=1NPT,iCiPR,i

Where N is number of multi-path signals. PT,i is Tx beamforming gain for ith-path signal. Ci is power of ith-path signal. PR,i is Rx beamforming gain for ith-path signal. It means that TX side needs to know the UE RX beam forming codebook in advance.

For this method, no simplified channel is needed as TX side can generate all kinds of channels. This method requires that RX beam pattern is known by TE in advance. However, since there are many UE vendors, then the test will be UE specific. we prefer to design a common pre-defined RX beam pattern for both TE and UE.

Observation 2: If Emulated signal considers both TX and RX beamforming gain, RX beam pattern is UE specific. In order to reduce test complex, prefer to design a common pre-defined RX beam pattern for both TE and UE.

  - Option 2: The emulated signal is before UE RX beamforming.

    - Option 2-1: Not reduce path number, but combine all TX signal by a fixed RX beamforming.

For option 2-1, the emulated signal is before UE RX beamforming, then the signal only considers TX beamforming gain, it will combine all TX path together by multiply TX beamforming gain. Before RX beamforming, multiple TX signal has already be combined into one path. UE needs to receive the combined TX signal by RX beam. the final input data for AI model will be:

PRi=1NPT,iCi

It means that the same RX beamforming gain, PR, is applied for different TX path signals. however, in reality, there will be different RX beamforming gain for different TX path signals. There will be some mismatch between emulated signal for test and original signals for training. The mismatch impact needs to be evaluated.

Observation 3: if a fixed RX beamforming gain is used for multi-path TX signals, there will be some mismatch between emulated signal for test and original signals for training. The mismatch impact needs to be evaluated.

    - Option 2-2: Simplify multi-path into single-path.

The following figure compare the L1-RSRP difference between different TX beam index with single path, the detail configuration and analysis can be found in[1]. By our simulation, the less the path number, the larger gap between different beams, which means that AI prediction performance will be relaxed. It’s more easiler for AI to distingwish the best Top-K TX beam. Therefore, we prefer not to reduce probe number to verify AI prediction performance.

Simplified CDL-C with 3 cluster, 1 AOA

Fig.1 CDF curve L1-RSRP difference between CDL-C and simplified CDL-C

Observation 4: For simplified channel model, L1-RSRP delta between different TX beams will increase if cluser number and AOA number is further reduced. It’s more easiler for AI to distingwish the best Top-K TX beam. The AI prediction requirement will be relaxed.

In summary, we prefer option 1 or option 2-1 for single AOA solutions to verify AI prediction accuracy.

For option 1, further discuss whether it’s possible to design common pre-defined RX beam pattern to generate emulated single AOA signals.

For option 2, further discuss the impact of mismatch between emulated signal for test and original signals for training.

Proposal 10: For single AOA, RAN4 to discuss whether it’s possible to design common pre-defined RX beam pattern to generate emulated single AOA signals.

### 2.5.2 Multi-AOA

For test part 2, multi-AOA can be used. For multi-AOA, due to probe limitation in chamber, simplified channel model needs to be studied.

For CDL model, there are several clusters and for each cluster, there are 20 rays. In order to reduce the total number of AOA, for each cluster, only one ray is considered.

Additionally, the number of clusters can be further reduced using the following options:

- Angle Offset Threshold:

  - If the AOA offset between two clusters is smaller than a threshold, these clusters can be merged into one. The AOA offset threshold may depend on the angle resolution of the RX antenna and test setup limitations, which can be further discussed in RAN4.

- Power Threshold:

  - Remove clusters with low power. The power threshold can be further discussed.

- Combination of Angle and Power Thresholds:

  - Apply both angle and power thresholds to reduce the number of clusters.

How to choose cluster for simplified CDL channel needs to be discussed. Here, we select 8 clusters with 3 probes for simplified CDL-C

- 1st probe: cluster {2,3,4}with the same angle

- 2nd probe: cluster{6,7,8}with the same angle

- 3rd probe: cluster{1,5} with small angle offset

| Cluster | Normalized delay | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4 | -46.6 | -101 | 97.2 | 87.6 |
| 2 | 0.2099 | -1.2 | -22.8 | 120 | 98.6 | 72.1 |
| 3 | 0.2219 | -3.5 | -22.8 | 120 | 98.6 | 72.1 |
| 4 | 0.2329 | -5.2 | -22.8 | 120 | 98.6 | 72.1 |
| 5 | 0.2176 | -2.5 | -40.7 | -127.5 | 100.6 | 70.1 |
| 6 | 0.6366 | 0 | 0.3 | 170.4 | 99.2 | 75.3 |
| 7 | 0.6448 | -2.2 | 0.3 | 170.4 | 99.2 | 75.3 |
| 8 | 0.6560 | -3.9 | 0.3 | 170.4 | 99.2 | 75.3 |

Proposal 11: For simplified multi-AOA CDL model, cluster number can be reduced from spatial angle offset and power aspect:

- Combine clusters if AOA offset between these clusters is smaller than a threshold

- Remove cluster with low power.

      - 2.6 Test procedure

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

it’s agreed that the ground truth of the predicted RSRP is the ideal measurement of RSRP on the predicted TX beam.

Test procedure

Fig.2 test procedure

As shown in Fig.2, the main steps for test beam or L1-RSRP accuracy are as below:

- Step 1: TE configures resource sets for measurement of set B and prediction report for set A

- Step 2: UE measures for beams in set B in T1

- Step 3: UE performs beam prediction for set A

- Step 4: UE report predicted best beam(s) and/or L1-RSRP(s)

- Step 5: TE configures resource set for measurement of set A

- Step 6: UE measures for beams in set A in T2 under high SNR

- Step 7: UE report ground truth of L1-RSRP of set A and/or best beam index

- Step 8: TE compares predicted beam with ground truth based on different metric

We identified that there are several issues that will have impact on the test metric accuracy.

### 2.6.1 Ground truth for best beam index

If the test metric is beam index prediction accuracy, UE will report the best beam index in set A after measurement. However, as we analysed in performance metric part, UE may not distinguish for the best 1st beam index and 2nd best beam index due to small L1-RSRP delta between the best 1st beam index and 2nd best beam index.

Therefore, UE may need to report for N best beam index. RAN4 can further discuss how to decide N.

Proposal 12: If the test metric is beam index prediction accuracy, UE may need to report ground truth of N best beam index for set A.

### 2.6.2 SNR impact

Same as SNR difference analysed for set B, there will be great SNR difference for 32 TX beams in set A either. If SNR for the worst TX beam is set as 10dB, then SNR for best TX beam will be too high. There may be some test issue. Suppose that if SNR of best several TX beams is set as 10dB, SNR for the worst 10 TX beam can be as low as -15dB. There will be great measurement error for the worst beam.

For set A, UE only needs to identify several best beam indexes and/or corresponding L1-RSRPs, UE doesn’t need to measure for all beams accurately. The reason is that since the L1-RSRP delta between best beam and worst beam is relatively large, the measured L1-RSRP for worst beam is still quite small, which will not exceed the value of L1-RSRP of best TX beams.

Therefore, for deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Observation 5: For deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Therefore, RAN4 needs to discuss how to set SNR condition for measurement of set A to derive ground truth.

Proposal 13: For deriving ground truth of set A, SNR level should satisfy that for Top-M beams, the measurement error is small. RAN4 to discuss how to decide M and how to set SNR.

### 2.6.3 Channel time variation impact

Since the beam prediction will be based on the measurement of set B at T1. Ground truth L1-RSRP will be measured at T2. There will be a time gap between T1 and T2. Due to doppler, L1-RSRP will also vary in time domain. L1-RSRP time domain variation will also have impact on ground truth.

For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation. While for BM case-2, UE will be assumed to move. Therefore, channel doppler can’t be set for 0. Doppler will depend on UE speed and UE trajectory model. The RSRP variation can be evaluated by different doppler and time gap. RSRP variation impact to BM case-2 can be considered in RSRP accuracy requirement.

Proposal 14: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 15: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

### 2.6.4 How to solve UE cheating issue

In legacy L1-RSRP test, ground truth is set by TE. Therefore, UE can honestly report the measured L1-RSRP results.

In current AI based BM test, UE will report both best predicted beam and ground truth value to TE. TE can only rely on the two reports from UE to decide whether UE can pass the test. It’s easy for UE to cheat if the configuration for set A at T1 and T2 are the same. For example, suppose that for both T1 and T2, the same resource is configured and beam is mapping to these resources with the same order. Then in T1, UE report the best predicted RS index is 7. And in T2, after measurement for all RSs, UE find out the best beam is in RS 6. However, UE can just report the best measured RS index with 7 to TE. since TE didn’t know which beam is the best, it will only compare two indexes from UE reported at T1 and T2. If the two index is the same, TE will assume that predicted best beam is correct. Therefore, RAN4 needs to discuss how to avoid the issue.

Observation 6: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it’s easy for UE to pass the test by cheating.

The issue can be solved by several possible options.

- Option 1: Configure different resource for set A in T1 and T2.

- Option 2: Configure same resource for set A in T1 and T2 while the mapping relation of RS index and beam index is different. For example, suppose the beam index and RS index mapping relation for T1 is shown in Fig.3(a). For T2, TE will re-transmit all beams in set A on all RSs. TE will change the mapping relationship between RS index and beam index as shown in Fig.3(b). In this way, UE can only honestly report the best measured beam in T2.

Fig.3(a) beam index and RS index mapping relation for set A in T1

Fig.3 (b) beam index and RS index mapping relation for set A for T2

Proposal 16: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth.

- Conclusion

In this contribution, we provide the following proposals:

Proposal 1: For BM-case 2, introduce a scaling factor Q to adjust the measurement time for RSRP predictions, where Q is determined by UE capability, to adopt to different UE implementations.

Proposal 2: For BM-case 2, RAN4 to define prediction window:

- Stating point of prediction window is:

- The most recent occasion of the CSI-RS/SSB resource in Set B for measurement + Reference time of the earliest time instance for the predicted results

- Ending point of prediction window is:

- Starting point + Ttime_gap *N, where Ttime_gap is the time gap between two consecutive future time instances configured by RRC, and N is the number of future time instance(s) configured by RRC.

Proposal 3: For BM-case 2, the scheduling and measurement restriction can be relaxed in the prediction window.

Proposal 4: Define a new UE capability with the following table:

Proposal 5: For BM case-1, the TCI state switch command is received within Tprediction_valid ms after latest predicted L1 RSRP report of the target TCI state. Where Tprediction_valid ms is prediction valid time which is related to channel coherence time. FFS Tprediction_valid ms.

Proposal 6: For BM case-1, when target TCI state is in set A but is not QCL-D to any measured RS before, there is no detectable RS and SNR condition.

Proposal 7: For BM case-1, if the predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B, the corresponding Rx beam is known.

Proposal 8: For BM case-2:

- The target TCI state is known when the following is met: The TCI state switch command is received within Tprediction_time_period + Tprediction_valid ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L1 RSRP report of the target TCI state

- there is no detectable RS and SNR condition.

Proposal 9: For OTA test, prefer to split the test into two parts.

- Part 1: Verify AI model prediction algorithm, using single AOA model.

- Part 2: Verify input data accuracy of model AI based on RX beam sweeping, using multi-AOA model.

Observation 2: If Emulated signal considers both TX and RX beamforming gain, RX beam pattern is UE specific. In order to reduce test complex, prefer to design a common pre-defined RX beam pattern for both TE and UE.

Observation 3: if a fixed RX beamforming gain is used for multi-path TX signals, there will be some mismatch between emulated signal for test and original signals for training. The mismatch impact needs to be evaluated.

Observation 4: For simplified channel model, L1-RSRP delta between different TX beams will increase if cluser number and AOA number is further reduced. It’s more easiler for AI to distingwish the best Top-K TX beam. The AI prediction requirement will be relaxed.

Proposal 10: For single AOA, RAN4 to discuss whether it’s possible to design common pre-defined RX beam pattern to generate emulated single AOA signals.

Proposal 11: For simplified multi-AOA CDL model, cluster number can be reduced from spatial angle offset and power aspect:

- Combine clusters if AOA offset between these clusters is smaller than a threshold

- Remove cluster with low power.

Proposal 12: If the test metric is beam index prediction accuracy, UE may need to report ground truth of N best beam index for set A.

Observation 5: For deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Proposal 13: For deriving ground truth of set A, SNR level should satisfy that for Top-M beams, the measurement error is small. RAN4 to discuss how to decide M and how to set SNR.

Proposal 14: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 15: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

Observation 6: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it’s easy for UE to pass the test by cheating.

Proposal 16: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth.

- Reference

[1] R1-2501595, FL summary 6 for AI/ML in beam management