3GPP TSG-RAN WG4 Meeting #116	R4-2509805

Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	D2R LLS SNR simulation results update

Agenda item:	7.22.3.1

Document for:	Discussion

- Introduction

This contribution updates our D2R SNR simulation results based on new approved assumptions including RAN1 midamble agreements.

- Discussion

In our simulation, the following assumptions are used. The highlighted parameters are the options we chose from the WF [1].

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 900 MHz |
| SCS | 15 kHz as baseline |
| Block structure | Depend on RAN1 |
| Channel model | AWGN |
| Number of Tx/Rx chains for Ambient IoT device | 1 |
| Number of BS antenna elements | 1 |
| Reference data rate | 1 kbps (M) |
| Message size | 96 bits |
| BLER target | 10% |
| Sampling frequency（SFO） | Randomly select a value from the range of [104 ppm~105 ppm] for device 1 |
| [OOK/BPSK chip rate] | 7.5 kcps for 2SB 15 kHz Transmission bandwidth |
| Receiver bandwidth | 3rd order Butterworth filter with 8kHz pass band |
| Waveform (CW) | unmodulated single tone |
| Modulation | BPSK |
| Line code | Manchester encoding |
| FEC | 1/3 Convolutional Code |
| ADC bit width | Ideal |
| D2R receiver | coherent receiver |
| Repetition | None |
| Preamble configuration | 32 bit M-sequence |
| Midamble configuration | 32 bit M-sequence |

We have the following simulation results using the above assumptions,

| preamble/midamble configuration | Message size | BLER target | SNR (dB) |
| --- | --- | --- | --- |
| Without SFO | 96bits | 10% | -7.3 |
| Preamble, without midamble | 96bits | 10% | 13 |
| Preamble + 1 midamble | 96bits | 10% | -7 |
| Preamble + 2 midamble | 96bits | 10% | -7 |

From above simulation results, it seems for AWGN, 1 midamble is sufficient to correct SFO impact.

Observation: 1 midamble is sufficient to correct SFO impact for the configurations in the simulation assumptions.

- Conclusion

This contribution provides our simulation results update. Our observation is that 1 midamble is sufficient to correct SFO impact for the configurations in the simulation assumptions. The SNR threshold in our results is -7 dB.

- References

[1] R4-2508101, “WF on A-IoT BS and CW requirements”, Huawei, RAN4#115