# 3GPP TSG-RAN WG4 Meeting #116	R4-2510262

Bengaluru, India, August 25th – 29th, 2025

Title:	Reply LS on requirements for phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3

Response to:	(R4-2505305_R1-2503071) LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3

Release:	Release 19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name: Shuai Zhou

E-mail Address: shuai.zhou@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN1 for the LS in R1-2503071.

RAN4 has discussed the phase continuity requirements for the duration of one OCC group with PUSCH and confirmed that the same requirements for DMRS bundling in Table 6.4.3.5-1 of TS 38.101-1 can still be applied, i.e. ≤25° between adjacent slots and ≤30° between the first slot and slot p.

The condition when above requirements are applicable can refer to Section 6.4.2.2 of TS 38.101-5 with one additional item.

The above requirements are applicable when all the following conditions are met within the measurement time window: - RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated. - Modulation order does not change. - No network commanded TA takes effect. - The TPMI precoder does not change. - There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE. - UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions. - Doppler conditions are set to zero and delay conditions are set to constant. - No UE specific and common TA updates.

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to take the above information into account for the phase continuity requirements for the duration of one OCC group with PUSCH.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #116-bis	13 - 17 Oct. 2025                    Prague, CZ

TSG-RAN WG4 Meeting #117	17 - 21 Nov. 2025                    Dallas, US