3GPP TSG-RAN WG4 Meeting #116	R4-2510307

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	(NR_redcap-Core) PC3 1Rx RedCap REFSENS

Agenda item:	4.2.2

Document for:	Discussion

Introduction

During the last RAN4 meeting, proposals were presented to relax the 1 RX REFSENS for RedCap devices for certain bands based on the fact the Rx noise impact is generally not the same for the primary and diversity receiver [1]. In this document we present our proposal regarding the same.

Discussion

Impacted release

The core arguments by the proponents for propagating these changes from Release 17 is that for 2Rx RedCap UE, the NR FR1 non-RedCap requirements were reused. For 1Rx, even though requirements should have been analyzed and specified band by band, they were lumped together and specified in a mostly band agnostic method during Release 17. However, it has been more than three years since Release 17 was frozen. There are already Release 17 compliant devices in the field, so such changes should only be applicable from Release 19 onwards and not Release 17.

It has been more than three years since Release 17 was frozen.

Release 17 compliant devices already exist in the field.

Agree to change the 1Rx delta to be band dependent only from Release 19 and onwards.

REFSENS value

1Rx devices have existed since LTE times and thus 1 Rx REFSENS requirements exist in the LTE specification. There is no technical reason why the delta between 2Rx and 1Rx REFSENS requirements be larger in NR as compared to LTE. Comparing the list of bands for which new relaxation is proposed with TS 36.101, there are three notable bands 8, 20 and 28 for which the 1Rx delta exists. For these bands, instead of defining new relaxation values, the values defined in TS36.101 should be utilized.

1Rx REFSENS requirements should not be more relaxed in NR as compared to LTE.

Agree to reuse the ΔR1R for n8, n20 and n28 from TS 36.101.

The methodology used by the proponents to derive the 1Rx delta is similar to what was used to derive the REFSENS delta tables for PC2 RedCap devices during the Release 19 period. This is one feasible way given that the test configuration defined by RAN5 does not exist. However, in the current case of PC3 1RX, test configuration exists. Thus, instead of reverse engineering and fudging numbers, the UE should be simulated with the given test configuration and then it should be derived whether any additional relaxation is required or not.

Test configuration already exists for the requirements that are proposed to be changed.

Analysis of the test configuration reveals that challenging frequency bands have already two types of relaxation: both in their REFSENS requirements and in their test configuration parameters. While reverse engineering based solely on current REFSENS values will suggest significant relaxation factors, actual simulations of these bands present substantially different results.

1Rx REFSENS relaxation should be based on measured or simulated performance taking into account the RAN5 test configuration.

Using similar assumptions as the proponent, we calculated the required 1Rx relaxation via two methods. Method 1 is similar to what proponents used and method 2 is using simulated performance with RAN5 configuration. The required ΔR1R via these two methods for different channel bandwidths is tabulated below in Table 1 and Table 2. As can be seen from these tables, the needed relaxation for higher channel bandwidths paints a completely different picture, where just based on current REFSENS number one can deduce that 3.9 dB of relaxation is needed, whereas simulated data shows that the current REFSENS along with the defined test configuration is very easy to meet and no relaxation is needed at all.

Table 1 ΔR1R via method 1 i.e., reverse engineering the self-interference using current REFSENS

| Band | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- |
| n5 | 3.0 | 3.0 | 4.0 |
| n8 | 3.0 | 3.1 | 4.0 |
| n12 | 3.0 | 3.9 | N/A |
| n20 | 3.0 | 3.2 | 3.2 |
| n26 | 3.2 | 3.2 | 3.9 |
| n28 | 3.2 | 3.0 | 3.3 |
| n71 | 3.0 | 3.1 | 4.1 |
| n74 | 3.0 | 3.0 | 4.0 |
| n85 | 3.0 | 3.9 | N/A |
| n105 | 3.0 | 3.1 | 3.7 |

Table 2 ΔR1R via method 2 i.e., simulated

| Band | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- |
| n5 | 3.1 | 3.1 | 0.0 |
| n8 | 3.1 | 2.5 | 0.0 |
| n12 | 3.2 | 0.0 | N/A |
| n20 | 3.1 | 2.1 | 2.2 |
| n26 | 3.3 | 3.3 | 0.4 |
| n28 | 3.2 | 3.1 | 1.7 |
| n71 | 3.0 | 2.5 | 0.0 |
| n74 | 3.0 | 3.1 | 0.0 |
| n85 | 3.2 | 0.0 | N/A |
| n105 | 3.0 | 2.5 | 0.0 |

Based on above mentioned factors, we propose the relaxation factors mentioned in Table 3 for the bands in question.

Table 3 Proposed ΔR1R for the 1Rx REFSENS

| Band | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- |
| n5 | 3.0 | 3.0 | 3.5 |
| n8 | 2.5 | 3.0 | 3.5 |
| n12 | 3.1 | 3.0 | N/A |
| n20 | 2.5 | 3.0 | 3.0 |
| n26 | 3.3 | 3.3 | 3.5 |
| n28 | 2.5 | 2.5 | 2.5 |
| n71 | 3.0 | 3.1 | 3.5 |
| n74 | 3.0 | 3.1 | 3.5 |
| n85 | 3.1 | 3.5 | N/A |
| n105 | 3.0 | 3.0 | 3.5 |

Agree to the 1Rx relaxation numbers presented in the accompanying CR R4-2510308

Conclusion

The following Observations and Proposals were made:

Observation 1: It has been more than three years since Release 17 was frozen.

Observation 2: Release 17 compliant devices already exist in the field.

Proposal 1: Agree to change the 1Rx delta to be band dependent only from Release 19 and onwards.

Observation 3: 1Rx REFSENS requirements should not be more relaxed in NR as compared to LTE.

Proposal 2: Agree to reuse the ΔR1R for n8, n20 and n28 from TS 36.101.

Observation 4: Test configuration already exists for the requirements that are proposed to be changed.

Proposal 3: 1Rx REFSENS relaxation should be based on measured or simulated performance taking into account the RAN5 test configuration.

Proposal 4: Agree to the 1Rx relaxation numbers presented in the accompanying CR R4-2510308

- References

- R4-2507936, CR to 38.101-1 on corrections for RedCap UE PC3 FDD band 1Rx REFSENS, Apple, Skyworks, RAN4#115, Malta, MT, 19th – 23rd May, 2025.