3GPP TSG-WG4 Meeting #116	R4-2510366

# Bangalore, India, 25th August 2025 – 29th August 2025

Title:	LS to RAN5 on HPUE MSD requirements

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_RF_Ph4-Perf

Source:	TSG RAN WG4

To:	TSG RAN WG5

Cc:

Contact Person:

Name:  Petri Vasenkari

E-mail Address:  petri.j.vasenkari@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN4 has implemented a change [1] how inter-band NR CA HPUE i.e. PC2 and PC1.5 reference sensitivity exceptions also called as maximum sensitivity degradation i.e. MSD are specified. So far RAN4 has discussed HPUE MSDs individually per NR CA configuration, similarly as default power class PC3 MSD. In future work after the CR [1] is implemented into Rel-19 specifications, HPUE MSDs can be directly calculated with the help of look up tables defined in 38.101-1. This new method applies to PC2 inter-band HPUEs implemented with 1Tx or 2Tx in one UL band and PC1.5 HPUEs implemented with 2Tx in one UL band. PCx = PC21Tx, PC22Tx, or PC1.52Tx.

New HPUE MSD formula is

PCx MSD = PC31Tx MSD + MSDmax

Where PC31Tx MSD is the MSD for default power class PC3 and MSDmax is derived from look up tables in 38.101-1 for NR CA.

There are separate sets of MSD look up tables, one set for 1UL band 1UL CC MSD due to harmonic, harmonic mixing, cross-band isolation and another set for 2UL band 2UL CC intermodulation interference MSD. The MSD look up table set consists of actual look-up-table for MSDmax and companying MSDmax correspondence look-up-table which maps in case of 1UL band 1UL CC a combination of source of interference and power class to correct column in  MSDmax table or in case of 2UL band 2UL CC a combination of power class and IMD order into correct column in  MSDmax table.

All specified PC2 MSD tables and PC1.5 MSD tables remain untouched in TS 38.101-1 and TS38.101-3

This change does not in RAN4 view change the way RAN5 defines HPUE MSD tests, it is just that the allowed MSD for HPUEs is defined differently.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully ask RAN5 to take this information into account.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #116bis	13 - 17 Oct, 2025		                  Prague, Czech Rep.

TSG-RAN WG4 Meeting #117	17 – 21 Nov, 2025		                  Dallas, US

4. References

[1] BigCR for R19