3GPP TSG-RAN WG4 Meeting #116	R4-2510376

Bengaluru, India, 25th – 29th August 2025

Source: 	Ericsson

Title:  	Discussion on RRM requirements of NTN with channel BW less than 5MHz

Agenda Item:	7.6.4.5

Document for:	Discussion

# Introduction

In the WI NR Enhanced requirements and test methodology for NR and IoT NTN, the objective: RRM of NTN with channel BW less than 5MHz was discussed in the last meeting, the consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on the issues concerning performance requirements as follow.

# Discussion

### Issue 2-2-3 Test cases configuations

In the light of the WF, our views on the configurations for the agreed test cases are summarized in the below table:

| Test case | Requirements | Configurations | Recommend WF/ Companies views |
| --- | --- | --- | --- |
| Common configuration |  | 15KHz, 3MHz BW, 12 PRB SSB, GSO/NGSO |  |
| HO-1: | 6.1C.1.2 NR SAN FR1 – NR SAN FR1 Handover | unknown cell |  |
|  |  | Intra-frequency |  |
|  |  | Same testing procedure as NTN can be reused |  |
|  |  | Other aspects |  |
| CHO-1 | 6.1C.2.2 NR SAN FR1 – NR SAN FR1 conditional handover or 6.1C.2.3 NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurements | With L3 measurement/ without L3 measurements | With L3 |
|  |  | Time based |  |
|  |  | Unknown target cell |  |
|  |  | Intra-frequency |  |
|  |  | Other aspects |  |
| SatSwitch-1 | 6.1C.3 NR SAN Satellite switching with re-synchronization | Hard switch |  |
|  |  | Unknown target cell |  |
|  |  | Other aspects: e.g. RACH based | RACH based |
| RLM-1 | 8.1C Radio Link Monitoring for Satellite Access | RLM OOS |  |
|  |  | FR1 PCell only |  |
|  |  | non-DRX/DRX | DRX |
|  |  | Other aspects |  |
| RLM-2 | 8.1C Radio Link Monitoring for Satellite Access | RLM IS |  |
|  |  | FR1 PCell only |  |
|  |  | non-DRX/DRX | Non-DRX |
| BFR-1 | 8.5C Link Recovery Procedures for Satellite Access | SSB based BFD |  |
|  |  | FR1 PCell only |  |
|  |  | non-DRX/DRX | Non-DRX |
|  |  | Other aspects |  |
| MR-1 | 9.2C NR intra-frequency measurements for SAN | with SSB index detection |  |
|  |  | Non-DRX/DRX | Non-DRX |
|  |  | No gap | No gap |
|  |  | Other aspects |  |
| MR-2 | 9.3C NR inter-frequency measurements for SAN | With SSB index detection |  |
|  |  | Non-DRX/DRX | DRX |
|  |  | No gap | Gap |
|  |  | Other aspects |  |
| IDLE-1 | 4.2C Cell Re-selection for NR UE for Satellite Access |  | Intra-frequency NR case, - Normal and enhanced requirements - Time-based |

# Conclusion

Proposal 1: Define the new test cases and corresponding configurations for the core requirements for NTN less than 5MHz:

| Test case | Requirements | Configurations | Recommend WF/ Companies views |
| --- | --- | --- | --- |
| Common configuration |  | 15KHz, 3MHz BW, 12 PRB SSB, GSO/NGSO |  |
| HO-1: | 6.1C.1.2 NR SAN FR1 – NR SAN FR1 Handover | unknown cell |  |
|  |  | Intra-frequency |  |
|  |  | Same testing procedure as NTN can be reused |  |
|  |  | Other aspects |  |
| CHO-1 | 6.1C.2.2 NR SAN FR1 – NR SAN FR1 conditional handover or 6.1C.2.3 NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurements | With L3 measurement/ without L3 measurements | With L3 |
|  |  | Time based |  |
|  |  | Unknown target cell |  |
|  |  | Intra-frequency |  |
|  |  | Other aspects |  |
| SatSwitch-1 | 6.1C.3 NR SAN Satellite switching with re-synchronization | Hard switch |  |
|  |  | Unknown target cell |  |
|  |  | Other aspects: e.g. RACH based | RACH based |
| RLM-1 | 8.1C Radio Link Monitoring for Satellite Access | RLM OOS |  |
|  |  | FR1 PCell only |  |
|  |  | non-DRX/DRX | DRX |
|  |  | Other aspects |  |
| RLM-2 | 8.1C Radio Link Monitoring for Satellite Access | RLM IS |  |
|  |  | FR1 PCell only |  |
|  |  | non-DRX/DRX | Non-DRX |
| BFR-1 | 8.5C Link Recovery Procedures for Satellite Access | SSB based BFD |  |
|  |  | FR1 PCell only |  |
|  |  | non-DRX/DRX | Non-DRX |
|  |  | Other aspects |  |
| MR-1 | 9.2C NR intra-frequency measurements for SAN | with SSB index detection |  |
|  |  | Non-DRX/DRX | Non-DRX |
|  |  | No gap | No gap |
|  |  | Other aspects |  |
| MR-2 | 9.3C NR inter-frequency measurements for SAN | With SSB index detection |  |
|  |  | Non-DRX/DRX | DRX |
|  |  | No gap | Gap |
|  |  | Other aspects |  |
| IDLE-1 | 4.2C Cell Re-selection for NR UE for Satellite Access |  | Intra-frequency NR case, - Normal and enhanced requirements - Time-based |

# References

- R4-2502589, WF for RRM of NTN with channel BW less than 5MHz, Xiaomi