3GPP TSG-RAN4 Meeting #116	R4-2510420

# Bengaluru, India 25th – 29th August 2025

Title:	Views on general aspects for SBFD

Source:	Qualcomm Incorporated

Agenda item:	7.21.2

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#114bis, a WF was agreed in [2] entailing several aspects regarding SBFD general system design and deployment. In this contribution we share our views on the remaining open items that were considered in [2].

# General aspects

## Declaration and flexibility for SBFD operation

In [2], the following was discussed:

- For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm selected from the range of [33, 38] dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - X value will be selected in next meeting.

Since converging views were reached during the last meeting to accommodate different implementations, RAN4 agreed on the power threshold criteria that was proposed in [2]. Additionally, Table 1 below represents the adopted BS output power for MR BSs during the study item (see Table 9.3.1.1-1 in TR 38.858 for self-interference analysis, note that similar values were considered for co-channel inter-subband interference analysis), where we can observe that the tentatively proposed 33 dBm for MR SBFD-capable BS falls more towards the minimum value that was presented during the feasibility study. Thus, to be aligned with envisioned deployments of MR BSs in the field, RAN4 should not adopt a limiting value for X dBm to ensure proper coverage resulting from MR SBFD-capable BS deployment. Given that in TS 38.104, the TRP the MR BS rated carrier TRP output power limit is set to be at most 47 dBm, the tentatively proposed 33 dBm will be too limiting. In this regard, it would be a viable way forward to consider a compromise between maximum TRP values given in TS 38.104 and representative value based on feasibility analysis in TR 38.858. Accordingly, we believe 38 dBm would be a good compromise to ensure sufficient MR SBFD-capable BS deployments.

Table 1 MR BS Output power considered during Rel-18 SBFD SI

| FR1 | Nokia | Ericsson |  |  |  | ZTE | Huawei | Samsung |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class | Medium Range BS | Medium Range (3GPP minimum requirements) | Medium range (Realistic) | Medium Range (Optimistic RX) | Medium Range (Realistic, lower power) | MR | Medium Range BS | Medium Range BS |
| BS TX Power dBPTX = ① dBm | 36 dBm | 38 dBm | 38 dBm | 38 dBm | 35 dBm | 30 | 38 dBm | 38 dBm |

Proposal 1: RAN4 to consider the X = 38 dBm as MR SBFD-capable power threshold value (X dBm) as a compromise between MR maximum BS rated carrier TRP output power limit given in Table 9.3.1-1, TS 38.104 (i.e., 43 dBm) and representative values adopted in Section 9.3, TR 38.858.

## Update for HPUE duty cycle evaluation

In [2], the following was discussed (i.e., WF from RAN4#114bis:

- RAN4 has discussed the two following points: - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. - FFS RAN4 specification impact by taking into account the above understanding for SBFD-aware UE.

Recall, as captured in [1], that SBFD can be configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon. Additionally, it is important to note that for the legacy duty cycle evaluation, the calculation is based on the legacy uplink symbols that are transmitted by the UE, not on the symbol assignment only. Thus, in the case of flexible symbol(s) assigned by the network to UL transmissions for a given UE, UE would need to account for such symbol(s) as part of the duty cycle evaluation.

Observation 1: The HPUE duty cycle evaluation is based on UL transmissions carried by the UE and not only on the symbol assignment by the network.

In this regard, comparing this operation to SBFD, the same should apply since UL transmissions carried out by the UE, whether they are based on flexible symbol(s) or SBFD symbol(s), need to be accounted for as part of the duty cycle evaluation framework. In our view, the duty cycle evaluation considers UL transmissions by a UE (SBFD aware or not) independent if such UL transmissions were based on UL symbols, flexible symbols assigned to UL by the network, or UL subband of an SBFD symbol.

Observation 2: Duty cycle evaluation considers UL transmissions by a (SBFD-aware or non SBFD-aware) UE independent if such UL transmissions were based on UL symbols, flexible symbols assigned to UL by the network, or UL subband of an SBFD symbol.

Proposal 2: No additional considerations for SBFD-aware UE is expected to the HPUE duty cycle evaluation given in TS 38.101-1.

# Conclusion

In this contribution, we provided our views on general aspects for SBFD operation with the following observations and proposals:

Proposal 1: RAN4 to consider the X = 38 dBm as MR SBFD-capable power threshold value (X dBm) as a compromise between MR maximum BS rated carrier TRP output power limit given in Table 9.3.1-1, TS 38.104 (i.e., 43 dBm) and representative values adopted in Section 9.3, TR 38.858.

Observation 1: The HPUE duty cycle evaluation is based on UL transmissions carried by the UE and not only on the symbol assignment by the network.

Observation 2: Duty cycle evaluation considers UL transmissions by a (SBFD-aware or non SBFD-aware) UE independent if such UL transmissions were based on UL symbols, flexible symbols assigned to UL by the network, or UL subband of an SBFD symbol.

Proposal 2: No additional considerations for SBFD-aware UE is expected to the HPUE duty cycle evaluation given in TS 38.101-1.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2508707, “Way Forward for [115][306] NR_duplex_evo_General”, RAN4#115, Samsung.