# 3GPP TSG-RAN WG4 Meeting #116	R4-2510429

# Bengaluru, India, 25th – 29th August, 2025

Title: 	Coexistence study for introduction of US 4.9GHz band

Agenda item:	6.14.2.1

Source:	Huawei, HiSilicon

Document for:	Discussion

# Introduction

The new WI was approved in the RAN 108th meeting to introduce a new US TDD band[1]. On the first quarter of the WI, a study phase should be conducted to ensure the coexistence requirements based on the FCC regulations, as detailed in the WID and reproduced below:

- Study coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz band. - Identify adjacent spectrum protection requirements - Identify any potential emission limits more stringent than the normal BS and UE emission masks - Identify other technologies operating in the 4940-4990 MHz band - Identify potential coexistence mechanism(s) - Frequency division sharing - Geographic separation - TDD synchronization - Identify any potential emission limits more stringent than the normal BS and UE emission masks

In this work the output power and Spectrum Emission Mask (SEM) requirements of the FCC are compared to the ones of 3GPP to analyse the coexistence.

# Discussion

In FCC document CFR Title 47 Part 90.210(m) and Part 90.1215, the SEM and maximum conducted output power are detailed. It is important to ensure that 3GPP requirements need to meet those values.

## SEM

The SEM in CFR Title 47 Part 90.210(m) is specified as below:

Emission Mask M. For high power transmitters (greater that 20 dBm) operating in the 4940-4990 MHz frequency band, the power spectral density of the emissions must be attenuated below the output power of the transmitter as follows: (1) On any frequency removed from the assigned frequency between 0-45% of the authorized bandwidth (BW): 0 dB. (2) On any frequency removed from the assigned frequency between 45-50% of the authorized bandwidth: 568 log (% of (BW)/45) dB. (3) On any frequency removed from the assigned frequency between 50-55% of the authorized bandwidth: 26 + 145 log (% of BW/50) dB. (4) On any frequency removed from the assigned frequency between 55-100% of the authorized bandwidth: 32 + 31 log (% of (BW)/55) dB. (5) On any frequency removed from the assigned frequency between 100-150% of the authorized bandwidth: 40 + 57 log (% of (BW)/100) dB. (6) On any frequency removed from the assigned frequency between above 150% of the authorized bandwidth: 50 dB or 55 + 10 log (P) dB, whichever is the lesser attenuation. (7) The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission and a video bandwidth of 30 kHz. The power spectral density is the power measured within the resolution bandwidth of the measurement device divided by the resolution bandwidth of the measurement device. Emission levels are also based on the use of measurement instrumentation employing a resolution bandwidth of at least one percent of the occupied bandwidth.

The SEM limits of 3GPP spec are specified in TS 38.101-1 6.5.2.2, as reproduced below:

Table 6.5.2.2-1: General NR spectrum emission mask ΔfOOB (MHz); Channel bandwidth (MHz) / Spectrum emission limit (dBm); Measurement bandwidth. 3; 5; 10, 15, 20, 25, 30, 35, 40, 45; 50, 60, 70, 80, 90, 100. ± 0-1; -13; -13; -13; 1 % of channel BW. ± 0-1; -24; 30 kHz. ± 1-5; -10; -10; -10; 1 MHz. ± 5-6; -25; -13. ± 6-10; -25. ± 5-BWChannel; -13. ± BWChannel-(BWChannel+5); -25.

As the measurement bandwidths and ΔfOOB are different for FCC and 3GPP, a comparison is not straightforward, hence in Figure 1, the SEM of a 20MHz CBW for a PC3 is plotted for FCC and 3GPP requirements. It can be observed that the FCC is significantly more stringent and a regular SEM requirement would not be enough, therefore a new network signalling is needed to ensure the FCC SEM requirements.

Figure 1 SEM evaluation: (left) 3GPP requirements, (right) FCC requirements

Proposal 1: The FCC SEM requirements should be captured as a new network signalling (NS_xx), as the general 3GPP SEM requirements would not be enough.

## Output power

The maximum conducted output power in CFR Title 47 Part 90.1215 is specified as below. On the other hand, it is mentioned that the maximum PSD is set to 21dBm/MHz and If transmitting antennas of directional gain greater than 9 dBi are used, both the maximum conducted output power and the maximum PSD should be reduced by the amount in decibels that the directional gain of the antenna exceeds 9 dBi.

From 3GPP perspective the conducted output power is CBW independent and set to 23dBm, 26dBm, 29dBm and 31 dBm for PC3, PC2, PC1.5 and PC1, respectively. looking at the CFR Title 47 Part 90.1215 Table 1, it can be observed that a PC3 UE can operate with all NR CBWs. However other power classes might not be able to transmit using some CBWs and respect the PSD requirement of FCC. In Table 1 the list of the CBWs that respect the conducted output power limits imposed by FCC is given. For PC2, only 3MHz CBW does not respect the limitations, however 3MHz is specified for bands below 1435 MHz, therefore 3MHz won’t be implemented in for the band of this WI (4940-4990 MHz). For PC1.5 and PC1, only the CBWs greater or equal to 10 and 20 MHz, respectively, can pass the FCC requirements.

CFR Title 47 Part 90.1215 Table 1 to Paragraph (a)(1). Channel bandwidth (MHz); Low power maximum conducted output power (dBm); High power maximum conducted output power (dBm). 1; 7; 20. 5; 14; 27. 10; 17; 30. 15; 18.8; 31.8. 20; 20; 33. 30; 21.8; 34.8. 40; 23; 36. 50; 24; 37.

Table 1 3GPP CBWs per power class that passes the FCC requirements in CFR Title 47 Part 90.1215 Table 1

| PC3 | All CBW |
| --- | --- |
| PC2 | All CBW (3MHz excluded) |
| PC1.5 | CBW ≥ 10MHz |
| PC1 | CBW ≥ 20MHz |

Proposal 2: PC3 and PC2 UE should be specified for this band, as they can support FCC regulations

Proposal 3: PC1.5 and PC1 UE, only the CBWs greater or equal to 10 and 20 MHz, respectively should be specified for this band, as they can support FCC regulations

# Conclusion

Proposal 1: The FCC SEM requirements should be captured as a new network signalling (NS_xx), as the general 3GPP SEM requirements would not be enough.

Proposal 2: PC3 and PC2 UE should be specified for this band, as they can support FCC regulations

Proposal 3: PC1.5 and PC1 UE, only the CBWs greater or equal to 10 and 20 MHz, respectively should be specified for this band, as they can support FCC regulations

# Reference

[1]	« RP-251887, New WID on Introduction of NR TDD 4.9GHz Band for US Operation, Fujitsu ».