3GPP TSG-RAN WG4 Meeting #116	R4-2510436

Bangalore, India, 25th – 29rd August, 2025

Agenda item:	7.4.6

Source:	Samsung

Title:	Discussion on UE demodulation and CSI performance requirements for 7MHz CHBW

Document for:	Discussion

- Introduction

In the RAN plenary RAN#106 meeting the WID [1] for 7MHz CHBW has been approved and further revised in RAN#107 meeting [2]. This is our second and the last meeting on demodulation and CSI reporting performance requirements for 7MHz CHBW WI. The objectives on performance part are as below.

Specify necessary UE/BS performance requirements for NR operation in dedicated FDD FR1 spectrum allocations for irregular channel bandwidths, corresponding to the core requirements: - Specify necessary RRM performance requirements if needed (RAN4) - Specify, if necessary, UE demodulation performance and CSI reporting requirements for the 7 MHz channel bandwidth (RAN4) - Specify, if necessary, BS demodulation performance requirements (RAN4) - Specify necessary BS conformance tests (RAN4)

In this contribution, we provide our analysis and views on the open issues of UE demodulation and CSI reporting requirements for the 7 MHz channel bandwidth.

- UE demodulation performance requirements

How to define requirements

In last RAN4#115 meeting, agreements were achieved to define PDSCH CA requirements for 7MHz channel bandwidth [3]. But about how to define requirements, either based on simulation results or reuse existing 5MHz/10MHz requirements, need further discussion.

Whether to define PDSCH CA requirments for 7MHz bandwidth Agreements: - Define normal PDSCH type A CA requirements for 7MHz - Not to consider HST-SFN and HST-DPS scenario. - Interested companies are welcome to bring results for 7MHz. - FFS how to define requirements: - Option 1: Define new requirements for 7MHz CC based on the simulations - Option 2: Reuse existing 5MHz/10MHz requirements

Currently, the requirements about FR1 FDD 5MHz/10MHz CA cases could be summarized as below

Table 5.2A.2.1-1: Single carrier performance for FDD 15 kHz SCS for CA configurations Bandwidth (MHz); Reference channel; Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 5; R.PDSCH.1-9.1 FDD; 16QAM, 0.48; TDLA30-10; 2x2, ULA Low; 70; 13.6. 10; R.PDSCH.1-2.2 FDD; 16QAM, 0.48; TDLA30-10; 2x2, ULA Low; 70; 13.6. … Table 5.2A.3.1-1: Single carrier performance for FDD 15 kHz SCS for CA configurations Bandwidth (MHz); Reference channel; Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 5; R.PDSCH.1-9.1 FDD; 16QAM, 0.48; TDLA30-10; 2x4, ULA Low; 70; 8.5. 10; R.PDSCH.1-2.2 FDD; 16QAM, 0.48; TDLA30-10; 2x4, ULA Low; 70; 8.5. …

Basically, we think both ways (based on simulation results, or reuse existing 5MHz/10MHz requirements) should be workable, the precondition is new simulation results don’t have big difference (e.g., more than 0.2dB) compared with existing 5MHz/10MHz requirements which probably comes from different number of contributors. It’s no doubt that RAN4 specification is contribution driven, but we should also promise our test requirements looks reasonable to spec readers, e.g., NR developer and verification engineers. For this case, the expected requirement for 7MHz should be close to that of 5MHz or 10MHz.

Proposal 1: Both ways (based on simulation results, or reuse existing 5MHz/10MHz requirements) should be workable, the precondition is new simulation results don’t have big difference (e.g., more than 0.2dB) compared with existing 5MHz/10MHz requirements.

Release independence for 7MHz bandwidth requirements

According RAN2’s reply LS [4] and the definition of release independent in 38.307, key information copied as below.

… We think it would be fine to add support for 7 MHz as a possible maximum or minimum supported bandwidth in a release later than Rel-15, as 35 and 45 MHz were added in SupportedBandwidth-v1700, while the bandwidths themselves are release independent from Rel-15. …

Release independent for NR channel bandwidths in 38.307

Table 5.1-3: NR channel bandwidths Feature; Duplex-mode; Release independent from; Requirements to be fulfilled (see TS 38.307 of the release in which the channel bandwidth was introduced). NR channel bandwidths, all unless otherwise stated; FDD, TDD, SDL, SUL; Rel-15; Table B.4.1-1, Table B.4.3-1. Channel bandwidth of 3 MHz; FDD; Rel-18; Table B.4.1-1.

It is reasonable to follow the same release independent rule for 7MHz demod requirement from Rel-15.

Proposal 2: RAN4 7MHz demod requirements follow the same release independent rule for 7MHz from Rel-15.

- Conclusion

In this contribution, we provide analysis and views on UE demodulation performance and CSI requirements for 7MHz CHBW WI.

The proposals and observations could be summarized as:

Proposal 1: Both ways (based on simulation results, or reuse existing 5MHz/10MHz requirements) should be workable, the precondition is new simulation results don’t have big difference (e.g., more than 0.2dB) compared with existing 5MHz/10MHz requirements.

Proposal 2: RAN4 7MHz demod requirements follow the same release independent rule for 7MHz from Rel-15.

- Reference

[1] RP-243323, New WID on 7 MHz Channel Bandwidth for n26 and n5, RAN#106 meeting, China Telecom, T-Mobile USA

[2] RP-250373, Revised WID on NR FR1 7 MHz Channel Bandwidth, RAN#107 meeting, T-Mobile USA, China Telecom

[3] R4-2508757, WF on demodulation and CSI requirements for 7MHz, RAN4#115 meeting, Huawei, HiSilicon

[4] R4-2505319_R2-2503201, Reply LS on Signalling for 7MHz Channel Bandwidth, RAN4#115 meeting, RAN2