3GPP TSG RAN WG4 Meeting #116	R4-2510488

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.19.3.2

Source:	vivo

Title:	Discussion on RAN4 impacts for measurement event prediction

Document for:	Discussion

- Introduction

In the previous meetings, the discussion on potential RRM impacts for measurement event prediction was discussed. The considerations from companies and the latest progress on RAN4 impacts for measurement event prediction were summarized in WF [1]. In this contribution, we would like to provide our further analysis on aspects on RRM requirements impacts for measurement event prediction.

- Discussion

Issue 3-1-1: Prioritization of cases for event prediction Agreement: - RAN4 should discuss both indirect and direct cases. If any prioritization will be agreed in RAN2, RAN4 to follow RAN2 prioritization Issue 3-1-2: Performance metric for event prediction use case Agreement: - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for the predicted event triggered reporting - RAN4 to study the requirement for [delay and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define requirement for the predicted RSRPs

In RAN4#114bis meeting, it concluded with the above agreements regarding measurement event prediction. For the potential RRM requirement impact for event prediction for indirect case, it includes two aspects: one is the "delay requirement for predicted event triggered reporting," and the other is the "metric for event prediction."

Regarding the "delay requirement for predicted event triggered reporting," let’s first analyze the existing requirement for Event Triggered Reporting. The measurement reporting delay is defined from the moment an event that will trigger a measurement report actually occurs until the UE transmits the measurement report. In legacy, the time when the event condition is met must always precede the UE’s transmission of the measurement report, ensuring that the UE can confirm the event condition is satisfied during measurement.

However, for measurement event prediction, the process is as follows: The UE inputs past/current measurement results into a model. → Based on measurements and/or prediction results, it determines whether an event will occur at a future time. → If the UE predicts that a measurement event will occur at a future time, it reports the predicted RSRP of serving/neighboring cells as an intermediate output, and the expected occurrence time of a certain measurement event as the final output.

Thus, the expected occurrence time of the measurement event corresponds to a future moment, which will necessarily be later than the UE’s transmission of the prediction report. In this case, the delay can not follow the existing logic of the "measurement reporting delay" definition. Given this issue, we tend not to define the related delay requirement.

Observation 1: In legacy, the time when the event condition is met must always precede the UE’s transmission of the measurement report, ensuring that the UE can confirm the event condition is satisfied during measurement. While for the measurement event prediction, the expected occurrence time of the measurement event corresponds to a future moment. It will be later than the UE’s transmission of the prediction report

Proposal 1: RAN4 not to study the delay requirement for the predicted event triggered reporting

Issue 3-1: Performance metric for event prediction use case Agreement: In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP.

For the metric for indirect measurement event prediction, the following conclusions were reached in the previous meeting. Absolute and/or relative accuracy of predicted RSRP will be adopted as one of the metrics, and RAN4 will define its requirements.

#### 9.2.4.3 Event Triggered Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled. The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

On the other hand, regarding whether and how to verify the accuracy of final put of measurement event prediction, according to the description of the following captures in TR38.744, the F1 score serves as the performance metric for evaluating the accuracy of measurement event prediction. For indirect prediction, the counters n1, n2, and n3 in the calculation formula are related to the length of the maximum ETD. When the ETD (i.e., the timing difference between the ground-truth event and the predicted event) is larger, the actual time of the event occurrence is more likely to fall within the time range of [the expected occurrence time of the event ± ETD], resulting in a better displayed F1 score. to verify the accuracy of event prediction, the F1 score under certain fixed ETD conditions can be candidate metric for further discussion.

TR 38.744 ### Evaluation methodology, metrics and assumptions The performance metric F1 score is defined as following: F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3) For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following: Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD] The ETD i.e. timing difference between ground-truth event and predicted event is illustrated in Figure 5.3.1-1: Figure 5.3.1-1: illustration of ETD As illustrated in Figure 5.3.1-1, only if the ETD between a predicted event and a ground-truth event e.g. ground-truth event 2 is less than or equal to maximum ETD, the ETD can still be tolerated. Otherwise, both false event and missed event are detected.

Proposal 2: F1 score under certain fixed ETD conditions can be served as the candidate performance metric for evaluating the accuracy of measurement event prediction.

For direct measurement event prediction (which differs slightly from indirect prediction), a measurement event could be predicted within an occurrence window spanning from the current time instance (t₀) to a future time instance (t₁) with a probability, rather than specifying the exact timing of the event. Correspondingly, we propose that the F1-score under certain fixed occurrence window lengths and probability conditions could serve as a candidate performance metric to evaluate the accuracy of measurement event prediction.

For direct prediction, the counter n1, n2 and n3 in the formula are defined as following: Counter n3 (true event prediction): it increases by 1 when a ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n1 (false event detection): it increases by 1 when no ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n2 (missed event detection): it increases by 1 when a ground-truth event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold For direct prediction method, a measurement event could be predicted within an occurrence window starting from current time instance i.e. t0 and future time instance t1 with a probability as illustrated in Figure 5.3.1-1. Figure 5.3.1-2: occurrence window of direct prediction method For measurement event prediction based on intra-frequency temporal domain case A or case B, the simulation assumptions for intra-frequency temporal domain case A or case B in Table 5.2.1-1, Table 5.2.1-2, Table 5.2.1-3 and Table 5.2.1-4 are reused respectively. On top of that, following additional simulation assumptions are used for measurement event prediction based on intra-frequency temporal domain case A in Table 5.3.1-1 and temporal domain case B in Table 5.3.1-2 respectively:

Proposal 3: F1 score under certain fixed occurrence window length with fixed probability conditions can be served as a candidate performance metric for evaluating the accuracy of measurement event prediction.

- Summary

Observation 1: In legacy, the time when the event condition is met must always precede the UE’s transmission of the measurement report, ensuring that the UE can confirm the event condition is satisfied during measurement. While for the measurement event prediction, the expected occurrence time of the measurement event corresponds to a future moment. It will be later than the UE’s transmission of the prediction report

Proposal 1: RAN4 not to study the delay requirement for the predicted event triggered reporting

Proposal 2: F1 score under certain fixed ETD conditions can be served as the candidate performance metric for evaluating the accuracy of measurement event prediction.

Proposal 3: F1 score under certain fixed occurrence window length with fixed probability conditions can be served as a candidate performance metric for evaluating the accuracy of measurement event prediction.

- References

- R4-2508268, WF on FS_NR_AIML_Mob_Part1, Nokia