# 3GPP TSG-RAN WG4 Meeting #116	R4-2510521
Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.6.3.2.1

Source: 	OPPO

Title: 	Discussion on Tx requirements for NR-NTN HPUE

Document for:	Discussion

- Introduction

In previous RAN4 meeting, the NR NTN HPUE of PC1, PC1.5 and PC2 operating in FR1-NTN bands were discussed and the most TX requirements for PC1, PC1.5 and PC2 have been agreed, but some remaining issues need further confirm in last R19 meeting.

This contribution further discussed the UE Tx RF requirements for NR NTN HPUE, including configured output power and AMPR for HPUE.

- Discussion

2.1 Configured transmitted power

In last meeting, the parameters of the configured transmitted power were discussed and some parameters need further confirm as follow issue:

Issue 1-1: Configured Tx power equation

Agreement:

- Agree to clarify and correct the configured transmitted power equation for NTN

  - [∆TC,c], ∆TIB,c , [∆TRxSRS] are not applicable to Rel-19 NR-NTN UE

  - FFS ΔPPowerClass

  - For ΔPPowerBoost, Rel-18 power boosting should be applicable to NR-NTN.

- Regarding how to address these in CR drafting, following approach is agreed.

  - By referring to 38.101-1 and then exclude non-applicable parameters, e.g. ‘except for...’

∆TC,c is only applied to those TN bands labelled by NOTE 3 in Table 6.2.1-1 in 38.101-1, it is due to these bands has a larger relative bandwidth caused the band filter has a larger insertion loss at the edge of the filter. Currently, FR1-NTN bands in spec don’t have this request, so ∆TC,c shouldn’t be applicable to NR-NTN UE not only for HPUE PC1/PC1.5/PC2 but also for PC3.

SRS signal is used for BS to do UL channel estimation. Usually this SRS signal is transmitted on Tx antenna which leads to BS cannot estimate the pure Rx antenna channel statusnnot estimate e the Rx antenna c on Tx antenna. ed in the market.ry and how to solve the concerns from  and MIMO performance in these antennas will be degraded. SRS switch function is designed to solve this issue by switching SRS transmission among Tx and Rx antennas. However, additional ILs will be introduced. But NR-NTN UE doesn’t support MIMO, it’s unnecessary to support SRS Tx switch, therefore, ∆TRxSRS shouldn’t be applicable to NR-NTN UE not only for HPUE PC1/PC1.5/PC2 but also for PC3.

ΔPPowerClass has two purposes in current TN Spec, one is to clarify the power backoff when the indicated P-max is lower than the power class the UE reported; another one to resolve SAR issues based on the duty circle method.

Proposal 1: ∆TC,c and ∆TRxSRS shouldn’t be applicable to NR-NTN UE, including HPUE PC1/PC1.5/PC2 and PC3.

Proposal 2: ΔPPowerClass is only applicable to clarify the power backoff when the indicated P-max is lower than the power class the UE reported.

2.2 AMPR for n256 NS_24

Currently, the carrier centre frequency ranges for NS_24 is defined for band n65, from below table we can see n256 UL lower frequency is 1980MHz and higher than 1920MHz of n65

| n65 | 1920 MHz – 2010 MHz | 2110 MHz – 2200 MHz | FDD4 |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |

Therefore, the carrier centre frequency ranges for NS_24 applied to band n256 should belong to the frequency ranges of 1980-2010MHz.

Proposal 3: The AMPR of NS_24 for NTN HPUE should be only applied to NTN band n256 and the carrier centre frequency ranges for different channel bandwidth are given based on n256 as follow table 2.2-1:

Table 2.2-1 The carrier centre frequency ranges for different channel bandwidth for NS_24 with PC2/PC1.5/PC1

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz |
| --- | --- |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |
| 10MHz | 1985 < Fc ≤ 1995 |
| 10MHz | 1995 < Fc ≤ 2000 |
| 15MHz | 1987.5 < Fc ≤ 1997.5 |
| 20MHz | 1990 < Fc ≤ 1995 |

Based on above carrier centre frequency range, current PC3 NS_24 AMPR have three region frames as follow figures  2.2-1 to 2.2-3：

Figure 2.2-1 Region frame for 5MHz of 1997.5 < Fc ≤ 2002.5 and 10MHz of 1985 < Fc ≤ 1995

Figure 2.2-2 Region frame for 5MHz of 1992.5 < Fc ≤ 1997.5 and 1987.5 < Fc ≤ 1992.5

Figure 2.2-3 Region frame for 10 MHz of 1995 < Fc ≤ 2000, 15MHz and 20MHz

In our companion contribution [2], we compared PC2 CP-OFDM QPSK AMPR simulation results with PC3 CP-OFDM QPSK AMPR simulation results from our simulation platform based on the divided regions for NS_24 of PC3 in current Spec as shown in figures 2.2-1 to 2.2-3, and observed:

Observation 1: For NS_24,

- The PC3 AMPR simulations results from our side have more margins compared the current region frame and related AMPR values in Spec.

- Some regions for PC2 are extended compared with PC3 due to higher output power, i.e., A5 region for 10MHz, 15MHz and 20 MHz, A1 region for 5MHz and 10MHz, A4 region for 5MHz.

- The AMPR value of A6 region in figure 2.2-1 is smaller 5dB than the value of A6 region in figure 2.2-3.

Therefore, we proposed:

Proposal 4: PC2/PC1.5/PC1 AMPR regions for NS_24 can reuse the same region frame with PC3 in Spec.

Proposal 5: The detail region ranges should be extended for PC2/PC1.5/PC1 based on the one of PC3, i.e., A5 region for 10MHz, 15MHz and 20 MHz, A1 region for 5MHz and 10MHz, A4 region for 5MHz.

Proposal 6: Define a new region name for A6 region in figure 2.2-3 for 10MHz, 15MHz, and 20MHz to distinguish with the A6 region in figure 2.2-1 for 5MHz and 10MHz, i.e., A8 replace A6:

Figure 2.2-4 Using A8 replace of A6 for 10 MHz of 1995 < Fc ≤ 2000, 15MHz and 20MHz

Here we further compared the NS_24 AMPR simulation results of PC2 10MHz (Fc=2000MHz) under different modulations with CP-OFDM and DFT-s-OFDM waveforms as given in below figures 2.2-5 and 2.2-6, the simulation results are based on fixed bias PA:

Figure 2.2-5 AMPR for PC2 10MHz CP-OFDM under different modulations

Figure 2.2-6 AMPR for PC2 10MHz DFT-s-OFDM under different modulations

Observation 2: For NS_24,

- For CP-OFDM, the PC2 AMPR region frame and the maximum AMPR values under the different modulations are almost the same.

- For DFT-s-OFDM, the PC2 AMPR region frame and the maximum AMPR values under the different modulations are almost the same.

We further compared the NS_24 AMPR simulation results for 5MHz (Fc=2002.5) and 10MHz (Fc=2000MHz) under PC2(1T), PC1.5 (2T) and PC1(4T) with CP-OFDM QPSK as shown in figures 2.2-7, the simulation results are based on fixed bias PA:

Figure 2.2-7 AMPR for 5MHz CP-OFDM QPSK under different power classes

Figure 2.2-8 AMPR for 10MHz CP-OFDM QPSK under different power classes

In the AMPR simulation for PC2 with 1T, PC1.5with 2Tx and PC1 with 4T, we use the same PC2 PA with fixed bias for these three power classes. From figure 2.2-7 and 2.2-8, we can see:

- The PC2 and PC1.5 have the similar AMPR region frame and values due to the maximum output power of each PA for PC2 and PC1.5 is 26dBm;

- The AMPR region frame for PC1 has more margin than PC2 and PC1, since the maximum output power of each PA for PC1 is 25dBm.

Observation 3: The NS_24 AMPR regions for PC2, PC1.5 and PC1 are similar, they can use the same region frequency ranges.

Based on the analysis in the companion contribution [1], we proposed to extend the regions of A1, A4 and A5 and reduce the regions of A2, A3 and A8 correspondingly based on the region frame of PC3, as shown in follow table:

Table 2.2-2 The AMPR regions for NS_24 under PC2/PC1.5/PC1

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | > 2.16 | A4 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | > 1.44 | A1 | > 2.7 | >1.08 ≤ 1.44 | A2 | ≤2.7 | ≤ 1.44 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | > 3.24 | A1 | ≥ 5.4 | > 2.16 ≤ 3.24 | A2 | < 5.4 | ≤ 3.24 | A3 |
|  |  |  |  |  |  | ≤ 2.16 | A6 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥ 5.4 |  | A5 | < 3.42 |  | A5 | ≥ 3.42 < 5.4 | >1.08 | A8 |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | < 5.94 |  | A5 | ≥ 5.94 <8.64 | >1.44 | A8 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <7.74 |  | A5 | ≥7.74 <11.52 | >1.44 | A8 |
| NOTE: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Proposal 7: Adopt the AMPR regions for NS_24 under PC2 as given in follow table:

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | > 2.16 | A4 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | > 1.44 | A1 | > 2.7 | >1.08 ≤ 1.44 | A2 | ≤2.7 | ≤ 1.44 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | > 3.24 | A1 | ≥ 5.4 | > 2.16 ≤ 3.24 | A2 | < 5.4 | ≤ 3.24 | A3 |
|  |  |  |  |  |  | ≤ 2.16 | A6 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥ 5.4 |  | A5 | < 3.42 |  | A5 | ≥ 3.42 <5.4 | >1.08 | A8 |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | < 5.94 |  | A5 | ≥ 5.94 <8.64 | >1.44 | A8 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <7.74 |  | A5 | ≥7.74 <11.52 | >1.44 | A8 |
| NOTE: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

We also simulate the AMPR values for NS_24 using APT PA model, here we compared the difference for different power classes under 5MHz CP-OFDM QPSK as shown in follow figures:

Figure 2.2-9 AMPR for PC2 5MHz CP-OFDM QPSK under different PA model

Figure 2.2-10 AMPR for PC1.5 5MHz CP-OFDM QPSK under different PA model

Figure 2.2-11 AMPR for PC1 5MHz CP-OFDM QPSK under different PA model

From figure 2.2-11, we can find there is no AMPR values for some RB allocations using APT PA model, especially for larger LCRB allocations. It’s due to larger power backoff values cause the input power of PA is lower than the minimum input power of the PA model in this power range.

In additional, we choose a fixed bias PA model with higher linearity and higher dynamic range, therefore, the AMPR values using fixed bias PA model are smaller the AMPR values using APT PA model.

Observation 4: For NS_24,

- The AMPR difference for PC2 between fixed bias PA model and APT PA model is 5 dB.

- The AMPR difference for PC1.5 between fixed bias PA model and APT PA model is 8 dB.

- The AMPR difference for PC1 between fixed bias PA model and APT PA model is larger than 10 dB.

Proposal 8: PC2 AMPR for NS_24 can be

| Modulation/ Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A8 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner |  | Outer |
| CP-OFDM QPSK /16QAM/64QAM | ≤ 16 | ≤ 10 | ≤ 10 | ≤ 9.5 | ≤ 26 | ≤ 12 | ≤ 17 | ≤ 8.5 |
| DFT-s-OFDM pi/2 BPSK /QPSK/16QAM/64QAM | ≤ 15 | ≤ 9 | ≤ 9 | ≤ 8.5 | ≤ 26 | ≤ 11 | ≤ 16 | ≤ 7.5 |

2.3 AMPR for n256 NS_100

Currently, there are no UTRA ACLR requirements for NR TN and NR NTN PC1 and PC1.5. Therefore, it’s proposed to not define the NS_100 AMPR for NR NTN PC1 and PC1.5.

NR NTN PC2 UE can reuse AMPR values for NR TN PC2 UE in table 6.2.3.1-2 of 38.101-1.

Proposal 9: For NS_100,

- For NR NTN PC1 and PC1.5, don’t define the AMPR.

- For NR NTN PC2 UE, reuse AMPR values of NR TN PC2 UE in table 6.2.3.1-2 of 38.101-1.

2.4 AMPR for n255 NS_02N

There is no AMPR for NS_02N for PC3, in the discussion, some companies think it is considered 10dB filter rejection for PC3 AMPR. Therefore, in last meeting, it was agreed the NS_02N AMPR for PC2/PC1.5/PC1 can also use 10dB filter rejection as following agreement.

Issue 1-3: AMPR assumption for n255

Agreement:

- Agree to use 10dB as filter rejection assumption for GNSS frequencies 1559-1610 MHz for PC2/PC1.5/PC1 AMPR analysis for n255.

And we reviewed TR 38.863 about PC3 NS_02N for band n255, it said the the expected filter attenuation is more than 10 dB below 1610 MHz, therefore, no A-MPR needs to be specified for n255.

In fact, the filter rejection at the frequencies 1559-1610 MHz is not a constant 10 dB attenuation but rather it has a slope. It’s hard to add the filter rejection without the slope factor, we didn’t introduce the filter rejection in our AMPR simulation platform at first.

In order to evaluate the rejection effect of the filter, we simulate the AMPR for PC3 CP-OFDM QPSK using fixed bias PA and APT PA, separately.

For PC3 CP-OFDM QPSK using fixed bias PA:

- The AMPR-MPR is 0dB for 5MHz, 10MHz and 15MHz without additional filter rejection.

- The AMPR-MPR is 3dB for 20MHz without additional filter rejection.

For PC3 CP-OFDM QPSK using APT PA:

- The AMPR-MPR is 0dB for 5MHz and 10MHz without additional filter rejection.

- The AMPR-MPR is 2dB for 15MHz, and 10.5dB for 20MHz without additional filter rejection.

From PC3 AMPR based on our simulation no matter which PA model are used, it seems the filter attenuation is needed for NS_02N for PC3.

To check the effect of filter rejection, we simulate the AMPR for PC2 CP-OFDM QPSK using APT PA w/o filter rejection and with a constant 10 dB filter attenuation at the frequencies 1559-1610 MHz, separately.

Figure 2.4-1 AMPR change for 10MHz based on a constant 10 dB filter rejection using APT PA

Figure 2.4-2 AMPR change for 10MHz based on a constant 10 dB filter rejection using APT PA

Figure 2.4-3 AMPR change for 10MHz based on a constant 10 dB filter rejection using APT PA

PC2 AMPR without filter rejection are shown in Table 2.4-1:

Table 2.4-1 AMPR for PC2 CP-OFDM QPSK using APT PA without filter rejection

|  | Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | RBstart*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| --- | --- | --- | --- | --- | --- |
| CP-OFDM QPSK | 10MHz | 1631.5 < Fc ≤ 1641.5 |  | >5.4 | 5 |
|  | 15MHz | 1634 < Fc ≤ 1649 |  | >5.4 | 6 |
|  | 20MHz | 1636.5 < Fc ≤ 1650.5 |  | >9 | 6 |
|  |  |  |  | ≥ 6.12 <9 | 4.5 |
|  |  |  | <1.44 | <6.12 | 7.5 |
|  |  |  | >17.64 | <0.72 | 4.5 |

PC2 AMPR with 10dB filter rejection are shown in Table 2.4-2:

Table 2.4-1 AMPR for PC2 CP-OFDM QPSK using APT PA without filter rejection

|  | Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | RBstart*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| --- | --- | --- | --- | --- | --- |
| CP-OFDM QPSK | 15MHz | 1634 < Fc ≤ 1649 |  | >10.8 | 4 |
|  | 20MHz | 1636.5 < Fc ≤ 1650.5 |  | >10.8 | 5.5 |
|  |  |  | <0.54 | <1.8 | 6.5 |

Here we further compared different modulations with CP-OFDM and DFT-s-OFDM waveforms for NS_02N based on 15MHz, as shown in Figure 2.4-3:

Observation 5: For NS_02N,

- For CP-OFDM, the PC2 AMPR region frame under the different modulations are almost the same, and the maximum AMPR values only have 0.5 dB difference.

- For DFT-s-OFDM, the PC2 AMPR region frame and the maximum AMPR values under the different modulations are almost the same except pi/2BPSK.

Proposal 10: Considering 10dB filter rejection, PC2 AMPR for NS_02N can be:

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | RBstart*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| --- | --- | --- | --- | --- |
| 15MHz | 1634 < Fc ≤ 1649 |  | >10.8 | A1 |
| 20MHz | 1636.5 < Fc ≤ 1650.5 |  | >10.8 | A2 |
|  |  | <0.54 | <1.8 |  |

|  |  | Modulation | A1 | A2 |
| --- | --- | --- | --- | --- |
| PC2 | DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 3 |
|  |  | QPSK/16QAM/64QAM | ≤ 3.5 | ≤ 6 |
|  | CP-OFDM | QPSK/16QAM/64QAM | ≤ 4 | ≤ 6.5 |

- Conclusion

In this paper we give further discussion on the remaining issues for NR-NTN HPUE. And we observed that:

Observation 1: For NS_24,

- The PC3 AMPR simulations results from our side have more margins compared the current region frame and related AMPR values in Spec.

- Some regions for PC2 are extended compared with PC3 due to higher output power, i.e., A5 region for 10MHz, 15MHz and 20 MHz, A1 region for 5MHz and 10MHz, A4 region for 5MHz.

- The AMPR value of A6 region in figure 2.2-1 is smaller 5dB than the value of A6 region in figure 2.2-3.

Observation 2: For NS_24,

- For CP-OFDM, the PC2 AMPR region frame and the maximum AMPR values under the different modulations are almost the same.

- For DFT-s-OFDM, the PC2 AMPR region frame and the maximum AMPR values under the different modulations are almost the same.

Observation 3: The NS_24 AMPR regions for PC2, PC1.5 and PC1 are similar and can use the same region frequency ranges.

Observation 4: For NS_24,

- The AMPR difference for PC2 between fixed bias PA model and APT PA model is 5 dB.

- The AMPR difference for PC1.5 between fixed bias PA model and APT PA model is 8 dB.

- The AMPR difference for PC1 between fixed bias PA model and APT PA model is larger than 10 dB.

Observation 5: For NS_02N,

- For CP-OFDM, the PC2 AMPR region frame under the different modulations are almost the same, and the maximum AMPR values only have 0.5 dB difference.

- For DFT-s-OFDM, the PC2 AMPR region frame and the maximum AMPR values under the different modulations are almost the same except pi/2BPSK.

And proposed:

Proposal 1: ∆TC,c and ∆TRxSRS shouldn’t be applicable to NR-NTN UE, including HPUE PC1/PC1.5/PC2 and PC3.

Proposal 2: ΔPPowerClass is only applicable to clarify the power backoff when the indicated P-max is lower than the power class the UE reported.

Proposal 3: The AMPR of NS_24 for NTN HPUE should be only applied to NTN band n256 and the carrier centre frequency ranges for different channel bandwidth are given based on n256 as follow table 2.2-1:

Table 2.2-1 The carrier centre frequency ranges for different channel bandwidth for NS_24 with PC2/PC1.5/PC1

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz |
| --- | --- |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |
| 10MHz | 1985 < Fc ≤ 1995 |
| 10MHz | 1995 < Fc ≤ 2000 |
| 15MHz | 1987.5 < Fc ≤ 1997.5 |
| 20MHz | 1990 < Fc ≤ 1995 |

Proposal 4: PC2/PC1.5/PC1 AMPR regions for NS_24 can reuse the same region frame with PC3 in Spec.

Proposal 5: The detail region ranges should be extended for PC2/PC1.5/PC1 based on the one of PC3, i.e., A5 region for 10MHz, 15MHz and 20 MHz, A1 region for 5MHz and 10MHz, A4 region for 5MHz.

Proposal 6: Define a new region name for A6 region in figure 2.2-3 for 10MHz, 15MHz, and 20MHz to distinguish with the A6 region in figure 2.2-1 for 5MHz and 10MHz, i.e., A8 replace A6:

Figure 2.2-4 Using A8 replace of A6 for 10 MHz of 1995 < Fc ≤ 2000, 15MHz and 20MHz

Proposal 7: Adopt the AMPR regions for NS_24 under PC2 as given in follow table:

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | > 2.16 | A4 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | > 1.44 | A1 | > 2.7 | >1.08 ≤ 1.44 | A2 | ≤2.7 | ≤ 1.44 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | > 3.24 | A1 | ≥ 5.4 | > 2.16 ≤ 3.24 | A2 | < 5.4 | ≤ 3.24 | A3 |
|  |  |  |  |  |  | ≤ 2.16 | A6 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥ 5.4 |  | A5 | < 3.42 |  | A5 | ≥ 3.42 <5.4 | >1.08 | A8 |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | < 5.94 |  | A5 | ≥ 5.94 <8.64 | >1.44 | A8 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <7.74 |  | A5 | ≥7.74 <11.52 | >1.44 | A8 |
| NOTE: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Proposal 8: PC2 AMPR for NS_24 can be:

| Modulation/ Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A8 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner |  | Outer |
| CP-OFDM QPSK /16QAM/64QAM | ≤ 16 | ≤ 10 | ≤ 10 | ≤ 9.5 | ≤ 26 | ≤ 12 | ≤ 17 | ≤ 8.5 |
| DFT-s-OFDM pi/2 BPSK /QPSK/16QAM/64QAM | ≤ 15 | ≤ 9 | ≤ 9 | ≤ 8.5 | ≤ 26 | ≤ 11 | ≤ 16 | ≤ 7.5 |

Proposal 9: For NS_100,

- For NR NTN PC1 and PC1.5, don’t define the AMPR.

- For NR NTN PC2 UE, reuse AMPR values of NR TN PC2 UE in table 6.2.3.1-2 of 38.101-1.

Proposal 10: Considering 10dB filter rejection, PC2 AMPR for NS_02N can be:

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | RBstart*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| --- | --- | --- | --- | --- |
| 15MHz | 1634 < Fc ≤ 1649 |  | >10.8 | A1 |
| 20MHz | 1636.5 < Fc ≤ 1650.5 |  | >10.8 | A2 |
|  |  | <0.54 | <1.8 |  |

|  |  | Modulation | A1 | A2 |
| --- | --- | --- | --- | --- |
| PC2 | DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 3 |
|  |  | QPSK/16QAM/64QAM | ≤ 3.5 | ≤ 6 |
|  | CP-OFDM | QPSK/16QAM/64QAM | ≤ 4 | ≤ 6.5 |

- Reference

[1]R4-25010522, Analysis on NTN AMPR  for n256 NS_24/n255 NS_02N, OPPO, RAN4#116.

- Annex

NS_24 AMPR simulation results

PC3 1T

Fixed bias PA

5MHz CP-OFDM QPSK

Fc=2002.5

Fc=1997.5

Fc=1992.5

10MHz CP-OFDM QPSK

Fc=2000M

Fc=1995

15MHz CP-OFDM QPSK

Fc=1997.5

20MHz CP-OFDM QPSK

Fc=1995

PC2 1T

Fixed bias PA

5MHz CP-OFDM QPSK

Fc=2002.5

Fc=1997.5

Fc=1992.5

10MHz CP-OFDM with different modulations

Fc=2000M

Fc=1995

Fc=1990

Fc=1985

10MHz DFT-s-OFDM with different modulations

Fc=2000M

Fc=1995

15MHz CP-OFDM QPSK

Fc=1997.5

Fc=1987.5

20MHz CP-OFDM QPSK

Fc=1995

Fc=1990

APT PA

5MHz CP-OFDM QPSK

Fc=2002.5

Fc=1997.5

Fc=1992.5

10MHz CP-OFDM QPSK

Fc=2000M

Fc=1995

Fc=1990

Fc=1985

PC1.5 2T

Fixed bias PA

5MHz CP-OFDM QPSK

Fc=2002.5

Fc=1997.5

Fc=1992.5

5MHz DFT-s-OFDM QPSK

Fc=2002.5

Fc=1997.5

Fc=1992.5

10MHz CP-OFDM QPSK

Fc=2000M

Fc=1995M

Fc=1990M

Fc=1985M

10MHz DFT-s-OFDM QPSK

Fc=2000M

Fc=1995M

Fc=1990M

Fc=1985M

15MHz CP-OFDM QPSK

Fc=1997.5

Fc=1993

Fc=1987.5

15MHz DFT-s-OFDM QPSK

Fc=1997.5

Fc=1992.5

Fc=1987.5

20MHz CP-OFDM QPSK

Fc=1995

Fc=1990

20MHz DFT-s-OFDM QPSK

Fc=1995

Fc=1990

APT PA

5MHz CP-OFDM QPSK

Fc=2002.5

Fc=1997.5

Fc=1992.5

10MHz CP-OFDM

Fc=2000M

Fc=1995

Fc=1990

Fc=1985

15MHz DFT-s-OFDM

Fc=1997.5

Fc=1987.5

PC1 4T

Fixed bias PA

5MHz CP-OFDM QPSK

Fc=2002.5

Fc=1997.5

5MHz DFT-s-OFDM QPSK

Fc=2002.5

Fc=1997.5

10MHz CP-OFDM QPSK

Fc=2000M

Fc=1995M

Fc=1990M

Fc=1985M

10MHz DFT-s-OFDM QPSK

Fc=2000M

Fc=1995M

Fc=1990M

Fc=1985M

15MHz CP-OFDM QPSK

Fc=1997.5

Fc=1992.5

Fc=1987.5

15MHz DFT-s-OFDM QPSK

Fc=1997.5

Fc=1992.5

Fc=1987.5

20MHz CP-OFDM QPSK

Fc=1995

Fc=1990

20MHz DFT-s-OFDM QPSK

Fc=1995

Fc=1990

APT PA

5MHz CP-OFDM QPSK

Fc=2002.5

Fc=1997.5

Fc=1992.5

10MHz CP-OFDM QPSK

Fc=2000M

Fc=1995

Fc=1990

Fc=1985

NS_02N AMPR simulation results

PC3 1T

Fixed bias PA

10MHz CP-OFDM QPSK

Fc=1631.5

15MHz CP-OFDM QPSK

Fc=1634

20MHz CP-OFDM QPSK

Fc=1636.5

APT PA

10MHz CP-OFDM QPSK

Fc=1631.5

15MHz CP-OFDM QPSK

Fc=1634

20MHz CP-OFDM QPSK

Fc=1636.5

PC2 1T

Fixed bias PA

10MHz CP-OFDM QPSK

Fc=1631.5

15MHz CP-OFDM with different modulations

Fc=1634

15MHz DFT-s-OFDM with different modulations

20MHz CP-OFDM QPSK

Fc=1636.5

APT PA

5MHz CP-OFDM QPSK

Fc=1629

10MHz CP-OFDM QPSK

Fc=1631.5

10db filter rejection

Fc=1641.5

15MHz CP-OFDM QPSK

Fc=1634

10db filter rejection

Fc=1649.5

15MHz DFT-s-OFDM QPSK

Fc=1634

Fc=1649.5

20MHz CP-OFDM QPSK

Fc=1636.5

No filter rejection

10dB filter rejection

Fc=1640.5

Fc=1650.5

PC1.5 2T

APT PA

5MHz CP-OFDM QPSK

Fc=1629

10MHz CP-OFDM QPSK

Fc=1631.5

Fc=1641.5

15MHz CP-OFDM QPSK

Fc=1634

Fc=1649

20MHz CP-OFDM QPSK

Fc=1636.5

Fc=1640.5

Fc=1650.5

PC1 4Tx

APT PA

5MHz CP-OFDM QPSK

Fc=1629

10MHz CP-OFDM QPSK

Fc=1631.5

Fc=1641.5

15MHz CP-OFDM QPSK

Fc=1634

Fc=1649

20MHz CP-OFDM QPSK

Fc=1636.5

Fc-1640.5

Fc=1650.5