# 3GPP TSG-RAN WG4 Meeting #116	R4-2510761

# Bengaluru, IN, 25th – 29th August 2025

Agenda Item:	7.19.2

Source: 	Ericsson

Title: 	Corrections to TP on RAN4 aspects in TR 38.744

Document for:	Approval

# Introduction

Text proposal on the framework of RAN4 part in TR 38.744 to capture the RAN4 agreements was approved in RAN4#115 [1]. In this contribution, corrections to the definition of relative accuracy of predicted L3-RSRP for intra-frequency prediction and inter-frequency prediction are provided.

# Text proposal

< Start of text proposal >

#### 6.2.1.2	Potential RRM requirements

The requirement impact corresponding to Point A1 as defined in Figure 9.2.4-1 in TS 38.300 for measurement prediction is not to be studied.

Both absolute and relative accuracy of predicted L3-RSRP are considered as the candidate metrics for RRM measurement prediction use cases for intra-frequency scenario and inter-frequency scenarios.

The absolute accuracy of predicted L3-RSRP is defined as:

- Absolute accuracy of predicted L3-RSRP = reported predicted L3-RSRP – ground truth of L3-RSRP.

The relative accuracy of predicted L3-RSRP for intra-frequency prediction is defined as:

- Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported L3-RSRP of cell 2) – (ground truth of L3-RSRP of cell 1 – ground truth of L3-RSRP of cell 2),

  - cell 1 and cell 2 are on the same frequency

  - the reported L3-RSRP of cell 2 can be measured or predicted.

- Editor Note: The relative RSRP accuracy for beam level measurements may be further discussed during WI phase depending upon RAN2 progress.

The relative accuracy of predicted L3-RSRP for inter-frequency prediction is defined as:

- Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported L3-RSRP of cell 2) – (ground truth of L3-RSRP of cell 1 – ground truth of L3-RSRP of cell 2),

  - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1

  - the reported L3-RSRP of cell 2 can be measured or predicted.

Editor Note: RAN4 will further discuss whether and how to consider time difference between the two reported L3-RSRPs when applying the definition of the relative RSRP accuracy.

As a baseline, the ground truth of L3-RSRP in FR2 is defined as the reported L3-RSRP measurement result under sufficient high SNR.

The ground truth of L3-RSRP in FR1 is based on:

- Alt1: The transmitted or reception power

- Alt2: The reported measurement value under certain conditions.

For intra-frequency temporal domain prediction, the impact on prediction accuracy by UE speed, MRRT, and OW/PW length will be considered. The number of measurements performed in OW could be also considered.

For inter-frequency prediction scenario, the impact on prediction accuracy by following elements can be considered:

- Side condition of frequency prediction (e.g., EPRE difference)

- Cluster approach, e.g.,

  - when measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

  - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

- RAN4 may further discuss whether and how correlation coefficient can be considered in simulation assumption for RAN4 requirements.

For both intra-frequency and inter-frequency prediction, RAN4 thinks RF and baseband errors on the measurements would impact the prediction accuracy.

Editor Note: RAN4 may continue discussion on the measurement error model in RAN4#116. Any further agreements will be further captured in this clause

<End of text proposal>

# References

- R4-2508419, TP on RAN4 aspects for TR 38.744, OPPO, Nokia.