3GPP TSG-RAN WG4 Meeting #116	R4-2510884

# Bengaluru, India, August 25th – 29th, 2025

Title: 	On demodulation and CSI requirements for R19 MIMO evolution

Source: 	Huawei, HiSilicon

Agenda item:	7.20.6

Document for:	Discussion

Background

In last meeting, a WF for MIMO demod was agreed, where many open issues were remained for discussion. This contribution provides our views on these open issues.

- UE performance requirements

Single panel I codebook

Test parameters for PMI requirements with 64 ports

CSI resources with more than 32 ports are introduced with aggregated existing CSI-RS resources. 2 aggregated NZP CSI-RS resources for 64 ports were agreed (K=2) in last meeting. We haven’t discussed detailed CSI-RS resource configuration. For CSI-IM and ZP-CSI-RS, it’s proposed to reuse the configuration of 32Tx PMI test. For NZP-CSI-RS, configuration for 32Tx PMI test could be generally reused with changes that 2 NZP-CSI-RS resources should be TDM within same slot and shall not be overlapped with ZP-CSI-RS and CSI-IM. The proposed CSI-RS configuration is depicted in Figure 2-1:

Figure 2-1: Proposed CSI-RS configuration

Table 2-1 summarizes the proposed NZP-CSI-RS configuration

Table 2-1: Proposed NZP-CSI-RS configuration

| NZP CSI-RS | CSI-RS resource 1 and 2 | CSI-RS resource Type |  | Aperiodic |
| --- | --- | --- | --- | --- |
|  |  | Number of CSI-RS ports (X) |  | 32 |
|  |  | CDM Type |  | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) |
|  | CSI-RS resource 1 | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (1, 7) |
|  | CSI-RS resource 2 | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17, (5, 12) |
|  | CSI-RS resource 1 and 2 | CSI-RS interval and offset | slot | Not configured |
|  |  | aperiodicTriggeringOffset |  | 0 |

Proposal 1: RAN4 to use following CSI-RS configuration for PMI test with Type I single panel codebook:

ZP-CSI-RS and CSI-IM: Reuse existing 32Tx port PMI test

NZP-CSI-RS: Table 2-1

CSI-RS and CSI report / apply pattern

According to [2], RAN1 specifies new UE CSI computation time for typeI-SinglePanel-r19, which is coped as following:

When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, the UE shall provide a valid CSI report for the n-th triggered report, - - if the first uplink symbol to carry the corresponding CSI report(s) including the effect of the timing advance, starts no earlier than at symbol Zref, and - - if the first uplink symbol to carry the n-th CSI report including the effect of the timing advance, starts no earlier than at symbol Z'ref(n), where Zref is defined as the next uplink symbol with its CP starting after the end of the last symbol of the PDCCH triggering the CSI report(s), and where Z'ref(n), is defined as the next uplink symbol with its CP starting after the end of the last symbol in time of the latest of: aperiodic CSI-RS resource for channel measurements, aperiodic CSI-IM used for interference measurements, and aperiodic NZP CSI-RS for interference measurement for a CSI-ReportConfig, or for all triggered sub-configurations if CSI-ReportConfig contains multiple sub-configurations, when aperiodic CSI-RS is used for channel measurement for the n-th triggered CSI report, and where Tswitch is defined in clause 6.4 and is applied only if Z1 of table 5.4-1 is applied. Z=maxm=0,…,M-1(Z(m))⁡ and Z'=maxm=0,…,M-1(Z'(m)), where M is the number of updated CSI report(s) according to Clause 5.2.1.6, (Z(m),Z'(m)) corresponds to the m-th updated CSI report and is defined as - (Z2,Z2') or Z2⋅P32,Z2'⋅P32, according to UE reported capability 1 or 2, respectively, with (Z2,Z2') of table 5.4-2, if codebookType is set to 'typeI-SinglePanel-r19', 'typeI-MultiPanel-r19', 'eTypeII-r19' or 'typeII-FePortSelection-r19', where P is the total number of CSI-RS ports across the K configured CSI-RS resources for channel measurement, or Table 5.4-2: CSI computation delay requirement 2 Z1 [symbols]; Z2 [symbols]; Z3 [symbols]. Z1; Z'1; Z2; Z'2; Z3; Z'3. 0; 22; 16; 40; 37; 22; X0. 1; 33; 30; 72; 69; 33; X1. 2; 44; 42; 141; 140; min(44, X3X2+ KB1); X2. 3; 97; 85; 152; 140; min(97, X3+ KB2); X3. 5; 388; 340; 608; 560; min(388, X5+ KB3); X5. 6; 776; 680; 1216; 1120; min(776, X6+ KB4); X6.

Two capabilities are defined, capability 1 means the same CSI computation time is not changed while capability 2 means the CSI computation time is proportionally extended. The CSI-RS/ CSI patten shall be defined according to capability 2 to cover all UE implementation. Figure 2-2 and 2-3 depict the legacy CSI pattern of 32Tx test for FDD and TDD respectively, CSI computation time is 4 slots (56 symbols) for FDD and 7 slots (98 symbols) for TDD, which cannot satisfy capability 2 where at least 74 symbols are needed for FDD/15kHz and 138 symbols are needed for TDD/30kHz.

Figure 2-2: Legacy CSI pattern for 32 Port, FDD

Figure 2-3: Legacy CSI pattern for 32 Port, TDD

Observation 1: The CSI computation time according to existing CSI pattern cannot satisfy capability 2.

We propose the following CSI pattern depicted in Figure 2-4 for FDD:

- CSI request: 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0

- Aperiodic Report Slot Offset :11

- CSI delay: 14ms

Figure 2-4: Proposed CSI pattern for 32 Port, FDD

We propose the following CSI pattern depicted in Figure 2-5 for TDD:

- CSI request: 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0

- Aperiodic Report Slot Offset :17

- CSI delay: 11.5ms

Figure 2-5: Proposed CSI pattern for 32 Port, TDD

Proposal 2: RAN4 to use following CSI-RS configuration for CSI pattern:

FDD

- CSI request: 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0

- Aperiodic Report Slot Offset :11

- CSI delay: 13ms

TDD

- CSI request: 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0

- Aperiodic Report Slot Offset :17

- CSI delay: 11.5ms

Correlation configuration

We have following options for correlation configuration:

Way forward: - Option 1: XP-Medium - Other options are not precluded

A simulation was conducted to verify the performance of XP-Medium and XP-High, in the simulation, MCS20 and Rank2 were used. The simulation results were shown in Figure 2-6:

Figure 2-6: Simulation results for follow PMI test with single panel I.

Simulation results show that both correlations lead huge gap between follow PMI and random PMI and higher correlation enlarge the gap. Med correlation would be sufficient to verify the follow PMI algorithm. Therefore, we propose medium correlation.

Proposal 3: RAN4 to use XPL medium correlation

(N1, N2 configuration)

(16,2) and (8,4) are two candidate options:

Way forward: - Select single one from below options - Option 1: (16,2) - Option 2: (8,4)

Based on our simulation results shown in figure 2-6, compared to (8,4), (16,2) has better performance for follow PMI and worse performance for random PMI, but both (N1,N2) configuration show huge gap between follow PMI and random PMI. Both configurations are reasonable, we slightly prefer option 1.

Proposal 4: RAN4 to use (N1,N2)=(16,2)

Test metric

The legacy PMI metric is defined as the TP ratio of follow PMI to random PMI when SNR point is corresponding to  90% maximum TP of follow PMI. However, the problem is that throughput of random PMI is 0 when SNR point is corresponding to 90% maximum TP of follow PMI, which leads to the infinite Gama value. To address this issue, we propose to set different SNR points for follow PMI and random PMI, denoted as SNR1 and SNR2 respectively, where SNR1 is corresponding to 90% of maximum TP of follow PMI and SNR2 is SNR1 plus XdB (X>0 and FFS for X), X could be specified to make the Gama value in the reasonable range.  Follow text summarize the test metric:

is 90 % of the maximum throughput obtained at using the precoders configured according to the UE reports, and is the throughput measured at +[XdB] with random precoding.

Proposal 5: Modify the legacy test metric to following for 64Port test:

is 90 % of the maximum throughput obtained at using the precoders configured according to the UE reports, and is the throughput measured at +[XdB] with random precoding. X could be specified to make the γvalue in the reasonable range.

MCS level

MCS 20 was agreed as starting point:

Agreement: - MCS20 (Table 1) as the starting point

Based on our simulation results with MCS20, the SNR point @ 90% of maximum throughput is around 8dB for 4Rx and 12~14dB for 2Rx, which are reasonable. Therefore, we confirm MCS20

Proposal 6: RAN4 to confirm MCS20

Correlation matrix

In last meeting, it was agreed to specify the procedure that generate correlation matrix for larger than 32 ports rather than specifying exact correlation matrix in the spec. The scaling factor a is used to ensure the final correlation matrix is positive semi-definite matrix, which is done using following equation:

The values in Table B.2.3.2.2-2, and Table B.2.3.2.2-4 have been adjusted to ensure the correlation matrix is positive semi-definite after round-off to 4 digit precision. This is done using the equation: or

a should be small as much as possible to minimum the impact on Rspatwith the condition that the final matrix is semi-definite. Our  simulation shows that a=0.00010 could make Rmedium semi-definite. Therefore we propose a=0.00010.

Proposal 7: RAN4 to use scaling factor a=0.00010.

Port mapping

RAN1 specifies two port mapping methods:

Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the mapping from CSI-RS resource index/port index per resource and port index to CSI/PMI calculation, support NW to configure UE with one of the following mapping methods via higher-layer (RRC) signaling, - Mapping method 1: Sequential ordering/indexing within (1st resource, 1st polarization), then (2nd resource, 1st polarization), …, then (Kth resource, 1st polarization), then (1st resource, 2nd polarization), then (2nd resource, 2nd polarization), …, then (Kth resource, 2nd polarization) - Mapping method 2: Sequential ordering/indexing within (where K*n2 = N2): - for the 1st polarization, (1st n2 ports in 1st resource, 1st polarization), (1st n2 ports in 2nd resource, 1st polarization), …, (1st n2 ports in Kth resource, 1st polarization), then (2nd n2 ports in 1st resource, 1st polarization), (2nd n2 ports in 2nd resource, 1st polarization), …, (2nd n2 ports in Kth resource, 1st polarization), … then (N1th n2 ports in 1st resource, 1st polarization), (N1th n2 ports in 2nd resource, 1st polarization), …, (N1th n2 ports in Kth resource, 1st polarization) , - and then for the 2nd polarization, (1st n2 ports in 1st resource, 2nd polarization), (1st n2 ports in 2nd resource, 2nd polarization), …, (1st n2 ports in Kth resource, 2nd polarization), then (2nd n2 ports in 1st resource, 2nd polarization), (2nd n2 ports in 2nd resource, 2nd polarization), …, (2nd n2 ports in Kth resource, 2nd polarization), … then (N1th n2 ports in 1st resource, 2nd polarization), (N1th n2 ports in 2nd resource, 2nd polarization), …, (N1th n2 ports in Kth resource, 2nd polarization)

To compare the performance between two different mapping methods, a simulation was conducted. In the simulation, XPL, medium, Rank2, (N1, N2)=(8,4) were assumed. The simulation results are shown in figure 2-7:

Figure 2-7: Simulation results for PMI results with different mapping methods.

It can be observed that two mapping methods have same performance. Therefore either method is feasible with method 1 preferred,

Proposal 8: RAN4 to use port mapping method 1.

Other codebook types

It is FFS whether to introduce PMI requirements with eType II-r19 and type II-Doppler-r19 codebook. As known, the feature is supporting up to 128 CSI-RS ports, which has been verified by Type 1 codebook PMI test. According to the RAN1 agreement, eType II- r19 and type II-Doppler-r19 codebook are just simple extension of eType II- r16 and type II-Doppler-r18 codebook by adding new (N1, N2) values, which means there is no codebook structure changes. We don’t need to define requirements for all codebooks with common feature.

Furthermore, specification supports up to 32 CSI-RS Ports before Rel-19, RAN4 defined 32 ports PMI cases only for Type1-SinglePanel Codebook and only 16 ports is covered for eType II-r19 and type II-Doppler-r19 codebook. That means Type1-SinglePanel rather than more advanced codebook is always used to verify supporting large number of CSI-Port. Follow the logic, we don’t need to define PMI requirements for larger than 32 ports for more advanced codebook.

Proposal 9: Not to introduce PMI requirements for eType II-r19 and type II-Doppler-r19 codebook.

CJT calibration offset reporting requirements

There is an open issue whether to introduce CJTC reporting requirements. Based on our understanding, like RSRP/RSRQ/TDCP reporting, this feature has more impact on RRM part, where UE report the measurement results of frequency, time and phase offset between TRPs to BS.

RRM has agreed to define measurement period requirements for delay and frequency offset reporting but FFS for defining accuracy requirements. If RRM decides to define accuracy requirements, the feature has been verified and demodulation requirements just to covert measurement errors to performance degradation, which is redundant test. Furthermore, we need to specify new TE behavior that how to utilize the reported results such as time, frequency and phase compensation, which will bring additional TE implementation complexity.

Therefore, we don’t see the need to introduce demodulation and CSI requirements for this feature.

Proposal 10: Not to introduce CJTC offset reporting requirements.

- BS performance requirements

There is an open issue whether to define PUSCH demodulation requirements for 3-antenna-port-based UL transmission. Based on our understanding, the feature is introducing new 3-Tx UL codebook. Considering PUSCH test is outer-loop test with fixed precoder rather than PMI test, BS only aware of number of layers (DMRS port) but not the number Tx for PUSCH demodulation. Furthermore, RAN4 has already defined requirements for 4 MIMO layers, we don’t see the need to introduce easier case with 3 MIMO layers. Therefore, we don’t support defining PUSCH requirements for R19 MIMO.

Proposal 11: Not to introduce BS performance requirements for R19 MIMO.

- Conclusion

In this contribution we provide our views on demodulation and CSI requirements for R19 MIMO. The proposals and observations are:

Proposal 1: RAN4 to use following CSI-RS configuration for PMI test with Type I single panel codebook:

ZP-CSI-RS and CSI-IM: Reuse existing 32Tx port PMI test

NZP-CSI-RS: Table 2-1

Observation 1: The CSI computation time according to existing CSI pattern cannot satisfy capability 2.

Proposal 2: RAN4 to use following CSI-RS configuration for CSI pattern:

FDD

- CSI request: 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0

- Aperiodic Report Slot Offset :11

- CSI delay: 13ms

TDD

- CSI request: 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0

- Aperiodic Report Slot Offset :17

- CSI delay: 11.5ms

Proposal 3: RAN4 to use XPL medium correlation

Proposal 4: RAN4 to use (N1,N2)=(16,2)

Proposal 5: Modify the legacy test metric to following for 64Port test:

is 90 % of the maximum throughput obtained at using the precoders configured according to the UE reports, and is the throughput measured at +[XdB] with random precoding. X could be specified to make the γvalue in the reasonable range.

Proposal 6: RAN4 to confirm MCS20

Proposal 7: RAN4 to use scaling factor a=0.00010.

Proposal 8: RAN4 to use port mapping method 1.

Proposal 9: Not to introduce PMI requirements for eType II-r19 and type II-Doppler-r19 codebook.

Proposal 10: Not to introduce CJTC offset reporting requirements.

Proposal 11: Not to introduce BS performance requirements for R19 MIMO.

- Reference

[1] Way Forward for [115][329] NR_MIMO_demod. Samsung

[2]  Introduction of CSI enhancements for NR MIMO Phase 5. Nokia