3GPP TSG-RAN WG4 Meeting #116	R4-2510888

# Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussions on 8Rx performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.13.3

Document for:	Discussion

Background

WF R4-2502275 [1] was agreed in RAN4-114 meeting, this contribution provides our views on related open issues.

- CQI for inter cell interference

Definition of MMSE-IRC receiver capability for 8Rx UE

One left issue is definition of MMSE-IRC receiver, the candidate options are listed as follows:

Options: - Option 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 2: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 3: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 4: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 5: Re-use existing definition in Rel-18.

We don’t support introducing terminology “MMSE-IRC” in the definition and name. As known, even all basic requirements are defined with MMSE-IRC assumption, but the purpose is to facilitate all UE receivers to be tested, that’s the reason we call the requirements “minimum requirements”. MMSE-IRC is the simplest receiver which performs bad in the real field and most UEs have implemented more advanced receiver. Specifying “MMSE-IRC”receiver in the definition means only UE with MMSE-IRC receiver can be tested, which will limit the test scope and against the meaning of “minimum requirements”. We don’t see any essential difference between option3,4 and 5, we slightly prefer option 5 since “MMSE-IRC” receiver on MU-MIMO scenario is SU-MIMO receiver actually, since Rel-18 definitions works.

Proposal 1: Re-use existing 8Rx receiver definition in Rel-18

CQI requirements

For CQI requirements, one open issue is Gamma and SINR:

Configuration for CQI requirements Agreement: - Use [-2dB] SINR for requirement definition. - TBD for the Gamma value. Companies are encouraged to propose the value for Gamma in the next meeting.

Our simulation results are captured in Table 2-1:

Table 2-1: Simulation results for CQI requirements with inter cell interference

| SINR | FDD |  |  | TDD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Baseline receiver | Simplified receiver | Gap Gama(Baseline receiver)- Gama(Simplified receiver) | Baseline receiver | Simplified receiver | Gap Gama(Baseline receiver)- Gama(Simplified receiver) |
| -6 | 2.93 | 5.17 | -2.24 | 2.64 | 5.46 | -2.82 |
| -5 | 3.97 | 4.56 | -0.59 | 3.71 | 4.53 | -0.82 |
| -4 | 4.41 | 3.31 | 1.1 | 4.43 | 3.05 | 1.38 |
| -3 | 3.55 | 2.67 | 0.88 | 3.26 | 2.51 | 0.75 |
| -2 | 3.00 | 2.91 | 0.09 | 2.85 | 2.68 | 0.17 |
| -1 | 2.85 | 3.33 | -0.48 | 2.87 | 3.03 | -0.16 |
| 0 | 3.04 | 3.12 | -0.08 | 2.96 | 2.77 | 0.19 |
| 1 | 3.42 | 2.67 | 0.75 | 3.48 | 2.37 | 1.11 |
| 2 | 3.89 | 2.40 | 1.49 | 3.97 | 2.30 | 1.67 |
| 3 | 3.99 | 2.51 | 1.48 | 4.09 | 2.56 | 1.53 |
| 4 | 3.42 | 2.66 | 0.76 | 3.71 | 2.70 | 1.01 |
| 5 | 2.59 | 2.64 | -0.05 | 2.53 | 2.84 | -0.31 |

Based on our simulation results, we propose SINR=-2dB and Gama=2.1

Proposal 2: Set SINR=-2dB and Gama=2.1.

- Conclusion

This contribution provides our simulation results for 8Rx requirements with interference. The proposals are:

Proposal 1: Re-use existing 8Rx receiver definition in Rel-18

Proposal 2: Set SINR=-2dB and Gama=2.1.

- Reference

[1]  R4-2508620 Way forward for [115][323] NR_demod_Ph5_Part2_UE. Nokia