3GPP TSG-RAN WG4 Meeting #116 	R4-2511212

Bengaluru, IN, August 25th – 29th, 2025

Agenda Item: 7.21.4.1

Source: MediaTek inc.

Title: Discussion on SBFD UE-to-UE CLI

Document for: Discussion

# Introduction

The work item (WI) for Evolution of NR duplex operation: Sub-band full duplex (SBFD) has the following objectives [1]:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In the last meeting, RRM group discussed several issues on the impacted RRM specification and whether new RRM requirements are needed to handle the new SBFD operation and CLI handling mechanism, where the issues and agreements are captured in the way forward (WF) [2] and collection of companies proposals in Topic summary [3]. In this contribution, we provide further discussion on the issues captured in the WF.

# Discussion on the RRM impact for CLI handling mechanisms

In legacy 5G NR TDD, the RRM specification cover the CLI measurements delay and accuracy for SRS-RSRP and CLI-RSSI. The UE reports these measurements to the gNB to help the dynamic allocation of uplink/downlink in time. Nevertheless, these measurements are based on L3 measurements, i.e. it requires L3 filtering, which has longer measurement delay compared to L1 measurements. Therefore, in SBFD Rel-19 WID objective, new CLI measurements and reporting shall be introduced for L1 based UE-to-UE CLI with new measurement quantities, such as L1-SRS-RSRP, L1-CLI-RSSI.

# Discussion on the general issues

In the previous meeting, RAN4 RRM reached agreement with related further study point, which are captured in the WF.

Issue 1-1-1: Requirement applicability:

Agreement (Monday online session) - For method #2, 3 and non-basic UE operation of method #1 - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is configured within active DL BWP - when CLI-RSSI measurement resource is configured within active DL BWP - FFS for basic UE operation for method #1 - Note: The assumption for basic UE operation is that UE cannot receive simultaneous PDSCH reception on two DL subbands, and whether the basic UE operation exists or not is pending on RAN1 discussion.

On the basic operation for method#1, RAN1 has left the following basic capability:

Therefore, RAN4 to wait for RAN1 to reach decision on the issue above.

- On the basic operation for method#1 and whether that UE cannot receive simultaneous PDSCH reception on two DL subbands, RAN4 to wait for RAN1 to reach conclusion.

Issue 1-1-3: L1 CLI measurement in non-SBFD symbols:

- Proposals - RAN4 to wait, and potentially take into consideration, RAN1 decision on whether a UE can be configured to perform L1 UE-to-UE CLI measurements in non-SBFD symbols in SBFD operation. - Recommended WF - Suggest to wait for RAN1 conclusion.

RAN1 has reached the following agreement:

Agreement For a CSI report associated with periodic/semi-persistent SRS-RSRP measurement resources or CLI-RSSI measurement resources, the valid symbol type for CSI derivation configured in CSI-ReportConfig is applicable for L1 UE-to-UE CLI measurement and reporting if SBFD symbols are configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the valid symbol type configured in CSI-ReportConfig The valid symbol type does not apply for a UE if SBFD symbols are not configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the configuration provided in CSI-ResourceConfig indicated by CSI-ReportConfig FG 60-1 is not the prerequisite of FG 60-8 and FG60-9

Valid symbol type defined CSI-ReportConfig (which can be SBFD or non-SBFD) is applicable for L1 UE-to-UE CLI measurements. Therefore, a UE can be configured to perform L1 UE-to-UE CLI measurements in non-SBFD symbols in SBFD operation.

- A UE can be configured to perform L1 UE-to-UE CLI measurements in non-SBFD symbols in SBFD operation.

# Discussion on the RRM impact for L1-CLI-RSSI

In the previous meeting, RAN4 RRM reached agreement with related further study point, which are captured in the WF.

Issue 1-3-1: Measurement period:

- Proposals - Proposal 1 (HW): - RAN4 to confirm that the number of samples for DUD case with method #1 and resource type#2 is 1, at least for the case when UE supports DL reception in two DL subbands. - RAN4 to wait for RAN1 conclusion on whether the case when UE does not support DL reception in two DL subbands exists or not, and if so, discuss whether and how to define requirements. - Proposal 2 (ZTE): - For L1-CLI-RSSI measurement based on P/SPS resource, 1 sample or 3 samples is performed and up to the configuration of timeRestrictionForChannelMeasurement. - Recommended WF - Discuss P2, i.e. whether to use 3 samples for measurement period for L1-CLI-RSSI when timeRestrictionForChannelMeasurement is not configured. - For P1, suggest to wait for RAN1 conclusion on UE capability for simultaneous DL reception in two DL subbands.

Regardless of the configuration of timeRestrictionForChannelMeasurement, RAN4 requirements shall be based on 1 sample.

- RAN4 RRM requirements shall be based on 1 sample regardless of the configuration of timeRestrictionForChannelMeasurement.

Issue 1-3-3: Measurement restriction:

- Proposals for collision between L1-CLI-RSSI measurement and L1/L3 RRM measurements in FR1 - Proposal 1a (CATT, ZTE) - Measurement requirements for L1-CLI-RSSI apply as default. - Proposal 1b (Samsung) - Measurement requirements for L1-CLI-RSSI apply, when SCS of L1-CLI-RSSI measurement and L1/L3 RRM measurements are same. - Proposal 1c (E////) - Measurement requirements for L1-CLI-RSSI apply, except when L1-CLI-RSSI measurement resources overlap MG - Proposal 1d (HW) - Measurement requirements for L1-CLI-RSSI apply, except when - L1-CLI-RSSI measurement resources overlap MG - L1-CLI-RSSI measurement resources are configured with a different SCS from the SSB used for L1/L3 measurement - Proposal 2 (Nokia) - Measurement requirements for L1-CLI-RSSI do not apply. - Recommended WF for collision between L1 CLI measurements and L1/L3 RRM measurements in FR1 - Discuss the following 2 options on high level: - Option 1: Measurement requirements for L1-CLI-RSSI apply as default, and define exception cases where requirements do not apply. - Option 2: Measurement requirements for L1-CLI-RSSI do not apply.

When a collision between L1-CLI-RSSI measurement and L1/L3 RRM measurements in FR1 occur, then it should be left to the UE implementation on which measurement to perform.

- When a collision between L1-CLI-RSSI measurement and L1/L3 RRM measurements in FR1 occur, then it should be left to the UE implementation on which measurement to perform.

# Summary

In this contribution, we have the following proposals:

Proposal 1: On the basic operation for method#1 and whether that UE cannot receive simultaneous PDSCH reception on two DL subbands, RAN4 to wait for RAN1 to reach conclusion.

Proposal 2: A UE can be configured to perform L1 UE-to-UE CLI measurements in non-SBFD symbols in SBFD operation.

Proposal 3: RAN4 RRM requirements shall be based on 1 sample regardless of the configuration of timeRestrictionForChannelMeasurement.

Proposal 4: When a collision between L1-CLI-RSSI measurement and L1/L3 RRM measurements in FR1 occur, then it should be left to the UE implementation on which measurement to perform.

# References

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June 2024.

- R4-2508274, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, Malta, May 2025.

- R4-2505568, “Topic summary for [115][219] NR_duplex_evo”, Huawei, HiSilicon, Malta, May 2025.