3GPP TSG-RAN WG4 Meeting #116	R4-2511222

# Bengaluru, India, August 25th – 29th, 2025

Source:	Nokia

Title:	RRM core requirements and testing framework for AI/ML positioning accuracy enhancement

Agenda item:	7.17.5

Document for:	Discussion

Introduction

The work item on artificial intelligence (AI)/machine learning (ML) for NR air interface (NR_AIML_Air) approved at RAN#102, was revised at RAN#108 [1] to refine the normative work of the general AI/ML framework for air interface and to enable the recommended use cases as depicted in the TR for the preceding study [2].

In this paper, open issues on RRM core and performance requirements for UE-based positioning (Case 1) and NG-RAN node assisted positioning (Cases 3a and 3b) from RAN4#115 are discussed and corresponding proposals are made.

Discussion

The open issues related to Cases 1, 3a and 3b are discussed in this section according to RAN4#115 WF [3] and Topic summary [4].

Requirements for Case1

This section discusses the requirements for the use cases with a UE-side model.

The following agreements were reached for Case 1 at RAN4#115 [3].

Issue 3-1: Requirements for case 1 Agreement: - Discuss how to define a measurement delay requirement (reporting delay) - Do not define any positioning accuracy requirement

Hence RAN4 needs to investigate how to define the measurement delay requirement for the reported quantity in Case 1, i.e. the reporting delay for the reported UE location while no positioning accuracy requirements will be specified.

Measurement delay requirements

The integration of AI/ML models into UE-based positioning introduces a complex interplay between measurement acquisition, model inference, and reporting mechanisms. In Case 1, where the AI/ML model resides and operates on the UE, the positioning process becomes highly scenario-dependent. Unlike traditional A-GNSS or OTDOA methods, the inference latency is not solely determined by signal reception but also by the computational characteristics of the model, such as architecture depth, input dimensionality, and hardware acceleration capabilities. This variability makes it difficult to define a one-size-fits-all processing delay requirement.

A comprehensive reporting delay can be defined by encompassing measurement, AI/ML inference, and reporting time intervals.

Moreover, the lack of standardized definitions for inference delay and model verification procedures poses a challenge for ensuring consistent performance across devices and vendors. For instance, a UE may infer its position based on a trained neural network using PRS measurements, but the time taken for inference can vary significantly depending on whether the model is lightweight or computationally intensive. Without a verification framework, it is unclear whether the inferred position meets any accuracy threshold, raising concerns about the reliability of the reported data.

It would be needed to categorize AI/ML inference delays based on model complexity and UE processing capabilities, to support flexible but bounded delay requirements.

RAN4 could be required to investigate the feasibility of defining a model verification framework to ensure that AI/ML-based positioning outputs meet minimum reliability and performance standards.

To address this, RAN4 can begin by anchoring the measurement period to existing requirements defined for RRC_CONNECTED mode, where measurements are performed within designated measurement gaps. This approach provides a stable reference point and aligns with current implementations in UE-assisted positioning. The start of the measurement period in UE-based positioning could be defined as the time of reception of the LMF command, analogous to the first PRS in UE-assisted positioning. This reuse of legacy definitions ensures continuity and simplifies implementation.

Defining the processing delay, however, remains a nuanced task. RAN4 may consider categorizing inference delays based on model complexity classes or hardware capabilities, allowing for flexible yet bounded delay requirements. Additionally, the reporting delay should account for the time taken to transmit the inferred position to the network via LPP procedures. This end-to-end delay, from PRS measurement to AI inference to location reporting, must be clearly delineated to support performance evaluation and conformance testing.

Another dimension that RAN4 must explore is the relationship between delay and positioning accuracy. In the absence of accuracy requirements, a UE could technically satisfy any delay constraint by reporting arbitrary or non-truthful location data. This undermines the integrity of the positioning system and could have implications for services relying on precise location information. Therefore, RAN4 should investigate whether delay requirements can be meaningfully defined without accompanying accuracy metrics, and whether mechanisms are needed to ensure the veracity of the reported position.

Furthermore, the interactive nature of LPP procedures implies that signaling latency may also influence the overall delay. RAN4 should collaborate with RAN2 to identify potential enhancements to LPP signaling that support timely and reliable transmission of AI-inferred location data. This includes evaluating whether additional signaling messages or protocol optimizations are needed to accommodate the unique characteristics of AI/ML-based positioning.

The evaluation of delay requirements for UE-based positioning in Case 1 should remain open-ended, as the integration of AI/ML introduces significant variability in inference timing and reporting behavior across devices. Rather than specifying reporting delays, RAN4 should discuss possible approaches that reflect the nature of the AI/ML inference delay and the current absence of verification, accuracy, and delay definitions.

RAN4 should consider whether a reporting delay requirement is appropriate for UE-based positioning in Case 1, given the variability introduced by AI/ML inference on the UE and the current lack of standardized delay, accuracy, and verification frameworks.

LCM-related aspects: delay requirements for activation and model performance monitoring

Model performance monitoring has been identified as a key consideration for AI/ML integration in RAN4, particularly for UE-side use cases like beam management and positioning. However, in Case 1 of UE-based positioning, the AI/ML model operates entirely on the UE, and no measurement reporting to the network is required. This fundamental difference reduces the relevance of performance monitoring mechanisms that rely on network-visible metrics.

While the general aspects paper [5] and prior discussions in RAN4 suggest using beam management as a baseline for monitoring, the applicability to positioning remains limited. Without measurement reporting, there is no practical basis for monitoring model performance on received measurements, and enforcing such requirements may introduce unnecessary complexity. Future implementations can still benefit from reliability principles established in beam management without mandating formal performance monitoring for positioning.

RAN4 should not define explicit performance monitoring requirements for UE-based positioning in Case 1.

Testing framework

The discussion on meaningful delay requirements for reporting positioning coordinates for UE based positioning is ongoing in RAN4. We propose to postpone the discussion on the testing framework until testable core requirements are specified.

Postpone the discussion on the testing framework for Case 1 until testable core requirements are specified.

Requirements for Case 3a

Testing framework in Case 3a

As for legacy gNB assisted NR positioning in Rel-16, Rel-17 and Rel-18, there is no need to specify a testing framework in 3GPP for gNB-assisted AI/ML positioning with gNB model in Case 3a.

Do not specify a testing framework for Case 3a.

Requirements for Case 3b

Testing framework in Case 3b

As for legacy gNB assisted NR positioning in Rel-16, Rel-17 and Rel-18, there is no need to specify a testing framework in 3GPP for gNB-assisted AI/ML positioning in Case 3b.

Do not specify a testing framework for path-based or sample-based measurements for Case 3b.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to AI/ML based positioning. In the paper, the following observations and proposals were made:

Case 1

- A comprehensive reporting delay can be defined by encompassing measurement, AI/ML inference, and reporting time intervals.

- It would be needed to categorize AI/ML inference delays based on model complexity and UE processing capabilities, to support flexible but bounded delay requirements.

RAN4 could be required to investigate the feasibility of defining a model verification framework to ensure that AI/ML-based positioning outputs meet minimum reliability and performance standards.

- RAN4 should consider whether a reporting delay requirement is appropriate for UE-based positioning in Case 1, given the variability introduced by AI/ML inference on the UE and the current lack of standardized delay, accuracy, and verification frameworks.

RAN4 should not define explicit performance monitoring requirements for UE-based positioning in Case 1.

Postpone the discussion on the testing framework for Case 1 until testable core requirements are specified.

- Case 3a/3b

Do not specify a testing framework for Case 3a.

Do not specify a testing framework for path-based or sample-based measurements for Case 3b.

- References

- RP-251186, Revised WID on AI/ML for NR air interface, RAN#108, Prague, Czech Republic, June 9-13, 2025

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm, RAN4#115

- R4-2507570, Topic summary for [115][131] NR_AIML_air_part1, Moderator (Qualcomm), RAN4#115

- R4-2509920, On General Aspects of AI/ML for NR Air Interface Phase 1, Nokia, RAN4#116