3GPP TSG-RAN WG4 Meeting #116	R4-2511333

Bengaluru, India, Aug. 2025

Source: 	Qualcomm

Title: 	TP for TR 38.774: ACS requirements for LP-WUS

Agenda Item:	7.24.3.2

Document for:	Approval

# Background

This contribution provides the TP for TR 38.774 on blocking requirements for LP-WUS.

# References

[1] R4-2503830, TR 38.774 V0.3.0

# Text Proposal

<Start of Text Poposal>

### 7.1.6	ACS requirements

#### 7.1.6.1	Motivation and general comments

The basis of jammer requirements for the LPWUS feature is that the LR and MR cohabit the same UE, and the UE must withstand the same jammers whether it is receiving PDSCH or monitoring the WUS in some power-saving mode.

In LPWUS monitoring mode, the UE can autonomously and transparently flip between using the LR and MR, as part of its RAN1- and RAN2-defined behavior. The overall goal is to preserve reliability of PDCCH/paging while supporting the LPWUS feature, compared to legacy operation without LPWUS. The inherent flexibility designed into the UE for the LPWUS feature means it is not possible to delineate LR behavior from MR behavior, it is only possible to specify UE-level behavior.

The analysis in subsequent sections assume that the LR’s NF variation across bands tracks that of the MR, This assumption allows band-agnostic analysis.

#### 7.1.6.2	Deriving the requirement from system level considerations, FR1

##### 7.1.6.2.1	General

The challenge for deriving the ACS requirement is translating ‘the UE must withstand the same jammers whether it is receiving PDSCH or monitoring the WUS in some power-saving mode’ into a mathematically relatable principle. Three were considered in the work phase:

- Extension of legacy ACS side conditions: Same jammer levels as legacy requirement side conditions would apply and wanted signal strength would relate to its REFSENS in the same way as the legacy case.

- Real world consideration 1: The coverage of PDSCH and the coverage of LPWUS both degrade by similar amounts in the presence of an interferer

- Real world consideration 2: Under the condition of same PSD for PDSCH and LPWUS, the SNR degradations for PDSCH reception and for LPWUS reception are comparable

##### 7.1.6.2.2	ACS consideration 1

LPWUS ACS requirements can be derived based on extension of legacy ACS requirement. Specifically, it is assumed that the same jammer levels as legacy requirement side conditions would apply and the LPWUS wanted signal strength would relate to its REFSENS in the same way as the legacy case (+14 dB).

For the main radio with diversity gain DG, the absolute interferer level for the legacy interferer at each UE input can be calculated as:

Intrfrlegacy=-174+NFlegacy-DG+10logCBW+10log101410-1+ACSlegacy

A similar equation can be written for the absolute level of the interferer for the LPWUS case, assuming the LPWUS (wanted) signal is also 14 dB stronger than the LPWUS REFSENS, and there is no diversity gain (1Rx case):

IntrfrLPWUS= ACSLPWUS+ NLPWUS+10logBWLPWUS+10log101410-1

If the absolute interferer power levels are the same for both cases (legacy and LPWUR), then the legacy interferer per connector and LPWUS interferer are equal, i.e. the right-hand sides of the 2 equations can be directly compared:

NFLPWUS+10logBWLPWUS+ACSLPWUS= -DG+NFlegacy+10logCBW+ACSlegacy

i.e.   ACSLPWUS= NFlegacy-NFLPWUS-DG+10logCBWBWLPWUS+ACSlegacy

i.e.  ACSLPWUS= ACSlegacy+10logNRBRBLPWUS+(NFlegacy-NFLPWUS)-DG   EQ 7.1.6.2.2-1

##### 7.1.6.2.3	ACS consideration 2

A real-world ACS performance expectation is ‘the coverage of PDSCH and the coverage of LPWUS both degrade by similar amounts in the presence of an interferer’. This expectation can be translated to ‘for a given interferer, the desensitization suffered by the UE for LPWUS detection is no greater than the desensitization suffered by the main radio’. This real-world scenario is analyzed below.

For the main radio with diversity gain DG, assume an interferer that causes ‘x’ dB desensitization of PDSCH. The absolute interferer level (dBm) at each UE input can be calculated as:

Intrfrlegacy=-174+NFlegacy-DG+10logCBW+10log10x10-1+ACSlegacy

In the real world, LPWUS is likely to have adjacent non LPWUS RBs. Desensitization due to these adjacent non-LPWUS RBs is assumed to be ‘d’ (dB). The ‘real-world principle’ to apply here is that that the adjacent channel interferer that causes ‘x’ dB desens in the MR causes no more than a further ‘x’ dB desensitization at the UE for LPWUS detection.  i.e the net desens for LPWUS reception would be ‘d+x’. The required ACS that should apply to the UE for LPWUS reception can then be calculated as:

ACSLPWUS= Intrfrlegacy+174- NFLPWUS-10logBWLPWUS-10log10x10-1 -d

Combining the two equations above by eliminating Intrfrlegacy:

ACSLPWUS= - NFLPWUS-10logBWLPWUS+NFlegacy-DG+10logCBW+ACSlegacy

i.e.       ACSLPWUS= NFlegacy-NFLPWUS-DG+10logCBWBWLPWUS+ACSlegacy

i.e.      ACSLPWUS= ACSlegacy+10logNRBRBLPWUS+(NFlegacy-NFLPWUS)-DG………. EQ 7.1.6.2.3-1

##### 7.1.6.2.4	ACS consideration 3

A second real-world consideration for LPWUS ACS is the UE’s radio environment when connected to a serving cell (for PDSCH and LPWUS) and in the presence of a neighbor cell interferer. Here the key constraint is equal PSD for all DL from the serving cell, at least for the case of no power boost. The no power boost condition is significant here because that is also the chosen condition for the ASCS requirement.

The SNR degradation in the MR when the adjacent channel interferer is present, relative to when the UE is interference-free is given by:

∆SNRACS,g=10log1+10IG-ACSlegacy+DG10CBW.10Nlegacy10

…. EQ 7.1.6.2.4-1

LPWUS operation is now considered for the same pair of UE and serving cell gNB. For LPWUS RBs, 0 dB power boost is assumed. i.e., when LPWUS is transmitted, it has the same ‘P’ dBm/Hz PSD as PDSCH.  Desensitization due to the adjacent non-LPWUS RBs is assumed to be ‘d’ (dB). Assuming filtering of the interferer by the amount ACSLPWUS, the SNR degradation in LPWUS reception due to the adjacent channel jammer ;

∆SNRACS,LPWUS=10log1+10Ig-ACSLPWUS10BWLPWUS.10NLPWUS10+10d10-1.10P-SNRLPWUS10 	…. EQ 7.1.6.2.4-2

The principle used for this real-world consideration is that under the condition of same PSD for PDSCH and LPWUS, the SNR degradation for PDSCH reception is comparable to the SNR degradation of LPWUS reception in the presence of an adjacent channel interferer. i.e.:

∆SNRACS,LPWUS=∆SNRACS,g

From equations 7.1.6.2.4-1 and -2:

10log1+10IG-ACSlegacy+DG10CBW.10Nlegacy10=10log1+10Ig-ACSLPWUS10BWLPWUS.10NLPWUS10+10d10-1.10P-SNRLPWUS10

i.e.   10IG-ACSlegacy+DG10CBW.10Nlegacy10=10Ig-ACSLPWUS10BWLPWUS.10NLPWUS10+10d10-1.10P-SNRLPWUS10

i.e.    10ACSLPWUS-ACSlegacy+DG10=CBW.10Nlegacy10BWLPWUS.10NLPWUS10+10d10-1.10P-SNRLPWUS10

ACSLPWUS-ACSlegacy+DG=Nlegacy+10logCBWBWLPWUS-10log10NLPWUS10+10d10-1.10P-SNRLPWUS10

ACSLPWUS-ACSlegacy+DG=Nlegacy-NLPWUS+10logCBWBWLPWUS-10log1+10d10-1.10P-SNRLPWUS1010NLPWUS10

Note that the threshold SNR for LPWUS reception is given by SNRLPWUS= RLPWUS-NLPWUS where ‘R’ is the PSD of the REFSENS and N is the PSD of the antenna referred noise (-174 + NF), so:

ACSLPWUS-ACSlegacy+DG=Nlegacy-NLPWUS+10logCBWBWLPWUS-10log1+10d10-1.10P-RLPWUS10

Or…     ACSLPWUS= ACSlegacy+NFlegacy-NFLPWUS+10logNRBRBLPWUS-DG-10log1+10d10-1.10P-RLPWUS10

While this expression is dependent on the DL PSD ‘P’, simplifications can be made. Specifically, the expression above must apply throughout the cell, especially at cell edge. For cell edge cases, ‘P’ is close to ‘R’ (PDSCH sensitivity PSD), and therefore comparable to ‘RLPWUS’. Consequently, the last term 10log1+10d10-1.10P-RLPWUS10 contributes negligibly, and the ACS expressions align for the LR types:

ACSLPWUS≈ ACSlegacy+NFlegacy-NFLPWUS+10logNRBRBLPWUS-DG

…… EQ 7.1.6.2.4-3

#### 7.1.6.3	UE Implementation considerations for ACS

The previous sections have evaluated UE ACS requirements demanded by the system, i.e. a ‘top-down’ approach. We now evaluate if there is a feasibility problem at the UE.

System design allows the UE to autonomously and transparently flip between using the LR and MR, as part of its expected behavior. The overall goal is to preserve reliability of PDCCH/paging while supporting the LPWUS feature and under jammer conditions, compared to legacy operation. This UE flexibility is key to implementing LPWUS support with LRs with ultra-low operational power consumption goals.

For example, consider a UE with an LR implementation featuring either relaxed filtering, or relaxed phase noise performance. This UE would opportunistically use the LR when interference conditions are low enough but fall back to MR operation when interferers can affect its LR implementation’s reliability.

A UE’s implementation choice of the LR and the handover logic between the LR or legacy operation with the MR are two sides of the same coin. For example, a UE design with an ultra-low consumption LR is more likely to fall back to legacy operation with the MR due to interferers, while another UE with sufficient filtering and LO quality will not need to fall back to MR operation as often. The handover logic is unique to each UE implementation and is integral to the support of this feature.

In conclusion, the ACS requirement derived from system-level considerations above do not preclude LRs with ultra-low power consumption. The flexibility to fall back to legacy operation using the MR allows a broad spectrum of LR implementations.

#### 7.1.6.6	LPWUS ACS requirement summary

The table below summarizes the implied ACS requirements for the 3 considerations or motivating principles identified above:

| Motivating principle | Implied LPWUS ACS Requirement | ACS Requirement for type 1 LR | ACS Requirement for type 2 LR |
| --- | --- | --- | --- |
| - RAN4 agreements on LPWUS | ACSLPWUS= Z+(NFlegacy-NFLPWUS) EQ. 7.1.6.2.2-1 | ACSLPWUS= Z-9.5 | ACSLPWUS= Z-5.0 |
| - The coverage of PDSCH and the coverage of LPWUS both degrade by similar amounts in the presence of an interferer | ACSLPWUS= Z+NFlegacy-NFLPWUS-d EQ. 7.1.6.2.3-1 | ACSLPWUS= Z-10.0 | ACSLPWUS= Z-5.0 |
| - Under the condition of same PSD for PDSCH and LPWUS, the SNR degradations for PDSCH reception and for LPWUS reception are comparable | ACSLPWUS=Z -NFlegacy-NFLPWUS EQ. 7.1.6.2.4-3 | ACSLPWUS= Z-9.5 | ACSLPWUS= Z-5.0 |
| NOTE: Z= ACSlegacy+10logNRBRBLPWUS-DG |  |  |  |

Regardless of system-level motivating principle, the ACS requirement for LPWUS works out to a bandwidth-corrected version of the legacy ACS requirement (parameter ‘Z’ in table above) with some relaxation. There is close alignment between the requirements, so it may be reasonable to choose the more relaxed candidate for the requirement

i.e.      ACSLPWUS= ACSlegacy+10logNRBRBLPWUS+NFlegacy-NFLPWUS-DG-d

#### 7.1.6.3	FR2-specific considerations

In general, agreements made for FR1 can be carried over to FR2. One legacy FR2 requirement assumptions that is useful to co-opt is that the jammer is ‘polarization matched to the wanted signal’. For simplicity this assumption can be carried over. For legacy operation with the main radio, the following expression relates the max. strength of the interferer to the ACS requirement and the side condition of providing the wanted signal 14 dB higher than REFSENS, where Gant=10logNelem+Gelemis the array gain in each pol. of the module

Intrfrlegacy= ACSlegacy+ Nlegacy+10logCBW+10log101410-1-Gant

Similarly, for LPWUS reception with a single element and pol. selection diversity (EQ. 5.2-1)

IntrfrLPWUS= ACSLPWUS+ NLPWUS+10logBWLPWUS+10log101410-1-CG-Gelem

Note here that CG = -3 for LPWUS (for pol. selection diversity) Setting both interferers to the same value,

ACSLPWUS= NFlegacy-NFLPWUS+CG-10logNelem+10logCBWBWLPWUS+ACSlegacy

For FR2 LPWUS, NF+IM is agreed (tentatively) as 12.0 dB for type 2 LR, where IM itself is 2.0 dB from legacy FR2 practice. It follows therefore that NFLPWUS is 10.0 dB. Finally, legacy assumption for the module was a 4x1 linear array. The equation above can be specialized for type 2 LR as:

ACSLPWUS= 9.0-10.0-3.0-6.0+10logCBWBWLPWUS+ACSlegacy

ACSLPWUS,type2= ACSlegacy+10logNRBRBLPWUS-10.0

i.e. the FR2 LPWUS ACS requirement also takes on a form similar to that of FR1.

<End of Text Poposal>