3GPP TSG-RAN WG4 Meeting #116	R4-2511527

Bengaluru, India, 25th – 29th August, 2025

Agenda item:		7.21.3.1

Source:	Nokia

Title:	On potentially new BS RF requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#115 [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the potentially new BS RF requirements for SBFD operation. Also, the open items are listed in the WF. This contribution displays Nokia’s view on the potentially new BS RF requirements for SBFD operation.

# Discussion

## In-channel adjacent subband leakage ratio

There was no new agreement in RAN4#115, and it was agreed to still follow WF in RAN4#114bis:

Agreement: - The requirement (In-channel adjacent subband leakage ratio) is renamed as “in-channel adjacent subband leakage requirement” (by removing “ratio”).

Way forward: - FFS whether/how this assumption can be captured in the RAN4 requirement for in-channel adjacent subband leakage.

Way forward: - Absolute limit for in-channel adjacent subband leakage: - FFS by considering the following exemplary cases by performing analysis including measurement or simulation: - FR1: - DU/UD: 80MHz (DL) – 20MHz (UL) - DUD: 40MHz (DL) – 20MHz (UL) – 40MHz (DL) - Other cases are not precluded - FR2-1: - DU/UD: 50MHz (DL) – 50MHz (UL); 100MHz (DL) – 100MHz (UL) - DUD: 75MHz (DL) – 50MHz (UL) – 75MHz (DL) - Other cases are not precluded - Note: The study on different configuration does not mean RAN4 will define separate requirements for different configuration. RAN4 still try to use a unified requirements for absolute limit.

For the absolute limit of the in-channel adjacent subband leakage, the following was agreed in RAN4#115:

Agreement - For absolute limit for in-channel adjacent subband leakage: - For FR2 BS, legacy ACLR absolute basic limit (Table 9.7.3.3-2 for FR2 in TS 38.104) is followed. - For FR1 BS, - For FR1 LA SBFD BS, no relaxation compared with absolute ACLR limit (DU configuration) or absolute CACLR limit (DUD) are needed. - For FR1 WA and MR SBFD-capable BS, - Option 1: [3]dB relaxation is provided. - Option 2: no relaxation from corresponding existing requirement

It was agreed in RAN4#114bis that RAN4 will still try to use a unified requirement for absolute limit. The unified limit should be applied in our view and the existing ACLR absolute basic limit values can be used for in-channel adjacent subband leakage requirements as described in Tables 6.6.3.2-2 and Table 9.7.3.3-2 in TS 38.104:

Table 6.6.3.2-2: Base station ACLR absolute basic limit

| BS category / BS class | ACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

Table 9.7.3.3-2: BS type 2-O ACLR absolute limit

| BS class | ACLR absolute limit |
| --- | --- |
| Wide area BS | -13 dBm/MHz |
| Medium range BS | -20 dBm/MHz |
| Local area BS | -20 dBm/MHz |

Use the existing ACLR absolute limits as the new in-channel adjacent subband leakage requirements.

Honestly, we feel it will be a mess if separate requirements will be introduced for all different configurations. Therefore, the most straightforward approach is to follow the existing absolute basic limits. RAN4 is also running out of time with the core part of the WI as the core part will be finished already in RAN4#116.

## In-channel adjacent subband blocking

The following way forward was agreed in RAN4#115:

Agreement - Wanted signal mean power, i.e., degradation level for Refsens level - For conducted in-channel adjacent subband blocking requirement, as defined for in-band blocking requirement, 6dB degradation from Refsens - 6dB degradation on top of OTA REFSENS degradation for in-channel adjacent subband blocking requirement, and own Tx is ON. - On the level of in-channel adjacent subband blocking requirement: - For FR1 WA BS: - [-31+3 = -28] dBm - FFS isolation between sectors can captured in specification - For FR1 MR BS: - [-32] dBm. - FFS isolation between sectors can captured in specification - For FR1 LA BS: - [-34] dBm.

FR1 WA SBFD-capable BS will be an AAS and most commonly deployed in a sectorized setup. Therefore, there will be two sectors causing in-channel adjacent subband blocking. This would be realized in practice by doubling the interference level resulting in an additional 3dB of interference. In our view, as FR1 MR BS’s are also commonly deployed in sectorized setup, this needs to be accounted also for FR1 MR BS.

Sectorized setup is also valid for FR1 MR BS.

Add 3dB also to FR1 MR BS requirement due to sectorized setup.

The WF has a value of -31dBm in the square brackets. According to our analysis in the previous RAN4 meeting, we proposed a value of -21dBm. This contribution was missing when the value of -31dBm in square brackets was proposed. We feel that this level is too low based on our evaluation. More reasonable value would be based on taking all the companies’ contributions into consideration. By calculating the average of all companies’ proposals, the result would be -28dBm. When considering the effect of sectorized deployment, the additional 3dB would be introduced resulting in a value of -25dBm.

Introduce the in-channel adjacent subband blocking level of -25dBm for FR1 WA BS.

# Conclusion

In this contribution, our views on the potentially new BS RF requirements for SBFD operation are presented. The following observations and proposals were listed:

- Sectorized setup is also valid for FR1 MR BS

- Use the existing ACLR absolute limits as the new in-channel adjacent subband leakage requirements.

- Add 3dB also to FR1 MR BS requirement due to sectorized setup.

Introduce the in-channel adjacent subband blocking level of -25dBm for FR1 WA BS.

# References

- R4-2508707 “Way Forward for [115][306] NR_duplex_evo_General”, RAN4#115, Malta, 19th-23rd May, 2025