3GPP TSG-RAN4 Meeting #116	R4-2511571

# Bangalore, India 25th – 29th August, 2025

Source: 	Qualcomm Incorporated

Title: 	CSI Compression: Testability and Interoperability

Agenda item:	7.18.3

Document for:	Discussion

# Introduction

In this paper, we present two types of simulation results:

- Results with fully specified option 3 based on mixed dataset of track 2 with fully specified decoder

- Results with partially specified option 4 based on mixed dataset of track 2 with the reference encoder

We also provide our views regarding option 4, in general.

# Past Agreements

## Model of Encoder-Decoder Pair

Following agreements occurred during the email discussion after RAN4 111 [1].

Based on the two rounds of discussions, the following parameters are proposed: Fig 1. Detailed model diagrams of encoder. Fig 2. Detailed model diagrams of decoder. Table 1. CNN model parameters for encoder. Model architecture parameters for encoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 47M (vs. 1.6M for eTypeII). Model size; 448Kbyte. Num of Resnet blocks; 6. Layer after resnet blocks; 2 layers (one FC layer, one Conv layer). Layer before resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,32). Size of message (feature map) passed across layers; (13,32). Table 2. CNN model parameters for decoder. Model architecture parameters for decoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 740M. Model size; 7.2Mbyte. Num of Resnet blocks; 6. Layer before resnet blocks; 2 layers (one FC layer, one Conv layer). Layer after resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,128). Size of message (feature map) passed across layers; (13,32). Other model related parameters: Activation function; ReLU. Latent message size; 32 float values. Payload size, bits; 64 per layer.. Quantization; 2 bits. System parameters: Frequency range; 2GHz (4GHz optional). MIMO Layers; 1 layer (2 layers optional, companies to. Sub-bands; 13. Channel estimation; Ideal. Metrics; SGCS of AI CSI feedback and eTypeII CSI feedback, both relative to ideal CSI.. Companies are also invited to provide the training parameters used.

Following agreements happened a couple of meeting ago [2].

Parameter/Configurations; Configuration for alignment. Channel/data-set related. UE antenna element assumptions; Ideal omni-directional antenna elements following RAN1 assumptions. Sub-band channel averaging; - Average of covariance channel matrices per PRBs in the sub-band - 1 sample per PRB is selected. Any other pre-processing; - Apply phase normalization: - Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - i.e. the phase of the first value of the eigenvector is 0. - L2 norm of the eigenvector is equal to 1.. Data collection assumptions; - Strive to ensure that the samples are decorrelated, especially when collected from the same UEs.. Training dataset size; - Aim to use the dataset around 600K. - At least 100K sample to be used.. Dataset quantization; - Float32. Other data-set characteristics/statistics; - Companies to propose. - Companies are encouraged to report some statistics about the dataset. - Statistics are FFS - Companies are encouraged to report the CDF of the SGCS across sub-bands and input samples. - If possible, also report CDF for eTypeII.. Model-related. Quantization of encoder output; - Report results without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report - Companies still need to report quantization results: - Companies to report whether/how quantization aware training is implemented (e.g., how quantization is treated at back propagation) - Companies can try quantization aware training with bypassing of the quantization in back propagation - Quantization options, companies to provide the details: - Fixed unform quantization with equal ranges - Trainable quantization. Normalization before quantization in the encoder; - For the results with quantization, to use a sigmoid function in the encoder for normalization - Sigmoid in the decoder is optional.. Use and configuration of drop-out; - Not to use drop-out layers. Training loss function, use of regularization; - Use (1-SGCS) as loss function as reference. Optimizer; - Use Adam. Learning rate. - Fixed learning rate of 10^-4. Batch size. - 256. (Max) Number of epochs. - 150 epochs. Stopping criteria. - No improvement over 25 epochs.. Weight initialization. - Default. Kernel Parameters: - Dilation - Padding Type The value of groups (VG); - Default. Additional data to be submitted: - CDF of SGCS per subband - CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format)

RAN4 agreed to the following during the last meeting [3]:

Issue 4-1: Simulation results discussion for Option 3 … Additional data to be submitted: - CDF of SGCS per subband - CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format)

## System Parameters

The system parameter assumptions of 38.843 were considered to be baseline for RAN4 performance evaluation.

Table 6.2.1-1: Baseline System Level Simulation assumptions for AI/ML based CSI feedback enhancement evaluations

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g., UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 2GHz as baseline, optional for 4GHz (if R16 as baseline) FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz (if R17 as baseline) |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz (if R16 as baseline) 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz (if R17 as baseline) |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU. |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g., 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | At least, FTP model 1 with packet size 0.5 Mbytes is assumed. Other options are not precluded |
| Traffic load (Resource utilization) |  | 20/50/70%. Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) CSI prediction: 100% outdoor (10, 20, 30, 60, 120 km/h) including outdoor-to-indoor car penetration loss per TR 38.901 if the simulation assumes UEs inside vehicles. No explicit trajectory modeling considered for evaluations. |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline. Up to companies to choose the error modelling method for realistic channel estimation. Ideal DL channel estimation is optionally taken into the baseline of evaluation methodology for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Up to companies to report whether/how ideal channel is used in the dataset construction and performance evaluation/inference. Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation. |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. The CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. - For AI/ML based solutions: The above-mentioned "CSI feedback overhead" is calculated as max allowed bits at the given rank. - For legacy Type II CB: Option 2b is mandatorily reported by companies, while Option 2a can be optionally reported up to companies if partial NZC report is assumed for the legacy Type II CB - Option 2a: The above-mentioned "CSI feedback overhead" is calculated as each CSI reported payload with a given rank - Option 2b: The above-mentioned "CSI feedback overhead" is calculated as max allowed bits at the given rank Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | For CSI compression: Companies need to report which option is used between: - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. Additional assumptions from R17 TypeII EVM: Same consideration with respect to utilizing angle-delay reciprocity should be considered for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline. Optionally, Type I Codebook (if it outperforms Type II Codebook) can be considered for comparing AI/ML schemes. For CSI-prediction: Both of the following are taken as baseline: - The nearest historical CSI without prediction - Non-AI/ML or AI/ML with collaboration Level x based CSI prediction for which corresponding details would need to be reported Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example. For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on CSI without compression (e.g., eigenvector from measured channel), which is taken as an upper bound for performance comparison. |
| Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions. The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations. |  |  |

## Different Options for Fully and Partially Specified Encoeder-Decoder Pair

RAN4 agreed to the following recently [4].

Issue 4-1: Option 3 (fully specified decoder) next steps - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

Issue 1-2: Option 3 next step - decoder(s) selection choose the decoders with Low, medium, high SGCS Agreement: among the decoders submitted by companies Continue the feasibility study on the two tracks below: Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset) - The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s). - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

RAN4 made the following agreement regarding reference encoder decoder pair a couple of meeting ago [4]

Issue 4-4: Options for Option 4 Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

Issue 1-3: Option 4a(Dataset based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3 - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1 - Or mixed dataset from track 2 - Step 5: Label selected dataset(s) (mapping of encoder input to latent message) [using encoder] corresponding to the selected test decoder(s) from option 3 - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s) - Performance alignment to be checked/discussed - Step 7: Conclude on overall feasibility of Option 4a - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders(using different structures) Selected test decoder and matching encoder pair from Option 3 can be used to generate the common dataset (that includes latent messages) based on aggregated/mixed dataset

Issue 1-4: Option 4b (encoder based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more encoder(s) for further analysis based on option 3 - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1 - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2 - Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s) - Performance alignment to be checked/discussed - Step 6: Conclude on overall feasibility of Option 4b - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)

## Parameters to be Specified and Captured for Option 3 and Option 4

### Option 3

Agreements from RAN4#114bis

- The encoder that corresponds to the standardized test decoder is stored within 3GPP

  - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS)

  - FFS whether several encoders would be captured (e.g. due to different complexity)

New agreement from RAN4#115:

The training dataset used to train the test decoder in 3GPP should be captured

- FFS whether channel, Eigenvectors or something else

### Option 4a

Agreements from RAN4#114bis

- Dataset is specified for option 4a.

- Assume reference decoder is captured.

New agreements from RAN4#115:

- Reference encoder should be captured

- FFS whether the labelled dataset is based on Eigenvector, channel estimation or something else

- The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure.

  - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters.

  - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated

### Option 4b

Agreements from RAN4#114bis

- Dataset is specified for option 4a.

- FFS on dataset is captured for option 4b.

- Assume reference decoder is captured.

- FFS on reference encoder for option 4a

- Test decoder structure is (i) fully specified or (ii) partially specified

- NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase.

New agreements from RAN4#115:

- Specify dataset to train frozen encoder and decoder

    - TE vendor shall use the dataset to train the test decoder

    - This is the same dataset as used to train the frozen encoder

    - FFS whether the datasets consist of Eigenvector, channel or something else

- The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure.

  - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters.

  - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated

# Discussion and Results

## Simulation Results with Option 3

Table 1: Simulation result with Option 3 Track 2 (resubmission from last meeting)

| Option 3 |  |  |  |
| --- | --- | --- | --- |
|  |  | Joint training (QC encoder with QC decoder) | New QC encoder training with (Samsung’s) frozen decoder |
|  |  | Training and testing on mixed | Training and testing on mixed |
| RAN4 Agreed encoder structure, CNN | Qualcomm | 0.683 | 0.7005 |

Table 1 shows the SGCS results with track 2, Qualcomm encoder-decoder with joint training and Qualcomm encoder based on Samsung’s decoder. Following steps were performed to generate the result of right hand side of Table 1.

- Samsung’s decoder is obtained from [9] (uploaded after RAN4 114 meeting) and considered as a frozen decoder.

- Qualcomm’s encoder is trained with Samsung’s frozen decoder based on mixed dataset of [10] (generated and uploaded after RAN4 114 meeting)

- Trained Qualcomm encoder from previous step and Samsung’s frozen decoder is tested based on mixed dataset of [10]

Table 1 shows that the SGCS obtained with separate encoder decoder training (QC encoder with Samsung’s frozen decoder) is very close to the SGCS obtained with Qualcomm’s joint encoder-decoder training. Hence, fully specified decoder in track 2 with mixed dataset seems to be a feasible approach.

Observation 1: Option 3 (fully specified decoder) leads to good SGCS in, at least, the scenario where encoder is trained with the same dataset as the decoder.

## Simulation Results with Option 4

The following steps were performed to evaluate the feasibility of option 4:

- Take Samsung reference encoder and mixed dataset. Use these to “label” the (encoder input, encoder output) training dataset. Train a new “own” nominal decoder using the labelled dataset. Then train an “own” encoder with the newly trained decoder

- Test “own” encoder with the nominal decoder of all companies, including own “nominal” decoder

  - Note: Nominal decoders of all companies are generated based on Samsung’s reference encoder

Table 2 shows that the SGCS obtained with all other companies’ nominal decoders are quite aligned and they are very similar to the SGCS obtained with Qualcomm’s own encoder and own decoder/

Table 2: Simulation results with Option 4 reference encoder with mixed dataset (resubmission from last meeting)

| Option 4 Table shows result of ‘own encoder’ with ‘own decoder’ and other companies’ nominal decoders (all developed based on Samsung’s reference encoder) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Dataset1=Dataset2=Dataset3=mixed |  |  |  |  |  |
|  |  | Qualcomm | MediaTek | VIVO | Nokia | Huawei, Hisilicon | Ericsson |
| RAN4 Agreed encoder, CNN | Qualcomm | 0.6972 | 0.6832 | 0.6894 | 0.6826 | 0.701 | 0.6909 |

Observation 2: Option 4 leads to good SGCS between company’s own encoder and nominal decoders of all other companies when all encoders and decoders are generated based on Samsung’s reference encoder and mixed dataset.

The following simulations were performed to evaluate the importance of dataset standardization in option 4b:

- Take Samsung reference encoder and mixed dataset. Use these to “label” the (encoder input, encoder output) training dataset. Train a new “own” nominal decoder using the labelled dataset. Then train an “own” encoder with the newly trained decoder

- Test “own” encoder with own “nominal” decoder

Table 3: Simulation results with Option 4 reference encoder with different datasets (new submissions from last meeting)

| Option 4 Table shows result of ‘own encoder’ with ‘own nominal decoder’ with different datasets (Reference encoder comes from Samsung) |  |  |
| --- | --- | --- |
|  | Qualcomm “nominal” decoder Dataset1= Dataset4 = mixed Dataset2=Dataset3=own dataset | Qualcomm “nominal” decoder Dataset1 = Dataset2 = Dataset3 = mixed Dataset4 = own dataset |
| RAN4 Agreed encoder, CNN Qualcomm “own” encoder | 0.4713 | 0.4577 |

Table 3 shows that the performance of option 4b can significantly drop if a different dataset is used during training the nominal decoder or during the tests. That’s why, during the last meeting, RAN4 has made the correct decision to capture the dataset to train the test decoder and frozen “nominal” decoder.

Observation 3: Performance of option 4b can significantly drop if a different dataset is used while training the nominal decoder and the test decoder.

## Preferred Option for Normative Work

During the study item, RAN4 has focused on both fully specified option 3 and partially specified option 4. Recently concluded RAN plenary captured the following texts in the WID of Rel-20 AI-ML NR air interface.

Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

Above texts of WID clearly show that RAN plenary requires RAN4 to define requirement for the fully specified test decoder, i.e., option 3.

A careful look at the agreements related to option 3 and option 4 reveal that both these options are quite similar in terms of capturing or specifying a reference encoder, a reference decoder and a training dataset. The major difference between the two options lies in the fact that option 4 allows test vendor to implement test decoder with different parameters (i.e., weights and biases) or with a partially different structure. But RAN4 has not yet investigated the interoperability in the case that the structure of the TE decoder is not fully specified.

RAN4 115 agreement: - The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure. - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters. - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated

Besides, designing test via option 4 may lead to unintended complications in the tests. Typically, when UE fails a test, UE needs to be debugged because the issue is expected to happen at the UE. In the partially specified option, UE’s failure during a test could stem from UE issue or from a potential TE issue where TE’s selected parameters for the test vendor are not suitable for the reference encoder and decoder. Hence, both UE and TE need to be debugged during a test that focuses on option 4. Hence, RAN4 should just focus on option 3 during the work item.

Observation 4: Recently agreed RAN plenary requires RAN4 to define requirement for the fully specified test decoder, i.e., option 3.

Observation 5: Both option 3 and option 4 are quite similar in terms of capturing or specifying a reference encoder, a reference decoder and a training dataset.

- Note: Option 4 allows test vendor to implement test decoder with different parameters or with a partially different structure

Observation 6: In option 4, UE’s failure during a test could stem from UE issue or from a potential TE issue where TE’s selected parameters for the test vendor are not compatible for the reference encoder and decoder

Proposal 1: RAN4 follows the guidance of the WID of Rel-20 AI/ML and focuses only on fully specified decoder option 3 during the WI.

## Channel Selection For Normative Work

RAN4 has, so far, focused on system level channel to simulate and investigate the performance of CSI compression. System level channel is more realistic for field deployment. RAN plenary has also agreed to use both the structure and parameters of fully defined encoder/decoder in option C and just the structure in option 4a-1.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

On the other hand, RAN4 has not calibrated system level channel for demod testing. RAN4 has only calibrated TDL based channel models for demod testing. The actual performance test can only happen in a link level TDL channel to avoid miscalibration issue.

Typically, RAN4 would select a calibrated dataset for testing and defining performance requirement. However, due to field deployment consideration and option C/A’s dependence on RAN4, RAN4 should discuss whether to consider system level channel to design the fully defined/specified reference model of “Direction 3” and the standardized encoder model structure of “Direction A, sub-option 3a-1”.

Observation 7: RAN4 has not calibrated system level channel for PMI testing. RAN4 has, so far, only calibrated TDL based channel models for PMI testing.

Observation 8: RAN plenary has also agreed to use both the structure and parameters of fully defined encoder/decoder in option C and just the structure in option 4a-1.

Proposal 2: RAN4 selects TDL channel to test two-sided CSI feedback mechanism of Rel-20.

Proposal 3: RAN4 discusses which of following channels to consider in the training data set to design the fully defined/specified reference model of “Direction C”.

- Link level channel (e.g. TDL)

- System level channel

- Generalized dataset containing dataset of system level plus link level channel

## Simulation Results with Low Complexity Encoder-Decoder

RAN4 captured the following during the last meeting [12]:

Further simulations (not agreement): In order to further elaborate option 3 track 2 considering lower complexity encoders, interested companies may contribute simulations to RAN4#116. The following guidance, and also the guidance in R4-2508084 is provided and recommended for companies to follow in order to obtained aligned simulations. For option 3, track 2 consider the following investigations for lower complexity encoder structures. - Performance assuming training and testing with the Samsung frozen decoder - Performance achieved with a jointly trained low complexity encoder and decoder - Performance achieved when a frozen decoder is trained based on all of the encoder structures (i.e. different encoder complexity) - Consider transformer, CNN, MLP

RAN1 has studied the feasibility of scalable model structure specification by adopting transformer as backbone. RAN1 considered transformer-based encoder structure of Figure 2 as an example in its study of feasibility and scalability. We use this transformer model to evaluate the performance of Option 3 track 2 for both joint and sequential training.

Figure 2: RAN1’s considered transformer based encoder structure for scalability and feasibility study

Table 2: SGCS performance of option 3 track 2 mixed dataset based training and testing (# of Flops = 8M)

| Training type | Enc-model | Dec-model | SGCS | Ref SGCS of SAM Enc-Dec |
| --- | --- | --- | --- | --- |
| joint training | TF | TF | 0.7084 | 0.7016 |
| joint training | TF | CNN | 0.7087 |  |
| seq training | TF | CNN (Samsung Decoder) | 0.7028 |  |

Table 2 shows that transformer-based encoder-decoder architecture can achieve similar SGCS as CNN based encoder-decoder architecture. The required number of flops for the considered transformer-based architecture is much smaller compared to the CNN based architecture that RAN4 is currently considering in study item. Both UE and TE vendors have no reason to consider a more complex model if an encoder-decoder with lower complexity can achieve same performance. Hence, RAN4 should use transformer as the backbone structure to define the standardized test structure in RAN4, and “Dir C” and “Direction A, sub-option 3a-1” in RAN4 during the work item.

Observation 9: RAN1 considered transformer-based encoder structure of Figure 2 as an example in its study of feasibility and scalability.

Observation 10: Transformer-based encoder-decoder architecture can achieve similar SGCS as CNN based encoder-decoder architecture.

- Sequential training of transformer-based encoder and CNN decoder generates same SGCS as CNN based encoder-decoder.

Observation 11: The required number of flops for the considered transformer-based architecture is much smaller compared to the CNN based architecture that RAN4 is currently considering in study item.

Proposal 4: RAN4 uses transformer as the backbone structure to define the standardized test structure in RAN4,  and “Dir C” and “Direction A, sub-option 3a-1” in RAN4 during the work item.

# Conclusions

Observation 1: Option 3 (fully specified decoder) leads to good SGCS in, at least, the scenario where encoder is trained with the same dataset as the decoder.

Observation 2: Option 4 leads to good SGCS between company’s own encoder and nominal decoders of all other companies when all encoders and decoders are generated based on Samsung’s reference encoder and mixed dataset.

Observation 3: Performance of option 4b can significantly drop if a different dataset is used while training the nominal decoder and the test decoder.

Observation 4: Recently agreed RAN plenary requires RAN4 to define requirement for the fully specified test decoder, i.e., option 3.

Observation 5: Both option 3 and option 4 are quite similar in terms of capturing or specifying a reference encoder, a reference decoder and a training dataset.

- Note: Option 4 allows test vendor to implement test decoder with different parameters or with a partially different structure

Observation 6: In option 4, UE’s failure during a test could stem from UE issue or from a potential TE issue where TE’s selected parameters for the test vendor are not compatible for the reference encoder and decoder

Observation 7: RAN4 has not calibrated system level channel for PMI testing. RAN4 has, so far, only calibrated TDL based channel models for PMI testing.

Observation 8: RAN plenary has also agreed to use both the structure and parameters of fully defined encoder/decoder in option C and just the structure in option 4a-1.

Observation 9: RAN1 considered transformer-based encoder structure of Figure 2 as an example in its study of feasibility and scalability.

Observation 10: Transformer-based encoder-decoder architecture can achieve similar SGCS as CNN based encoder-decoder architecture.

- Sequential training of transformer-based encoder and CNN decoder generates same SGCS as CNN based encoder-decoder.

Observation 11: The required number of flops for the considered transformer-based architecture is much smaller compared to the CNN based architecture that RAN4 is currently considering in study item.

Proposal 1: RAN4 follows the guidance of the WID of Rel-20 AI/ML and focuses only on fully specified decoder of option 3 during the WI.

Proposal 2: RAN4 selects TDL channel to test two-sided CSI feedback mechanism of Rel-20.

Proposal 3: RAN4 discusses which of following channels to consider in the training data set to design the fully defined/specified reference model of “Direction C”.

- Link level channel (e.g. TDL)

- System level channel

- Generalized dataset containing dataset of system level plus link level channel

Proposal 4: Select transformer as the backbone structure to define the standardized test structure of RAN4,  and the structure of “Dir C” and “Direction A, sub-option 3a-1” of RAN1 during the work item.

# References

[1] R4-[111][133] NR_AIML_air-email discussions after RAN4#111 meeting, https://list.etsi.org/scripts/wa.exe?A2=3GPP_TSG_RAN_WG4;aa5369d9.2406C&S=

[2] R4-2414447, “Alignment of model training and data set parameters for AI/ML based CSI compression”, Nokia,  3GPP TSG-RAN4 Meeting #112.

[3] R4-2413169, “CSI Compression: Testability and Interoperability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112.

[4] R4-2414323, “WF on AI/ML”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112

[5] R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #112bis.

[6] R4-2415889, “CSI Compression: Testability and Interoperability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112-bis

[7]https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_113/Inbox/Drafts/%5B113%5D%5B129%5D%20NR_AIML_air_part1/CSI_Compression_Results_Summary/%5B113%5D%5B129%5DAIML_Summary_of_CSI_Compression_Sim_Results_v3_Nokia2_QC2.xlsx

[8]https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression/Datasets/R4_113

[9] https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression/Models/R4_114

[10] https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression/Datasets/R4_114

[11] R4-2505107, WF on AI-ML for air interface study, 3GPP TSG WG RAN1 #114bis.

[12] R4-2508051, WF on AI/ML part 2, 3GPP TSG WG RAN1 #115.

[13] RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, 3GPP TSG RAN Meeting #108.