3GPP TSG-RAN WG4 Meeting #116	R4-2511649

Bengaluru, IN, August 25 – 29, 2025

Agenda Item:	7.14.3.1

Source:	MediaTek Inc.

Title:	Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The objective of R19 NR RRM Phase-5 work item is captured below in the WID [1]. In this paper, we continue the discussion on the remaining issues in first objective, particularly on optimizing Rx beam sweeping factor for L3 measurement delay reduction when UE support multiple-Rx simultaneous reception.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

In the last meeting, RAN4 discussed the conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” as captured in the WF [2]:

### Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1. Way forward: For re-evaluation mechanism for scenario 1/2/3/4: - Check whether the following mechanism is agreeable: - UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode.

The remaining issue is regarding the re-evaluation of the triggering conditions to activate multi-Rx for L3 measurement for scenarios (1/2/3/4) again after quitting. This will depend on whether UE quit based on UAI or based on timer expiration. If UE quit based on UAI e.g., due to overheating issue, then UE can decide its readiness to activate multi-Rx again by indicating “multiple” to re-enter multi-Rx mode (as shown below) when the activation conditions of multi-Rx are still met.

multiRx-PreferenceFR2 Indicates the UE's preference on single FR2 Rx operation to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2..

Proposal 1: If UE quit based on UAI e.g., due to overheating, UE can decide its readiness to re-enter multi-Rx mode again by indicating “multiple” in the UAI when the activation conditions of multi-Rx are still met.

On the other hand, if the quit was due to the expiration of the timer, UE can to re-evaluate the triggering conditions to activate multi-Rx for L3 measurement again and restart the timer if the conditions are met.

Proposal 2: If UE quit due to timer expiration, UE can to re-evaluate the triggering conditions to activate multi-Rx for L3 measurement again and restart the timer if the conditions are met.

# Summary

In this contribution, the following observations and proposals are concluded:

Proposal 1: If UE quit based on UAI e.g., due to overheating, UE can decide its readiness to re-enter multi-Rx mode again by indicating “multiple” in the UAI when the activation conditions of multi-Rx are still met.

Proposal 2: If UE quit due to timer expiration, UE can to re-evaluate the triggering conditions to activate multi-Rx for L3 measurement again and restart the timer if the conditions are met.

# References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2508266, WF on RRM requirements for NR_RRM_Ph5_Part2