3GPP TSG-RAN WG4 Meeting #116	R4-2510048

Bangalore, India, 25th - 29th Aug, 2025

Source:	Samsung, KDDI, LGE

Title:	TP to TR 38.746 for CA_n74A-n77(2A) with UL n77(2A) PC2

Agenda item:			6.8.3

Document for:	Approval

- Introduction

The revised basket WID was approved in RAN#108[1].

CA_n74A-n77(2A) with n77(2A) PC3 is submitted in this meeting in TP R4-2510047.

- References

[1] RP-251450, Revised WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)

Text Proposal

<Start of Text Proposal>

5.X	CA_n74-n77

5.X.1	Configurations

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n74A-n77(2A) | n77(2A)8 | n74 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

5.X.2	Maximum output power

Already in clause 5.X.1.

5.X.3	REFSENS requirements

Based on co-existence studies presented for PC3(in R4-2510047), IMD6/7 of ULCA_n77(2A) falls into the n74DL, and the MSD requirements are defined as following.

Table 5.x.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n74-n77 | n74 | N/A | 5 | N/A | 1500 | 4.6 | FDD | IMD6 |
|  | n7712 | 3505 | 10 | 1(Rbstart=26) | 3505 | N/A | TDD | N/A |
|  |  | 4005 | 10 | 1(Rbstart=26) | 4005 |  |  |  |
|  | n74 | N/A | 5 | N/A | 1505 | 1.7 | FDD | IMD7 |
|  | n7712 | 3305 | 10 | 1(Rbstart=26) | 3305 | N/A | TDD | N/A |
|  |  | 3905 | 10 | 1(Rbstart=26) | 3905 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

5.X.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

<End of Text Proposal>