3GPP TSG-RAN Working Group 4 (Radio) Meeting #116						R4-2512573

# Bangalore, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.941 | CR | - | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.941: Addition of technical background for enhancement of co-location requirements in clause 6.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This draft CR is created based on structure agreed last meeting in R4-2508785 and findings from Rel-19 work. Technical background and frindings from the coupling loss evalaution (based on measurements and simulations) is collected. This is a revision of R4-2510228. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of technical background in clause 6.4.4.1. - Addition of measurement and simulation results in clause 6.4.4.2. - Addition of technical background for individual co-location requirements in clause 6.4.4.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the outcome of Rel-19 work will not be captured in technical report. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Clause 6.4:

## 6.4	Co-location measurements

### 6.4.1	General

Co-location measurements are required for the OTA co-location requirements which are based on assuming the BS is co-located with another BS of the same base station class. They ensure that both co-located systems can operate with minimal degradation to each other.

Co-location requirements are specified as power levels into or out of the conducted interface of the co-location reference antenna. For conformance testing the requirements are translated to the input or output of a co-location test antenna (CLTA).

CLTA is a practical passive antenna that is used for conformance testing of the co-location requirements and is based on the definition of the co-location reference antenna. Basic principle of the CLTA are outlines in clause 6.4.2. For more details on the co-location test antenna characteristics, refer to TS 37.145-2 [4] clause 4.15.2.2, or TS 38.141-2 [6] clause 4.12.2.2. The CLTA is suitable for testing BS implemented with a planar antenna array.

The alignment between the BS under test and the co-location test antenna is depicted in figure 6.4.1-1. For more details on the co-location test antenna alignment and test arrangement, refer to TS 37.145-2 [4] clause 4.15.2.3, or TS 38.141-2 [6] clause 4.12.2.3. The same physical alignment applies to in-band and out-of-band co-location requirements.

Co-location requirements are not applicable to FR2.

Figure 6.4.1-1: Alignment of BS and CLTA

### 6.4.2	Co-location test antenna

The co-location test antenna is a practical passive antenna based on the definition of the co-location reference antenna, and it is used for conformance testing of the co-location requirements. Thus the CLTA should be within a certain tolerance of the co-location reference antenna definition, so that commonly available passive BS antennas can be used to test the co-location requirements without having to use a large number of test antennas and without having to design specialized co-location test antennas.

For example, where the co-location reference antenna definition specifies that the co-location reference antenna, should be a single column passive antenna which has the same vertical radiating dimension (h) as the composite antenna of BS, this would be very restrictive when selecting a test antenna. Hence the CLTA should be within a certain tolerance for the vertical radiating dimension (h) such that a suitable practical BS antenna can be found for each BS.

The CLTA is hence defined as follows:

-	The CLTA is a single column passive antenna which has a vertical radiating dimension (h) which is equal to the vertical radiating dimension of the BS composite antenna ±30 %.

-	For in-band requirements, the CLTA supports the same frequency range and polarizations as the BS composite antenna.

-	For out-of-band co-location requirements, the half power vertical beamwidth of the CLTA equals the narrowest declared vertical beamwidth ±3 °.

-	The polarization should match the in-band CLTA polarization.

-	The CLTA has a horizontal half-power beamwidth of 65 ° ± 10 °.

-	The front faces of the BS and the CLTA need to be aligned in a common plane perpendicular to the mechanical bore-sight direction.

-	The centre of the vertical radiating regions of the CLTA and the BS composite antenna need to be aligned.

### 6.4.3	Standard test antenna

Co-location requirements may also be tested with a standardised test antenna such as a dipole. This has a number of advantages:

-	Standard test antennas are easily available and easier to specify.

-	Repeatability between different conformance testing runs will be greater.

-	Using standard test antennas, fewer antennas will be required for conformance testing.

-	A standard test antenna can have higher gain in the direction of the BS than a CLTA, so that signal levels can be higher relative to the noise floor in emission measurements and lower test levels can be used in interference measurements.

In order to test with a standard test antenna, the translation between measured power levels in the standard test antenna and the co-location reference antenna must be shown. The method for translating the power levels in the co-location reference antenna to the standard test antenna and vice versa is not covered by the present release of this technical report.

### Co-location measurements for higher frequency bands

#### Background on CLTA alternatives discussion

#### BS-to-BS isolation analysis

The BS-to-BS isolation between two co-located base stations have been evaluated from OTA measurement and OTA co-loation simulation results. The coupling loss have been evaluated for intra-band and inter-band scenarios, considering the coupling loss as sub-Array-to-Sub-Array (SA-to-SA) coupling loss, Array-to-Sub-Array (A-to-SA) coupling loss and Array-to-Array (A-to-A) coupling loss.

Coupling loss for different scenarios is listed in Table 6.4.4.2-1 and Table 6.4.4.2-2.

Table 6.4.4.2-1: Intra band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| Vertical Separation | SA-to-SA | M: 54 | M: 53 | M: 56 |  |
| Horizontal Separation | SA-to-SA | M: 31 | M: 34 | M: 46 |  |
|  | A-to-SA |  | M: 35 S: 55±15 | M: 51 M: 69 | M: 55 S: 70±15 |
|  | A-to-A |  |  |  | S: 77±15 |
| M: measurement results S: simulation results. |  |  |  |  |  |

Table 6.4.4.2-2: Inter band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |
| --- | --- | --- |
|  |  | 3.5 GHz to 7 GHz |
| Horizontal separation | A-to-SA | S: 57±10 |
|  | A-to-A | S: 54±10 |
| M: measurement results S: simulation results. |  |  |

The outcome of coupling loss evaluation indicates that the assumption taken from the conducted co-location scenario where BS-to-BS port coupling loss of 30 dB for BS type 1-C at high FR1 frequencies is not relevant anymore.

From the combined result from measurements and simulations it can be concluded that the general trend indicates coupling loss increase for bands in the upper region of FR1.