3GPP TSG-RAN WG4 Meeting #116                                                    	R4-2512852

Bengaluru, India, August 26 – 29, 2025

Source:	ZTE Corporation, Sanechips

Title:	TP for TR 37.719-21-11 to introduce 2L1N EN-DC configuration DC_7A-20A_n7A

Agenda item:			6.2.3 (DC_R19_xBLTE_yBNR_3DL2UL)

Document for:	Approval

# 1. Introduction

The 2L1N EN-DC configuration DC_7A-20A_n7A was requested in Rel-19 basket WID [1]. This TP is to introduce DC_7A-20A_n7A to TR 37.719-11-11.

Proposal 1.  It is suggested to approve the following TP for DC_7A-20A_n7A to TR 37.719-21-11.

2. Reference

[1] RP-250966, Revised WID: Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands, Nokia, CHTTL, LG Electronics, Samsung.

[2] TR 37.719-21-11, Rel-19 Dual Connectivity of EN-DC and NE-DC configurations consisting of 2 bands LTE inter-band CA (2DL/1UL) and 1 band NR (1DL/1UL), or consisting of 1 band LTE (1DL/1UL) and 2 bands NR inter-band CA (2DL/1UL)

# 3. Text Proposal

<Start of Text Proposal>

6.x	DC_7-20_n7

6.x.1	Configurations for DC

Table 6.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE X) |
| --- | --- |
| DC_7A-20A_n7A | DC_7A_n7A2 DC_20A_n7A |
| NOTE 2: Only single switched UL is supported |  |

6.x.2	Co-existence analysis for DC

Table 6.x.2-1: Band 20 and Band n7 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1638 – 1738 |  | 3332 – 3432 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 776 – 906 |  | 4138 – 4308 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4164 – 4294 |  | 5832 – 6002 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 – 86 |  | 6638 – 6878 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3276 – 3476 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4996 – 5156 |  | 8332 – 8572 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6664– 6864 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9138 – 9448 |  | 758 – 948 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5776 – 6046 |  | 2414 – 2644 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10832 – 11142 |  | 5828 – 6018 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9164 – 9434 |  | 7496 – 7726 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.x.2-1, 3rd IMD and 5th IMD product of band 20 and band n7 may fall into the DL of band 20, and it has been specified for the fallback combination DC_20_n7. And no IMD product of band 20 and band n7 may fall into Rx frequencies of band 7, given that there is no simultaneous Rx/Tx between n7 UL and B7 DL.

6.x.3	∆TIB and ∆RIB values

Table 6.x.3-1: ΔTIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| DC_7-20_n7 | 0.3 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

Table 6.x.3-2: ΔRIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| DC_7-20_n7 | - | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

6.x.4	Analysis of MSD requirements

To follow the similar MSD values as used for DC_20A_n7A in current spec, the MSD for DC_7A-20A_n7A are given in the table as below.

Table 6.x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-20A_n7A | 7 | N/A | 5 | N/A | 2631.5 | 2.5 | IMD5 |
|  | 20 | 834.5 | 5 | 25 | 793.5 | N/A | N/A |
|  | n7 | 2567.5 | 5 | 25 | 2687.5 | N/A | N/A |

<End of Text Proposal>