3GPP TSG-RAN WG4 #116BIS Meeting						       R4-2513038

Prague, Czech, 13th – 17th October 2025

Agenda Item: 	8.6

Source: 	MediaTek Inc.

Title: 	6G Candidate Spectrums

Document for: 	Discussion and Decision

# Introduction

Following the 6G SI [1] approved in RAN#108, the first 6G discussion starts from this RAN4 meeting. In this paper, we provide our views on the 6G candidate spectrum, 6G band combination scheme to simply support CA and DC and others aspects:

- 6G candidate Spectrum

- Band combination definition/simplification

- UL/DL decoupling

# 6G Candidate Spectrum

The following spectrums are considered in both WRC-23 and WRC-27 for new IMT (6G) telecommunication service. For new IMT (6G) service, the candidate bands are listed with 4GHz/7GHz/8GHz/14GHz in AI 1.7 in WRC-27 as following spectrum plan;

Figure 1: 6G Candidate spectrums based on WRC-23 and WRC-27 Agenda for spectrum

Figure 2: ITU-R recommendation for detailed maps for all Regions globally

## Candidate 6G spectrum in Region 1

For the Region1, the candidate 3.3~3.4 GHz band will be used for IMT Mobile communication over 17 countries regardless of 5G or 6G service. Their government can be relaxed some limitation of the spectrum usages in near future.

Russia also will be considered to use the 4.4~4.8GHz spectrum for 6G IMT service since WRC already decided this spectrum is harmonized NATO usage in Europe.

In Europe, the n102 (5925~6425 MHz) will be still considered as the unlicensed spectrum for WiFi and Bluetooth. However, the n104 (6425~7125MHz) will be considered as a licensed spectrum for IMT mobile communication service in Europe, as is the spectrum usage in China. Hence, the spectrum ranges of 6425~7125MHz will be considered as 6G IMT candidate spectrum in Europe.

For the WRC-27, the 7125~8.4 GHz and 14.8~15.35 GHz will be further discussed. However, NATO will be considered as military spectrum usage for those two spectrum ranges 7.25~8.4GHz and 14.8 ~15.35 GHz which spectrum would not support as IMT service due to NATO military spectrum usage.

Based on these reports, we can share the following observation

Observation 1: The following spectrum will be considered as a IMT (6G) Candidate spectrum in region 1.

- 3.3~3.4 GHz

- 4.4 ~4.8 GHz (Russia only)

- 6.425 ~7.125 GHz

- [7.125 ~7.25GHz] if feasibility study between military service and IMT service are proved, then these frequency range can be used for IMT service. (except with 7.25~8.4GHz as military service)

## Candidate 6G spectrum in Region 2

For the Region2, the candidate 3.3~3.8 GHz band will be used for IMT Mobile communication over 15 countries. Their government can be relaxed some limitation of the spectrum usages in near future.

US FCC and other America countries also has auction plan to use the 4.4~4.9 GHz spectrum for 6G IMT service with limitation to protect legacy system.

In US, the new government announced OBBB (One Big Beautiful Bill) spectrum auction plan between 1.3 ~ 10.5 GHz except 3.1 ~3.45 GHz and 7.4 ~8.4GHz (as military satellites usage) at July 2025.

- Competitive bidding for full-power commercial licensed usage in US (1.3 ~10.5 GHz)

  - 3.98 ~ 4.2 GHz

  - 2.7 ~ 2.9 GHz

  - 4.4 ~ 4.9 GHz

  - 7.25 ~ 7.4 GHz

  - 6.425 ~7.125 GHz: NTIA identification activity between Wi-Fi and IMT usage (FCC Changed the previous decision that only allow for unlicensed spectrum usage)

For the WRC-27, the 14.8 ~ 15.35 GHz will be further discussed in Region 2. However, this spectrum usage is not clear auction planning and development plan in Region 2 until now.

Based on these reports, we can share the following observation

Observation 2: The following spectrum will be considered as a IMT (6G) Candidate spectrum in region 2.

- 2.7 ~ 2.9 GHz

- 3.98 ~ 4.2 GHz

- 4.4 ~ 4.9 GHz

- 6.425 ~ 7.125 GHz (only Brazil, Mexico, Still further identification between WiFi and IMT in US)

- 7.25 ~ 7.4 GHz

## Candidate 6G spectrum in Region 3

For the Region3, the candidate 4.8~4.499 GHz band will be used for IMT Mobile communication over 40 countries. Their government can be relaxed some limitation of the spectrum usages in near future.

In China, the 6.425 ~ 7.125 GHz is main target for 6G IMT usage. It was some different between US and China. However, US FCC still ongoing discussion by NTIA identification activity between WiFi and IMT part to finalize the spectrum usage.

In Korea/Japan, they still not decide the spectrum usage since US government could be changed the spectrum usage for 6.425 ~7.125 GHz. If the activity is not change the legacy decision, the IMT candidate spectrum is only 7.25 ~7.4 GHz in around 7 ~8 GHz if they follow the US government decision.

Observation 3: The following spectrum will be considered as a IMT (6G) Candidate spectrum in region 3.

- 4.8 ~ 4.99 GHz

- 6.425 ~ 7.125 GHz (China and a few South Asia countries)

- [7.125 ~ 8.4GHz] (Korea/Japan still not decide the spectrum usage as same US)

## TN and NTN candidate spectrum in WRC-27

In WRC-27 agenda Item, there are two approaches to treat new mobile satellite services as follow

- Option 1: Mobile Satellite Service (MSS) spectrum: Define NTN spectrum as like in 5G

  - Define new MSS spectrum in 6G candidate spectrums

- Option 2: Mobile Service (MS) spectrum :

  - Use terrestrial spectrum : from 694MHz ~2.7GHz, complement terrestrial mobile coverage at IMT band

  - Need to discuss on the regulatory framework and parameters need to define the PSD threshold for protecting TN system.

Observation 4: TN/NTN harmonized bands will be further discussed based on WRC-27 decision. RAN4 need to discuss how can define the PSD threshold to protect terrestrial IMT system when ITU-R decide with option 2.

Proposal 1: For the Upper 6GHz and the lower FR3 (7.125 ~ 8.4GHz) will consider as 6G target spectrum in early stage. The FR3 mid band (10~15.35GHz) can be consider in later stage.

Proposal 2: Based on Proposal 1, RAN4 can extend FR1 definition from 410~7125MHz to 410 ~ 8400MHz in 6G.

# Band combination definition/simplification

For 6G Specification simplification, RAN4 need to consider simple way to specify the CA/DC UL/DL configuration.

Similar concept is now discussed in RAN1, focusing on the baseband aspects, e.g., a single cell with multiple carriers across multiple operating bands. The impact of such a concept to RAN4 RF requirement is not clear now. Further consider it as a way to simplify the band combo needs more discussions.

In 5G (NR) cellular communication system, carrier aggregation (CA) band combinations had been widely introduced in current specs in 38.101-1 for FR1, 38101-2 for FR2 and 38.101-3 for MR-DC. In 5G, the CA framework is used where there is always a one-to-one mapping between a logical cell and a physical carrier. Also, there are thousands of CA combination in the existing specs. The large number of 5G NR CA/DC/MRDC combinations bring heavy signalling overhead in air interface. UE-specific signaling and carriers configurations complexity is proportional with the number of aggregated carriers. All of these leads to unnecessary system overhead.

To reduce the large overhead, in the RAN1 discussion, it is considered to decouple one-to-one mapping between a physical carrier and a logical cell and enable multiple carriers with similar channel properties to be allocated to the same cell (multi-carrier cell) when carriers are sufficiently synchronized. For an example, in order to enable a very fast carrier switch, practical thresholds of conditions may include Maximum Receive Timing Difference (MRTD) < 3us (or CP), frequency error < 0.1ppm, and similar total received power for an intra-band-like UE receiver, where a common time/frequency synchronization reference and AGC can be used at a UE.

      - Example for Single Cell group with Multiple bands

  - Cell 1: less than 1GHz FDD band grouping (n5, n8, n12, n13, n14, n18, n20,…)

  - Cell 2: 1~2.6GHz FDD band grouping (n1, n2, n3, n7,…)

  - Cell 3: 1.4 ~3.2GHz TDD band grouping (n34, n38, n39, n40, n41, n50, n51,…)

  - Cell 4: 3.2~5.925 GHz TDD band grouping (n48, n77, n78, n79, n46, n47, …)

  - Cell 5: 5.925~8.4GHz band grouping (n96, n102, n104, new 6G spectrum,…)

  - Cell 6: 13.5 ~15.35GHz band Grouping (6G new spectrum)

Cell 3

Cell 2

13.5-15.35GHz

1

2

3

4

5

6

7

GHz

FDD sub-3

TDD sub-6

5/6GHz

Cell 1

Cell 5

7-8.4GHz

Cell 6

Cell 4

Figure 3: 6G Single cell concept with multiple operating bands

Proposal 3: RAN4 to wait for RAN1 on the conclusions of multi-carrier cell concept and check its potential impact to RAN4 RF requirement.

In addition to the multi-carrier cell, there are some other aspects that RAN4 needs to take into account for band combo simplification, e.g., flexible Tx/Rx switching, DL-UL decoupling (which will be described more in the next section). Therefore, we suggest RAN4 to consider also these concepts together with the band combo simplification discussions.

Proposal 4: When discussing band combo simplification, RAN4 needs to also consider concept like flexible Tx/Rx switching and DL-UL decoupling together.

# UL/DL decoupling

To reduce the large overhead for CA or MRSS, in the last RAN1 discussion in August, it is considered to decouple one-to-one mapping between a physical carrier and a logical cell and enable multiple carriers with similar channel properties to be allocated to the same cell (multi-carrier cells) when carriers are sufficiently synchronized. The concept of uplink/downlink decoupling is also raised to allow more flexible spectrum utilization and radio resources. 6G Network can schedule carrier in lower frequency band for enhanced UL coverage and carriers in higher frequency band for maximized DL throughput.

Observation 5: For better UL coverage and DL throughput performance, UL/DL decoupling can be considered.

However, there are incumbent bands and legacy cellular service such as 4G and 5G networks. To accommodate legacy network, the FR1 legacy FDD/TDD band definition shall be re-used in 6G. It can further be considered whether band index can be simplified when the frequency range is fully overlapped.

Observation 6: For the incumbent spectrum blocks migration from 5G to 6G, maintain the FR1 legacy FDD/TDD band definition and duplex can be considered as starting point.

How to correct report UE’s CA capability, maximum supported number of carriers as well as number of maximum MIMO layer in uplink and downlink perspective is also important for 6G signalling design that are being discussed in RAN2 working group. From RAN4 perspective, the antenna performance due to wide aggregated BW shall be further discussed since 200MHz even 400MHz maximum channel bandwidth is also discussed.

Proposal 5: Incumbent frequency band/band index shall be re-used in 6G as starting point. FFS on whether band index can be simplified, considering RAN2’s potential enhancement on UE capability reporting.

Proposal 6: To enable agile spectrum utilization, the decoupling of uplink and downlink (UL/DL) can be considered, considering other impact on cross-carrier scheduling, RF/PHY implementation, Antenna performance for wide aggregated CBW, …, etc.

# Summary

In this paper, we provide our views on the 6G candidate spectrum as follow;

Observation 1: The following spectrum will be considered as a IMT (6G) Candidate spectrum in region 1.

- 3.3~3.4 GHz

- 4.4 ~4.8 GHz (Russia only)

- 6.425 ~7.125 GHz

- [7.125 ~7.25GHz] if feasibility study between military service and IMT service are proved, then these frequency range can be used for IMT service. (except with 7.25~8.4GHz as military service)

Observation 2: The following spectrum will be considered as a IMT (6G) Candidate spectrum in region 2.

- 2.7 ~ 2.9 GHz

- 3.98 ~ 4.2 GHz

- 4.4 ~ 4.9 GHz

- 6.425 ~ 7.125 GHz (only Brazil, Mexico, Still further identification between WiFi and IMT in US)

- 7.25 ~ 7.4 GHz

Observation 3: The following spectrum will be considered as a IMT (6G) Candidate spectrum in region 3.

- 4.8 ~ 4.99 GHz

- 6.425 ~ 7.125 GHz (China and a few South Asia countries)

- [7.125 ~ 8.4GHz] (Korea/Japan still not decide the spectrum usage as same US)

Observation 4: TN/NTN harmonized bands will be further discussed based on WRC-27 decision. RAN4 need to discuss how can define the PSD threshold to protect terrestrial IMT system when ITU-R decide with option 2.

Proposal 1: For the Upper 6GHz and the lower FR3 (7.125 ~ 8.4GHz) will consider as 6G target spectrum in early stage. The FR3 mid band (10~15.35GHz) can be consider in later stage.

Proposal 2: Based on Proposal 1, RAN4 can extend FR1 definition from 410~7125MHz to 410 ~ 8400MHz in 6G.

For the Band combination simplification, we proposed as follow;

Proposal 3: RAN4 to wait for RAN1 on the conclusions of multi-carrier cell concept and check its potential impact to RAN4 RF requirement.

Proposal 4: When discussing band combo simplification, RAN4 needs to also consider concept like flexible Tx/Rx switching and DL-UL decoupling together.

For the 6G migration features with legacy system, we proposed as follow;

Observation 5: For better UL coverage and DL throughput performance, UL/DL decoupling can be considered.

Observation 6: For the incumbent spectrum blocks migration from 5G to 6G, maintain the FR1 legacy FDD/TDD band definition and duplex can be considered as starting point.

Proposal 5: Incumbent frequency band/band index shall be re-used in 6G as starting point. FFS on whether band index can be simplified, considering RAN2’s potential enhancement on UE capability reporting.

Proposal 6: To enable agile spectrum utilization, the decoupling of uplink and downlink (UL/DL) can be considered, considering other impact on cross-carrier scheduling, RF/PHY implementation, Antenna performance for wide aggregated CBW, …, etc.

# References

- [1] RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator)

- [2] WRC27 Agenda (Resolution 813, WRC23): https://www.itu.int/dms_pub/itu-r/oth/0c/0a/R0C0A0000110038PDFE.pdf

- [3] ITU-R preparatory studies: https://www.itu.int/en/ITU-R/study-groups/rcpm/Pages/wrc-27-studies.aspx

- [4] ITU Radio Regulations 2024 (free downloads): https://www.itu.int/pub/R-REG-RR-2024

- [5] CEPT groups: https://cept.org/ecc/groups-list