3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513044

# Prague, CZ, 13th - 17th, Oct 2026

Source: 	Samsung

Title: 	Discussion on general RF and UE RF for 6GR

Agenda Item:	8.4

Document for:	Approval

- Introduction

In RAN#108, a SI for a new journey on “6G RAN” was approved, and updated in RAN#109 [1]. RAN4 as one of the important WGs for enabling 6G also discussed the plan and arrangements to clarify the scope of each 6G topic concerned with RAN4 in RAN4#116. This contribution outlines Samsung’s views on several aspects, specifically focusing on Tx EVM relaxation, coherent UL MIMO, and the power class framework.

- Discussion

## 2.1 Tx EVM relaxation (MPR reduction)

2.1.1 Motivation

In general, the ever-growing data rate demands in mobile communications motivate the exploitation of higher order modulations (HOM) such as 256QAM and 1024QAM. At a given transmitter-receiver distance, the HOM requires larger transmission power than QPSK, 16QAM, and 64QAM due to its reduced distance between constellation points, which results in smaller coverage for HOM. A natural way to improve the HOM coverage would be to increase the transmission power level. However, the possible increase of transmission power level is limited by the maximum power that can be provided by the power amplifier (PA). To maximize the HOM coverage, it is expected that the PA output power should be as close to its saturation level as possible. This would, however, result in severe performance loss due to the nonlinear distortion caused by typical characteristics of radio frequency (RF) components such as PA. It is clear that such non-linear distortion needs to be compensated at either transmit side (TX) with pre-distortion mechanism or receive side (RX) with equalization to ensure block error rate (BLER) or coverage requirements for HOM.

To leverage HOM, it is essential to develop a more effective scheme to compensate the nonlinear distortion. Two viable alternatives are to embrace advanced BS receiver with artificial intelligence (AI) or Non-AI solutions, both of which have been recognized as promising approaches for UE coverage enhancement.

Non-AI algorithms are typically based on predefined models and hence have the well-known problem of “model deficit” in reality. AI solution is good choice to avoid the model deficit problem as it can be used for any types of PA having different non-linearity characteristics and different channel environments. With a EVM relaxation from 3.5% to 6% for 256QAM, AI solution outperforms Non-AI solution 2dB at BLER of 10%.

2.1.2 Proposed study scope for Tx EVM relaxation for 5G-A

The following Note, including a check point, was captured in Rel-20 WID (New WID: UE RF enhancements for NR FR1, Phase 5) [2] during RANP#109 as working guidance. Motivated by the potential commonality of non-linearity modeling of transmission signals and feasibility study steps, which are likely to be agnostic of 5G-A and 6G solutions, the group decided to combine the 5G and 6G discussion into one track to avoid the duplicated efforts and inconsistence. Specifically, it was emphasized that the 5G-A solution should be non-AI based and should not impact RAN1. Additionally, it is natural to assume all discussions are based on 5G waveform and mechanism/assumption. While for 6G, new DMRS waveforms and/or AI-based solution can be considered.

For the 5G-A non-AI based solution, according to the above Note, 2025 Q4 will primarily focus on the scope discussions. Once the scope is clearly defined, feasibility discussions will commence from 2026 Q1 and continue through Q3.

In alignment with our views expressed in RAN#109[3], for core part study, we propose the study scope as in Proposal 1. In our view, the spirit of this study is to increase the possibility of low-end UE transmitting higher power for higher modulation schemes, with advanced compensation solutions implemented at BS side.

I/Q imbalance, which includes phase imbalance and amplitude imbalance, causes distortion of the constellation diagram, making it elliptical or rotated. This results in larger EVM. More specific, in the case of amplitude imbalance, the inconsistence of I and Q paths causes the points on the constellation diagram to deviate from their ideal positions. In the case of phase imbalance, a phase difference between I and Q signals deviating from 90 degree rotates the constellation diagram. Higher modulation such as 64QAM and 256QAM, having smaller spacing between constellation points, demanding higher signal quality.

Observation 1: The goal of the work is to enable low-end UE to transmit higher power, by leveraging the capabilities of advanced BS receiver.

Observation 2: The EVM of higher modulation is significantly affected by I/Q imbalance.

Therefore, to emulate the realistic UE behavior, we believe the non-linearity model(s) selected to evaluate BS compensation capabilities and UE MPR reduction limit should also account for other RFIC impairments including at least I/Q imbalance, other factors such as fractional timing error are not precluded. This is reflected in Proposal 1.

In addition, to simplify the discussion considering the time constrains, it is advisable to prioritize FR1 single CC operation, for PC3 with 1Tx and PC2 with both 1Tx and 2Tx. Further, selecting CP-OFDM as the waveform is recommended which is suitable for both Layer-1 and Layer-2.

For the performance study part, whether BS performance requirements are needed to verify the BS’s capability to compensate for the non-linearity of transmission signals with modulation orders of 64QAM and 256 QAM can be deferred to the WI phase (if any).

Proposal 1: Adopt the following study scope of 5G-A solution.

Reduction of UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders - Study the feasibility to reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM, 256QAM - The study is performed based on non-AI-based approach at BS receiver under existing and relaxed UE TX EVM requirements - Non-linearity model(s) of transmission signals is studied to capture PA non-linearity and other RFIC impairment at least including I/Q imbalance. - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged. - The study is performed based for FR1 single CC operation. - Example bands as n41, n77/n78 - PC3 with 1Tx, PC2 with both 1 TX and 2Tx - Waveform: CP-OFDM

2.1.3 General assumption for both AI and Non-AI solutions

In accordance with the chair's guidance, discussions under the UE RF agenda should focus solely on non-AI-related UE RF topics, while AI-related matters are to be addressed in a separate AI agenda. However, it is our understanding that certain aspects, such as the non-linearity model(s) selected for evaluation purpose and steps to confirm feasibility, can be determined in UE RF session. These are agnostic to AI/Non-AI solutions and can help best organize the study. Additionally, by assuming the same waveform and non-linearity model(s), results for both AI and non-AI solutions can be effectively compared. This approach ensures a more organized and efficient study process.

At the initial stage of 6G discussion, while the discussion on the new waveform and new PA modeling for 6G are going, it is advisable to prioritize the study of AI and non-AI solutions based on 5G waveforms and 5G PA models. Once 6G waveforms and 6G PA models are agreed, they can be incorporated into the study.

Proposal 2: Non-linearity model(s) of transmission signals (selected for the evaluation of BS compensation capabilities and UE MPR reduction limits) and the working steps for feasibility confirmation, are determined in UE RF session, which are common for BS compensation with AI and Non-AI solutions

To evaluate the effectiveness of proposed techniques related to nonlinearity, PA models in TR 38.803 can be considered for evaluation purpose, e.g., Rapp model and generalized memory-based polynomial (GMP) model, in conjunction with modelling of other non-ideal factors including I/Q imbalance, fractional timing error and so on.

Proposal 3：Non-linearity model(s) of transmission signals shall account for both PA non-linearity and other RFIC impairment including at least I/Q imbalance

- For evaluation purpose, adopt Rapp model and generalized memory-based polynomial (GMP) model in TR 38.803

For IQ imbalance, the following modelling can be considered.

Let st=sIt+jsQt and sLO,It=cos2πf0t denote perfect baseband signal and perfect local oscillator in-phase output, respectively. Then perfect I/Q modulation

rIQ,idealt=sItcos2πf0t+sQtcos2πf0t+π2=sItcos2πf0t-sQtsin2πf0t

Let gI and gQ denote the amplitude impairment of in-phase reference signal and quadrature reference signal, respectively. Furthermore, define phase impairment of in-phase reference signal and quadrature reference signal as θI and θQ. With such distorted reference signals, I/Q modulation output is given by:

rIQt=gI∙sItcos2πf0t+θI-gQ∙sQtsin2πf0t+θQ

Corresponding baseband equivalent signal of the in-phase part and quadrature part of rIQt can be expressed as follows:

rI,bbt=LPFrIQtcos2πf0t=gI∙sItcosθI-gQ∙sQtsinθQ

rQ,bbt=LPF-rIQtsin2πf0t=gI∙sItsinθI+gQ∙sQtcosθQ

Then the baseband equivalent signal of rIQt is given by:

rIQ,bbt=rI,bbt+jrQ,bbt

=gI∙sItcosθI-gQ∙sQtsinθQ+jgI∙sItsinθI+gQ∙sQtcosθQ

=β∙st+γ∙s*t

where β=12gIejθI+gQejθQ and γ=12gIejθI-gQejθQ  represent the coefficients of st and its mirrored component s*t, respectively.

Imbalance between the in-phase and quadrature components of signal output from RF receivers can be cost-effectively compensated rather than improving the analog front-end RF hardware. If gI≈gQ and θI≈θQ after compensation, β≈1 and will be significantly greater than γ. In such case, mirrored component s*t can be ignored and I/Q imbalance can be approximated as a slight amplitude scaling and a slight phase rotation.

In general, it is enough to model I/Q imbalance with the above frequency-independent model. If imperfect RF frequency response in in-phase and quadrature brunches are further considered, the two multiplication operations in rIQ,bbt=β∙st+γ∙s*t should be replaced with convolutions:

rIQ,bbt=βt*st+γt*s*t

where βt and γt are the inverse Fourier transform of the imperfect frequency response in in-phase and quadrature brunches, respectively.

βt=IFT12HIf∙gIejθI+HQf∙gQejθQ

γt=IFT12HIf∙gIejθI-HQf∙gQejθQ

Observation 3: For evaluation purpose, the above I/Q imbalance modelling can be considered.

Furthermore, considering that operators have a commercial interest in higher uplink modulation schemes such as 1024QAM for 6G, we recommend evaluating 1024QAM for the 6G Tx EVM relaxation study once 1024QAM is confirmed as one of the modulation schemes for 6G.

Proposal 4: Uplink 1024 QAM can be evaluated as part of the 6G Tx EVM relaxation study once it is confirmed as one of the 6G modulation schemes.

-Downlink 1024 QAM MCS table can be reused

2.1.4 Working steps for feasibility study for both AI and Non-AI solutions

To efficient the complex discussion, it is essential to establish clear criteria for confirming feasibility. We propose to agree on the following steps listed in Proposal 5. As detailed below, the feasibility study involves extensive demodulation work. However, to facilitate the discussion and avoid scattering the Tx EVM relaxation topics across 3 agendas, we suggest addressing demodulation work in UE RF agenda for Non-AI solution.

Proposal 5: Adopt the following working steps for feasibility study for both AI and Non-AI solutions.

  - Study item phase (demod centric)

    - Step-1: Selection of Non-linearity model(s) of transmission signals

- UE RF session firstly to determine the non-linearity model(s)

    - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

      - Note: For feasibility study, demodulation relevant evaluation is performed in UE RF session for non-AI based solution, demodulation relevant evaluation is performed in AI session for AI based solution

- Including the non-linearity model(s) into the RAN4 PUSCH demodulation test case(s) for both 64QAM and 256QAM

- For a given Tx EVM value derived based on the non-linearity models, evaluate the performance with following assumptions on selected RAN4 test cases to find the target SNR to meet RAN4 requirement or test metric, e.g., 70% of maximum throughput or 10% BLER

  - Performance evaluation

    - Case 1: Conventional receiver including non-linearity model: SNR-a

    - Case 2: Non-AI based non-linearity (NC) receiver with including non-linearity model: SNR-b

    - Case 3: Conventional receiver without including any non-linearity model: SNR-c

    - Note: Case 3 is considered as upper bound performance, Case 1 is expected to be low bound performance

  - Performance comparison

    - Compare SNR-b and SNR-c, the results should be comparable

      - FFS the criteria to be claimed as “comparable” (e.g., within certain x dB), and the target SNR-b(c) should be feasible for network deployment

- Perform evaluation with different Tx EVM values, find/determine the maximum Tx EVM with the target SNR-b derived based on Case 2 to meet RAN4 requirement or test metric and comparable with the target SNR-c derived based on Case 3.

  - Perform different PA models, if needed

  - WI phase (UE RF centric)

    - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation:

  - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

  - For instance for 256QAM Tx EVM, 3.5% -> 6%,

    - For selected PA model(s), evaluate the allowed TX power boosting (i.e., MPR reduction) for different power class

    - FFS the necessity for new EVM model for RAN4 demodulation test cases.

Note: Above approaches are applicable for both 5G waveforms and 6G waveforms

  - Coherent UL MIMO

Coherent Uplink (UL) MIMO is increasingly seen as a cornerstone technology for the upcoming 6G era, promising significant advancements in network performance. While the concept isn't entirely new, it has yet to see widespread commercial adoption in the market. The industry has only recently begun to deploy UL MIMO in earnest, and so far, these implementations have been non-coherent. However, if a device is already equipped with the hardware for UL MIMO, upgrading to a coherent system is the logical next step to unlock its full potential.

The primary hurdle standing in the way of coherent UL MIMO is the challenge of precise synchronization between multiple antennas. To function correctly, all antenna channels must be perfectly aligned in both phase and amplitude. In reality, this is incredibly difficult to achieve. Discrepancies in the length of RF paths and minor inaccuracies in oscillators can introduce phase offsets. Furthermore, unavoidable manufacturing variations in components like PAs and filters lead to inconsistent channel responses. Achieving perfect synchronization across different RFICs, for instance by using a shared Phase-Locked Loop (PLL) and quadrature generator, remains a significant engineering challenge, making coherent UL MIMO a promising, yet demanding, feature for the future of 6G.

It is good timing to see whether the legacy requirements (FR1 requirements copied as below) could represent coherent UL MIMO in terms of phase/power error within the time window. For example, the existing requirement exclusively defines coherency based on the maximum relative phase and power error measured against the most recently transmitted SRS. This SRS-dependent approach, while a functional starting point, has several fundamental issues that need to be addressed as RAN4 had discussed over the last 2-3 releases.

Table 6.4D.4-1: Maximum allowable difference of relative phase and power errors in a given slot compared to those measured at last SRS transmitted

| Difference of relative phase error | Difference of relative power error | Time window |
| --- | --- | --- |
| 40 degrees | 4 dB | 20 msec |

Proposal 6: Coherent UL MIMO should be studied in 6G study, with relevant 5G NR requirements as baseline to study the feasibility of practical implementation in 6GR.

  - Power class framework and PCMAX

During the 5G era, extensive discussions took place to explore the full utilization of hardware capabilities to transmit higher power, which leads to the introduction of HPUE, increasing higher power limit feature, several power boosting features. Additionally, many attempts have been made to optimize the PCMAX equation to enable higher power as well but unfortunately not adopted. In 6G era, there is no doubt that improving UL coverage remains a critical objective. For 6G discussion, it is advisable to first address the power class concept for both single CC and 6G-6G CA, as other requirements are subsequently built upon this foundation.

In terms of single CC, for UE equipped with single PA, we donot expect any changes on the power class concept. However, the PCMAX equation can be further simplified. For UE equipped with multiple PAs, there were proposals on asymmetric TxD/MIMO in 5G, which is also to advocate the sum of each PA capability according to the proponents. While UEs can achieve higher conductive power, the gain from the OTA perspective remains questionable. Further, this approach would further complicate CA implementation. We are open to discussing it further but would like to see more justification.

In terms of CA, one pain point in 5G is the contiguous delay of HPUE deployment due to the “step-by-step” RAN4 CA introduction procedure. More specific, HPUE is always introduced for single CC first and then extended to CA. However, the CA HPUE introduction needs to go through certain procedures, such as specifying from child BCs to parent BC with the guarantee of no fallback missing, and from PC3 to PC2 to PC1.5. Before the specification work, new BC request has to be made on the reflector and merged into the revised WID, subject to the approval of the RAN-P. In real deployment, the “step-by step” CA introduction procedure has led to a phenomenon that has been widely criticized, i.e., when more CC is configured for DL, UE has to decrease its power if HPUE applicability Note is not in place in specification, even though the hardware is capable of transmitting higher power. This issue has been addressed by a RAN task for DL CA only [4][5], but it remains unresolved for ULCA cases as of now. There was also an attempt to remove the HPUE applicability Note [6] by leveraging the introduction of HPUE MSD LUT, which has garnered wide support but unfortunately did not reach consensus in the end.

For CA, the aforementioned two attempts, while representing some progress, are still built on the existing power class concept. One issue has been complained about in 5G is the existence several power class capabilities including per band power class, per band per BC power class, per band per BC power class, for which the relationship among each other are complicated thus not friendly for implementation. As such, 6G presents an opportunity to reconsider and streamline the power class concept and its associated capabilities. One popular proposal is to remove the per BC power class concept. Instead, sum up the per band power class instead for inter-band CA, and allow to transmit up to the per-band power class for intra-band CA. This approach aligns with our our interest.

The pros and cons of aforementioned proposals are summarized in Observation 3.

Observation 4: The pros and cons for different approaches to fully utilize PA capabilities are summarized in the following Table.

| Single CC or CA | Proposal | Details | Pros and Cons |
| --- | --- | --- | --- |
| Single CC | Asymmetric TxD Asymmetric UL MIMO | Like 26dBm+23dBm Like 26+23+23+23 | Pros: Fully utilize PA capability, higher power for conductive Cons: Questionable gain for OTA; Even complex for 4Tx-TxD; Further complicate CA implementation; Need to consider whether to define req. for non-integer PC (i.e., duty cycle for SAR, MPR/AMPR, ACLR etc) and if new conformance test rule needed |
| CA operation | Remove HPUE applicability Notes in clause 5 (Refer to CR R4-2510045) | Along with the success of HPUE MSD LUT | Pros: No delay of NW deployment nor UE implementation by replacing the step-by-step RAN4 BC introduction procedure Cons: Maybe concerned by some chipset/UE vendor as a mass of combos will be allowed for HP; Maybe unfair to BC TP/CR reviewer for thorough checking of MSD exception (significant workload); |
|  | Extend “RAN task” method (Refer to CR R4-2420520 and WF R4-2420525) | Extend the ideas for “single UL CC configured” to “multiple UL CC configured” | Pros: Moderate spec impact&RAN4 basket procedure impact Cons: Interim solution (as exception), not the best way for ecosystem |
|  | Remove per BC Power class concept (at least for inter-band CA) | The sum of the power capability from each band | Pros: Fully utilize PA capability, higher power achieved; No delay of NW deployment nor UE implementation by replacing the step-by-step RAN4 BC introduction procedure Cons: Need to consider whether to define requirements for non-integer PC (Most req. are per-band req. in 5G except MSD, duty cycle for SAR) and if new conformance test rule needed |

Proposal 7： RAN4 to discuss power class framework and PCMAX optimization, for both single CC and 6G-6G CA.

-Unified framework can be considered for FR1-TN and FR1-NTN below 10GHz.

In 5G, there are primarily two approaches to address SAR issue, i.e., P-MPR (Not specified in 3GPP) and duty cycle solution (specified in 3GPP). The duty cycle solution, though specified for both single CC and CA, has never been implemented by any UE vendors due to the clear drawbacks. These includes the big 3 dB step-size power fallback, and the unclear estimated evaluation window, which have hindered its implementation. For 6G, it is advisable not to specify the duty cycle solution anymore. The P-MPR solution remains viable, though it is not specified in 3GPP.

Proposal 8: Do not specify duty cycle solution for SAR issue.

# 3. Conclusion

For Tx EVM relaxation:

Observation 1: The goal of the work is to enable low-end UE to transmit higher power, leveraging the capabilities of advanced BS receiver.

Observation 2: The EVM of higher modulation is significantly affected by I/Q imbalance.

Proposal 1: Adopt the following study scope of 5G-A solution.

Proposal 2: Non-linearity model(s) of transmission signals (selected for the evaluation of BS compensation capabilities and UE MPR reduction limits) and the working steps for feasibility confirmation, are determined in UE RF session, which are common for BS compensation with AI and Non-AI solutions

Proposal 3：Non-linearity model(s) of transmission signals shall account for both PA non-linearity and other RFIC impairment including at least I/Q imbalance

- For evaluation purpose, adopt Rapp model and generalized memory-based polynomial (GMP) model in TR 38.803

Observation 3: For evaluation purpose, the above I/Q imbalance modelling can be considered.

Proposal 4: Uplink 1024 QAM can be evaluated as part of the 6G Tx EVM relaxation study once it is confirmed as one of the 6G modulation schemes.

-Downlink 1024 QAM MCS table can be reused

Proposal 5: Adopt the following working steps for feasibility study for both AI and Non-AI solutions.

For Coherent UL MIMO:

Proposal 6: Coherent UL MIMO should be studied in 6G study, with relevant 5G NR requirements as baseline to study the feasibility of practical implementation in 6GR.

For Power class framework and PCMAX

Observation 4: The pros and cons for different approaches to fully utilize PA capabilities are summarized in the following Table.

Proposal 7: RAN4 to discuss power class framework and PCMAX optimization, for both single CC and 6G-6G CA.

-Unified framework can be considered for FR1-TN and FR1-NTN below 10GHz.

Proposal 8: Do not specify duty cycle solution for SAR issue.

# 4. Reference

[1] RP-251881	New SID: Study on 6G Radio		RAN#109

[2] RP-252952, New WID: UE RF enhancements for NR FR1 Phase 5, RAN#109

[3] RP-252066, Views on 5G-A Rel-20 UE RF topics, RAN#109, Samsung

[4] R4-2420520

[5] R4-2420525

[6] R4-2510045