# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513101

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	7.8.4

Source:	OPPO

Title:	Discussion on RRM core requirements for R20 LB CA

Document for:	Approval

- Introduction

RANP #109 meeting agreed a new WID in [1] and the objectives are duplicated below. The majority objectives are similar as R19 LB CA and it is encouraged to reuse R19 mechanism and requirements as much as possible. The main enhanced part is to change case 2 (Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1) to case 3 (Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1). Based on the objectives and the agreements reached in R19 LB CA, this contribution will provide our initial considerations on RRM core requirements for R20 LB CA.

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

- Discussion

Issue 1-2-2: Necessity of interruption requirement in RRM when more carriers are considered Agreement: - According to the WID, RANA only defines requirements for CA with FDD PCC and one SDL SCC.

As shown above, R19 LB CA precluded the scenario when more carriers are configured. We think R20 is the same as R19 and the above agreement should still apply.

Proposal 1: RAN4 will only define requirements for CA with FDD PCC and one SDL SCC.

Issue 1-1-3: switching pattern applicability for RRM requirement in R19 LB CA via switching Agreement: - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirements for deactivated SDL SCell measurement. Note: the above FFS parts will be discussed and decided in RAN4.

[Agreement]: - Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied. - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms

Based on the WID, the semi-static switching pattern based on RRC configuration will be reused in R20. Regarding to how to apply this configured pattern, RRM has reached the agreements in R19. As shown above, this switching pattern is applied when the SCell is activated or in activation procedure. When the SCell is deactivated, UE is not expected receive data on the SDL and thus the switching pattern is not needed. This logic still works for R20 and the above applicability can be reused.

Proposal 2: The switching pattern applicability in R19 could be reused in R20, i.e. the switching pattern is applied to the activated SCell and the SCell in activation procedure, not for deactivated SCell.

Issue 1-3-5: Activation of Switching Pattern - Recommended WF - Check if followings are agreeable: - At the end of SCell activation (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), spec allows UE a minimum time separation of X ms between transmitting CQI report and applying the semi-statically configured switching pattern. - X=3ms (QC) - X=4ms (Apple) - X=0ms (vivo) - [Online discussion]: Whether or not DCI triggered AP CQI reporting is considered in this SDL SCell activation procedure in LB-CA. - FFS: After sending valid non-zero CQI report and waiting for spec allowed minimum time separation), UE waits until the next transition point from PCell to SCell and switches to SCell during that transition point. - Note: UE can be scheduled in PCell while it waits until the next transition point from PCell to SCell to switch to SCell. [Agreement]: - Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied. - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - In maintenance stage, RAN4 to FFS how UE performs switching according to the following two options. - For MAC CE based SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(THARQ+3ms+ TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(THARQ+3ms+ TLBCA)/slot-length. - Other options are not precluded. - For RRC based direct SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(TRRC_Process + TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(TRRC_Process + TLBCA)/slot-length. - Other options are not precluded. Note: if the timeline for application and for switching has ambiguity, RAN4 may revise the above application timeline.

Issue 1-3-5a: Deactivation of Switching Pattern [Agreement]: - The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms+ TLBCA)/slot-length). - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - FFS in maintenance stage: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately. The carrier switching, if applicable, shall be placed at the end of a slot.

If proposal 2 is agreed, the next issue is the (de)activation delay of switch pattern in case is SCell (de)activation. As shown above, an additional time margin TLBCA may be considered as the UE processing delay. But the exact value of TLBCA and the timeline of switching to SDL SCell in case of SCell activation are still open. UE behavior in R20 may be a little different from that in R19. For example, when SCell is in activation procedure, UE needs to turn off FDD Tx and Rx, and switch to SDL in R19. However, R20 UE needs to turn off FDD Tx, maintain the FDD Rx and turn on the SDL in R20. We are not sure whether the same value of TLBCA could apply in the two cases and it is better to wait more conclusions in R19 LB CA.

Proposal 3: Postpone (de)activation delay of switch pattern in case of SDL SCell (de)activation after the conclusion in R19 LB CA is agreed.

For activated SDL SCell L3 measurement Online Agreement on Tuesday: - RAN4 to define the requirement for activated SDL SCell L3 measurement. Online Agreement on Tuesday: - For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. Agreement in online session: - if the activated SCC measurement is MG-based in LB-CA, the legacy intra-frequency identification/measurement requirement with MG can be applied. - reuse the existing measurement requirements with gap for inter-frequency measurement and inter-RAT measurement in LB-CA. - Not consider the scenario and not define the requirement for measurement without gap for inter-frequency measurement and inter-RAT measurement in LB-CA. Agreement in online session: agree on the following approach, and further update on the wording is not precluded. Further discuss on the CR drafting. - if the activated SCC measurement is MG-less in LB-CA, Kp factor can be updated for activated SCC identification/measurement requirement (intra-frequency measurement without MG) for LB-CA. - Kp factor for MG-less activated SCC identification/measurement requirement in LB-CA can be updated as following: - Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows: - For a window W of duration max(SMTC period, xRP_max, switch pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and switch pattern periodicity is the periodicity of the NW configured switch pattern for LB CA via switching, and starting from the beginning of any SMTC occasion: - Ntotal is the total number of SMTC occasions within the window, including those overlapped with MG and those overlapped with FDD PCC duration based on the switching pattern within the window, and - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions and not overlapped with FDD PCC duration based on the switching pattern within the window W. - No requirement when Navailable = 0.

For deactivated SDL SCell L3 measurement Online agreement on Tuesday: - RAN4 to define the requirement for deactivated SDL SCell L3 measurement. Agreement on Thursday: - The legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement.

BFD/CBD requirements for SDL SCell Agreement: - If RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching, the BFD/CBD on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern. [Agreement]: - RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching - follow the principle in issue 1-1-1 to define the BFD/CBD requirement for SDL SCell. - P factor in the existing BFD/CBD requirement can be updated for LB-CA via switching, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal/ Navailable - Ntotal is total number of BFD-RS or CBD-RS in W window. - Navailable is number of available BFD-RS or CBD-RS in W window. - No requirement when Navailable = 0. - FFS in maintenance stage for P3.

TCI state switch delay requirement for SDL SCell [Agreement]: - RAN4 to define the TCI state switch delay requirement for LB-CA via switching - In TCI state switch delay requirement for SDL SCell, Tfirst-SSB_LB is time to first available SSB reception based on switching pattern after MAC CE command is decoded by the UE.

L1-RSRP/SINR measurement requirements for SDL SCell Agreement in AH: - For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern. [Agreement]: - follow the principle in issue 1-1-1 to define the L1-RSRP/SINR requirement for SDL SCell. - P factor in the existing L1-RSRP/SINR measurement requirements can be updated for LB-CA via switching, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal / Navailable - Ntotal is total number of L1-RSRP/SINR RS of SCell in W window. - Navailable is number of available L1-RSRP/SINR RS overlapping with SCell duration according to the RRC configured switching pattern in W window. - No requirement when Navailable = 0.

Comparing R19 LB CA and R20 LB CA, the major difference is that UE could receive data from FDD carrier 1 when SDL carrier 2 is turned ON. From the perspective of SDL SCell, UE behavior is almost the same, i.e. periodically switch to the SDL SCell. In R19 LB CA, the related measurement delay requirements are extended considering the unavailable occasions due to LB CA switching pattern. We this principle could be reused.

Proposal 4: The updated measurement delay requirements on SDL SCell in R19 LB CA could be reused for R20 LB CA, including

- Activated SDL SCell L3 measurement (gap-less only)

- Deactivated SDL SCell measurement (reuse legacy requirement before R19)

- BFD/CBD

- TCI state switch delay

- L1-RSRP/SINR

Interruption requirement design of deactivated SDL SCell measurement Agreement on Thursday: - For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. - FFS: X, and X can be same or different for different SDL SCC measurement cycle (i.e., configured measCycleSCell). [Agreement]: - For R19 LB CA via switching, interruption length on PCC due to measurements on deactivated SDL is (tuning time in slot level + TSMTC_duation + retuning time in slot level), - Tuning time from PCell to SCell is equal to the RRC configured switching gap from PCell to SCell LBCA-SwitchingGap-Duration-PCelltoSCell-r19 - Returning time from SCell to PCell is equal to the RRC configured switching gap from SCell to PCell, i.e., LBCA-SwitchingGap- SCelltoPCell-r19 - Missed ACK/NACK rate For R19 LB CA via switching, missed ACK/NACK rate due to measurements on deactivated SDL are allowed with up to X= YmeasCycleSCell, where Y=2* interruption length.

One impact on FDD PCell is the interruption due to deactivation SDL SCell measurement. As shown above, both interruption length and miss ACK/NACK ratio are defined in R19 LB CA. In R20 LB CA, we think the impacts on FDD DL and UL should be separately considered. For interruption length on FDD UL, the same methodology could be applied, that is the interruption length = tuning time + TSMTC_duation + retuning time (in slot level), where the tuning/retuning time is updated based on RAN1 conclusion.

Proposal 5: Interruption length on FDD UL due to deactivated SCell measurement = tuning time + TSMTC_duation + retuning time.

For interruption length on FDD DL, at least the SMTC duration should be excluded since UE is capable to Rx on both FDD DL carrier and SDL carrier. During the switching gap, interruption on FDD DL may be needed but we are open to discuss.

Proposal 6: Interruption length on FDD DL due to deactivated SCell measurement = tuning time + retuning time.

Based on proposal 5 and proposal 6, the miss ACK/NACK rate could be calculated as below.

Proposal 7: Missed ACK/NACK rate due to deactivated SDL measurements are allowed with up to X= YUL+YDLmeasCycleSCell, where YUL is the interruption length on FDD UL, and YDL is the interruption length on FDD DL.

Issue 1-4-1: RLM requirement for PCell [Agreement]: - For RLM in LB-CA, follow principle in issue 1-1-1 to define RLM requirements for PCell in LB CA. - P factor in the existing RLM measurement requirements can be updated, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal/ Navailable - Ntotal is total number of RLM-RS in W window. - Navailable is number of available RLM-RS in W window. - No requirement when Navailable = 0. - FFS in maintenance stage: RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritize to receive RS for faster link recovery. Principle of scheduling restriction or interruption requirement agreed for the SCell activation to be reused for the link recovery procedure. Issue 1-4-2: others RRM requirement of FDD PCell in LB CA via switching [Agreement]: The principle of agreed activated SDL SCell requirement in LB-CA can be reused for the corresponding PCell requirement in LB-CA.

For FDD PCell measurement including RLM, BFD, CBD, intra-frequency SSB L3 measurement without gap and L1-RSRP/SINR measurement, R19 LB CA extends the measurement delay requirements considering the switching pattern. However, UE in R20 could still maintain FDD Rx even when SDL is turned on. The measurement delay requirements do not need to be extended and the legacy requirements before R19 LB CA can apply. Then how to differentiate R19 LB CA and R20 LB CA, and reflect in the spec can be further discussed.

Proposal 8: For FDD PCell, the legacy delay requirements without considering R19 LB CA extension should apply for R20 LB CA, including:

- Intra-frequency L3 measurement (gap-less only)

- RLM

- BFD/CBD

- L1-RSRP/SINR

- PL-RS switch delay

- TCI state switch delay

The potential issues and requirements for R20 LB CA are summarized in the table below.

|  | Requirement | R19 LB CA | R20 LB CA |
| --- | --- | --- | --- |
| SDL SCell | Activated SDL SCell L3 measurement (Intra-freq SSB without gap) | New extended delay requirement | Reuse R19 LB CA requirement |
|  | Deactivated SDL SCell measurement | Legacy delay requirement (no extension) |  |
|  | BFD/CBD | New extended delay requirement |  |
|  | L1-RSRP/SINR | New extended delay requirement |  |
|  | TCI state switch delay | New requirement, update Tfirst-SSB based on switch patter |  |
|  | SDL SCell activation/deactivation and corresponding interruption | New extended delay requirement considering TLBCA | Postpone after R19 LB CA |
| FDD PCell | Intra-freq SSB without gap | New extended delay requirement | Reuse legacy requirement without R19 LB CA extension. FFS how to implement spec. |
|  | RLM | New extended delay requirement |  |
|  | BFD/CBD | New extended delay requirement |  |
|  | L1-RSRP/SINR | New extended delay requirement |  |
|  | PL-RS switch delay | New extended delay requirement |  |
|  | TCI state switch delay | New requirement, update Tfirst-SSB based on switch patter |  |
|  | Interruption due to deactivated SCell measurement (interruption length, missed ACK/NACK) | New requirements | 1) Separate interruption length on FDD DL and UL 2) Miss ACK/NACK ratio is based on the total interruption length on FDD DL and UL |

- Conclusion

This contribution gave our considerations on RRM requirements for R20 LB CA and the following proposals.

Proposal 1: RAN4 will only define requirements for CA with FDD PCC and one SDL SCC.

Proposal 2: The switching pattern applicability in R19 could be reused in R20, i.e. the switching pattern is applied to the activated SCell and the SCell in activation procedure, not for deactivated SCell.

Proposal 3: Postpone (de)activation delay of switch pattern in case of SDL SCell (de)activation after the conclusion in R19 LB CA is agreed.

Proposal 4: The updated measurement delay requirements on SDL SCell in R19 LB CA could be reused for R20 LB CA, including

- Activated SDL SCell L3 measurement (gap-less only)

- Deactivated SDL SCell measurement (reuse legacy requirement before R19)

- BFD/CBD

- TCI state switch delay

- L1-RSRP/SINR

Proposal 5: Interruption length on FDD UL due to deactivated SCell measurement = tuning time + TSMTC_duation + retuning time.

Proposal 6: Interruption length on FDD DL due to deactivated SCell measurement = tuning time + retuning time.

Proposal 7: Missed ACK/NACK rate due to deactivated SDL measurements are allowed with up to X= YUL+YDLmeasCycleSCell, where YUL is the interruption length on FDD UL, and YDL is the interruption length on FDD DL.

Proposal 8: For FDD PCell, the legacy delay requirements without considering R19 LB CA extension should apply for R20 LB CA, including:

- Intra-frequency L3 measurement (gap-less only)

- RLM

- BFD/CBD

- L1-RSRP/SINR

- PL-RS switch delay

- TCI state switch delay

The potential issues and requirements for R20 LB CA are summarized in the table below.

|  | Requirement | R19 LB CA | R20 LB CA |
| --- | --- | --- | --- |
| SDL SCell | Activated SDL SCell L3 measurement (Intra-freq SSB without gap) | New extended delay requirement | Reuse R19 LB CA requirement |
|  | Deactivated SDL SCell measurement | Legacy delay requirement (no extension) |  |
|  | BFD/CBD | New extended delay requirement |  |
|  | L1-RSRP/SINR | New extended delay requirement |  |
|  | TCI state switch delay | New requirement, update Tfirst-SSB based on switch patter |  |
|  | SDL SCell activation/deactivation and corresponding interruption | New extended delay requirement considering TLBCA | Postpone after R19 LB CA |
| FDD PCell | Intra-freq SSB without gap | New extended delay requirement | Reuse legacy requirement without R19 LB CA extension. FFS how to implement spec. |
|  | RLM | New extended delay requirement |  |
|  | BFD/CBD | New extended delay requirement |  |
|  | L1-RSRP/SINR | New extended delay requirement |  |
|  | PL-RS switch delay | New extended delay requirement |  |
|  | TCI state switch delay | New requirement, update Tfirst-SSB based on switch patter |  |
|  | Interruption due to deactivated SCell measurement (interruption length, missed ACK/NACK) | New requirements | 1) Separate interruption length on FDD DL and UL 2) Miss ACK/NACK ratio is based on the total interruption length on FDD DL and UL |

References

- RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Huawei, RANP #109