3GPP TSG-RAN WG4 Meeting #116bis		 										R4-2513105

Prague, Czech Republic, October 13th – 17th, 2025

Source: 	OPPO

Title: 	On RRM performance requirements of OD-SSB SCell operation

Agenda Item:	6.16.2.1

Document for:	Discussion

Introduction

In this document, we provide our views on the RRM performance and test case requirements of OD-SSB SCell operation in R19 NES.

Discussion

Issue 3-1-4: Test case list Agreement RAN4 to at least define the following OD-SSB test cases. - SCell activation in Case 1 - Intra-frequency measurement(deactivated SCell) in Case 1 - L1-RSRP measurement with OD-SSB parameter update(Scenario 3B) RAN4 to further discuss whether the following OD-SSB test cases can be agreed. - SCell activation in Case 2 Alt Time C-1 - OD-SSB based multiple SCell activation - OD-SSB based direct SCell Activation at SCell addition in Case 1 - OD-SSB based direct SCell Activation at SCell addition in Case 2 - OD-SSB based PUCCH SCell Activation in Case 1 - OD-SSB based PUCCH SCell Activation in Case 2 - Fast SCell Activation with OD-SSB; - OD-SSB based SCell Activation with the L3 reporting during activation - Intra-frequency measurement (deactivated SCell) in Case 2 - Intra-frequency measurement in Case 1 - Intra-frequency measurement in Case 2 - Interruption at OD-SSB activation/deactivation - Interruptions during measurements on deactivated NR SCC

In core part, RAN4 has agreed to define requirements 3 aspects due to OD-SSB, including

- SCell activation

- Intra-frequency measurement

- Interruption

The core requirements are divided into different clauses, differentiated by case 1 OD-SSB only and case 2 OD-SSB+AO-SSB. In our view, RAN4 needs to test the new cases which are brought due to OD-SSB, like FMW for deactivated Scell measurement, OD-SSB based Scell activation for scenario 2 and 2a. For case 1 and 2, most procedure and requirements are similar except the effective periodicity of OD-SSB and AO-SSB in certain scenarios. We prefer to only choose to justify this.

In last meeting, RAN4 agreed to at least define the following OD-SSB test cases.

- SCell activation in Case 1

- Intra-frequency measurement (deactivated SCell) in Case 1

- L1-RSRP measurement with OD-SSB parameter update (Scenario 3B)

For OD-SSB based multiple SCell activation, OD-SSB based direct SCell Activation at SCell addition in Case 1, and OD-SSB based PUCCH SCell Activation in Case 1, we think there is no need to further involve these cases for testing.

Proposal 1: RAN4 to test one new case for Scell activation in case 1, but no need to test other Scell activation like direct Scell activation, PUCCH SCell activation.

Observation 1: For case 1 and 2, most procedure and requirements are similar except effective SSB periodicity.

Proposal 2: RAN4 to only choose one test cases for both case 1 and case 2 to justify the similar core requirements, including SCell activation, Intra-frequency measurement, Interruption.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1: RAN4 to test one new case for Scell activation in case 1, but no need to test other Scell activation like direct Scell activation, PUCCH SCell activation.

Observation 1: For case 1 and 2, most procedure and requirements are similar except effective SSB periodicity.

Proposal 2: RAN4 to only choose one test cases for both case 1 and case 2 to justify the similar core requirements, including SCell activation, Intra-frequency measurement, Interruption.

- Reference

[1]. R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1.