3GPP TSG-RAN WG4 Meeting #116-bis               	                R4-2513120

Prague, CZ, 13th - 17th Oct 2025

Title:	RRM performance requirements on conditional Intra-CU LTM

Source:	China Telecom

Agenda item:			6.17.2.3

Document for:	Discussion

- Introduction

In the RAN4 #116 meeting, the performance requirements were discussed and WF [1] has been approved. In this paper, we will continue to discuss the RRM performance requirements on conditional Intra-CU LTM.

- Discussion

In previous meetings, we have defined the core part requirements on CLTM, and we should define test cases now.

We have agreed to cover the scenario including intra-frequency/inter-frequency, FR1/FR2, RACH-based/RACH-less, L1 measurement/L3 measurement, with early DL synchronization/without early DL synchronization, initial CLTM/subsequent CLTM. If we want to cover all test cases, the workload is huge and unnecessary, because the almost procedure between LTM is CLTM is same.

In R18 LTM, we defined the following test cases, in this case, we think we only need to verify the delta part, i.e. cell switch is triggered by UE instead of network (cell switch command) and avoid overlap.

| A.6.3.4.1 | RACH-based Intra-frequency PCell switch from FR1 to FR1 |
| --- | --- |
| A.6.3.4.2 | RACH based Inter-frequency LTM PCell switch from FR1 to FR1 |
| A.6.3.4.3 | RACH-less Intra-frequency PCell switch from FR1 to FR1 |
| A.6.3.4.4 | RACH-less Intra-frequency PCell switch from FR1 to FR1 without L1-RSRP measurement |
| A.6.3.5.1 | RACH-based intra-frequency LTM PSCell switch from FR1 to FR1 |
| A.7.3.4.1 | RACH based Intra-frequency cell switch from FR2 to FR2 |
| A.7.3.4.2 | RACH-less Intra-frequency PCell switch from FR2 to FR2 |
| A.7.3.4.3 | RACH-less Inter-frequency LTM PCell switch from FR2 to FR2 |
| A.7.3.5.1 | Intra-frequency RACH-based LTM PSCell switch from FR2 to FR2 |
| A.6.3.2.4 | LTM PDCCH-order Random Access |
| A.7.3.2.4 | LTM PDCCH-order Random Access |

So, for CLTM, we think we should introduce the following test cases for CLTM and subsequent CLTM,

- RACH-less intra-frequency PCell switch from FR1 to FR1

- RACH-based inter-frequency LTM PCell switch from FR2 to FR2

- RACH-less intra-frequency subsequent CLTM from FR1 to FR1

- RACH-based inter-frequency subsequent CLTM from FR2 to FR2

For PSCell switch, the objective of R19 CLTM does not include DC scenarios, so it could be excluded.

For PDCCH-order Random Access, we have agreed that there is no need to change existing PDCCH-ordered RACH delay requirements, so it could be excluded.

Proposal 1: Introduce the following test cases for CLTM and subsequent CLTM,

- RACH-less intra-frequency PCell switch from FR1 to FR1

- RACH-based inter-frequency LTM PCell switch from FR2 to FR2

- RACH-less intra-frequency subsequent CLTM from FR1 to FR1

- RACH-based inter-frequency subsequent CLTM from FR2 to FR2

Proposal 2: Exclude PSCell switch and PDCCH-order Random Access test cases.

- Summary

In this paper, we provide our views on measurements related enhancements for LTM. From this discussion we have derived the following proposals:

Proposal 1: Introduce the following test cases for CLTM and subsequent CLTM,

- RACH-less intra-frequency PCell switch from FR1 to FR1

- RACH-based inter-frequency LTM PCell switch from FR2 to FR2

- RACH-less intra-frequency subsequent CLTM from FR1 to FR1

- RACH-based inter-frequency subsequent CLTM from FR2 to FR2

Proposal 2: Exclude PSCell switch and PDCCH-order Random Access test cases.

- References

[1] R4-2512141, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple Inc.