3GPP TSG-RAN WG4 Meeting #116bis                      R4-2513280

Prague, Czech, October 13th – 17th, 2025

Agenda item:			8.5

Source:	Xiaomi

Title:	Overview for 6GR co-existence study

Document for:	Discussion

Background

In the RAN#108 meeting, the new SID study on 6G Radio was approval by the meeting. intra-3GPP co-existence studies is one of major objective for RAN4 6G study.

In the endorsed 6G RAN4 work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. BS RF including co-existence study is one of the major area.

In NR, from Rel-14, there are several coexistence studies finshed by 3GPP RAN4 to determine the ACLR, ACS, blocking and spurious for FR1 and FR2 base stations and UEs including FR2 co-existence study in Rel-14, NTN co-existence study on FR1/FR2 NTN bands in Rel-17/18/19, co-existence study for SBFD operation and co-existence study for IMT parameters on 4GHz, 8GHz and 15GHz . In the 6G Radio study period, coexistence study is still very important for the determine the core requirment of 6G Radio.

In this document, initial analysis and views on potenail 6G co-existence study are provided.

Disucussion

In general RAN4 co-existence study can be categorized into following areas:

- Operation on new frequency e.g. FR2, FR3

- Updated system parameters e.g. BS, UE MOP and antenna pattern

- New operation service e.g. NTN

- New duplex mode e,g. SBFD

RAN4 should analyses the effect of coexistence if there are new configurations or scenarios for 6GR.

The target operating frequency for 6GR including FR1, FR2 and operating frequeuncy between FR1 and FR2 (7~24GHz). In Rel-19 RAN4 has established a study item for IMT parameters including 4400-4800 MHz, 7125-8400MHz and 14800-15350MHz with analysis on paramertes in BS and UE side with detailed assumption.

The coexistence of the 14-15 GHz band was already studied in Release 18 for IMT parameter development submitted to ITU-R WP 5D. During this study, a set of assumptions and parameters for both base stations (BS) and user equipment (UE) in this frequency band were proposed by companies and the coexsitence situation have been fully studied in RAN4.

However, as this was a preliminary study, not all participating companies had fully considered the implications of this band for 6G at that time. This may result in slightly changed or new configurations for this band, as well as the introduction of new scenarios. As the new beginning of 6GR, 3GPP should not close the gate for proposing new parameters and scenarios of this new frequency band.

Should such changes or new scenarios emerge, RAN4 would need to assess their impact to determine whether the coexistence study warrants extension.

Traditional co-existence study under FR1 and FR2 including  SBFD operation is already covered by the 5GA.

RAN4 also covered new frequency range between 7-24 GHz in previous IMT system parameter study.

RAN4 should analyses the effect of coexistence if there are new configurations or scenarios for 14-15GHz.

3GPP RAN4 has initiated SBFD research in Release-18, which focuses on SBFD-TDD coexistence for FR1 and FR2, defining interference coordination frameworks and slot allocation rules to mitigate self-interference from concurrent transmission/reception. In Release-20 5GA, the coexistence study for SBFD will extends to SBFD-FDD scenarios, analyzing duplex mode switching impacts on adjacent-channel interference for FR1 and FR2.

To give the meeting a full picture, against the backdrop of the continuous evolution of communication technologies, it must be mentioned that the 3GPP RAN4 working group has indeed already formally initiated and actively pursued research on SBFD (Sub-band Full Duplex) within Release-18. To be more specific, this research currently places its primary focus on addressing the coexistence of SBFD-TDD in the FR1 and FR2 frequency bands. To this end, they have fully discuss the coexistence issues of SBFD-TDD, with different scenarios. Now, moving forward to the Release 20, the research on SBFD coexistence is evidently set to expand significantly in both scope and depth and turning their attention to SBFD-FDD application scenarios. Within this new direction, one of the core tasks of the research will be to meticulously analyze the precise impacts of duplex mode switching (such as dynamic adjustments in SBFD-FDD), particularly its effects on adjacent-channel interference within the FR1 and FR2 frequency bands – this represents a critical point requiring focused examination.

Due to the different antenna configurations and channel models expected in new frequency ranges, the resulting interference can be different compared to FR1 and FR2.Considering that the combination of SBFD with these potential 6G new frequency range  maybe require new evaluation. Similar to the studies in 5GA , the two main coexistence scenarios – SBFD vs. TDD and SBFD vs. FDD – should be evaluated and discussed within 6GR RAN4 meetings. However, due to the significantly different antenna configurations and channel models expected in these new bands, the resulting interference issues cannot be ignored.

Furthermore, during the early stages of 6GR standardization , parameters related to FR1 and FR2 (such as antenna configurations and maximum transmit power) may undergo changes as discussions progress. Some of these assumptions and parameter adjustments could have impact the interference situation. Therefore, the impact of these new parameters requires thorough analysis by RAN4.

SBFD in 6GR new frequency may be require new co-existence study.

RAN4 can discuss the necessity of SBFD coexistence study in new frequency ranges.

In RAN1 disucssion, the agreements about the new 6G duplex modes shown as below:

Agreement

Study and identify the lessons learned from NR duplex modes.

On 6GR duplexing study, RAN1 considers at least following duplex types:

•	FD-FDD

•	Semi-static TDD

•	gNB semi-static SBFD

•	HD-FDD on UE side

•	Dynamic TDD

Study whether to consider following duplexing types

•	gNB dynamic SBFD

•	UE SBFD

•	gNB FD

•	Note: Other duplex modes are not precluded.

Potential new duplexing types depending on RAN1 progress including “FD-FDD, Semi-static TDD, gNB semi-static SBFD, HD-FDD on UE side and Dynamic TDD”.

RAN4 can discuss the necessity of coexistence study of the duplex types of “FD-FDD, Semi-static TDD, gNB semi-static SBFD, HD-FDD on UE side and Dynamic TDD” as the beginning.

RAN4 should not study those new potential duplex modes “gNB dynamic SBFD, UE SBFD and gNB FD” until RAN1 have new conclusion.

Multi-Radio System Spectrum Sharing (MRSS) is poised to be a key topic in RAN4 6GR discussions. Key coexistence scenarios likely under consideration include 6G vs 5G and 6G vs 4G coexistence, where RAN4 will address ACLR and ACS determination.

Regarding coexistence studies, an initial step would be to evaluate the necessity based on the relevant parameters and assumptions.

From a spectrum perspective, MRSS coexistence considerations are primarily relevant for FR1 and FR2 bands. For 6GR, if the parameters and assumptions for FR1 and FR2 systems remain unchanged from 5G, the existing ACLR and ACS requirements can be reused, negating the need for new coexistence studies. However, coexistence studies become necessary when significant changes to parameters or assumptions occur that could materially impact study results. Examples include substantial increases or decreases in transmit power, number of antenna elements, or maximum EIRP.

Coexsitence study between 6G and 5G, 6G and 4G is not necessary unless there are signifigate parameters and assumptions changes.

Conclusion

- Traditional co-existence study under FR1 and FR2 including  SBFD operation is already covered by the 5GA.

RAN4 also covered new frequency range between 7-24 GHz in previous IMT system parameter study.

SBFD in 6GR new frequency may be require new co-existence study.

Potential new duplexing types depending on RAN1 progress including “FD-FDD, Semi-static TDD, gNB semi-static SBFD, HD-FDD on UE side and Dynamic TDD”.

- RAN4 should analyses the effect of coexistence if there are new configurations or scenarios for 6GR.

RAN4 should analyze the effect of coexistence if there are new configurations or scenarios for 14-15GHz.

RAN4 can discuss the necessity of SBFD coexistence study in new frequency ranges.

RAN4 can discuss the necessity of coexistence study of the duplex types of “FD-FDD, Semi-static TDD, gNB semi-static SBFD, HD-FDD on UE side and Dynamic TDD” as the beginning.

RAN4 should not study those new potential duplex modes “gNB dynamic SBFD, UE SBFD and gNB FD” until RAN1 have new conclusion.

Coexsitence study between 6G and 5G, 6G and 4G is not necessary unless there are signifigate parameters and assumptions changes.