3GPP TSG-RAN WG4 Meeting #116bis	R4- 2513313

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Initial discussion on 6G RRM

Agenda Item:	8.7

Document for:	Discussion

- Introduction

The RAN WG study on 6G radio has been approved by RAN#108 as SID in [1].The relevant objectives of RAN4 are copied below for reference.

- Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

Also, as indicated in [2], multiple topics are provided for initial 6G RAN4 discussion from Q4 2025 to Q1 2026. RRM, which is a critical part from RAN4 perspective for several decades, is one of the topics for initial 6G discussion and our initial considerations on 6G RRM is provided in this contribution.

- Discussion

  - RRM related to energy efficiency

From [1] on RAN1 aspect, energy efficiency and energy saving for both network and device is one critical aspect of 6GR technology framework. The initial discussion on energy efficiency was carried out at RAN1#122 meeting, various aspects/solutions of this topic have been discussed and the potential RRM impact are provided in this section.

- 2.1.1 Overview on power saving techniques

2.1.1.1 Legacy power saving techniques for UE

In RRC idle/inactive mode, paging monitoring and RRM measurement dominate UE power consumption. The existed power saving techniques for RRC idle/inactive are all in time domain, including the following:

- paging monitoring based on I-DRX or eDRX cycle;

- paging early indication (PEI);

- RRM neighbor cell measurement relaxation;

- LR (low power receiver) based LP-WUS monitoring, RRM measurement relaxation and serving cell RRM measurement offloading.

These solutions will bring different levels of power consumption reduction. According to [4] and [5], up to more than 90% UE power saving gain is observed by using I-DRX operation with LP-WUS/WUR, compared with I-DRX operation with and without PEI, if sufficient relaxation to MR RRM measurement is applied. And with comparable UE power saving gain, I-DRX operation with LP-WUS/WUR has 23x lower latency than eDRX operation with PEI .

For RRC connected mode, unlike idle/inactive state where most power saving techniques are exploited at the time domain, various power saving techniques at different domains were specified in 5G NR including [7]:

- Time domain: C-DRX, Downlink Control Information of Power Saving (DCP), PDCCH skipping/SSSG switching, RLM/BFD relaxation, LP-WUS/WUR, SCell dormancy,

- Frequency domain: BWP adaptation,

- Spatial domain: DL MIMO layer adaptation, UL MIMO layer adaptation

- Power domain: PA efficiency etc.

For the time domain, various techniques are available and most of them obtain power saving through reducing unnecessary PDCCH monitoring. The achieved power saving gain is dependent on the traffic arrival pattern in time, however not on the packet size. For example, C-DRX based technique is beneficial to periodic traffic however its power saving gain become less or the latency impact becomes large when such designs applied to aperiodic traffic. Compared with techniques including Rel-15 long/short C-DRX cycle, Rel-16 DCP, Rel-17 PDCCH skipping/SSSG switching and Rel-18 enhanced C-DRX for non-integer periodicities, LP-WUS/WUR based technique achieves up to more than 60% power saving gain and up to more than 10% UPT improvement for FTP and IM traffic and up to more than 10% power saving gain and marginal capacity impact for XR traffic according to [4].

The frequency domain power saving techniques targets adapting the UE bandwidth at RF and/or baseband according to the traffic need.  The power saving gain achieved is highly dependent on the traffic volume and packet size. For burst traffic with larger packet size, utilizing a larger bandwidth allows a shorter transmission and reception time which can provide more power saving gain than the case with smaller bandwidth.

In spatial domain, DL and UL MIMO layers adaptation is supported in NR which allows UE to switch ON/OFF receiver/transmitter RF chains according to the traffic need based on network indication

2.1.1.2 Legacy power saving techniques for Network

For the power saving at the network side, power saving can be achieved from time, frequency and spatial domain. For the frequency domain, the TX/RX bandwidth can be dynamically updated according to needs to reduce unnecessary energy consumption. At the spatial domain, more and more antenna ports and TRPs are being deployed at BS side and dynamic adaptation/shutdown of antenna ports according to needs can reduce unnecessary energy consumption.

Particularly, NES was introduced in Rel-18 as a study item and further studied as a work item in Rel-18/Rel-19 time frame. At the time domain, on-demand transmission of SIB1 through UL WUS and on-demand transmission of SSB, adaptation of common signals such as SSB were specified.

Due to the limitation of the time frame, R19 has some limitations in terms of application scenarios, which restrict the extent of its energy-saving benefits. For example for the on-demand SIB1, NES cell with on-demand triggered SIB1 can only be a capacity cell since the triggering on the on-demand SIB1 still requires the assistance from cell A and in addition, SSBs of NES cell are still periodically transmitted, further limits the NES gain. These limitations together with other new solutions will be further addressed in the 6G era.

In the following sections, we try to provide our initial considerations on the RRM impacts due to various 6G energy efficiency techniques based on RAN1 122 meeting studies.

- 2.1.2 EE techniques at different domain

At [3], techniques for 6G EE were summarized. For the UE EE, usually, among these techniques, at the connected state, time domain techniques could provide better power saving gain compared with frequency or spatial domain techniques. Even when these techniques at different domains are combined, the major contributor for the energy efficiency is still time domain techniques. For example when frequency domain techniques are used on top of time domain techniques, the additional power saving gain by frequency domain techniques is less attractive since most of the power saving benefits are achieved by time domain techniques. In addition as mentioned in section 2.1.1, LP-WUS/WUR based technique provides either a relative higher power saving gain or a good tradeoff between power saving gain and other performance metrics like latency. Compared with connected state, for the UE EE at idle/inactive state, time domain solutions are even more dominant compared with techniques at other domains.

- For power saving techniques summarized in section 2.1.1 and 2.1.2, it was observed that the time domain power saving designs could provide better power saving gain.

- 2.1.3 SSB

The following 2nd round proposal from [3] provides a good summary on possible change for SSB and the possible RRM impact due to SSB change is provide in this section.

FL Proposal 2.2-16:

Study and evaluate NW energy saving from increasing the default periodicity of cell-defining SS/PBCH on synchronization raster. Additionally, study and evaluate UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering:

- SS/PBCH types (e.g., always-on, on-demand),

- SS/PBCH transmission adaptation

- SS/PBCH periodicity(ies),

- Synch raster search complexity,

- SS/PBCH detection performance (incl. #RBs, #symbols, beamforming),

- Other mechanisms are not precluded,

for at least initial access, RRM measurement procedure.

2.1.3.1 SSB default periodicity extension

The SSB periodicity, which is likely to be further extended in 6G for both the network and UE energy efficiency purpose, was discussed in various topics such as 6G energy efficiency, initial access etc.

For the idle/inactive state UE measurement requirement for mobility purpose, the requirement for serving cell and neighbour cell measurement are based on I-DRX with the value from 320ms to 2.56s as the example for intra-frequency neighbour cell measurement illustrated below. The SSB periodicity for 6G deployment could be 160ms or other value between 20ms to 160ms. With the assumption that the SSB periodicity is less than or equal to 320ms, the idle state RRM measurement requirement will not be impacted.

Table 4.2.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1Note1 | FR2-2 Note2 |  |  |  |
| 0.32 | 1 | 8 | 12 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | 5 | 8 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 6 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 5 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| NOTE 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. NOTE 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length. NOTE 3: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. |  |  |  |  |  |  |

For the connected state requirement, in 5G cell identification with/without SSB index acquisition are defined. The cell identification is further composed of:

- Sync signal detection time;

- Measurement period of SSB based measurement

- Time period used to acquire the index of the SSB being measured

Whether the cell identification requirements follow the same structure/have the same components should be studied once there are more RAN1 conclusions since there are many factor will impact the cell identification requirement, such as SSB signal structure, SSB repetition, OD-SSB etc. Currently, the cell identification requirement and corresponding components are impacted by the follow factors:

- SMTC period

- DRX cycle

- MGRP if gap is used

- various scaling factors due to collision or limitation of searchers

Focus only on the SSB periodicity extension, the new SSB periodicity will be reflected in the SMTC period. If the max function max(factor x, y, z) in requirements, as the example illustrated below, is still used in the cell identification and other requirements, the impact of the SSB periodicity is easily to be handled.

Table 9.3.4-1: Time period for PSS/SSS detection (FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(8 * Kgap) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(8*1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(8 * Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

- For SSB periodicity extension, for the idle(inactive) state requirements, there is very limited impact providing the idle state requirements are based on I-DRX cycle and the new SSB periodicity is less than or equal the minimum I-DRX cycle. For the connected state requirements such as cell identification requirements, if the max function structure in the current specs are reused, the impact due to new introduced SSB periodicity can be easily handled.

- The impact of the SSB extension on the 6G RRM requirement should be studied by RAN4.

- On the methodology/principles/structure for 6G RRM requirements, I-DRX based requirement structure defined in 5G for idle (inactive) state could be considered as the starting point. For 6G connected state cell identification requirements, the requirement structure using max function to handle impacts from different factors such as SSB/SMTC period, DRX (if necessary) and MGRP (if necessary) defined in 5G, could be considered as the starting point.

2.1.3.2 On-demand SSB and other SSB modification

As mentioned before, in Rel-19, SSBs of NES cell are still periodically transmitted, further limits the NES gain. There was discussion in RAN1 that Cell A-assisted on demand SSB&SIB1, i.e., a UE receives WUS config from cell A and transmits UL-WUS to NES cell to obtain OD-SSB and OD-SIB1. For this solution, the delay of the availability of SSB, which may impact idle state measurement, may need further investigation by RAN4.

For the SSB transmission for SCell, in NR, SSB-less SCell operation has been supported for intra-band CA scenario in Rel-15 and extended to inter-band CA scenario with certain limitations (i.e. co-located, power difference, QCL relations and etc.) in Rel-18. In Rel-19, on demand SSB is supported and could be activated for SCell measurement when needed. Therefore, for the scenarios where the SSB-less SCell condition is not satisfied, on-demand SSB is provided by network to achieve energy saving for SCell. To our understanding, SSB-less mechanism and on demand SSB mechanism for SCell operation designed in NR are effectively to ensure no SSB or sparse SSB transmission in SCell to obtain sufficient energy efficiency. Hence, further reduction of SSB transmission in SCell to achieve greater energy saving is not urgent and upon to RAN1 final design, RAN4 may not need further enhance.

For the on-demand SSB for SCell activation, in 5G measurement behaviour and requirements have been specified, the base of the requirements is the fast measure window (FMW). In 6G the on-demand SSB may be used for more general measurement purpose instead of only for SCell activation. The detailed design is still upon further discussion.

- For on-demand SSB, RAN4 may need study on-demand SSB related requirement in 6G time frame. Particularly, RAN4 may need investigate requirements when OD-SSB is used for other measurement purpose at connected state or any impact when OD-SSB is implemented in NES cell.

- 2.1.4 On-demand SIB-1

In R19 NES, cell A-assisted OD-SIB1 is specified to reduce SIB1 transmission of capacity cells [6], the specified procedure is illustrated in the following figure.

| Figure 1-1: illustration of R19 OD-SIB1 | Figure 1-2: Procedure of R19 OD-SIB1 |
| --- | --- |

As shown above, in R19, UE firstly camps on cell A and receives WUS config from cell A, where cell A is a coverage cell. UE may transmit UL-WUS to NES cell if the conditions of cell reselection is met, where NES cell is a capacity cell. Then, UE receives RAR and OD-SIB1 from NES cell during a time window.

In 6G, as discussed in RAN1 122, a single cell on-demand SIB1 scenario was discussed, i.e., on-demand SIB1 can be required by a UE without the assistant from cell A as the procedure in Rel-19. In addition, as discussed above, cell A assisted both on-demand SSB and SIB1 is another direction for 6G study.

The common part of the potential new solution on on-demand SIB1, together with legacy solution in Rel-19, is the delay to acquisition of SIB1. In legacy cell A assisted on-demand SIB1 require in Rel-19, the UL-WUS is triggered when the conditions of cell reselection is met, it is not clear whether the same triggering conditions will be reused in 6G ear or not. Anyway, in legacy cell reselection procedure in 5G, SIB1 reception and other SIBs reception are considered under the conditions without UL-WUS requests. For the UL-WUS based on-demand SIB1 acquisition, the acquisition delay of the SIB1, which depends on the RACH process, may need further investigated by RAN1. In addition, when both SSB/SIB1 are on-demand, the timing issue needs be further studied since in Rel-19, the NES cell always transmit SSB hence the timing reference maybe already available whereas when the SSB of the NES cell is also on-demand, the extra time to obtain the timing reference may also need study.

- RAN4 may study on-demand SIB1 related requirements in 6G time frame.

  - Upton detail procedure for on-demand SIB1 by RAN1/2, for on-demand SIB1, no matter for the single cell on-demand SIB1 scenario or cell A assisted on-demand SIB1 scenario, the SIB1 acquisition time may need further studied by RAN4. For the scenario when both SSB/SIB1 of the NES cell are on-demand, besides the SIB1 acquisition time, the period to obtain the timing reference may need study.

- 2.1.5 Impact from DL WUS/WUR

The DL-WUS/WUR was discussed at RAN1 122 meeting and the following summary proposal was suggested in [3]:

FL Proposal 2.4-30:

Study further use cases and potential energy efficiency gains for an OFDM-based DL WUS, apart from wake-up indication, e.g.,

- UE energy savings potential,

- gNB energy costs,

- Synchronization,

- Coverage,

- RRM measurements (e.g., serving cell and/or neighbor cells),

- Small control information and/or data,

- [Cell identification]

- Etc.

Based on RAN1 122 discussion, the possible impact on RRM requirements from the potential 6G DL-WUR/WUR solutions are as the following:

- Expand DL-WUS coverage

For 6GR LP-WUS, expanding WUS coverage is critical to achieve better UE power saving gains than NR and enable identical solution is applicable to all UEs within a cell regardless of its location. However, the coverage expansion comes with a price on overhead increment. The reasonable coverage expansion target, together with the trade-off between other factors such as signalling overhead and possible new solution due to the coverage expansion such as the interference handling at the cell edge, will be further investigated within 6G time frame.

From RAN4 RRM requirements point of view, if WUS is expanded, requirements related to WUS signal may need further studied including accuracy requirements, measurement delay etc.

- Measurements based on DL-WUS

It has been discussed in RAN1 122 meeting that more measurements may be based on the DL WUS signal, for example neighbour cell measurement for both idle and connected state. And handover related measurement could be also based on the WUS signal. From RAN4 RRM perspective, corresponding measurement requirements, such as intra/inter-frequency measurement based on WUS signal, should be further studied once the corresponding solution is stable.

- For the impact of the DL WUS/WUR for the energy efficiency, if WUS is expanded for coverage purpose, requirements related to WUS signal may need further studied including accuracy requirements, measurement delay etc.

- For the impact of the DL WUS/WUR for the energy efficiency, if more measurements are be based on the DL WUS signal, corresponding measurement requirements, such as intra/inter-frequency measurement based on WUS signal, should be further studied once the corresponding solution is stable.

  - Initial access

Initial access is a crucial part of technical design for each generation mobile communication system. In the initial access design, usually the performance metrics like coverage, access latency, capability for RACH, cell search and DL synchronization, suitability for various device types will be considered. Besides above aspects, in the 6G study, energy efficiency is a new dimension to be considered.

From energy efficiency perspective, 6G SSB design shall provide better energy efficient compared with 5G. To achieve energy efficiency/saving, a larger SSB period compared to NR can be studied. However, this means UE needs to do blind PSS detection for longer time causing longer initial cell search latency, the latency will be further exacerbated especially when UE needs process a large number of sync raster.

To reduce the initial cell search time with longer SSB period, several solutions were discussed in RAN1. For example one solution is to have more simple sync signals for initial cell search to ensure more sync raster can be processed simultaneously. Another solution is to have 2 stage SSBs, i.e., a 2nd stage SSB can be provided while maintaining similar SSB structure as in NR. Sparser sync raster is another alternative solution to reduce the access latency when the SSB periodicity is much longer compared with 5G.

From RRM requirement perspective, in 4G and 5G there is no corresponding requirements for the cell selection delay. The logic maybe that the initial cell selection is more like a one-shot behavior instead of a UE behaviour happens frequently. In the 6G the SSB periodicity is likely to be increased for energy efficiency purpose and various solutions try to reduce latency were proposed as discussed before. Hence whether it is necessary to introduce corresponding requirements could be discussed although such requirements are hard to be introduced. The same principle as that of 4G/5G, i.e., no corresponding requirements, can be used as the starting point.

In addition, due to the sparse of SSB and the possible enhancement on the SSB design to reduce the negative impact due to the sparsity of 6G SSB, the cell detection will be impacted if there is new SSB design and the cell detection related requirements should be studied.

- The cell detection should be studied based on 6G SSB design. In addition, RAN4 may discuss whether necessary to introduce cell selection delay requirement whereas the same principle as that of 4G/5G, i.e., no corresponding cell selection delay requirement, can be used as the base.

  - Spectrum aggregation

Carrier aggregation has become a crucial feature for improving system throughput for LTE and NR since it emerged at Rel-10. In CA, bandwidth expansion is realized by aggregating multiple contiguous or non-contiguous carriers called component carriers (CCs). Among these CCs, one is used as a primary cell (PCell) with others are treated as secondary cells (SCells).

However, in practice this feature suffers significant latency due to complicated signaling procedures and necessary synchronization, measurement and reporting for the SCell activation. Such delay becomes a bottleneck for exploiting the fully potential of CA. This issue has been handled in 5G and several solutions try to reduce the SCell activation delay has been studied in 5G time frame. In 6G, it was proposed that fast SCell activation should be studied to reduce SCell activation delay. Some mechanisms from NR can be inherited, such as SSB-less SCell, on-demand SSB, A-TRS for fast SCell activation. In addition, further enhancements based on these schemes can be considered, e.g., early measurement for SCell addition and activation, further minimizing the time for AGC setting, fine synchronization and accurate channel measurements.

From RAN4 requirements perspective, requirements should be defined once these enhanced solution, if there is any, is available.

- RAN4 should consider to define new requirements for solutions to improve SCell activation delay, if there is any.

As wireless networks evolve toward 6GR, the demand to utilize fragmented spectrum arises, including the re-farmed frequencies with smaller or irregular bandwidth in sub-1GHz low bands. However, utilizing existing CA framework in this scenario is inefficient in terms of complexity and system overhead. In 6GR, Single Cell Multi-Carriers (SCMC) framework is proposed to be studied for aggregating fragmented spectrum as a single cell, rather than multiple CCs.

Figure 2, Overall framework of SCMC operation (example)

In general, the applicable usages of SCMC include the following:

- aggregate non-contiguous spectrum across different band groups as a single cell

- aggregate non-contiguous spectrum within a band group as a single cell

From RAN4 RRM requirement perspective point of view, if fragmented spectrum is aggregated as a single cell. Requirements for single cell seems can be reused directly, from the first glance. However from UE perspective, due to constraints on RF implementations, current devices typically employ separate RF chains for different band and such RF implementation are expected to remain in 6G. Hence, the transition/switch related requirements and corresponding interruption requirement may need studied by RAN4, once the SCMC based solution is adopted and stable.

- For the Single Cell Multi-Carriers solution, the transition/switch related requirements and corresponding interruption requirement may need be studied by RAN4, once the SCMC based solution is adopted and stable.

  - MIMO and multi-TRP

MIMO related aspects are evolved continuously in multiple releases in 5G, and it is still expected to be an important key feature for coverage enhancement and throughput improvement in 6G. In 5G MIMO enhancement in previous releases, RAN4 mainly focus on developing requirements for beam management, mTRP scenarios and UE multiple panels. For beam management, RAN4 define RRM requirements for L1-SINR measurements in Rel-16. In Rel-17, TCI state switching requirements based on unified TCI framework are defined, which is further extended to mTRP in Rel-18. In Rel-19 RAN4 define requirements for UE initiated/event-triggered beam management for further enhancement. For mTRP scenarios, RAN4 define requirements for TRP specific BFR and L1 measurement for ICBM in Rel-17. In Rel-19, asymmetric DL/UL TRPs scenarios are further investigated. In addition to MIMO specific work items, other MIMO related RRM aspects are also investigated in different releases, e.g., SRS antenna switching requirements. In particular, RAN4 study the requirements for UE equipped multiple panels with simultaneous reception in Rel-18.

Base on the history of RRM discussion for MIMO/mTRP related aspects, some lessons could be learnt and should be more carefully considered in 6G. For instance, it could be observed that there are features/designs introduced in different releases/WIs serve similar purpose. RAN4 define TCI state switching requirement, UL spatial relation switching and Pathloss RS switching requirements separately. In later release, RAN4 define Unified TCI state switching requirements. Another example is that: RAN4 define requirements for TRP specific BFR and also UE initiated/event triggered BM, and similar UE behaviour and requirements are also considered in LTM. The group spend quite many efforts on requirements discussion in different release for similar functionalities and features, and it will inevitably improve the complexity and costs to implement these features. Another issue is that the UE implementation details and constraints sometimes are under estimated. Taking Multi-Rx as example, it is enabled in the specification since Rel-16/17, but when RAN4 start defining requirements in Rel-18, and it was identified that it is quite challenging for UE to support simultaneous reception and quite many conditions are defined. Thus, the RRM requirements for STxMP are no defined in RAN4.

For 6G study, RAN4 needs to study the RRM requirements for MIMO/mTRP (e.g., measurement for beam management, TCI state switching) based on concrete assumptions and conclusion to avoid duplicated requirements discussion. And RAN4/RRM specific issues/details may needs to be considered in early stage (e.g., UE multi-panel, Rx/Tx timing difference).

- Proposal X: MIMO/mTRP related RRM requirements (e.g., measurement for beam management, TCI state switching) need be studied based on concrete RAN1 assumptions and progress, and early RAN4 evolvement is needed w.r.t UE implementation constraint (e.g., UE multi-panel, Rx/Tx timing difference).

  - RAN4 driven RRM aspects

Measurement gap

The measurement gap is a key feature to enable a UE perform inter-frequency/inter-RAT and intra-frequency measurement. When a UE using measurement gap to perform measurement, the transmission/reception will be stopped hence it has negative impact on UE throughput. In 5G time frame, various map gap enhancement mechanism was studied with the intention to either improve the efficiency of the measurement gap or reduce the negative impact of the measurement gap on the throughput. In early 5G specs, gaps are classified into per-UE gap and per-FR gap. In Rel-16, inter-frequency measurement gap was introduced. In Rel-17, the following new gap types are introduced:

- Concurrent gap with the intention to handle the scenario where the SMTC between different measurement objects are not perfectly aligned.

- NCSG, similar to NCSG in LTE which tried to minimize the negative impact of measurement gap by introducing VIL at each end of a gap where the scheduling is not possible only for the VIL part within a gap.

- Pre-MG, with the intention to handle the mismatch at the time domain between relative slow RRC reconfiguration and relative fast DCI/timer triggered BWP switch.

In Rel-18 the following enhancements on gaps was introduced:

- Pre-MG/NCSG with concurrent gap;

- Measurement without gap with interruption

- MUSIM gaps

In addition, more than 20 types of measurement gap patterns are available in 5G whereas the number of mandatory gap in 5G is very limited, which means quite a few of gap patterns may never been used in practice.

With all these gap type, gap pattern from 5G on the table and all advantage/disadvantage learned from 5G, it is important for RAN4 to evaluate all these gap related techniques at 6G day 1 and determine which gap techniques should be supported from day 1.

- RAN4 should evaluate the existing gap based techniques defined in 5G based on 6G scenario at 6G day 1 and determine which gap techniques could be supported from 6G day 1.

To our understanding, the following gap techniques may be supported from 6G day-1:

per-UE, which is the base for measurement gap

per-FR, depending on the frequency range defined in 6G, at least the principle of per-FR gap could be reused

measurement without gap related techniques including NCSG

- For measurement gap related techniques, the per-UE gap, per-FR gap and measurement without gap related techniques including NCSG could be considered to be supported from 6G Day 1. Other gap related techniques should be studied once more detail RAN1/2 6G design is available.

In addition, gap sharing scheme, which only considers the gap sharing between 5G and LTE, need be studied to include the gap sharing between 5G and 6G.

- Gap sharing between 5G and 6G needs be studied by RAN4. Whether to maintenance the same number of gap patterns in 6G should be studied by RAN4.

- For the number of measurement gap patterns, depending on 6G design, less measurement gap patterns and more mandatory gap patterns compared with 5G may be studied by RAN4.

BWP switch

In NR Rel-15, BWP is a very important concept and motivated by the following usage scenarios [6]

- Usage scenario #1: Supporting reduced UE BW capability

- Usage scenario #2: Supporting reduced UE energy consumption by means of bandwidth adaptation

- Usage scenario #3: Supporting FDM of different numerologies

For scenario 2, using BWP framework for UE power saving is mainly from adapting the UE’s Rx RF/BB bandwidth size perspective, which means switching from a wide BWP to a narrow BWP for PDCCH monitoring only or for small traffic. However, existing BWP switch scheme has several issues which limit the usage for the BWP switch:

- Heavy BWP configuration and signalling overhead. Although numerous parameters are shared across BWPs, the network must configure each shared parameter individually for every BWP.

- Slower BWP switching time, 1-3 ms are defined in RAN4 requirements.

For the above issue, there are proposals to simplify the BWP framework, for example, through identify the parameters that has major impact to UE power saving or minimize the number of BWP-dedicated parameters. In addition, reducing RF/baseband switching delay is also proposed.

From RAN4 requirements point of view, BWP switch delay and interruption requirement has been defined in Rel-15. In Rel-16, BWP switch requirements are further extended to include requirement for multiple BWP switch scenario. In 6G, the assumption for UE parsing time used in 5G can be studied if the BWP framework is further simplified. At the same time, RAN4 could study whether the RF retuning time can be further improved or not.

- For BWP switch, reduction on BWP switch time may need study once BWP design (if there is any) in 6G is clear. Particularly, the reduction on the duration for UE parsing time may be studied if the BWP framework is further simplified. At the same time, RAN4 could study whether the RF retuning time can be further improved or not.

L1/L3 measurement and mobility

L3/L1 measurement and reporting is essential functionality to guarantee RRM performance including mobility performance. For mobile communications, L3 based mobility was the only solution until LTM was introduced in Rel-18. For 6G, a more unified L3/L1 mobility mechanism could be potential supported, especially if L1-based neighbour cell measurement is supported from first release of 6G.

2.5.3.1 L3/L1 Intra/Inter-frequency measurement definition

In 5G, the L3 measurement intra-frequency measurement has the following definition:

A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the SSB of the serving cell and the centre frequency of the SSB of the neighbour cell indicated for measurement are the same, and the subcarrier spacing of the two SSBs are also the same.

And the following is the definition for the inter-frequency measurement:

A measurement is defined as an SSB based inter-frequency measurement provided it is not defined as an intra-frequency measurement according to clause 9.2 (TS38.133).

In the 5G RRM measurement, based on the former intra-frequency and inter-frequency measurement definition, the following categories on the measurement requirements are defined:

- Intra-frequency measurements with measurement gaps

- Intra-frequency measurements without measurement gaps

- Inter-frequency measurement with measurement gaps

- Inter-frequency measurements without measurement gaps

The structure of requirements is complicated and the classification between intra-frequency measurement and inter-frequency measurement are marred since each measurement have requirements with/without measurement gaps.

For SSB-based L1 measurement for LTM, similar intra-frequency measurement definition was used. For inter-frequency measurement, requirements for both gap-based and gap-less L1 measurement were defined.

For CSI-RS L1 measurement for LTM, there were long discussions on intra-frequency measurement, it was agreed that the definition is based on active BWP rather than centre frequency of reference signal. Although only gap-less measurement was supported in 5G, it is indeed a good practice for 6G discussions.

In 6G time frame, the definition of the intra-frequency and inter-frequency measurement should be investigated. The main discussion point is that the relationship between intra-frequency measurement and gap-less measurement, intra-frequency measurement and intra-frequency mobility (handover/cell switch), even the necessity of intra-frequency definition.

- Study the definition of the intra-frequency and inter-frequency measurement for both L3 and L1 measurement and the following aspects could be discussed as the starting point:

- The relationship between intra-frequency measurement and gap-less measurement

- The relationship between intra-frequency measurement and serving cell measurement

- The relationship between intra-frequency measurement band intra-frequency mobility (handover/cell switch)

- The reference for intra-frequency measurement, e.g., center frequency of reference signal, active BWP etc.

- Necessity of intra-frequency measurement definition.

2.5.3.2 L1 measurement for serving cell

In 5G, L1 measurements for serving cell, e.g., radio link monitoring, L1-RSRP measurement, link recovery procedure, are performed outside measurement gap. In early releases of 5G, only L1 measurement based on reference signal within active BWP was supported. In Rel-18, L1 measurement outside active BWP was supported per commercial interest.

In 6G, the L1 measurement procedures are not clear yet. However, RAN4 can start discussion whether and how to support L1 measurement outside active BWP. For example, larger BW is used and no interruption is caused, or gap-based measurement is used etc.

For L1 measurement, it is performed outside measurement gap when gap is configured. In 6G, other mechanism may be considered, especially when L1 neighbour cell measurement is also supported.

- In 6G, RAN4 to study necessary enhancement for L1 measurement for serving cell compared to 5G.

2.5.3.3 L3 measurement

There are many factors that may impact the L3 measurement requirements and even the standardization efforts including maintenance efforts in later releases. The requirements are getting more complicated from release to release.

The CSSF is actually a good approach for defining the requirements. It makes the measurement requirements itself simple and clear. However, CSSF is getting complicated when more and more features are introduced. On the other hand, the CSSF may lead to relaxed measurement requirements depending on MO configurations. Therefore, it would be better for RAN4 to discuss simplified and forward compatible CSSF design.

Assumption on number of searchers also has impact on measurement performance and in the end on mobility performance. It was assumed in 5G that 2 searchers are the baseline. In Rel-19, 3 searchers based on UE capability was supported. It would be good for RAN4 also discuss the assumption in the study.

- In 6G, RAN4 to study potential requirements enhancement compared to 5G for L3 measurement on CSSF, number of searchers etc.

2.5.3.4 L3/L1 measurement sharing

There were many discussions in 5G how to share the results for L3 measurement and L1 measurement, especially for FR2. In Rel-18 LTM, L3 measurement results used for L1 measurement reporting in FR1 was supported. It is a important aspect that can be studied in 6G which could improve the measurement performance or could bring power saving if some of the measurements can be skipped due to measurement sharing.

- In 6G, RAN4 to study possible sharing between L3 measurement and L1 measurements.

Issues related to key components - UE baseband processing time/UE RF retuning time

In the 5G specification it can be found different requirements include some identical components. As an example the key components could be the UE baseband processing time which appears many times in handover related requirements and RF retuning time which appears in gap pattern definition, BWP switch requirements etc. Using the UE baseband processing time as an example, in the handover and DAPS handover the UE baseband processing time could be up to 20ms or 40ms depending on whether the handover is within the same FR or not. In the NR handover with PSCell, the UE processing time have different value. It can be observed or the lesson can be learned is that discussing the same component value for different scenario in different releases could result in duplicated discussion, possible inconsistent results and reducing spec readability.

In 6G, based on the former experience in 5G described above, these key component and corresponding value could be generalized/categorized at the top level then used in different cases instead of appearing in different requirements with different value without any further explanation.

- In 6G, RAN4 could identify key components, e.g., RF retuning time, baseband processing time for typical scenarios, which will be used in various requirement and generalize/categorize these key components in RAN4 specs and optimize their corresponding value.

- Conclusion

- For SSB periodicity extension, for the idle(inactive) state requirements, there is very limited impact providing the idle state requirements are based on I-DRX cycle and the new SSB periodicity is less than or equal the minimum I-DRX cycle. For the connected state requirements such as cell identification requirements, if the max function structure in the current specs are reused, the impact due to new introduced SSB periodicity can be easily handled.

- The impact of the SSB extension on the 6G RRM requirement should be studied by RAN4.

- On the methodology/principles/structure for 6G RRM requirements, I-DRX based requirement structure defined in 5G for idle (inactive) state could be considered as the starting point. For 6G connected state cell identification requirements, the requirement structure using max function to handle impacts from different factors such as SSB/SMTC period, DRX (if necessary) and MGRP (if necessary) defined in 5G, could be considered as the starting point.

- For on-demand SSB, RAN4 may need study on-demand SSB related requirement in 6G time frame. Particularly, RAN4 may need investigate requirements when OD-SSB is used for other measurement purpose at connected state or any impact when OD-SSB is implemented in NES cell.

- RAN4 may study on-demand SIB1 related requirements in 6G time frame.

  - Upton detail procedure for on-demand SIB1 by RAN1/2, for on-demand SIB1, no matter for the single cell on-demand SIB1 scenario or cell A assisted on-demand SIB1 scenario, the SIB1 acquisition time may need further studied by RAN4. For the scenario when both SSB/SIB1 of the NES cell are on-demand, besides the SIB1 acquisition time, the period to obtain the timing reference may need study.

- For the impact of the DL WUS/WUR for the energy efficiency, if WUS is expanded for coverage purpose, requirements related to WUS signal may need further studied including accuracy requirements, measurement delay etc.

- For the impact of the DL WUS/WUR for the energy efficiency, if more measurements are be based on the DL WUS signal, corresponding measurement requirements, such as intra/inter-frequency measurement based on WUS signal, should be further studied once the corresponding solution is stable.

- The cell detection should be studied based on 6G SSB design. In addition, RAN4 may discuss whether necessary to introduce cell selection delay requirement whereas the same principle as that of 4G/5G, i.e., no corresponding cell selection delay requirement, can be used as the base.

- RAN4 should consider to define new requirements for solutions to improve SCell activation delay, if there is any.

- For the Single Cell Multi-Carriers solution, the transition/switch related requirements and corresponding interruption requirement may need be studied by RAN4, once the SCMC based solution is adopted and stable.

- Proposal X: MIMO/mTRP related RRM requirements (e.g., measurement for beam management, TCI state switching) need be studied based on concrete RAN1 assumptions and progress, and early RAN4 evolvement is needed w.r.t UE implementation constraint (e.g., UE multi-panel, Rx/Tx timing difference).

- RAN4 should evaluate the existing gap based techniques defined in 5G based on 6G scenario at 6G day 1 and determine which gap techniques could be supported from 6G day 1.

- For measurement gap related techniques, the per-UE gap, per-FR gap and measurement without gap related techniques including NCSG could be considered to be supported from 6G Day 1. Other gap related techniques should be studied once more detail RAN1/2 6G design is available.

- Gap sharing between 5G and 6G needs be studied by RAN4. Whether to maintenance the same number of gap patterns in 6G should be studied by RAN4.

- For the number of measurement gap patterns, depending on 6G design, less measurement gap patterns and more mandatory gap patterns compared with 5G may be studied by RAN4.

- For BWP switch, reduction on BWP switch time may need study once BWP design (if there is any) in 6G is clear. Particularly, the reduction on the duration for UE parsing time may be studied if the BWP framework is further simplified. At the same time, RAN4 could study whether the RF retuning time can be further improved or not.

- Study the definition of the intra-frequency and inter-frequency measurement for both L3 and L1 measurement and the following aspects could be discussed as the starting point:

- The relationship between intra-frequency measurement and gap-less measurement

- The relationship between intra-frequency measurement and serving cell measurement

- The relationship between intra-frequency measurement band intra-frequency mobility (handover/cell switch)

- The reference for intra-frequency measurement, e.g., center frequency of reference signal, active BWP etc.

- Necessity of intra-frequency measurement definition.

- In 6G, RAN4 to study necessary enhancement for L1 measurement for serving cell compared to 5G.

- In 6G, RAN4 to study potential requirements enhancement compared to 5G for L3 measurement on CSSF, number of searchers etc.

- In 6G, RAN4 to study possible sharing between L3 measurement and L1 measurements.

- In 6G, RAN4 could identify key components, e.g., RF retuning time, baseband processing time for typical scenarios, which will be used in various requirement and generalize/categorize these key components in RAN4 specs and optimize their corresponding value.

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

- R4-2511652, Plan on RAN4 6G discussion and meeting arrangement in Rel-20, Apple, RAN4 115

- R1-2506602, Final Summary of 6GR Energy Efficiency Study, Ericsson and MediaTek, RAN1 122.

- 3GPP TR 38.869 V2.0.0 (2023-12), Study on low-power wake up signal and receiver for NR, (Release 18)

- R1-2305983 Evaluation methodologies and results for LP-WUS/WUR

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #117 Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.

- R1-2505420, Discussion on UE and network energy efficiency, RAN1 122, vivo