3GPP TSG-RAN WG4 Meeting #116bis		    R4-2513315
Prague, Czech Republic, 13 – 17 Oct 2025

Title:	On 6GR system parameter

Source:	OPPO

Agenda item:	8.3

Document for:	Approval

Introduction

In the RAN plenary (RAN#108), a study item on ‘6G RAN’ was approved [1]. This contribution discusses system parameters as per RAN4 guideline of 6GR discussion.

Discussion

Waveform

NR uses CP-OFDM for both downlink and uplink to ensure compatibility with MIMO and wideband operation for high-throughput scenario with better SNR and DFT-s-OFDM for uplink to reduce peak-to-average power ratio (PAPR) to enhance the UL coverage.

The 5G NR waveform is summarized as below table 2.1-1:

Table 2.1-1 5G NR waveform

| Waveform | Support |
| --- | --- |
| CP-OFDM | DL and UL both FR1 and FR2 |
| DFT-s-OFDM | UL for FR1 |

From RAN4 perspective, the waveform discussion needs to consider the spectrum utilization, implementation complexity and spectrum confinement technology together. During the NR early waveform discussion in RAN4, filter-OFDM and WOLA technology are well discussed. Although in the end there is no agreement on the exact spectrum confinement technology, the spectrum utilization agreement has already captured the results of all the company’s simulation and measurement results including such spectrum confinement technology.

In 6GR, since the waveform has quite good performance in the past decade, it is proposed to agree on the baseline waveform will be kept as CP-OFDM and DTF-s-OFDM. While further consider the spectrum confinement technology to have better spectrum shaping, PAPR, spectrum utilization is the motivation.

During the last RAN1 meeting, it has also been agreed that the CP-OFDM and DFT-s-OFDM will be set as basis for 6GR. The detail agreement is captured as below. Hence we can see the agreement is quite aligned with our proposals.

Agreement (first agreement for 6G!!)

CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink

- Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions

- Other OFDM based waveforms are not precluded.

Other OFDM based waveforms are not precluded from RAN1 study.

CP-OFDM and DTF-s-OFDM can be used as baseline to start the RAN4 6G study, other waveforms can be considered later pending on RAN1 conclusions.

As the DFT-s-OFDM enjoys better PAPR compared to CP-OFDM, which is shown below in figure 2.1-1. It has been proposed to study the DFT-s-OFDM waveform also for DL in 6GR. It can provide NES gain in initial access and coverage gain in PDCCH/PDSCH channels.

The DFT-s-OFDM for DL can provide NES gain and coverage gain in different channels.

Although we have to agree on the PAPR performance but for DL with two different waveforms, UE is required to differentiate these two different waveforms which will increase UE complexity. Furthermore, when UE is in blind detection for initial access, it requires UE to differentiate two different waveforms.

The two DL waveforms (DFT-s-OFDM and CP-OFDM) will increase UE implementation complexity.

Figure 2.1-1 CCDF of PAPR for different waveform

To study the DL DFT-s-OFDM waveform impact on RAN4 RF requirements.

Modulation

Higher Order modulation

NR use QPSK, 16QAM, 64QAM and 256QAM for both DL and UL modulation and also π/2-BPSK was introduced for coverage-limited scenarios for UL. Later in Rel-16/17 enhancement, 1024QAM in DL for FR2 was introduced. In Rel-18, DL 1024QAM has been extended to FR1 for FWA and UL 256QAM has been extended to FR2. Higher order modulation can bring us larger throughput however, during the study, it also shows that better environment as larger SNR is needed and hence usually the coverage is poor. Also, the trade-off of EVM requirement and modulation order needs to be compromised.

Table 2 5G NR Modulation

| Frequency range | Support Modulation |  |
| --- | --- | --- |
| FR1 | UL | QPSK, 16QAM, 64QAM, 256QAM |
|  | DL | QPSK, 16QAM, 64QAM, 256QAM, 1024QAM(FWA) |
| FR2 | UL | QPSK, 16QAM, 64QAM, 256QAM |
|  | DL | QPSK, 16QAM, 64QAM, 256QAM, 1024QAM |

Before introducing higher order modulation, many RAN4 works needs to be done.

- MPR study, including EVM requirement for higher order modulation which is based on MPR simulation.

- Link level study, including channel model and required SNR which is based on link level simulation.

- Throughput and coverage study, including throughput increase with certain SNR and coverage loss which is based on system level simulation.

Before introducing higher order modulation, MPR study, Link level study, tthroughput and coverage study are needed in RAN4.

During the RAN1 discussion on modulation, many new concept and technology has been proposed. From RAN4 perspective, we need to consider the true throughput benefit in certain SNR environment for higher order modulation if we want to introduce in 6GR. Hence it is proposed to follow the above three aspects to further study the new modulation.

For high modulation, to further study the MPR, required SNR and throughput and coverage in RAN4.

Spectrum shaping

During RAN1 discussion, geometric shaping (GS) and probabilistic shaping (PS) have been raised for spectrum shaping technology to be introduced in 6GR. The geometric shaping technology use non-uniform constellation by changing the signal space of the diagram without directly changing the PSD of the base band signal. The non-uniform GS constellation is shown below figure 2.2-1. The GS system performance is shown below in figure 2.2-1. It can be seen around 1dB gain can be reached for higher order modulation.

Figure 2.2-1 Example of geometric shaping for 256QAM          Figure 2.2-2 GS System performance

Geometric shaping has non-uniform constellation.

For higher order modulation, 1dB gain can be reached by the non-uniform constellation.

Further study Geometric Shaping impact on spectrum utilization and UE RF requirement if introduced in RAN1.

The probabilistic shaping still uses the uniform constellation but change the probability of each point from equal probability to the smaller power points with larger probability. Figure 2.2-3 below shows a uniform constellation while the points with larger black circle means larger probability. The PS method helps to reduce the probability of large power points and hence reduce PAPR and also helps to get system performance gain.

Figure 2.2-3 PS Constellation                                            Figure 2.2-4 PS System performance

Probabilistic shaping uses uniform constellation but with different probability of each point.

1dB gain can be reached by the PS with QAM constellation.

Further study Probabilistic Shaping impact on spectrum utilization and UE RF requirement if introduced in RAN1.

FFT

The Fast Fourier Transform (FFT) size in 5G NR has been a critical parameter influencing channel bandwidth, numerology flexibility, and implementation complexity. The maximum FFT size has been agreed to be 4096 points with that the supported maximum channel bandwidth has been agreed as 50MHz for FR1 (with 30kHz SCS and 4096 points) and 400MHz for FR2 (with 120kHz SCS and 4096 points).

In the 6GR, evolution can be foreseen to increase the largest FFT size from 4096 to 8192. In such case with 15kHz SCS, the maximum channel bandwidth can go up to 100MHz to improve the throughput of such low bands who in 5G NR the largest channel bandwidth is only 50MHz.

To introduce 8192 maximum FFT size in 6GR.

Numerology

The standardization of subcarrier spacing (SCS) in 5G NR was a fundamental design choice to enable flexible deployments across diverse frequency bands and use cases. It has been agreed that to define SCS as 15×2ⁿ kHz (n=0-4), supporting 15, 30, 60, 120, and 240 kHz to accommodate different channel conditions.

Table 2.4-1 5G NR numerology

| Frequency range | Support |
| --- | --- |
| FR1 | 15/30/60kHz |
| FR2 | 60/120/240kHz |

The 15kHz SCS with longer CP matches better for delay spread hence for low FDD bands, 15kHz is used for increasing coverage. However, 30kHz and 60kHz with smaller latency have better performance in high Doppler spread scenarios. Thus higher frequency bands especially the TDD bands of n41, n77, n78 and n79 apply 30kHz SCS. As it is foreseeable that most of the FR1 bands can be re-farmed to 6GR, for better co-existence with 5G, the same SCS for FR1 is proposed as for 6GR FR1, TDD bands apply 30kHz SCS and FDD bands apply 15kHz SCS.

In SID, it has indicated to avoid multiple numerologies for the same band / sub-range

Most of the FR1 bands can be re-farmed to 6GR, for better co-existence with 5G, the same SCS for FR1 is good for MRSS as for 6GR FR1

For 6GR FR1, TDD bands apply 30kHz SCS and FDD bands apply 15kHz SCS.

For FR3, current 6GR new spectrum allocation is shown as below figure 2.4-1. Currently the around 7GHz and around 15GHz frequency ranges has been allocated by WRC-27 as shown below figure 2.4-1.

Figure 2.4-1 6GR new spectrum allocation

The new frequency range allocated for 6GR are around 7GHz and around 15GHz.

The frequency range for around 7GHz and around 15GHz are not typical FR2 bands, and can be considered to be harmonized to FR1.

Below table 2.4-2 has shown the CP and corresponding scenarios for different SCS. As observed also above, the around 7GHz can be treated as the extension of FR1 and hence 30kHz SCS is proposed. For around 15GHz, 60kHz is proposed to some extend to against the phase noise of higher frequency.

Table 2.4-2 SCS, CP and corresponding scenario

| SCS (kHz) | symbol length (us) | CP percentage | Scenario |
| --- | --- | --- | --- |
| 15 | 66.7 | 7% | Coverage, co-existence with LTE |
| 30 | 33.3 | 7% | Balance for mobility and capacity |
| 60 | 16.7 | 14% | Low latency |
| 120 | 8.3 | 29% | Against phase noise |

For 6GR, 30kHz SCS for around 7GHz range and 60kHz SCS for around 15GHz range is proposed.

CBW

The channel bandwidth discussion is separated into four different parts as minimum channel bandwidth, maximum channel bandwidth, CBW step size and flexible channel bandwidth. Below we list the corresponding FR range and SCS, FFT, minCBW and maxCBW and in following sub-clause, detail discussion is provided.

Table 2.5-1 Proposed 6GR SCS, minCBW and maxCBW

| Frequency range |  | SCS( kHz) | Max FFT size | MaxCBW (MHz) | MinCBW for different bands(MHz) |
| --- | --- | --- | --- | --- | --- |
| FR1 | TDD | 30 | 8192 | 200 | 5/10/20 |
|  | FDD | 15 |  | 50/100 | 3/5 |
| Around 7GHz |  | 30 | 8192 | 200 | 10 |
| Around 15GHz |  | 60 | 8192 | 400 | 50 |
| FR2-1 |  | 120 | 8192 | 800 | 50 |

Minimum Channel bandwidth

The discussion and evolution of the minimum channel bandwidth (minCBW) in 5G NR have been a focal point in 3GPP RAN4 meetings, reflecting the need to balance spectral efficiency, implementation complexity, and backward compatibility.

Initially, proposals in 2017 suggested minCBW settings for Sub-6GHz bands: 5MHz for 15kHz SCS and 10MHz for both 30kHz and 60kHz SCS for FR1 and 50MHz for both 60 and 120kHz SCS for FR2. Subsequent evolution has addressed band-specific needs, such as introducing 5MHz CBW in n41/n90, often tied to regional spectrum fragmentation. In Rel-18, the less than 5MHz WID is accomplished to introduce 3MHz minCBW particularly for Railway communication system, public safety network as well as the legacy LTE refarming bands with fragmented 3MHz allocation. Also, the SSB for 3MHz has been developed and corresponding sync raster has been defined. Currently, band n5, n12, n26, n28, n31, n72, n85, n87, n88, n100, n106 and n110 as total 12 bands has introduced 3MHz.

5MHz for 15kHz SCS and 10MHz for 30kHz has been agreed as minCBW for FR1 in early 5G NR.

3MHz has been introduced in total 12 bands as minCBW at the end of Rel-19.

From the 6GR perspective, the minCBW should also be defined carefully especially from the following aspect:

- Initial access

  - The sync raster granularity depends on the minCBW as the larger minCBW, the coarser sync raster design corresponding to less blind detect time. From this aspect, larger minCBW is preferred.

- Spectrum fragmentation/allocation

  - In some of the LTE re-farming bands, the spectrum allocation of each operator is smaller than 5MHz. As the FR1 FDD bands are captured and shown below table 2.5.2-2. It can be observed that for low frequency bands, only small number of bands are limited by the spectrum allocation of 3MHz (Marked as Green with band n110).

- Spectrum utilization

  - As will also be discussed in the following chapter that with smaller CBW, the SU is always smaller than that with larger CBW. In such case, the small CBW seems not an efficient allocation. From this aspect, larger minCBW is preferred.

When define min CBW, initial access, spectrum status and spectrum utilization need to be considered.

So, to facilitate spectrum holder’s request while allowing larger SU and faster blind detection, we believe the per band basis minCBW is suitable to fulfill different needs.

For 6GR, for FDD bands, 3MHz has been introduced in NR and it is suitable to accommodate operator’s request. While still keep 5MHz for some of the bands which doesn’t need such small CBW as 3MHz is beneficial to initial access and also the spectrum utilization.

For TDD bands, 5/10/20 MHz has been agreed in NR as per band basis per operator’s request. The 5/10/20 MHz is still suitable for FR1.

5MHz for TDD and 3MHz for FDD bands as the 6GR system minCBW and each band can determine larger minCBW based on spectrum status and operator request.

It is proposed to agree on 3/5 MHz for FDD bands and 5/10/20 MHz for FR1 TDD bands, 10/40MHz for around 7GHz bands, 50MHz for around 15GHz and FR2-1 as minCBW.

Maximum Channel bandwidth

The discussion and evolution of the maximum channel bandwidth (maxCBW) in 5G NR have been shaped by spectrum availability, regulatory constraints, and technical feasibility across different frequency ranges (FR1, FR2, and above 52.6 GHz).

Spectrum availability, regulatory constraints, and technical feasibility are used to determine the max CBW.

In FR1 (Sub-6 GHz), maxCBW was initially set at 100 MHz for bands like n41/n78, balancing spectral efficiency and UE complexity. For mmWave (FR2, 24.25–52.6 GHz), 400 MHz was standardized, leveraging wider contiguous spectrum and advanced RF front-end designs. The key factor to define the maxCBW is the SCS and FFT size. In 5G NR, it has been considered 4096 FFT size and hence with 15kHz SCS, only 50MHz can be supported as maxCBW for FDD bands and for TDD bands with 30kHz SCS, up to 100MHz can be supported.

50MHz and 100MHz has been proposed as maxCBW in FR1 considering 15kHz SCS and 30/60kHz SCS in 5G.

In 6GR, as proposed in chapter 2.3, the FFT size is proposed to be increased to 8192. Hence, even with 15kHz SCS, the FDD bands can enjoy 100MHz maxCBW and TDD bands with up to 200MHz maxCBW.

Table 2.5.2-1: Example CBW for various SCS with 4096 and 8192 FFT size

| subcarrier spacing (kHz) | FFT size | Suitable Supported Max BW (MHz) | FFT size | Suitable Supported Max BW (MHz) |
| --- | --- | --- | --- | --- |
| 15 | 4096 | 50 | 8192 | 100 |
| 30 | 4096 | 100 | 8192 | 200 |
| 60 | 4096 | 200 | 8192 | 400 |
| 120 | 4096 | 400 | 8192 | 800 |
| 240 | 4096 | 800 | 8192 | 1600 |

Besides the FFT size and SCS, the contiguous spectrum available for one carrier of particular band as well as the implementation feasibility/complexity needs to be further discussed.

Currently the 5G FDD bands with the largest frequency span is the band n65 with UL and DL both 90MHz. Besides, take band n1 for example, in 5G since it is agreed that the maxCBW is 50MHz so even 60MHz is the frequency span, however, only 50MHz is supported for band n1. For other FDD bands below who has larger than 50MHz as available spectrum are only 9 bands out of 36 bands and hence it seems 50MHz maxCBW suits for most of the FDD bands. Furthermore, 50MHz as maxCBW can use 4096 FFT size instead of 8192 FFT size which can reduce UE implementation complexity and power consumption.

Currently the 5G FDD bands with the largest available spectrum is band n65 with UL and DL both 90MHz.

27 of 36 FDD bands has smaller than 50MHz available spectrum.

Use 50MHz for FDD bands as maxCBW can enjoy less UE implementation complexity and power consumption.

For 6GR FR1 FDD bands, the maxCBW is proposed to be 50 MHz. Further study if 100MHz can be used for maxCBW.

Table 2.5.2-2 NR FDD bands

| NR band | UL (MHz) | DL (MHz) | Available spectrum UL/DL (MHz) | NR band | UL (MHz) | DL (MHz) | Available spectrum UL/DL (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 1920 - 1980 | 2110 - 2170 | 60/60 | n66 | 1710 - 1780 | 2110 - 2200 | 70/90 |
| n2 | 1850 - 1910 | 1930 - 1990 | 60/60 | n68 | 698 - 728 | 753 - 783 | 30/30 |
| n3 | 1710 - 1785 | 1805 - 1880 | 75/75 | n70 | 1695 - 1710 | 1995 - 2020 | 15/25 |
| n5 | 824 - 849 | 869 - 894 | 25/25 | n71 | 663 - 698 | 617 - 652 | 35/35 |
| n7 | 2500 - 2570 | 2620 - 2690 | 70/70 | n72 | 451 - 456 | 461 - 466 | 5/5 |
| n8 | 880 - 915 | 925 - 960 | 35/35 | n74 | 1427 - 1470 | 1475 - 1518 | 43/43 |
| n12 | 699 - 716 | 729 - 746 | 17/17 | n85 | 698 - 716 | 728 - 746 | 18/18 |
| n13 | 777 - 787 | 746 - 756 | 10/10 | n87 | 410 - 415 | 420 - 425 | 5/5 |
| n14 | 788 - 798 | 758 - 768 | 10/10 | n88 | 412 - 417 | 422 - 427 | 5/5 |
| n18 | 815 - 830 | 860 - 875 | 15/15 | n91 | 832 - 862 | 1427 - 1432 | 5/30 |
| n20 | 832 - 862 | 791 - 821 | 30/30 | n92 | 832 - 862 | 1432 - 1517 | 30/85 |
| n24 | 1626.5 - 1660.5 | 1525 - 1559 | 34/34 | n93 | 880 - 915 | 1427 - 1432 | 5/35 |
| n25 | 1850 - 1915 | 1930 - 1995 | 65/65 | n94 | 880 - 915 | 1432 - 1517 | 35/85 |
| n26 | 814 - 849 | 859 - 894 | 35/35 | n100 | 874.4 - 880 | 919.4 - 925 | 5.6/5.6 |
| n28 | 703 - 748 | 758 - 803 | 45/45 | n105 | 663 - 703 | 612 - 652 | 40/40 |
| n30 | 2305 - 2315 | 2350 - 2360 | 10/10 | n106 | 896 - 901 | 935 - 940 | 5/5 |
| n31 | 452.5 - 457.5 | 462.5 - 467.5 | 5/5 | n109 | 703 - 733 | 1432 - 1517 | 30/85 |
| n65 | 1920 - 2010 | 2110 - 2200 | 90/90 | n110 | 1390 - 1395 | 1432 - 1435 | 5/3 |

The NR TDD bands and its frequency span is shown as below table 2.5.2-3. Currently 100MHz is defined even we can see some of the bands have as much as 500,700 and 900MHz span. One of the reasons is that the exact spectrum holder usually doesn’t have such large spectrum and also to consider the UE implementation and complexity. With that, together consider the FFT size evolution, it is proposed to use 200MHz as maxCBW for 6GR TDD bands.

Considering the exact spectrum allocation, UE implementation and PA bandwidth, the maxCBW in 5G NR FR1 is 100MHz.

For 6GR FR1 TDD bands, the maxCBW is proposed to be 200MHz.

Table 2.5.2-3 NR TDD bands

| NR band | Frequency Range (MHz) | Available spectrum (MHz) | NR band | Frequency span (MHz) | Available spectrum (MHz) |
| --- | --- | --- | --- | --- | --- |
| n34 | 2010 - 2025 | 15 | n53 | 2483.5 - 2495 | 11.5 |
| n38 | 2570 - 2620 | 50 | n54 | 1670 - 1675 | 5 |
| n39 | 1880 - 1920 | 40 | n77 | 3300 - 4200 | 900 |
| n40 | 2300 - 2400 | 100 | n78 | 3300 - 3800 | 500 |
| n41 | 2496 - 2690 | 194 | n79 | 4400 - 5000 | 600 |
| n46 | 5150 - 5925 | 775 | n90 | 2496 - 2690 | 194 |
| n47 | 5855 - 5925 | 70 | n101 | 1900 - 1910 | 10 |
| n48 | 3550 - 3700 | 150 | n102 | 5925 - 6425 | 500 |
| n50 | 1432 - 1517 | 85 | n104 | 6425 - 7125 | 700 |
| n51 | 1427 - 1432 | 5 |  |  |  |

For around 7GHz frequency ranges, it can be seen as band n104, the available spectrum of such band can go up to 700MHz. As the SCS is proposed to be 30/60kHz, then the maxCBW is proposed to be 200/400MHz considering the 8192 FFT size. As this around 7GHz range is more like the FR1 extension the same SCS is proposed as 30kHz and with 8192 FFT size, 200MHz is proposed as the maxCBW. Also, to consider the PA and filter CBW performance, especially at this high frequency, to reduce UE implementation complexity, 200MHHz is more suitable.

While for around 15GHz frequency range, 30/60/120kHz SCS is proposed and hence the maxCBW can be proposed to be 400/800MHz. However as current around 15GHz range from 14.8GHz to 15.3GHz and only 500MHz spectrum is available, so 400MHz seems more suitable for this range.

Together consider the SCS and UE implementation complexity, 200MHz is more suitable for around 7GHz range.

For 6GR around 7GHz bands, the maxCBW is proposed to be 200MHz corresponds further SCS study.

The spectrum available in 15GHz is only 500MHz

For 6GR around 15GHz bands, the maxCBW is proposed to be 400 corresponds further SCS study.

CBW step size

After determining the minCBW and maxCBW, the regular/fixed CBWs between min CBW and rnaxCBW, some step sizes need to be considered. Step size too large will lead to potential spectrum low utilization especially the spectrum is irregular CBW. Step size too small will cause high implementation/test complexity. Current channel bandwidth form of 5G is captured below in table 2.5.3-1. It can be seen that the step size is 5MHz from 5 to 50MHz and 10MHz from 50 to 100MHz.

Table 2.5.3-1 Channel bandwidth form of 5G

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n1 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |

The step size is 5MHz from 5 to 50MHz and 10MHz from 50 to 100MHz as smaller step size in the small CBW and larger step size in the larger CBW.

Currently for the spectrum that is not exactly equal to the defined CBW, CA and irregular channel bandwidth are introduced in 5G. CA might be more proper for the large CBWs considering the complexity of supporting too many flexible CBWs. While for irregular channel bandwidth, currently it is based on operator request and will be introduced one single irregular CBW at one time like the 7MHz. Considering the operator’s spectrum are quite fragmented, flexible CBW is very useful especially when the spec rum is small and no CA is feasible or concerns on the low efficiency with CA.

In 5G, irregular bandwidth has been introduced with only one specific CBW at one time which cannot match the large number of fragmented spectrums hold by operators.

Below CBW step size is proposed and further flexible CBW concept is discussed in next chapter 2.5.4.

Figure 2.5.3-1 Step size of CBW

Propose 5MHz step size for 5 to 50MHz; 10MHz step size for 50 to 100MHz and 20MHz step size for 100 to 200MHz.

Flexible Channel bandwidth

Background on irregular CBW

During the 5G NR evolution, irregular channel bandwidth has been introduced due to operator’s request as the spectrum allocation is often not regular enough to follow 3GPP channel bandwidth definition. In LTE, 1.4/3/5/10/20 MHz are defined. During the NR discussion, 5/10/15/20/25/30/35/40/45/50/60/70/80/90/100 MHz channel bandwidth are defined. It can be observed that 5MHz granularity is used for smaller channel bandwidth and 10MHz granularity is used for larger channel bandwidth. This will have problem when smaller granularity of 5/10MHz is needed such as 7MHz which might need 1MHz granularity to facilitate such channel bandwidth. A more detail irregular spectrum hold by operators are shown below in table 2.5.3-1.

Smaller granularity than 5/10MHz is needed such as 7MHz which might need 1MHz granularity,

Table 2.5.3-1 Irregular spectrum hold by operators

| Range of bandwidth | Irregular spectrum hold by operators (MHz) | Applicable 3GPP CBW |
| --- | --- | --- |
| <5MHz | 1, 2.4, 2.46, 2.49, 2.5, 2.51, 2.6, 3.4, 3.69, 3.7, 3.75, 3.8, 4, 4.2, 4.4, 4.6, 4.8, 4.9, 4.92 | No 3GPP CBW can be applied |
| [5MHz, 10MHz) | 5.03, 5.2, 5.4, 5.5, 5.6, 5.8, 6, 6.2, 6.25, 6.3, 6.4, 6.5, 6.6, 6.8, 7, 7.2, 7.5, 7.6, 7.8, 8, 8.2, 8.3, 8.4, 8.6, 8.8, 9, 9.1, 9.2, 9.3, 9.34, 9.6, 9.7, 9.8, 9.9 | 5MHz (Many spectrums will be wasted) |

Many operators’ spectrum doesn’t have suitable 3GPP CBW which makes the spectrum cannot be fully utilized.

Current solution for irregular CBW

To solve the irregular CBW problem, 5G has introduced different methods to solve this problem. Below has summarized those solutions.

Option 1: To choose the nearest regular CBW. For example, if the 7MHz is the exact irregular CBW, then it falls within 5MHz and 10MHz. Hence whether to choose 5MHz or 10MHz to deploy the cell. The question here is if we use 5MHz which is smaller than 7MHz, then 2MHz will be wasted. If we use 10MHz which is larger than 7MHz, then the filter for 10MHz cannot work for the range from 7 to 10MHz.

Figure 2.5.4.2-1 To choose the nearest regular CBW method

If choose the nearest small regular CBW, then the additional spectrum is wasted.

If choose the nearest large regular CBW, then the filter design does not match the bandwidth which will suffer interference.

Option 2: Base station support irregular CBW while UE uses two smaller regular CBW with overlapping. If we want to make full use of this 7MHz then the two 5MHz component carrier should be RB aligned and channel raster aligned which will not always be the case. Furthermore, the overlapping part should be large enough to place a whole SSB and Coreset0 while for smaller than 10MHz, this is also not always fulfilled.

Figure 2.5.4.2-2 BS support irregular CBW while UE support regular CBW

If BS support irregular CBW while UE support regular CBW, the component carrier RB level alignment, channel raster SCS level alignment and large enough overlapping part to put SSB and Correset0 are hard to guarantee.

Option 3: The overlapping CA method on BS side. BS use two regular CBW with overlapping CA to support the irregular CBW. Then two SSB need to be put in this BS CA range which is not possible.

Figure 2.5.4.2-3 BS CA 7MHz method

For small irregular CBW, it is difficult to put two SSB non-overlapping with the BS CA method.

So, the above 3 options are all not perfect. In the end, the 7MHz is introduced separately. If that is the method, any irregular CBW will need to be introduced one by one while as can be seen from table 2.5.3-1, too many number of irregular CBW are waiting to be studied.

Current solution doesn’t work well with irregular CBW.

To introduce flexible CBW in 6G to solve the 5G irregular CBW.

Flexible CBW concept in 6G for irregular CBW

The flexible channel bandwidth has been proposed and discussed during the 5G NR SI and WI. Due to time limitation companies are willing to finish the 5G NR first release in a simple way and further enhancement is agreed. However, as the frame of channel bandwidth has been agreed, it is hard to add smaller granularity as 1MHz or RB level. In 6GR, as the spectrum is more and more treasures, the flexible channel bandwidth is proposed as below figure 2.5.3-1. With the flexible channel bandwidth, operators can use fully of their irregular channel bandwidth.

Figure 2.5.3-1 Flexible channel bandwidth

Following are aspects that needs to study during the SI phase.

- UE RF requirement applicability: Some UE RF requirement is scalable with CBW while some are not.

- Guard band definition: How to calculate the guard band based on allocated RBs.

- Test burden: How to reduce test burden with small CBW granularity.

The introduction of flexible channel bandwidth need to further study the UE RF requirement applicability, guard band definition and reduce test burden.

Flexible CBW impacts to UE RF requirements

For the UE RF requirements, currently in NR, some requirements are CBW scalable while some of them are fixed without CBW change. To summarize, below table 2.5.4.3-1 shows different situation of current 5G NR UE RF requirements.

Table 2.5.4.3-1 Requirement corresponding to channel bandwidth

| Scalable with bandwidth | Fixed without bandwidth change |
| --- | --- |
| Minimum output power | UE maximum output power |
| Occupied bandwidth | MPR/A-MPR |
| REFSENS (FDD bands) | Transmit off power (Measurement bandwidth and SCS) |
| Max input level | Transmit ON/OFF time mask |
| ACS (smaller than 2700MHz) | Power control |
| In-band blocking | EVM |
| Out-of-band blocking | Carrier leakage |
| Narrow band blocking | In-band emission |
| Spurious response | Frequency error |
| RX inter-modulation | ACLR (Measurement bandwidth and SCS) |
|  | SEM (Requirement frequency range) |
|  | Spurious emission (OOB boundary) |
|  | Transmit intermodulation (Wanted signal and interference signal bandwidth) |
|  | Spurious emission |

Generally, it can be found that most of the TX requirements are non-related to channel bandwidth while for RX requirements, all of them except for spurious emission, are all related to channel bandwidth. For those requirements not related to CBW, it is straight forward to apply similarly with flexible channel bandwidth. For The requirements related to CBW besides the A-MPR and REFSENS requirements who need to be discussed separately, the other remaining RX requirements, who currently is scaled with CBW and same scaling can be remained to flexible channel bandwidth.

All requirements can either scale with bandwidth or be irrelevant with the bandwidth.

For the requirements not related to CBW, same requirement apply when introduce flexible channel bandwidth.

For requirements are scalable with bandwidth, the requirements can be scaled with flexible channel bandwidth same as NR principle.

Flexible CBW impacts to System parameter requirements

For the guard band calculation and SU for the flexible channel bandwidth, currently the SU is defined by each “regular” channel bandwidth and the minimum guard band is then calculated by formula. For flexible channel bandwidth, two options can be used to calculate the SU or number of RBs can be scheduled.

Scaling method: To scale the SU of the flexible channel bandwidth falling into the two adjacent “regular” bandwidth. Then calculate the corresponding RB number using the scaled SU.

Nearest SU method: To scale the corresponding RB numbers with the same SU of the nearest “regular” channel bandwidth.

Below we give an example on how to calculate the SU and guard band. Take 9MHz as example. Below table 2.5.4.3-2 is the SU for 3/5/10MHz. The 9MHz falls into 5 and 10MHz hence the SU for 5MHz as 90% and SU for 10MHz as 94% will be used.

Table 2.5.4.3-2 SU table for 3/5/10 MHz

| SCS (kHz) | 3 | 5 | 10 |
| --- | --- | --- | --- |
|  | MHz | MHz | MHz |
|  | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 |
| NR SU | 0.90 | 0.90 | 0.94 |

For scaling method, the SU for 9MHz will be 93.2% as shown below.

SU9M = SU5M+(SU10M-SU5M)/(10MHz-5MHz)*(9MHz-5MHz) = 93.2%

Then in this case the RB for 9MHz will be floor(9MHz * 93.2% / 0.18MHz) = 46. So 46RB is proposed to 9MHz.

For nearest SU method, the smaller SU of the two nearest SU will be used. In this case, SU for 5MHz as 90% is sued.

Then in this case the RB for 9MHz will be floor(9MHz * 90% / 0.18MHz) = 45. So 45RB is proposed to 9MHz.

Examples has been shown for different methods with different RBs for flexible CBW.

The nearest SU method as smaller SU is chosen to guarantee the guard band but the scaling method helps to guarantee the SU

This can be further studied with the 6GR SU discussion.

To further study the two methods as nearest SU method and scaling SU method and their impacts to SU.

Flexible CBW impacts to Signaling

In current 5G UE CBW signalling, the carrier bandwidth is from SIB1 of SCS-SpecificCarrier field as shown below:

Initial access

Connected

Currently the signalling of 5G already support the carrierBandwidth to be configured as any integer number of RBs and hence similar approach can be foreseen in 6GR in both initial access and RRC connected mode.

Current 5G carrierBandwidth for both initial access and RRC connected mode can support the flexible RB configuration for irregular CBW.

Further study the signalling design of 6GR to guarantee the flexible CBW work properly.

How to solve Flexible CBW test complexity

As previously indicated in this chapter the RF requirements are discussed and corresponding requirements can be defined for flexible channel bandwidth. However, as if all the flexible CBW needs to be tested, their will be large test burden.

RF performance usually is contiguous in terms of signal bandwidth. Hence nnow in UE design, usually only some of the CBWs are calibrated considering the time costs in production and CBW in middle will reuse the calibration results of nearest CBW. Therefore, it is reasonable to only test the regular CBW as provided in chapter 2.5.3.

To reduce test burden, specific regular channel bandwidth will be defined and the test only apply to regular channel bandwidth.

UL/DL asymmetric CBW

In NR, the UE channel bandwidth per operating band is specified for both the TX and RX path together, the UE only supports the symmetric channel bandwidth between downlink and uplink, if no specified asymmetric UL and DL channel bandwidth combinations for this band, as shown in Figure 2.5.5-1:

Figure 2.5.5-1 Symmetric channel bandwidth in downlink and uplink

And for FDD band, the carrier centre frequency separation between UL and DL needs to meet the default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation specified for each operating bands in Spec.

In addition, NR has specified asymmetric channel bandwidth in downlink and uplink for some FDD bands, correspondingly, the default Tx-Rx carrier centre frequency separation can be with a maximum deviation (ΔFTX-RX = | (BWDL – BWUL)/2) as shown in Figure 2.5.5-2:

Figure 2.5.5-2 Asymmetric channel bandwidth in downlink and uplink

When a UE supports the asymmetric channel bandwidth in downlink and uplink in the bands as specified in the Spec, the UE need report it by the capability of asymmetricBandwidthCombinationSet.

In 5G, besides of symmetric CBW, the asymmetric CBW in downlink and uplink are specified separately, and UE needs to indicate whether it can support asymmetric CBW through signalling.

Supporting asymmetric CBW especially smaller UL CBW is benefit in UE power consumption. Current large CBW + smaller BWP approach doesn’t save much RF power due to RFFE components are configured based CBW. Besides, usually UE UL throughputs are much smaller than DL. Therefore, as an optional feature, it might be useful to allow asymmetric CBW for all FDD bands in 6G.

Proposal 8: All bands can apply symmetric/asymmetric CBW in downlink and uplink in 6G day one.

Number of RX and TX

The discussion around the number of transmit (TX) and receive (RX) antennas in 5G NR has been crucial for balancing performance, complexity, and device form factors across different use cases. Below table 2.6-1 has provided a summary for different UE types of number of TX and RX for 5G.

Table 2.6-1 NR UE baseline number of RX and TX

|  | Rel-15 NR UE | Rel-16 | Rel-17 | Rel-18 | Rel-18 XR | Rel-19 |
| --- | --- | --- | --- | --- | --- | --- |
| Number of RX and TX | FR1 FDD: 1T2R FR1 TDD: 1T4R | V2X: 2RX UL MIMO: 2TX | Redcap: FR1 FDD: 1T1R FR1 TDD: 1T2R | eRedCap: FR1 FDD: 1T1R FR1 TDD: 1T2R | FR1 FDD/TDD: 1T2R | 6RX |
|  |  |  | FWA: 8RX | 3TX |  |  |

As can be seen in the early release the NR UE is defined, then in Rel-16 for V2X, 2RX is introduced due to the vehicle antenna deployment complexity. UL MIMO with 2TX is introduced to increase the UL throughput and for FR2 4RX is introduced to improve the coverage. Then in Rel-17 and Rel-18, Redcap and eRedCap are introduced with 1T1R for FDD bands and 1T2R for TDD bands. Larger number of RX for FWA as 8RX is introduced in Rel-17 and further 6RX is introduced in Rel-19 with also hand-held UE supported.

For hand-held UE, maximum capability is 2TX and 6RX till the end of NR evolution.

For reduced capability UE, 1T1R for FR1 FDD and 1T2R for FR1 TDD have been agreed.

For the assumption of 6GR UE, the number of TX and RX depends on the device type definition. In later chapter 2.9 device types, we give four different kinds of device types and each device type has its corresponding baseline of number of TX and RX is proposed as below table 2.6-1.

The number of TX/RX below is proposed based on different device types:

Table 2.6-1 Proposed Number of TX and RX for different device type

|  | Device type |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 6G IoT (Lowest-tier) | eMBB (Handheld UE) |  |  | FWA (e.g., CPE) |
|  |  | FR1 FDD | FR1 TDD | FR3 |  |
| number of Tx/Rx | Baseline: 1T1R Optional: 2R | Baseline: 1T2R Optional: 2T/3T; 4R | Baseline: 1T4R Optional: 2T/3T; 6R | Baseline: 2T4R Optional: 3T; 6R | Baseline: 4T8R Optional: 6T;12R |

Synchronization signal and raster

Sync raster

Background of 5G sync raster design

5G’s sync signal and raster evolution balances fast cell acquisition with forward compatibility for future spectrum types.

Currently there is two sets of synchronization raster for different minimum channel bandwidth as 3MHz and 5MHz and corresponding SSB design has a little bit difference. The sync raster design has considered the LTE 100kHz channel raster and is combined with SCS level alignment of sync raster point and channel raster point for FR1 lower than 3GHz bands. As a result, 1.2MHz sync raster granularity has been proposed and also an LS has been sent to RAN1 to facilitate the SCS level floating. In consequence, the kssb is introduced in RAN1 specification since the RB alignment is denied and kssb is needed for an idle UE to find the exact frequency boundary of a carrier which is a compromised solution as the sync raster design agreement in RAN4 is agreed at the end of the WI and RAN1 has no further time to discuss.

SCS level alignment of sync raster and channel raster has been agreed in 5G NR and kssb is introduced to find the RB edge.

Figure 2.7.1-1 SSB position with Kssb

For larger than 3GHz bands, 1.44MHz sync raster granularity is introduced based on SCS level channel raster and for different minimum channel bandwidth, the scaling factor are also different to enable larger sync raster granularity.

For 5MHz minCBW, the SS granularity is 1.2MHz for smaller than 3GHz and 1.44MHz for larger than 3GHz.

For 3MHz minCBW, the SS granularity is 0.6MHz.

From 6GR perspective, further enhancement for reducing the sync raster entries are helpful for UE initial cell search and blind detection. Currently there are different principle of sync raster for different bands, minCBW, SCS, frequency range, channel raster which makes the spec hard to read and also the complexity of UE implementation. From our understanding, a unified design will be helpful. To consider different minCBW of each band and corresponding sync raster entries can be scalable based on minCBW which follows the NR principle can be one of the solutions.

Define unified sync raster design for different bands, minCBW, SCS and channel raster in 6G.

RB based sync raster design

Currently we follow the Rule 1 as for each carrier of minCBW, there should be a whole SSB included in frequency range to define the sync raster. Then the sync raster point separation should fulfill below equation 1.

Sync raster separation ≤ minCBW + Channel raster – BWSSB		(Equation 1)

Figure 2.7.1-2 Sync raster separation

The equation 1 is illustrated as above figure 2.7.1-2. For the CBW1 till CBW2, they all can use SSB1 as their carrier SSB. While for CBW3 as its offset to CBW2 is the channel raster in 5G equation 1, the SSB1 is no longer fully contained by CBW3 and hence SSB2 is proposed. With this, the largest sync raster separation equation 1 is formulated. Based on the equation 1, if we want to have larger sync raster separation compared to NR as 1.2MHz and 1.44MHz for 5MHz minCBW and 600kHz for 3MHz minCBW, we will need larger minCBW, channel raster with smaller SSB bandwidth.

- minCBW: The minCBW has been discussed in chapter 2.5.1. It should follow the regulation. However, we can define different minCBW for different bands as proposed previously so that larger minCBW can apply conditionally.

- Channel raster: In 5G the granularity of each initial access cell is channel raster. While enlarging channel raster might be helpful to reduce the sync raster point, this might cause less channel raster points which will limit the operator deployment and also the SU might be impacted. Also, as observation

- BWSSB: Smaller SSB helps to enlarge the sync raster separation and also improves network energy saving while the SSB performance will be impacted and need to be further studied. Currently the 12RB SSB structure has been proposed in RAN1 and we believe this could be a good starting point to enjoy the UE power saving with smaller SSB bandwidth,

MinCBW, Channel raster, BWSSB are the main factors that impact the sync raster design. Mi CBW will limit the sync raster steps. Make sure all possible channel has SSB.

With current 5G sync raster design, especially to guarantee that for every minCBW in every channel raster there should be one SSB fully contained by the minCBW, the sync raster points cannot be coarser if we still follow such rule. The most straight forward method to have coarser sync raster is to make compromise on the deployment flexibility with the sync raster searching complexity. Here we calculate the equation 1 with replacing the channel raster to a reference sparse channel raster. By using the reference sparse channel raster, then the Rule 1 as for each carrier of minCBW, there should be a whole SSB included cannot apply. But we can have coarser sync raster points and also to avoid the Kssb to save SIB1 information bits.

By obeying the Rule 1 as for each carrier of minCBW, there should be a whole SSB included, we can have coarser sync raster points.

Consider allowing some channels that do not have SSB like pure data channel.

To have coarser sync raster points means faster cell search and better UE power saving. To also try to avoid the Kssb as introduced in the end of NR WID because of the SCS level floating sync raster agreed late in RAN4. These two main concerns in NR needs to be solved in 6GR new sync raster design.

6GR sync raster design to have coarser sync raster points and try to avoid Kssb.

Using 1RB as reference channel raster can naturally guarantee the SSB and channel RB grid is RB aligned. In this case, the UE after blind detection can directly know the RB grid edge and hence the Kssb is no longer needed.

Using 1RB as reference channel raster can avoid Kssb since the grid are RB level aligned.

Since the SCS level channel raster has been used in 5G, we try to make improvement by taking the pros of RB level channel raster while resolve the problem that it brings to us. A specific initial access carrier is proposed which means from initial access perspective, the IA carrier can use larger than SCS or as proposed here 1RB for reference channel raster as in the figure 2.7.1-2 above while after the IA process, the data carrier based on the SCS level channel raster can be established. This helps to keep the flexibility of data carrier with smaller channel raster while helps to reduce the sync raster points by enlarging the channel raster of Initial Access carrier.

Figure 2.7.1.2-3 Specific Initial Access carrier and Data carrier

The reference channel raster is coarser than the channel raster.

Here we calculate the sync raster granularity by using 15 RB configuration for 3MHz minCBW, 12 RB for SSB and 1RB as the reference channel raster. Table 2.7.1.2-1 below shows the sync raster granularity. The largest possible granularity is calculated and listed in the last by second column. Here we try to optimize the granularity as can be seen the last column based is the final proposed ones. With the granularity of different minCBW are integer numbers of times of 3MHz granularity, the all-other sync raster points of larger minCBW are subset of sync raster points of 3MHz. This is further illustrated as figure 2.7.1.2-4.

Table 2.7.1.2-1 Sync raster granularity

| Minimum BW | SCS for SS | NRB,Carrier | Max ΔFSS,Raster | ΔFSS,Raster |
| --- | --- | --- | --- | --- |
| 3 | 15 | 15 | 0.72 | 0.72 |
| 5 | 15 | 25 | 2.52 | 2.16 |
| 10 | 30 | 24 | 4.68 | 4.32 |
| 15 | 30 | 38 | 9.72 | 9.36 |
| 20 | 30 | 51 | 14.4 | 14.4 |

With the granularity of different minCBW are integer numbers of times of 3MHz granularity, the all-other sync raster points of larger minCBW are subset of sync raster points of 3MHz.

This ensure with different minCBW, no new sync raster points are introduced.

Figure 2.7.1.4-1 illustration of different minCBW corresponding to the sync raster points

Below table 2.7.1.2-2 has shown the sync raster point reduction with 1RB as Initial Access carrier with 12RB SSB structure.

Table 2.7.1.2-2 Sync Raster reduction with 1RB channel raster

| minCBW | Sync raster granularity for SCS channel raster | Sync raster granularity for RB channel raster | Sync raster points reduction percentage |
| --- | --- | --- | --- |
| 3 MHz | 600 kHz | 720 kHz | 20% |
| 5 MHz | 1.2 MHz | 2.16 MHz | 80% |
| 5 MHz | 1.44 MHz | 2.16 MHz | 50% |

With 1RB reference channel raster, for different minCBW cases, large sync raster points reduction can be reached.

It is proposed to use 1RB as reference channel raster for specific initial access carrier.

The per band basis minCBW apply and the sync raster granularity can be defined accordingly.

| Minimum BW | SCS for SS | NRB,Carrier | ΔFSS,Raster |
| --- | --- | --- | --- |
| 3 | 15 | 15 | 0.72 |
| 5 | 15 | 25 | 2.16 |
| 10 | 30 | 24 | 4.32 |
| 15 | 30 | 38 | 9.36 |
| 20 | 30 | 51 | 14.4 |

In 6GR, we propose 1RB as reference channel raster for initial access carrier that can at the same time to solve the two concerns and below table 2.7.1.2-3 shows the pros and cons of SCS channel raster and RB channel raster.

Table 2.7.1.2-3 Pros and Cons of different calculation in sync raster design

|  | Pros | Cons |
| --- | --- | --- |
| SCS level channel raster in Sync raster calculation | Larger SU: Small granularity of channel raster to make full use of spectrum Flexibility: More channel raster points to establish a carrier. | Slower Cell Search: Smaller sync raster separation leading to more blind detection time. Additional MIB information: Kssb is needed to show the RB boundary. |
| Reference sparse channel raster in Sync raster calculation | Faster Cell Search: Larger Sync raster separation leading to less blind detection time. Simplified MIB information: The sync raster and channel raster are RB level aligned and the RB boundary is naturally determined. | Lose SU: Large granularity of channel raster might lose 1RB of SU Less flexibility: Less channel raster to establish a carrier. |

To avoid the cons of Reference sparse channel raster in Sync raster calculation, the data carrier after IA can use channel raster to establish the data carrier itself than it can enjoy the full SU as well as the flexibility of deployment in frequency raster.

The data carrier can use channel raster to enjoy the full SU as well as the flexibility of deployment.

The data carrier after IA can use channel raster to establish the specific carrier.

Channel raster

During the NR discussion, the channel raster has been discussed and two proposals as 1 RB and SCS has been proposed and discussed. Due to the large granularity, the spectrum spill over might occur and 1 RB of SU might be sacrificed. So in the end the SCS has been agreed as the channel raster for 5G new bands.

While for LTE re-farming bands, 100kHz channel raster is still kept for better co-existence of NR and LTE. With the evolution, the conflict of 100kHz and 15/30kHz in FR1 has come out and 7.5kHz shift is introduced. Also, in the case of Cell bandwidth and UE specific CBW is configured with ODD/EVEN or EVEN/ODD RBs then the alignment of the two centers is impossible for 100kHz channel raster. Then the enhanced channel raster of 10kHz has been developed to solve such question.

5G has introduced 10MHz enhanced channel raster to get rid of 100kHz raster from LTE.

As from the above evolution of 5G, it seems the big question is two sets of channel raster as 100kHz from LTE and SCS for NR are used. In 6GR, we would like to get rid of the 100kHz channel raster’s impact, otherwise this 100kHz will be kept in all the following release. 5kHz as the greatest common divisor of 10kHz and 15kHz, can play a good role in channel raster for both MRSS as well as the channel spacing.

The NRARFCN is also based on 5kHz of global frequency raster which is the minimum granularity that is used for NR-ARFCN definition.

5kHz is the GCD of 10kHz and 15kHz and used as global frequency raster in NR-ARFCN.

Hence for the FR1 re-farming bands with 100kHz channel raster, to use 5kHz common channel raster can helps to get rid of LTE 100kHz which has caused lots of problem and also work as the GCD of 10/100/15kHz to achieve better MRSS.

So for 6GR, it is proposed as:

For re-farming FR1 bands with 100khz channel raster, using 5khz common channel raster, and avoid diverse channel raster in these bands. For other FR1 bands and new bands, SCS based channel raster is adopted

Spectrum Utilization

Room for spectrum utilization

Spectrum utilization in 5G NR has been a critical topic, focusing on maximizing efficiency while ensuring compliance with RF requirements (e.g., EVM, ACLR, SEM). The initial discussion has proposed greater than 90% SU compared to LTE. This can be easier to reach for larger channel bandwidths while hard for small bandwidth with large SCS. A table summarized current 5G SU for FR1 has been shown as below. We also listed the most PRBs that can be allocated in theory for comparison and to see how much improvement in 6GR the SU can reach.

Table 2.8.1-1 Max theory possible RB and SU for each channel bandwidth

| SCS (kHz) | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| NR SU | 90.0% | 90.0% | 93.6% | 94.8% | 95.4% | 95.8% | 96.0% | 96.7% | 97.2% | 96.8% | 97.2% |  |  |  |  |  |
| Full RB | 16 | 27 | 55 | 83 | 111 | 138 | 166 | 194 | 222 | 250 | 277 |  |  |  |  |  |
| MAX SU | 96.0% | 97.2% | 99.0% | 99.6% | 99.9% | 99.4% | 99.6% | 99.8% | 99.9% | 100.0% | 99.7% |  |  |  |  |  |
| RB Dif | 1 | 2 | 3 | 4 | 5 | 5 | 6 | 6 | 6 | 8 | 7 |  |  |  |  |  |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| NR SU |  | 79.2% | 86.4% | 91.2% | 91.8% | 93.6% | 93.6% | 94.6% | 95.4% | 95.2% | 95.8% | 97.2% | 97.2% | 97.7% | 98.0% | 98.3% |
| Full RB |  | 13 | 27 | 41 | 55 | 69 | 83 | 97 | 111 | 125 | 138 | 166 | 194 | 222 | 250 | 277 |
| MAX SU |  | 93.6% | 97.2% | 98.4% | 99.0% | 99.4% | 99.6% | 99.8% | 99.9% | 100.0% | 99.4% | 99.6% | 99.8% | 99.9% | 100.0% | 99.7% |
| RB Dif |  | 2 | 3 | 3 | 4 | 4 | 5 | 5 | 5 | 6 | 5 | 4 | 5 | 5 | 5 | 4 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |
| NR SU |  |  | 79.2% | 86.4% | 86.4% | 89.3% | 91.2% | 90.5% | 91.8% | 92.8% | 93.6% | 94.8% | 95.7% | 96.3% | 96.8% | 97.2% |
| Full RB |  |  | 13 | 20 | 27 | 34 | 41 | 48 | 55 | 62 | 69 | 83 | 97 | 111 | 125 | 138 |
| MAX SU |  |  | 93.6% | 96.0% | 97.2% | 97.9% | 98.4% | 98.7% | 99.0% | 99.2% | 99.4% | 99.6% | 99.8% | 99.9% | 100.0% | 99.4% |
| RB Dif |  |  | 2 | 2 | 3 | 3 | 3 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 3 |

Theoretically, still some potential RBs can be further used to enhance the SU.

Spectrum utilization evaluation and improvement

The SU evaluation should consider below aspect as below table 2.8.2-1 summarized the evaluation consideration.

Table 2.8.2-1 SU evaluation consideration

| Impact factors | Considerations |
| --- | --- |
| Waveform | Waveform for 6G is still be CP-OFDM/DFT Take the agreed baseline waveform to further study the impact on SU. |
| Modulation | Take the uniform constellation modulation as baseline to study the impact on SU. |
| SEM | The existing NR bands can reuse NR requirements while for newly defined bands especially in around 7GHz and 15GHz, the requirement can be further discussed. Current SEM relaxation of MPR reduction in NR Rel-19 WID can also be considered. |
| EVM | A possible of relaxing the UE TX EVM requirement for higher order modulation can be considered. |
| ACLR | Requirement will be based on co-existence study. For FR1 re-farming bands, NR legacy requirement can be reused. |
| Demodulation | This will be waveform related and discussed together with the waveform. |
| New PA models | Together with possible new default power class: PA model needs to be updated with APT PA to be considered. Also, the default UE power class might be newly defined as higher power class has already been introduced among plenty of bands in NR. |

The waveform, modulation, SEM, EVM, ACLR, demodulation and new PA models are factors need to be considered in the SU evaluation.

Start with the agreed baseline waveform, modulation, SEM, EVM, ACLR, demodulation and new PA models, to evaluate how much RB can be improved in the SI.

In 5G the UE RF requirement relaxation has been carried out for MPR reduction purpose. Currently the relaxation of requirements applies to ACLR/SEM/EVM requirements. Although these requirement relaxations are limited by network configuration and UE capability, similar thoughts can apply to 6GR requirement study and SU improvement.

NR has agreed the relaxation of ACLR/SEM/EVM to improve the MPR

Proposal x:	Study the relaxation of ACLR/SEM/EVM requirements to improve the SU in 6G.

Device Type

NR device background

The standardization of 5G device types has evolved to address diverse use cases, form factors, and deployment scenarios, balancing performance, cost, and power efficiency. Below table 2.9.1-1 has captured the different device types and their evolution during the 5G timeline.

Table 2.9.1-1 Device types of 5G with time evolution

|  | Rel-15 | Rel-16 | Rel-17 | Rel-18 | Rel-19 |
| --- | --- | --- | --- | --- | --- |
| Smartphone | FR1: 1T2R/4R | FR1: 2TX |  |  | 6RX with 4 layer |
| Fixed wireless access | FR1: 1T2R/4R |  | 8RX |  | 6RX with 6 layer |
| Vehicular |  | 2RX |  |  |  |
| Redcap |  |  | 1Rx/1Tx relaxed BW | eRedcap 5MHz BB bandwidth. |  |
| UAV |  | aerial UE |  |  |  |
| XR |  |  | 2RX |  |  |
| NTN |  |  | FR1 | VSAT IoT-NTN PC2/PC1.5 | Ku/Ka band |

From the table 2.9-1 we can see different device types has been introduced as some of them are depend on the usage scenario such as vehicular, UAV, XR and NTN while some of them are depend on capability such as smartphone, FWA and redcap. These types of UEs are not mutually excluded, for example, a smartphone can support NTN and in some case can fall back to a Redcap UE. Also, the differentiation of these UE types has considered: number of TX and RX, supported bandwidth, power class, and so on.

The 5G device types are not mutually exclusive. Different device types have been introduced as some of them are depend on the usage scenario such as vehicular, UAV, XR and NTN while some of them are depend on capability such as smartphone, FWA and redcap.

NR device capabilities

Below we give some detail of different device types who are to some extent as individual types and its capability.

Table 2.9.2-2 Detail differentiation of different UE capability

|  | R15 NR UE | R17 RedCap | R18 eRedCap | R18 XR |
| --- | --- | --- | --- | --- |
| UE number of Tx/Rx | FR1 FDD：1T2R FR1 TDD: 1T4R | FR1 FDD: 1T1R FR1 TDD: 1T2R | FR1 FDD: 1T1R FR1 TDD: 1T2R | FR1 FDD/TDD：1T2R |
| maxCBW | FR1: 100MHz | FR1: 20MHz | FR1:RF 20MHz， BB 5MHz | FR1: 100MHz |
| Duplex mode | FDD, TDD | HD-FDD, FDD, TDD | HD-FDD, FDD, TDD | FDD, TDD |
| Max MIMO layer | FR1 FDD：2 layer for UL and DL FR1 TDD: 2 layer for UL and 4 layer for DL | FR1 FDD: 1 layer for DL FR1 TDD: 2 layer for DL | FR1 FDD: 1 layer for DL FR1 TDD: 2 layer for DL | FR1 FDD/TDD：2 layer for UL and DL |
| CA | Support | Not Support | Not Support | Support |

Above in table 2.9-2 we have listed some of the aspects that we believe is most important to differentiate device types from UE RF capability perspective and they are number of TX/RX, maxCBW, duplex mode, Max MIMO layers and CA support. The proposed device type differentiation is shown as below in table 2.9-3.

Differentiate device types from UE RF capability perspective and they are number of TX/RX, maxCBW, duplex mode, Max MIMO layers and CA support.

6G device type consideration with mandatory baseline functionality set

In 6G, the scalable design for diverse device types have been discussed in RAN1#122 and RANP#109 and the agreements are captured as below:

RAN1#122 agreement

Agreement

Study a scalable 6GR design for diverse device types, considering aspects:

- What should be commonly applicable to all 6G device types

- FFS: add-on features dedicated to specific device types, if any

And in RAN#109, the device types were also discussed with the followings agreed.

Agreement

To investigate further:

  - Motivations/justifications behind the proposed diverse device types, which should be a limited set

  - Whether/how to have one or more device types for eMBB or 6G IoT

  - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc.

  - Whether/how to explicitly standardize device types

  - Ensuring forward compatibility

  - Minimizing/avoiding potential market fragmentation

Note: the terminology “device type” is subject to further discussion and possible refinement.

Agreement

In terms of diverse device types, study further:

  - Possible parameters/factors, e.g.:

  - Number of Tx antennas/chains

  - Number of Rx antennas/chains

  - Power classes

  - Maximum UE bandwidth (DL/UL)

  - Peak data rate (DL/UL)

  - Maximum MIMO layers (DL/UL)

  - Duplex mode

  - Max modulation order (DL/UL)

  - CA/spectrum aggregation (DL/UL)

  - UE processing capabilities

  - Coverage

  - Energy efficiency

  - Mobility/speed

  - Sensing

  - AI

Note: some of the above parameters/factors may be related with form factor

Note: aim to have a focused/limited set of parameters/factors for a device type

  - The value(s) for the identified parameters for a device type

The Mandatory baseline functionality set includes mandatory baseline functionalities for all device types. Hence,

- It should be a “lean” functionality set, in which each functionality shall be well justified to be truly mandatory for all device types.

- It does not need to include attributes. Purely device type-specific attributes are not necessarily included in Mandatory baseline functionality set.

For example, different device types have different requirements on mobility/speed. eMBB UEs and low-end IoT UEs are typically required to support 120km/h moving speed. But FWA CPE may only need to support 3km/h moving speed. UE power class is another example for “device type-specific functionalities”. eMBB, IoT and FWA UEs have different requirements for power class. Therefore, the Mandatory baseline functionality set may not need to include the two attributes.

Define a “lean” Mandatory baseline functionality set, in which each functionality shall be well justified to be truly mandatory for all device types.

Purely device type-specific attributes (e.g., mobility/moving speed, UE power class, etc.) are not necessarily included in Mandatory baseline functionality set.

Low-end IoT is the lowest end device type in 6G on most of aspects, which is very suitable to provide the template for identifying the Mandatory baseline functionality set. From our perspective, eMBB and 6G IoT are most important device types for the 1st release of 6G system. And each 6G MBB device in “low-data rate” mode is naturally a 6G IoT device. For example, when a smartphone is in a “screen off” status or communicates with IoT devices, only the low-rate “background data” are received and transmitted, and only “IoT-like” traffic is conveyed for the smartphone. Therefore, the low-tier 6G IoT provides a proper design template for 6GR Mandatory baseline functionality set. Starting from the template, some functionalities specially optimized for low-end IoT devices and not fit to other device types (e.g., extreme coverage enhancement for MTC sensors) can be cut off.

Therefore, a proper working approach is to firstly define the mandatory functionality set for low-tier6G IoT, then use the low-tier 6G IoT functionality set as a template to form the Mandatory baseline functionality set, where existing functionalities unjustified for other device types should be taken out, and necessary “outside” functionalities could be added in.

Figure 2.9.3-1 Low-end IoT provides a good template for defining Basic feature set

Use the lowest-tier 6G IoT functionality set as a template/starting point for definition of the Mandatory baseline functionality set for 6GR.

Functionalities specially optimized for low-tier 6G IoT (e.g., extreme coverage enhancement) does not have to be included in the Mandatory baseline functionality set.

To define the Mandatory baseline functionality set, three device types (6G IoT, eMBB and FWA) are listed in Table 2.9.3-1 for example.

Taking the categorization similar as in Table 2.9.3-1 as a starting point for the Mandatory baseline functionality set and device types.

Table 2.9.3-1: Mandatory baseline functionality set

| Mandatory Capability | Mandatory baseline functionality set | Examples of device types |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6G IoT (Lowest-tier) | eMBB (Handheld UE) | FWA (e.g., CPE) |
| Max. UE bandwidth | [5~20MHz] | [5~20MHz] | [200MHz] | [400MHz] |
| Peak data rate | [DL: 5Mbps/ UL: 5Mbps] | [DL: 5Mbps/ UL: 5Mbps] | [DL: 5Gbps/ UL: 2Gbps] | [DL: 5Gbps/ UL: 2Gbps] |
| Max. MIMO layers | 1T1R | [1T1R] | [3T6R] | [4T8R] |
| UE power class | N.A. | [26dBm] | [29dBm] | [32dBm] |
| Duplex mode | FDD/TDD | FDD/TDD/HD-FDD | FDD/TDD/SBFD | FDD/TDD/SBFD |
| Max. DL modulation order | 64QAM | 64QAM | [1024QAM] | [4096QAM] |
| UE energy efficiency | Basic power saving | Enhanced power saving | Basic power saving | Basic power saving |
| Coverage | Basic coverage enhancement | Extreme coverage enhancement | Basic coverage enhancement | Basic coverage enhancement |
| Mobility/speed | [3km/h] | [120]km/h | 120km/h | 3km/h |
| … |  |  |  |  |

Conclusion

This paper discussed the 6GR system parameter, and got below observations and proposals.

Waveform

Observation 1: 	Other OFDM based waveforms are not precluded from RAN1 study.

Proposal 1: 	CP-OFDM and DTF-s-OFDM can be used as baseline to start the RAN4 6G study, other waveforms can be considered later pending on RAN1 conclusions.

Observation 2: 	The DFT-s-OFDM for DL can provide NES gain and coverage gain in different channels.

Observation 3: 	The two DL waveforms (DFT-s-OFDM and CP-OFDM) will increase UE implementation complexity.

Proposal 2: 	To study the DL DFT-s-OFDM waveform impact on RAN4 RF requirements.

Observation 4: 	Before introducing higher order modulation, MPR study, Link level study, tthroughput and coverage study are needed in RAN4.

Proposal 3: 	For high modulation, to further study the MPR, required SNR and throughput and coverage in RAN4.

Observation 5: 	Geometric shaping has non-uniform constellation.

Observation 6: 	For higher order modulation, 1dB gain can be reached by the non-uniform constellation.

Proposal 4: 	Further study Geometric Shaping impact on spectrum utilization and UE RF requirement if introduced in RAN1.

Observation 7: 	Probabilistic shaping uses uniform constellation but with different probability of each point.

Observation 8: 	1dB gain can be reached by the PS with QAM constellation.

Proposal 5: 	Further study Probabilistic Shaping impact on spectrum utilization and UE RF requirement if introduced in RAN1.

Proposal 6: 	To introduce 8192 maximum FFT size in 6GR.

Numerology

Observation 9: 	In SID, it has indicated to avoid multiple numerologies for the same band / sub-range

Observation 10: 	Most of the FR1 bands can be re-farmed to 6GR, for better co-existence with 5G, the same SCS for FR1 is good for MRSS as for 6GR FR1

Proposal 7: 	For 6GR FR1, TDD bands apply 30kHz SCS and FDD bands apply 15kHz SCS.

Observation 11: 	The new frequency range allocated for 6GR are around 7GHz and around 15GHz.

Observation 12: 	The frequency range for around 7GHz and around 15GHz are not typical FR2 bands, and can be considered to be harmonized to FR1.

Proposal 8: 	For 6GR, 30kHz SCS for around 7GHz range and 60kHz SCS for around 15GHz range is proposed.

Minimum Channel bandwidth

Observation 13: 	5MHz for 15kHz SCS and 10MHz for 30kHz has been agreed as minCBW for FR1 in early 5G NR.

Observation 14: 	3MHz has been introduced in total 12 bands as minCBW at the end of Rel-19.

Proposal 9: 	When define min CBW, initial access, spectrum status and spectrum utilization need to be considered.

Proposal 10: 	5MHz for TDD and 3MHz for FDD bands as the 6GR system minCBW and each band can determine larger minCBW based on spectrum status and operator request.

Proposal 11: 	It is proposed to agree on 3/5 MHz for FDD bands and 5/10/20 MHz for FR1 TDD bands, 10/40MHz for around 7GHz bands, 50MHz for around 15GHz and FR2-1 as minCBW.

Maximum Channel bandwidth

Observation 15: 	Spectrum availability, regulatory constraints, and technical feasibility are used to determine the max CBW.

Observation 16: 	50MHz and 100MHz has been proposed as maxCBW in FR1 considering 15kHz SCS and 30/60kHz SCS in 5G.

Observation 17: 	Currently the 5G FDD bands with the largest available spectrum is band n65 with UL and DL both 90MHz.

Observation 18: 	27 of 36 FDD bands has smaller than 50MHz available spectrum.

Observation 19: 	Use 50MHz for FDD bands as maxCBW can enjoy less UE implementation complexity and power consumption.

Proposal 12: 	For 6GR FR1 FDD bands, the maxCBW is proposed to be 50 MHz. Further study if 100MHz can be used for maxCBW.

Observation 20: 	Considering the exact spectrum allocation, UE implementation and PA bandwidth, the maxCBW in 5G NR FR1 is 100MHz.

Proposal 13: 	For 6GR FR1 TDD bands, the maxCBW is proposed to be 200MHz.

Observation 21: 	Together consider the SCS and UE implementation complexity, 200MHz is more suitable for around 7GHz range.

Proposal 14: 	For 6GR around 7GHz bands, the maxCBW is proposed to be 200MHz corresponds further SCS study.

Observation 22: 	The spectrum available in 15GHz is only 500MHz

Observation 23: 	For 6GR around 15GHz bands, the maxCBW is proposed to be 400 corresponds further SCS study.

CBW step size

Observation 24: 	The step size is 5MHz from 5 to 50MHz and 10MHz from 50 to 100MHz as smaller step size in the small CBW and larger step size in the larger CBW.

Observation 25: 	In 5G, irregular bandwidth has been introduced with only one specific CBW at one time which cannot match the large number of fragmented spectrums hold by operators.

Proposal 15: 	Propose 5MHz step size for 5 to 50MHz; 10MHz step size for 50 to 100MHz and 20MHz step size for 100 to 200MHz.

Flexible Channel bandwidth

Observation 26: 	Smaller granularity than 5/10MHz is needed such as 7MHz which might need 1MHz granularity,

Observation 27: 	Many operators’ spectrum doesn’t have suitable 3GPP CBW which makes the spectrum cannot be fully utilized.

Observation 28: 	If choose the nearest small regular CBW, then the additional spectrum is wasted.

Observation 29: 	If choose the nearest large regular CBW, then the filter design does not match the bandwidth which will suffer interference.

Observation 30: 	If BS support irregular CBW while UE support regular CBW, the component carrier RB level alignment, channel raster SCS level alignment and large enough overlapping part to put SSB and Correset0 are hard to guarantee.

Observation 31: 	For small irregular CBW, it is difficult to put two SSB non-overlapping with the BS CA method.

Observation 32: 	Current solution doesn’t work well with irregular CBW.

Proposal 16: 	To introduce flexible CBW in 6G to solve the 5G irregular CBW.

Observation 33: 	The introduction of flexible channel bandwidth need to further study the UE RF requirement applicability, guard band definition and reduce test burden.

Observation 34: 	All requirements can either scale with bandwidth or be irrelevant with the bandwidth.

Proposal 17: 		For the requirements not related to CBW, same requirement apply when introduce flexible channel bandwidth.

Proposal 18: 	For requirements are scalable with bandwidth, the requirements can be scaled with flexible channel bandwidth same as NR principle.

Observation 35: 	Examples has been shown for different methods with different RBs for flexible CBW.

Observation 36: 	The nearest SU method as smaller SU is chosen to guarantee the guard band but the scaling method helps to guarantee the SU

Proposal 19: 	To further study the two methods as nearest SU method and scaling SU method and their impacts to SU.

Observation 37: 	Current 5G carrierBandwidth for both initial access and RRC connected mode can support the flexible RB configuration for irregular CBW.

Proposal 20: 	Further study the signalling design of 6GR to guarantee the flexible CBW work properly.

Proposal 21: 	To reduce test burden, specific regular channel bandwidth will be defined and the test only apply to regular channel bandwidth.

UL/DL asymmetric CBW

Observation 38: 	In 5G, besides of symmetric CBW, the asymmetric CBW in downlink and uplink are specified separately, and UE needs to indicate whether it can support asymmetric CBW through signalling.

Proposal 22: 	Proposal 8: All bands can apply symmetric/asymmetric CBW in downlink and uplink in 6G day one.

Number of RX and TX

Observation 39: 	For hand-held UE, maximum capability is 2TX and 6RX till the end of NR evolution.

Observation 40: 	For reduced capability UE, 1T1R for FR1 FDD and 1T2R for FR1 TDD have been agreed.

Proposal 23: 	The number of TX/RX below is proposed based on different device types:

Table 2.6-1 Proposed Number of TX and RX for different device type

|  | Device type |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 6G IoT (Lowest-tier) | eMBB (Handheld UE) |  |  | FWA (e.g., CPE) |
|  |  | FR1 FDD | FR1 TDD | FR3 |  |
| number of Tx/Rx | Baseline: 1T1R Optional: 2R | Baseline: 1T2R Optional: 2T/3T; 4R | Baseline: 1T4R Optional: 2T/3T; 6R | Baseline: 2T4R Optional: 3T; 6R | Baseline: 4T8R Optional: 6T;12R |

Sync raster

Observation 41: 	SCS level alignment of sync raster and channel raster has been agreed in 5G NR and kssb is introduced to find the RB edge.

Observation 42: 	For 5MHz minCBW, the SS granularity is 1.2MHz for smaller than 3GHz and 1.44MHz for larger than 3GHz.

Observation 43: 	For 3MHz minCBW, the SS granularity is 0.6MHz.

Proposal 24: 		Define unified sync raster design for different bands, minCBW, SCS and channel raster in 6G.

Observation 44: 	MinCBW, Channel raster, BWSSB are the main factors that impact the sync raster design. Mi CBW will limit the sync raster steps. Make sure all possible channel has SSB.

Observation 45: 	By obeying the Rule 1 as for each carrier of minCBW, there should be a whole SSB included, we can have coarser sync raster points.

Proposal 25: 	Consider allowing some channels that do not have SSB like pure data channel.

Proposal 26: 	 6GR sync raster design to have coarser sync raster points and try to avoid Kssb.

Observation 46: 	Using 1RB as reference channel raster can avoid Kssb since the grid are RB level aligned.

Observation 47: 	The reference channel raster is coarser than the channel raster.

Observation 48: 	With the granularity of different minCBW are integer numbers of times of 3MHz granularity, the all-other sync raster points of larger minCBW are subset of sync raster points of 3MHz.

Observation 49: 	This ensure with different minCBW, no new sync raster points are introduced.

Observation 50: 	With 1RB reference channel raster, for different minCBW cases, large sync raster points reduction can be reached.

Proposal 27: 	It is proposed to use 1RB as reference channel raster for specific initial access carrier.

Proposal 28: 	The per band basis minCBW apply and the sync raster granularity can be defined accordingly.

| Minimum BW | SCS for SS | NRB,Carrier | ΔFSS,Raster |
| --- | --- | --- | --- |
| 3 | 15 | 15 | 0.72 |
| 5 | 15 | 25 | 2.16 |
| 10 | 30 | 24 | 4.32 |
| 15 | 30 | 38 | 9.36 |
| 20 | 30 | 51 | 14.4 |

Observation 51: 	The data carrier can use channel raster to enjoy the full SU as well as the flexibility of deployment.

Proposal 29: 	The data carrier after IA can use channel raster to establish the specific carrier.

Channel raster

Observation 52: 	5G has introduced 10MHz enhanced channel raster to get rid of 100kHz raster from LTE.

Observation 53: 	5kHz is the GCD of 10kHz and 15kHz and used as global frequency raster in NR-ARFCN.

Proposal 30: 	For re-farming FR1 bands with 100khz channel raster, using 5khz common channel raster, and avoid diverse channel raster in these bands. For other FR1 bands and new bands, SCS based channel raster is adopted

Spectrum Utilization

Observation 54: 	Theoretically, still some potential RBs can be further used to enhance the SU.

Observation 55: 	The waveform, modulation, SEM, EVM, ACLR, demodulation and new PA models are factors need to be considered in the SU evaluation.

Proposal 31: 	Start with the agreed baseline waveform, modulation, SEM, EVM, ACLR, demodulation and new PA models, to evaluate how much RB can be improved in the SI.

Observation 56: 	NR has agreed the relaxation of ACLR/SEM/EVM to improve the MPR

Proposal 32: 	Study the relaxation of ACLR/SEM/EVM requirements to improve the SU in 6G.

Device Type

Observation 57: 	The 5G device types are not mutually exclusive. Different device types have been introduced as some of them are depend on the usage scenario such as vehicular, UAV, XR and NTN while some of them are depend on capability such as smartphone, FWA and redcap.

Observation 58: 	Differentiate device types from UE RF capability perspective and they are number of TX/RX, maxCBW, duplex mode, Max MIMO layers and CA support.

Proposal 33: 	Define a “lean” Mandatory baseline functionality set, in which each functionality shall be well justified to be truly mandatory for all device types.

Proposal 34: 	Purely device type-specific attributes (e.g., mobility/moving speed, UE power class, etc.) are not necessarily included in Mandatory baseline functionality set.

Proposal 35: 	Use the lowest-tier 6G IoT functionality set as a template/starting point for definition of the Mandatory baseline functionality set for 6GR.

Proposal 36: 	Functionalities specially optimized for low-tier 6G IoT (e.g., extreme coverage enhancement) does not have to be included in the Mandatory baseline functionality set.

Proposal 37: 	Taking the categorization similar as in Table 2.9.3-1 as a starting point for the Mandatory baseline functionality set and device types.

Table 2.9.3-1: Mandatory baseline functionality set

| Mandatory Capability | Mandatory baseline functionality set | Examples of device types |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6G IoT (Lowest-tier) | eMBB (Handheld UE) | FWA (e.g., CPE) |
| Max. UE bandwidth | [5~20MHz] | [5~20MHz] | [200MHz] | [400MHz] |
| Peak data rate | [DL: 5Mbps/ UL: 5Mbps] | [DL: 5Mbps/ UL: 5Mbps] | [DL: 5Gbps/ UL: 2Gbps] | [DL: 5Gbps/ UL: 2Gbps] |
| Max. MIMO layers | 1T1R | [1T1R] | [3T6R] | [4T8R] |
| UE power class | N.A. | [26dBm] | [29dBm] | [32dBm] |
| Duplex mode | FDD/TDD | FDD/TDD/HD-FDD | FDD/TDD/SBFD | FDD/TDD/SBFD |
| Max. DL modulation order | 64QAM | 64QAM | [1024QAM] | [4096QAM] |
| UE energy efficiency | Basic power saving | Enhanced power saving | Basic power saving | Basic power saving |
| Coverage | Basic coverage enhancement | Extreme coverage enhancement | Basic coverage enhancement | Basic coverage enhancement |
| Mobility/speed | [3km/h] | [120]km/h | 120km/h | 3km/h |
| … |  |  |  |  |

Reference

RP-251881	New SID: Study on 6G Radio, NTT DOCOMO (Moderator)