3GPP TSG-RAN WG4 Meeting #116bis		R4-2513316
Prague, Czech Republic, 13 – 17 Oct 2025

Title:	6G UE RF

Source:	OPPO

Agenda item:	8.4

Document for:	Approval

Introduction

RAN4 starts the 6GR study from this meeting. This paper shares our view on what can be and need to be studied at this stage. Based on the experiences that learned from 5G NR, the 6GR design in RAN4 needs to be more careful to make the spec concise and accurate.

Below, we will start with reviewing the UE RF requirement framework, and find out the enhancing areas and the requirements which may depends on RAN1 conclusions. Then we will analyse the detailed RF requirements which needs to be studied further and give the proposals based on our analysis.

Discussion

RF requirement framework

The UE RF requirement frameworks for OFDM system have been established in 4G, and then reused in 5G. It is expected that most of the requirements will continue to be reused in 6G, considering RAN1 has decided in their first meeting that the OFDM will be used the waveform in 6GR. And the waveform has much impact to RAN4 especially the Tx requirements.

Observation 1:   RAN1 has decided to continue use OFDM in 6GR. From RAN4 perspective, most of the 5G NR UE RF requirement frameworks can be reused for 6GR.

Below gives the initial view on the RF requirements for 6GR where some requirements may depend on RAN1/2 discussion while some requirements can reuse the 5G NR framework especially for the legacy bands. RAN4 discussion can start from the requirements which have less RAN1/2 dependants.

Proposal 1:	Consider take below table 1 as starting point for 6G UE RF discussion.

Table 1 Initial view on UE RF requirements for 6GR

| RF requirements | RAN4 scope | Whether depends on RAN1/2 |
| --- | --- | --- |
| Max Tx power | - Discuss power class framework for single CC and for band combinations in 6G - Discuss default power class for FR1/2/3 | MIMO capability and UE types discussion in RAN1 may have impact on this topic |
| MPR | - Streamline MPR tables for 1T/2T/MIMO/TxD/different UE types… - Discuss MPR reduction and improvement schemes | Waveform, modulation, and UE types discussion in RAN1 may have impact on this topic |
| AMPR | - Depends on whether new special regulation requirements for the new bands | N |
| Pcmax | - Discuss how to solve “power class fallback” schemes in 5G, e.g., SAR related - Discuss how to simplify the Pcmax definitions, e.g., SRS IL, power boosting - Discuss the approaches to enable UE fully utilize its PA capabilities | RAN1/2 power control and PHR reporting may have impact on this topic |
| Min Tx power | - Most likely can reuse 5G framework | N |
| OFF power | - Most likely can reuse 5G framework | N |
| ON/OFF time mask | - Reuse 5G framework in general ON/OFF time mask - FFS on how to harmonize the switching related time masks | RAN1/2 Tx/Rx switching related schemes may have impact on this topic |
| Power control tolerance | - Most likely can reuse 5G framework | RAN1/2 power control may have impact on this topic |
| Freq error | - Most likely can reuse 5G framework | N |
| EVM | - Discuss EVM requirements for high modulation, and non-uniform constellations - Discuss EVM relaxation schemes for high modulation | RAN1/2 highest modulation order and constellation design may have impact on this topic |
| Carrier leakage | - Most likely can reuse 5G framework | RAN1 physical design may have impact on this topic, e.g., whether the DC will carry data |
| IBE | - Most likely can reuse 5G framework | N |
| Occupied BW | - Most likely can reuse 5G framework for some CBWs - The spectrum utilization discussion in system parameter part will impact this requirement | RAN1 spectrum utilization discussion may impact this topic |
| SEM | - FFS on whether new SEM requirement is needed for the new spectrum, if not can reuse 5G framework | N |
| ASEM | - Depends on whether new special regulation requirements for the new bands | N |
| ACLR | - Most likely can reuse 5G framework for PC3/2/1.5 ACLR - FFS on the new power class demands | RAN1 physical design may have impact on this topic |
| SE | - Most likely can reuse 5G framework | N |
| Tx intermodulation | - Most likely can reuse 5G framework | N |
| Rx sensitivity | - Discuss the REFSENS for new high bands, e.g., NF and SNR | RAN1 physical design may have impact on this topic, e.g., the SNR demodulation target |
| Max input level | - Most likely can reuse 5G framework | N |
| ACS | - FFS on new high bands | RAN1 physical design may have impact on this topic |
| IBB | - FFS on new high bands | N |
| OOB | - FFS on new high bands | N |
| Narrow band blocking | - Most likely can reuse 5G framework | N |
| Spurious response | - FFS on new high bands | N |
| Rx intermodulation | - FFS on new high bands | N |

Power class for FR1

    - General view on power class

Power class is the most important requirement among the UE RF Tx requirements. A good power class definition should make sure UE can fully utilize the PA power capabilities while distinguish different levels of UEs in terms of Tx power.

Proposal 2:	Power class definition should make sure UE can fully utilize the PA power capabilities while distinguish different levels of UEs in terms of Tx power.

Currently in RAN4, the power class definition mainly includes the power class for single CC and power class for band combinations. Below will review the current status and see what can be optimised based on the experience in 5G.

    - Power class for 1CC

Power steps among power classes

In NR, the power class for FR1 1CC has specified PC5, PC3, PC2, PC1.5, PC1. Among them the PC5 is for unlicensed bands, and PC1 is for public safety or cab-radio UE. And for handheld UE, PC3/2/1.5 are mainly used.

In below figure 1, it summarizes all the power classes in FR1. It can be seen that 3dB power step has been followed among PC5/3/2/1.5, but the step becomes 2dB between PC1.5 and PC1. In Rel-20, there are some discussions in RAN#109 on whether to introduce a new PC1bis for UE with 4x26dBm PAs.

When PC1 was introduced in the past, it was based on one public safety PA which can only achieve 31dBm. Now for UE with multiple handheld PAs, it can achieve 32dBm which is 3dB step comparing to PC1.5. If further consider the +/-2dB tolerance, then the power levels will be much overlapped between PC1.5 and PC1 for handheld UE.

The power class definition in RAN4 should be streamlined from the beginning and it is proposed to keep 3dB power step for all the power classes.

Observation 2:   Less than 3dB power step makes the power classes are not well distinguished and has much overlapping power.

Proposal 3:	Keep 3dB power step among power classes.

Figure 1 summary of power class definition in 5G

How to handle the power gap b/w UE max Tx power and power class reporting

For UE with multiple PAs in single CC, the total power sometimes is not exactly the same as the power class definition as shown in below table 2. It can be seen, in some PA implementations, the gap between max Tx power and power class can reach 1.8dB.

Although RAN4 has defined the +/-2dB power tolerance together with the power class, this tolerance is mainly for the purpose of accommodating massive production variations. It cannot be used to solve this power gap.

Observation 3:   For UE with multiple Tx PAs, the gap between max UE power capability and power class can reach 1.8dB.

Observation 4:   The +/-2dB power tolerance was introduced to accommodate massive production variations.

Table 2 Gap between max Tx power and power class

| UL MIMO | UE PA capability | Max Tx power (dBm) | Power class report | Gap between max Tx power and power class (dB) |
| --- | --- | --- | --- | --- |
| 2Tx | PC3+PC2 | 27.8 | PC2 | 1.5 |
| 3Tx | PC3+PC3+PC3 | 27.8 | PC2 | 1.5 |
|  | PC3+PC2+PC2 | 30 | PC1.5 | 1 |
|  | PC2+PC2+PC2 | 30.8 | PC1.5 | 1.8 |
| 4Tx | PC3+PC3+PC3+PC2 | 30 | PC1.5 | 1 |
|  | PC3+PC3+PC2+PC2 | 30.8 | PC1.5 | 1.8 |
|  | … | … | … | … |

This problem is not severe in the past because most of the commercial UE can only support one PA with PC3 or PC2 mostly or support two equal PC3/PC2 PAs. However, in 6G, it is very likely that UE will support more Tx antennas like 2Tx/3Tx/4Tx with various PA capabilities then the total power will be different from the defined power class.

Whether and how to solve this upcoming issue needs to be considered in the beginning of the 6G power class definition. One way might be to borrow the 5G power boosting scheme that allow UE to indicate the increased power comparing to power class and keep the +/-2dB power tolerance.

Figure 2 fully utilize UE PA capabilities by power boosting

Proposal 4:	Study how to handle the issue of power gap b/w UE max Tx power capability and power class reporting in the beginning of 6G

Proposal 5:	Consider reuse the power boosting scheme that allow UE to report the boosted power comparing to the reported power class.

Power class for band combination

In 5G, the total power of the band combination has been used to define the power class. However, this kind of power class definition has put many restrictions on fully utilize of UE power capabilities especially when the total power is not exactly the same as the power class. With many painful discussions in 5G era, it has been recognized that total power limitation is unnecessary for band combinations.

Therefore, in 6G, it is proposed to remove the power class definition of band combinations and allow UE to transmit the max power of each band in the BC. However, it should be noticed that the per band power capability in a band combination might be different from the single band.

As shown in figure 3, for band A and band B each has two PC2 PAs, and can reach PC1.5 for single band, then in the band combination, this UE can transmit with one PA in each band, i.e., 26dBm of each band, which is different from the single band power class. Therefore, the max power of a band combination is determined by per band per BC power class.

Figure 3 power class definition for the band combination

Proposal 6:	For band combination, not define the total power limitation. Rely on the power capability of each band in the BC (ue-PowerClassPerBandPerBC) to determine the max Tx power.

Power class for different MIMO layers

In 5G, UE only report one power class for each band and no differentiation of MIMO layers. However, as shown in figure 4, for UEs not support TxD (most of commercial UEs don’t support it), the max power capability for single port configuration is much lower than 2/3/4 Layer UL MIMO. This leads to UE MOP capability actually is different for different MIMO configurations.

If only report one power class for each band, then the UE in figure 4 can only report PC2 which makes 3dB power loss for the UL MIMO.

Today this problem might be not severe due to no (or not many) UE supports PC1.5 in the market, later on when PC1.5 is widely adopted in 6G, then this inconsistency will be widely shown. And this issue needs to be considered in the power class definition and allow UE to report different power class for different MIMO Layers.

Figure 4 power class difference among MIMO layers

Observation 5:   For UEs not supporting TxD, the power capability could be different for different MIMO layer configurations, like PC2 in single port, but PC1.5 for UL MIMO. This power difference cannot be differentiated with 5G NR per band power class reporting.

Proposal 7:	For single band power class reporting, allow UE to report different power class for different MIMO Layers.

Default power class

In 5G NR, PC3 was defined as the default power class, and then HPUE like PC2/1.5 and lower power class like PC5 were introduced in a later stage. Currently, PC2 has been widely supported on the market and be considered useful to improve the UL coverage.

In 6GR, considering the current industry status, it might be useful to define PC2 as the baseline especially for TDD bands. For the FDD bands, there might be still some difficulty in supporting PC2. Whether PC2 can be defined as the baseline is FFS.

Observation 6:   For TDD bands, PC2 has been widely supported on the market.

Proposal 8:	For TDD bands, PC2 is defined as baseline. FFS whether PC2 or PC3 is baseline for FDD bands.

MPR

MPR definition

MRP requirements were defined based on configurations of waveform, modulation, RB allocation, power class, UE types (antenna isolation), MIMO configuration, and PA/RFIC models. And it has some dependent on RAN1 discussion especially the waveform and modulation. RAN4 may needs to wait for RAN1 conclusions before can really start the requirement discussions for 6G.

However, in the SI, RAN4 indeed can consider which aspects could be improved inside RAN4 like the PA models/RFIC impairments and how to align between measurements and simulations. Below table 3 has summarized the factors which impact MPR definition and also the status.

Table 3 Factors impacts MPR definition and its status

| Factors impact MPR | Status and what RAN4 can do |
| --- | --- |
| waveform | - RAN1 has decided to continue use DFT-S-OFDM and CP-OFDM as baseline for the UL, but still keep open for further optimisations or even new waveforms. - RAN4 can use DFT-S-OFDM and CP-OFDM as starting point in MPR discussion. |
| modulation | - RAN1 has decided to use uniform modulations up to 256QAM as basis for UL, but FFS on other enhancements like non-uniform modulations - RAN4 can use uniform modulations as starting point in MPR discussion. |
| power class | - RAN1 has some discussion on the mandatory power class in each device type - RAN4 needs to support all power classes in the MPR definition, therefore can start the work without impact from RAN1. |
| UE types (antenna isolation) | - RAN1 has discussed a lot on device types, however, mainly on capability reporting/mandator features perspective - The UE types considered in RAN4 mainly from UE requirement definition perspective which is somehow different from RAN1 discussion. In MPR discussion, RAN4 can focus on how much dB antenna isolation need to be considered and how this impacts the RIMD in the MPR simulation/measurements. |
| MIMO configuration | - Although RAN1 has not start the discussion in MIMO, the 5G NR MIMO configurations can be used as starting point. |
| PA model | - RAN4 has used the polynomial PA models to simulate the MPR/AMPR for a long time. RAN4 can discuss which kind of PA models to be used in 6G like fixed biased model, APT model, etc. to better align with the measurements. |
| RFIC models | - Carrier leakage, IQ image, CIM3 has been modeled in the MPR simulation to mimic the RFIC impairments. These factors were agreed in the beginning of 5G, RAN4 may need to review these RFIC impairments in 6G. |
| Simulation and measurements | - Both methods have been used in 5G to define the requirements, however, sometimes they are not aligned well. How to better align them might need to be further considered. |

Observation 7:   In MRP requirements definitions, waveform, modulation, RB allocation, power class, UE types (antenna isolation), MIMO configuration, and PA/RFIC models need to be considered. Among them some of the factors have some dependent on RAN1.

Proposal 9:	RAN4 can focus on below aspects to study the 6G MPR

        - Waveform: use DFT-S-OFDM and CP-OFDM as starting point

        - Modulation: use uniform modulations as starting point

        - Power class: take PC2 as starting point

        - UE types: use handheld UE as starting point, and FFS on how much dB antenna isolation need to be considered for other UE types

        - MIMO: single port as starting point

        - PA model: discuss which kind of PA models to be used in 6G, e.g., fixed biased model, APT model, etc. and also consider the new high bands

        - RFIC impairment: use 5G carrier leakage, IQ image, CIM3 assumptions as starting point and further discuss whether some updates can be made

        - Study how to better align simulation and measurements results.

MPR reduction with requirement relaxation

In 5G Rel-19, MPR reduction has been discussed a lot under several requirement relaxation conditions like ACLR/SEM relaxation, and in Rel-20 the MPR reduction based on EVM relaxation will also be considered. However, all these MPR reductions are optional features based on UE capability report and NW enable/disable. It means, UE still needs to comply with the legacy requirements. In 6G, it might be more meaningful to study whether these requirement relaxation scenarios could be used as baseline to increase UE Tx power.

Proposal 10:	Study whether the requirement (ACLR/SEM/EVM) relaxation scenarios could be used as default in 6G to make it more useful.

MPR calibration updates

In 5G, RAN4 has defined some PA calibration configurations (table 4) to align the simulation results and also measurement results. Many of these configurations are inherited from LTE like the CBW/RB allocation/waveform, etc. At the beginning of 6G, it might be a good chance to review these calibration conditions to better fit for 6G requirement definition.

Table 4 PA calibration configurations in 5G

|  | 5G |
| --- | --- |
| CBW, scs, RB allocation | 20MHz, 15khz scs, 100RB@0 |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Targe power level to meet ACLR | 1dB MPR |
| carrier leakage | -28dBc |
| IQ image | -28dBc |
| CIM3 | -60dB |

CBW/RB allocations

In RAN4#108 and following meetings, there were some discussions on whether 20MHz with 100RB allocation can be updated to 40MHz with full RB allocation considering the 20MHz with 100RB allocation is not the worst case in terms of spectrum utilization as can be seen in below figure [1]. And it pointed out that current less stringent PA calibration point will lead to worse PA calibration results and then lead to higher MPR defined. However, in the following meetings, some other paper [2] also did some analysis and pointed out that the calibration CBW impact might be different for different UEs and considering the potential impacts to legacy UE this discussion was ended without change of the calibration conditions.

Figure 5 spectrum utilization comparison between 20M/100RB and 40M/full RB

Observation 8:   The CBW/RB allocation used in 5G MPR calibration is not the worst case in terms of spectrum utilization which makes the calibrated PA has worse linearity and finally large MPR requirements.

At the beginning of 6G, no legacy UEs need to be considered which makes it a good chance to further review this CBW and RB allocation used in the MPR calibration.

To better understand how much the impact of using 20MHz/100RB@0 comparing to 40MHz/full RB, we recalibrated the PAs with these two configurations, and under different RFIC impairments, as can be seen in figure 6. The calibrated PA linearity difference seems quite small.

Figure 6 PA calibration results with 20MHz/100RB@0 comparing to 40MHz/full RB

The MPR simulation results for QPSK and 64QAM with these two calibrated PA can be found in figure 7. It can be seen no much difference can be observed.

Figure 7 MPR results with PAs calibrated with 20MHz/100RB@0 comparing to 40MHz/full RB

Observation 9:   The PA calibration results and MPR simulation results don’t show obvious difference between the calibration conditions of 20MHz/100RB@0 and 40MHz/full RB.

Therefore, the CBW and RB allocations used in 5G (20MHz/100RB@0) can be reused in 6G.

Proposal 11:	Continue to use 20MHz/100RB@0 as the MPR PA calibration condition in 6G.

Carrier leakage/IQ image

For carrier leakage and IQ image, in 5G NR, -28dBc has been used in the calibration with QPSK modulation. However, for the high modulations like 256QAM, this value needs to be tightened to -34dBc to pass the EVM requirements. Fundamentally, there is no difference in RFIC carrier leakage and IQ image in terms of modulation orders, therefore, it might be straight forward to tighten carrier leakage and IQ image from -28dBc to -34dBc.

Observation 10:	To pass the EVM requirements of 256QAM, -34dBc for carrier leakage and IQ image has been used in 5G MPR simulation/measurements. There is no difference in RFIC carrier leakage and IQ image in terms of modulation orders.

The impacts of tightening carrier leakage/IQ image to PA calibration have been analysed with simulations as shown in figure 6, and MPR requirement impacts to 64QAM and 256QAM as shown in figure 8. It can be seen, the carrier leakage/IQ image tighten to -34dBc can improve the MPR for 64QAM around 0.6dB, and is necessary for 256QAM in central RB allocations.

Figure 8 MPR results with PAs calibrated with -28dBc comparing to -34dBc

Observation 11:	Carrier leakage/IQ image tighten to -34dBc can improve the MPR for 64QAM around 0.6dB in some allocations, and is necessary for 256QAM.

Proposal 12:	The carrier leakage and IQ image condition for MPR calibration is tightened from -28dBc to -34dBc in 6G.

PA models

In MPR requirement definition, PA models are used in the simulation platforms to derive the final MPR requirements. The PA models used in the simulation play a key role. However, there is no aligned single PA model in the LTE and NR MPR discussions due to various PAs on the market and implemented in the UE. In reality, the PA models used in each company’s platform are different, for example, some are based on measured PA models used in the product, others are based on PA models shared in papers. This is one of the root causes that even after PA calibration the MPR simulation results are still different. It is expected this trend will continue in 6G.

Observation 12:	Due to various PAs on the market and implemented in UE, no single PA model can be universally used in the simulations. This trend will continue in 6G.

Proposal 13:	The PA models used in 6G MPR simulation is still based on each company’s own choice.

Regarding the detailed PA models, fixed biased PA model has been widely used in the past, however, this kind of PA is different from the real PAs used in the products which is either APT PA or ET PA. This is one of the reasons that lead to MPR difference between simulation and measurement.

To better align with the real products, APT PA is preferred in principle. The question is how much difference between fixed bias PA and APT PA which is unknow up to now. To better understand this issue, the APT PA model from real PA and also fixed bias PA from real PA are both implemented in our MPR simulation platform. Some comparisons have been made in terms of MPR requirements.

It is understood that for a typical APT PA, it may include several operating modes/statuses like high power mode (HPM), middle power mode (MPM), and low power mode (LPM), etc. The APT PA will change its operating status when the desired output power is different, for example change from HPM to MPM when the output power is below the targeting power to reduce power consumption but with the sacrifice of PA linearity. This will lead to higher MPR desired comparing to the fixed bias PA.

Observation 13:	APT PA is widely used in UE implementation, and the APT PA will change its working modes when the output power is different which could reduce the power consumption but with the sacrifice of PA linearity, i.e., larger MPR.

Based on the simulation results with fixed bias PA and APT PA, it was observed that when the required MPR is not large with fixed bias PA, e.g., less than 7dB for PC3 (this value could be different for different APT PA), then there is no difference comparing to APT PA. However, when the MPR is larger than that it will be different. Among the simulation results, it was observed that the difference could be larger than 5dB in some configurations.

Therefore, in 6G the APT PA should be used in MPR simulations especially when the MPR is large to better align with the real products and also measurements.

Observation 14:	From simulation results it can be seen that the MPR required for APT PA could be 5dB higher than fixed bias PA in some configurations.

Proposal 14:	The APT PA models is encouraged to be used in 6G MPR simulation to better align with the real products and measurements.

MPR summary

To summarize, the below table is proposed as starting point for 6G MPR simulations.

Proposal 15:	The 6G MPR simulation calibration conditions is as below table.

|  | 6G |
| --- | --- |
| CBW, scs, RB allocation | 20MHz, 15khz scs, 100RB@0 |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Targe power level to meet ACLR | 1dB MPR |
| carrier leakage | -34dBc |
| IQ image | -34dBc |
| CIM3 | -60dB |

Pcmax

Pcmax,h and power boosting

Pcmax gives the UE Tx power higher and lower limit under certain configurations. And the lower limit has drawn more attention usually than the higher limit since the lower limit gating UE from working worse. However, the higher limit now also has become more and more limiting UE performance especially for UEs with several Tx PAs as discussed in section 2.2.2.2.

Observation 15:	Pcmax,h has become the gating factor for UE to transmit higher power especially when UE has multi-Tx PAs.

To allow UE to transmit higher Tx power whenever possible, power boosting could be one of the solutions which has been adopted in 5G, e.g., delta Ppowerboost in the equation. Another solution is to remove this Pcmax,H and allow UE to transmit as high Tx power as possible, this may draw the concern of coexistence. Both solutions could be discussed in 6G study phase, and using power boosting could be more feasible.

Observation 16:	Power boosting or remove the Pcmax,h are two potential approaches to allow UE transmit higher power whenever possible.

Proposal 16:	Study how to enable UE to fully utilize its PA capabilities. And power boosting approach is preferred comparing to remove the Pcmax,h limits.

SAR and power fallback

Schemes to solve SAR issue have been widely studied in 5G including single CC HPUE and band combination HPUE, and for different power classes. Much meeting time has been spent on this “SAR solutions”, but unfortunately none of the SAR solutions except PAPR has been implemented in the UE.

Meeting time has been wasted in debating how the duty cycle solutions could be designed, and what is the duty cycle relationships among different power classes, and its power fallback. This has made the spec very complex. This kind of “SAR solution” discussion shall be avoided in 6G.

Observation 17:	Duty cycle-based SAR solutions in RAN4 spec is very complex, but it hasn’t been implemented in UE. Only PMPR is applied.

Proposal 17:	In 6G, only rely on PMPR to solve SAR issue and no other SAR solution is to be defined.

EVM

EVM requirement is tightly connected with RAN1 modulation definitions, and in last RAN1 meeting, it has been decided that the uniform modulations are supported as basis. And other modulation schemes are FFS.

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Actually, the agreed uniform modulations are same as 5G, and it is expected that the EVM requirements will be the same for these modulations. What RAN4 needs to pay attention is the potential enhancements in 6G no matter even higher modulations like 1024QAM in UL with AI/non-AI based modulation, or non-uniform modulations like probabilistic shaping as shown in below figure 9 or geometric shaping as shown in figure 10.

All these higher modulation, new non-uniform modulations may need RAN4 to fully study its impacts in terms of implementation complexity, requirements and performance.

Figure 9 Example of probabilistic shaping for 64QAM

Figure 10 Example of geometric shaping for 256QAM

Observation 18:	Besides of the uniform modulations already defined in 5G, some enhancements including even higher modulation, and non-uniform modulations are under discussion in RAN1 which will have impact to RAN4 EVM requirements.

Proposal 18:	RAN4 needs to closely follow the RAN1 discussion on even higher modulations in UL like 1024QAM with AI/non-AI based schemes, and the non-uniform modulations like probabilistic shaping or geometric shaping. And study the impacts in terms of implementation complexity, requirements and performance.

  - Coherent MIMO

Coherent UL MIMO is one of the important MIMO schemes defined in RAN1 in 5G, and RAN4 has specified the requirements in 38.101-1 as below table, however, this scheme hasn’t been implemented in UE. In the past releases, there were some discussions on how to enable this feature in the real network like calibration in UE side, etc. but was not able to be well studied. In 6G, probably this can be studied.

Figure 11 Coherent UL MIMO requirement

Observation 19:	Coherent UL MIMO was defined in 5G, but no be supported in UE/network.

Proposal 19:	Study how to enable coherent UL MIMO to improve UE performance.

REFSENS

REFSENS is the basis of UE RF Rx requirements. In 5G, it has been defined based on below equation where the SNR is determined by the demodulation simulations which depends on RAN1 physical design, and the NF is connected with the targeting bands.

For 6G, the equation can be reused, and RAN4 can start with the discussion of targeting bands and its related NF. After the spectrum utilization in system parameters is clear then the REFSENS will be much clearer.

REFSENS (dBm) = -174dBm/hz + 10lg(RxBW) + NF + 10lg(SU) – DivGain + SNR + IM - SNR = -1dB - DivGain = 3dB - IM = 2.5dB - NF = 10 for n78 and n79

Proposal 20:	The REFSENS calculation equation below can be reused in 6G, and 3dB for diversity gain with 2Rx antennas, 2.5dB implementation margin. For the NF, RAN4 can start with the discussion of targeting bands and analyze its related NF.

- REFSENS (dBm) = -174dBm/hz + 10lg(RxBW) + NF + 10lg(SU) – DivGain + SNR + IM

  - Requirements for different UE types

Device types have been widely discussed in RAN1 and last RAN plenary in terms of the mandatory features. From RAN4 UE RF requirement perspective, the UE types need to be considered is how RAN4 should group the UEs and define requirements for them. In other words, RAN1 may define many device types with different mandatory features, but in RAN4 they might share same UE RF requirements. This is the difference from RAN1 discussion.

Therefore, the discussion of UE types here will focus on the applicable requirements instead of mandatory features which will be covered by system parameter agenda.

Observation 20:	Device types discussed in RAN1 and RAN plenary mainly focus on the mandatory features for each device type which is different from RAN4 requirement definition. RAN1 may define many device types but in RAN4 they may share same RF requirements.

In RAN4 spec, especially 38.101-1, many UE types have been mentioned like Handheld, FWA, FWA form factor, CPE, vehicle, XR, IoT, Redcap, ATG, UAV, UE with large/small antenna isolations…Some of these UE types were introduced into RAN4 spec with some specific purposes, while some are not even carefully considered. This leads to confusions in the industry on which requirement should be applied and what is the difference among these UE types. And for the UE types that are not explicitly mentioned in the spec like laptop and pad which requirement should be applied.

This has led to some chaos in the industry and frequently CRs are brought to RAN4 to clarify this but with no conclusion due to impact on many legacy UEs. For example, whether to use FWA form factor or FWA like UE or FWA UE or CPE or FWA CPE…

Observation 21:	RAN4 has defined many UE types but not be well organized in the 5G spec. This has caused many confusions in the industry in terms of which requirement to be applied to which UE types.

In 6G, RAN4 shall think about the UE types very carefully on how to differentiate UEs and requirements. The UE types used in RAN4 to differentiate requirements should be as less as possible, and these UE types should cover the potential future UEs on the market.

From RF perspective,

- handheld is clearly a most important UE type which is different from other UE types in terms of compact size and sensitive to cost and power consumption.

- Among non-handheld UE,

  - there is no need to further differentiate FWA and CPE which are both with large size, and in the spec they are typically considered as with large (20dB) antenna isolation.

  - The vehicle UE was considered in 5G because of supporting 2Rx antennas in 4Rx mandatory bands and with 10dB antenna isolation.

  - XR introduced in 38.101-1 was also for the sake of relaxing Rx antennas from 4Rx to 2Rx

It can be seen that for most of UE types defined in RAN4, the antenna numbers and antenna isolations were the main concern in 5G.

- For the antenna numbers, it might be not avoidable to mention about specific UE type in REFSENS (as shown in figure 12) due to the concerns from NW side on the exception of supporting less antennas than the mandatory.

- For the antenna isolation, this mainly impacts the MPR requirements (as shown in figure 13), which is not bounded with UE types actually.

Figure 12 Two Rx UE types in REFSENS

Figure 13 FWA form factor special MPR requirement

Observation 22:	For most of UE types defined in RAN4, the antenna numbers and antenna isolations were the main concern in 5G. Antenna numbers mainly about the relaxation from mandatory 4Rx to 2Rx for some specific UE type, and antenna isolation is mainly for the MRP requirement definition.

Therefore, as starting point it might be possible to not specifically mention which UE type is applied for the MPR requirements, but use the technical wording like antenna isolations. Any UE types that meet the antenna isolation conditions can apply the corresponding requirements.

Proposal 21:	Consider using antenna isolations as the discriminator to differentiate applicable MPR requirements only and no bounding of UE types. Any UE that meets the antenna isolation conditions can apply the corresponding requirements.

  - Requirements for different UE architectures

In 5G, when UE RF requirements are defined, the principle of not restricting UE implementation is followed. And when a new feature comes, RAN4 will study the possible RF architectures and make some down selection. Most of the time, in order to accommodate different implementations like different PA capabilities and different antenna isolations, etc. the requirements finally defined are multiple sets of requirements and UE is required to reporting which requirement is applied. This actually complex the specification and burden the signalling reporting.

In 6G, the above-mentioned principle might need to be reviewed, e.g., whether RAN4 needs to cover all possible implementations or can based on one single implementation like the worst case or the most common case, and then define only one set of requirements.

Proposal 22:	RAN4 strive to define only one set of requirements for each feature and avoid multiple sets of requirements whenever possible in 6G.

Proposal 23:	RAN4 further discuss whether the worst case or the most common case implementation should be adopted in requirement definition based on case by case.

UE capability signaling simplification

As pointed out in section 2.9, in 5G RAN4 has defined many UE capabilities to differentiate which requirement UE is complied. And this kind of signaling is not useful in BS scheduling which is only for the testing purpose. However, in the testing/certification, UE declaration approach can be applied to make it clear which requirements should be tested. This makes these signaling are meaningless.

Observation 23:	In the testing/certification, UE declaration approach can be applied to make it clear which requirements should be tested. There is no need to define UE capabilities just for the purpose of differentiating the applicable requirements for a UE.

In 6G, this kind of testing purpose singling should be avoided and when new signaling is introduced it shall be justified that it is useful for BS scheduling.

Besides, the UE capability granularity (per band/BC/FS/CC) also needs to be carefully considered. And avoid defining capabilities with unnecessary small granularity

Proposal 24:	In 6G, the testing purpose singling should be avoided and when new signaling is introduced it shall be justified that it is useful for BS scheduling.

Proposal 25:	In 6G, avoid defining UE capabilities with small granularity when it is unnecessary.

  - FR2 UE RF

In 5G, RAN4 has spent much time in defining the requirements for FR2 in the first release. Then in the following releases some efforts on the enhancements like UL/DL CA and multi-panel transmission, etc. However, in the real field it seems these kinds of enhancements were not deployed. Even the FR2 single CC network deployment is rare, and not much UE support FR2.

In 6G, the FR2 spectrum is part of the targeting spectrum, and RAN4 will define requirements for the FR2 UEs. However, based on the progress on the market, it probably more convenient to reuse the 5G NR FR2 requirements whenever possible. And refrain from enhancements in FR2 like multi-carrier or multi-panel aspects.

Proposal 26:	In 6G, reuse 5G FR2 requirements as much as possible and focus on 1CC in the beginning.

Conclusion

This paper analyzed the UE RF aspects that need to be considered in the study item and got below initial views and proopsals.

RF requirement framework

Observation 1:   RAN1 has decided to continue use OFDM in 6GR. From RAN4 perspective, most of the 5G NR UE RF requirement frameworks can be reused for 6GR.

Proposal 1:	Consider take below table 1 as starting point for 6G UE RF discussion.

Table 1 Initial view on UE RF requirements for 6GR

| RF requirements | RAN4 scope | Whether depends on RAN1/2 |
| --- | --- | --- |
| Max Tx power | - Discuss power class framework for single CC and for band combinations in 6G - Discuss default power class for FR1/2/3 | MIMO capability and UE types discussion in RAN1 may have impact on this topic |
| MPR | - Streamline MPR tables for 1T/2T/MIMO/TxD/different UE types… - Discuss MPR reduction and improvement schemes | Waveform, modulation, and UE types discussion in RAN1 may have impact on this topic |
| AMPR | - Depends on whether new special regulation requirements for the new bands | N |
| Pcmax | - Discuss how to solve “power class fallback” schemes in 5G, e.g., SAR related - Discuss how to simplify the Pcmax definitions, e.g., SRS IL, power boosting - Discuss the approaches to enable UE fully utilize its PA capabilities | RAN1/2 power control and PHR reporting may have impact on this topic |
| Min Tx power | - Most likely can reuse 5G framework | N |
| OFF power | - Most likely can reuse 5G framework | N |
| ON/OFF time mask | - Reuse 5G framework in general ON/OFF time mask - FFS on how to harmonize the switching related time masks | RAN1/2 Tx/Rx switching related schemes may have impact on this topic |
| Power control tolerance | - Most likely can reuse 5G framework | RAN1/2 power control may have impact on this topic |
| Freq error | - Most likely can reuse 5G framework | N |
| EVM | - Discuss EVM requirements for high modulation, and non-uniform constellations - Discuss EVM relaxation schemes for high modulation | RAN1/2 highest modulation order and constellation design may have impact on this topic |
| Carrier leakage | - Most likely can reuse 5G framework | RAN1 physical design may have impact on this topic, e.g., whether the DC will carry data |
| IBE | - Most likely can reuse 5G framework | N |
| Occupied BW | - Most likely can reuse 5G framework for some CBWs - The spectrum utilization discussion in system parameter part will impact this requirement | RAN1 spectrum utilization discussion may impact this topic |
| SEM | - FFS on whether new SEM requirement is needed for the new spectrum, if not can reuse 5G framework | N |
| ASEM | - Depends on whether new special regulation requirements for the new bands | N |
| ACLR | - Most likely can reuse 5G framework for PC3/2/1.5 ACLR - FFS on the new power class demands | RAN1 physical design may have impact on this topic |
| SE | - Most likely can reuse 5G framework | N |
| Tx intermodulation | - Most likely can reuse 5G framework | N |
| Rx sensitivity | - Discuss the REFSENS for new high bands, e.g., NF and SNR | RAN1 physical design may have impact on this topic, e.g., the SNR demodulation target |
| Max input level | - Most likely can reuse 5G framework | N |
| ACS | - FFS on new high bands | RAN1 physical design may have impact on this topic |
| IBB | - FFS on new high bands | N |
| OOB | - FFS on new high bands | N |
| Narrow band blocking | - Most likely can reuse 5G framework | N |
| Spurious response | - FFS on new high bands | N |
| Rx intermodulation | - FFS on new high bands | N |

Power class for FR1

Proposal 2:	Power class definition should make sure UE can fully utilize the PA power capabilities while distinguish different levels of UEs in terms of Tx power.

Observation 2:   Less than 3dB power step makes the power classes are not well distinguished and has much overlapping power.

Proposal 3:	Keep 3dB power step among power classes.

Observation 3:   For UE with multiple Tx PAs, the gap between max UE power capability and power class can reach 1.8dB.

Observation 4:   The +/-2dB power tolerance was introduced to accommodate massive production variations.

Proposal 4:	Study how to handle the issue of power gap b/w UE max Tx power capability and power class reporting in the beginning of 6G

Proposal 5:	Consider reuse the power boosting scheme that allow UE to report the boosted power comparing to the reported power class.

Proposal 6:	For band combination, not define the total power limitation. Rely on the power capability of each band in the BC (ue-PowerClassPerBandPerBC) to determine the max Tx power.

Observation 5:   For UEs not supporting TxD, the power capability could be different for different MIMO layer configurations, like PC2 in single port, but PC1.5 for UL MIMO. This power difference cannot be differentiated with 5G NR per band power class reporting.

Proposal 7:	For single band power class reporting, allow UE to report different power class for different MIMO Layers.

Observation 6:   For TDD bands, PC2 has been widely supported on the market.

Proposal 8:	For TDD bands, PC2 is defined as baseline. FFS whether PC2 or PC3 is baseline for FDD bands.

MPR

Observation 7:   In MRP requirements definitions, waveform, modulation, RB allocation, power class, UE types (antenna isolation), MIMO configuration, and PA/RFIC models need to be considered. Among them some of the factors have some dependent on RAN1.

Proposal 9:	RAN4 can focus on below aspects to study the 6G MPR

        - Waveform: use DFT-S-OFDM and CP-OFDM as starting point

        - Modulation: use uniform modulations as starting point

        - Power class: take PC2 as starting point

        - UE types: use handheld UE as starting point, and FFS on how much dB antenna isolation need to be considered for other UE types

        - MIMO: single port as starting point

        - PA model: discuss which kind of PA models to be used in 6G, e.g., fixed biased model, APT model, etc. and also consider the new high bands

        - RFIC impairment: use 5G carrier leakage, IQ image, CIM3 assumptions as starting point and further discuss whether some updates can be made

        - Study how to better align simulation and measurements results.

Proposal 10:	Study whether the requirement (ACLR/SEM/EVM) relaxation scenarios could be used as default in 6G to make it more useful.

Observation 8:   The CBW/RB allocation used in 5G MPR calibration is not the worst case in terms of spectrum utilization which makes the calibrated PA has worse linearity and finally large MPR requirements.

Observation 9:   The PA calibration results and MPR simulation results don’t show obvious difference between the calibration conditions of 20MHz/100RB@0 and 40MHz/full RB.

Proposal 11:	Continue to use 20MHz/100RB@0 as the MPR PA calibration condition in 6G.

Observation 10:	To pass the EVM requirements of 256QAM, -34dBc for carrier leakage and IQ image has been used in 5G MPR simulation/measurements. There is no difference in RFIC carrier leakage and IQ image in terms of modulation orders.

Observation 11:	Carrier leakage/IQ image tighten to -34dBc can improve the MPR for 64QAM around 0.6dB in some allocations, and is necessary for 256QAM.

Proposal 12:	The carrier leakage and IQ image condition for MPR calibration is tightened from -28dBc to -34dBc in 6G.

Observation 12:	Due to various PAs on the market and implemented in UE, no single PA model can be universally used in the simulations. This trend will continue in 6G.

Proposal 13:	The PA models used in 6G MPR simulation is still based on each company’s own choice.

Observation 13:	APT PA is widely used in UE implementation, and the APT PA will change its working modes when the output power is different which could reduce the power consumption but with the sacrifice of PA linearity, i.e., larger MPR.

Observation 14:	From simulation results it can be seen that the MPR required for APT PA could be 5dB higher than fixed bias PA in some configurations.

Proposal 14:	The APT PA models is encouraged to be used in 6G MPR simulation to better align with the real products and measurements.

Proposal 15:	The 6G MPR simulation calibration conditions is as below table.

|  | 6G |
| --- | --- |
| CBW, scs, RB allocation | 20MHz, 15khz scs, 100RB@0 |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Targe power level to meet ACLR | 1dB MPR |
| carrier leakage | -34dBc |
| IQ image | -34dBc |
| CIM3 | -60dB |

Pcmax

Observation 15:	Pcmax,h has become the gating factor for UE to transmit higher power especially when UE has multi-Tx PAs.

Observation 16:	Power boosting or remove the Pcmax,h are two potential approaches to allow UE transmit higher power whenever possible.

Proposal 16:	Study how to enable UE to fully utilize its PA capabilities. And power boosting approach is preferred comparing to remove the Pcmax,h limits.

Observation 17:	Duty cycle-based SAR solutions in RAN4 spec is very complex, but it hasn’t been implemented in UE. Only PMPR is applied.

Proposal 17:	In 6G, only rely on PMPR to solve SAR issue and no other SAR solution is to be defined.

EVM

Observation 18:	Besides of the uniform modulations already defined in 5G, some enhancements including even higher modulation, and non-uniform modulations are under discussion in RAN1 which will have impact to RAN4 EVM requirements.

Proposal 18:	RAN4 needs to closely follow the RAN1 discussion on even higher modulations in UL like 1024QAM with AI/non-AI based schemes, and the non-uniform modulations like probabilistic shaping or geometric shaping. And study the impacts in terms of implementation complexity, requirements and performance.

Coherent MIMO

Observation 19:	Coherent UL MIMO was defined in 5G, but no be supported in UE/network.

Proposal 19:	Study how to enable coherent UL MIMO to improve UE performance.

REFSENS

Proposal 20:	The REFSENS calculation equation below can be reused in 6G, and 3dB for diversity gain with 2Rx antennas, 2.5dB implementation margin. For the NF, RAN4 can start with the discussion of targeting bands and analyze its related NF.

- REFSENS (dBm) = -174dBm/hz + 10lg(RxBW) + NF + 10lg(SU) – DivGain + SNR + IM

Requirements for different UE types

Observation 20:	Device types discussed in RAN1 and RAN plenary mainly focus on the mandatory features for each device type which is different from RAN4 requirement definition. RAN1 may define many device types but in RAN4 they may share same RF requirements.

Observation 21:	RAN4 has defined many UE types but not be well organized in the 5G spec. This has caused many confusions in the industry in terms of which requirement to be applied to which UE types.

Observation 22:	For most of UE types defined in RAN4, the antenna numbers and antenna isolations were the main concern in 5G. Antenna numbers mainly about the relaxation from mandatory 4Rx to 2Rx for some specific UE type, and antenna isolation is mainly for the MRP requirement definition.

Proposal 21:	Consider using antenna isolations as the discriminator to differentiate applicable MPR requirements only and no bounding of UE types. Any UE that meets the antenna isolation conditions can apply the corresponding requirements.

Requirements for different UE architectures

Proposal 22:	RAN4 strive to define only one set of requirements for each feature and avoid multiple sets of requirements whenever possible in 6G.

Proposal 23:	RAN4 further discuss whether the worst case or the most common case implementation should be adopted in requirement definition based on case by case.

UE capability signaling simplification

Observation 23:	In the testing/certification, UE declaration approach can be applied to make it clear which requirements should be tested. There is no need to define UE capabilities just for the purpose of differentiating the applicable requirements for a UE.

Proposal 24:	In 6G, the testing purpose singling should be avoided and when new signaling is introduced it shall be justified that it is useful for BS scheduling.

Proposal 25:	In 6G, avoid defining UE capabilities with small granularity when it is unnecessary.

FR2 UE RF

Proposal 26:	In 6G, reuse 5G FR2 requirements as much as possible and focus on 1CC in the beginning.

Reference

R4-2311151 PA Calibration using 40MHz CBW waveforms for FR1, Skyworks, #108 2023-08

R4-2318781 Reference waveform for PA calibration, Qualcomm, #109 2023-11