3GPP TSG-RAN WG4 Meeting #116bis	R4-2513433

Prague, Czech Republic, October 13 – 17, 2025

Title: 	On performance requirements for LP-WUS

Source: 	Huawei, HiSilicon

Agenda item:	6.16.4

Document for:	Discussion

Background

In last meeting, a WF for NR_LPWUS_Demod was agreed, this contribution provides our views on related open issues.

- Discussions

One issue is whether to define a test mode for LP-WUS/WUR:

Whether to define a test mode for LP-WUS/WUR - Agreement: Interested companies can provide an analysis of the expected test durations for the different modes next meeting. Further discuss if a test mode is required based on the provided analysis.

Counting MDR based on ACK/NACK in connected state or MSG1/3 in IDLE state can’t correctly verify LP-WUS received by WUR as per the RAN1 core specification definition. So the test should directly check the number of correctly detected LP-WUS signal received by LP-WUR corresponding to the group ID belongs to the UE, otherwise we can’t distinguish the feedback is from MR rather than LR., a UE using MR rather than LR to monitor PDCCH can also report ACK/NACK in connected mode or transmit MSG1/3 in IDLE mode that is the normal UE behaviour without LP-WUR implemented, so. Therefore, a proper approach is to introduce a test mode that enables UE to directly report to the TE the number of detected LP-WUS signal corresponding to the group ID belongs to the UE. The correctly detected number of LP-WUS compared to the total number of LP-WUS transmitted by TE should be equal or less than 1%.  A specific test cases should be defined to properly verify the performance of LP-WUR to correctly detect LP-WUS rather than the mixed performance of LP-WUR and WR.

Proposal 1: The test metric should be the number of correctly detected LP-WUS signal received by LP-WUR corresponding to the group ID belongs to the UE.

Proposal 2: Introduce a test mode that enables UE to directly report to the TE the number of detected LP-WUS signal received by LP-WUR corresponding to the group ID belongs to the UE.

One issue is whether to introduce the FAR test.

Whether to introduce FAR test - Proposals - Option 1: Yes - Option 2: No

It’s proposed not to introduce FAR test, high FAR means high power consumption, which is less important comparted to high MDR which means high missing data or paging occasion. However, FAR impact MDR performance since it impacts the energy threshold. Hence, it’s necessary to unify the FAR when running MDR simulation, otherwise, it would be hard to align the results.

Proposal 3: Not to introduce dedicate FAR test. For the MDR simulation alignment, set FAR to 1%.

One issue is Whether to define separate test cases for idle/inactive and connected modes

Whether to define separate test cases for idle/inactive and connected modes - Proposals - Option 1: Define separate test cases procedure for the modulator in idle and connected modes.

Supporting IDLE/INACTIVE mode and CONNECTED mode is based on UE capabilities, which means a UE supporting LP-WUS operation in IDLE/INACTIVE mode does not necessarily to support operation in CONNECTED mode, and vice versa. Hence the test mode should support both connected and IDLE models. UE could detect LP-WUS and count number of correct detected group IDs in connected or IDLE mode based on UE’s capability and report the results to TE in connected mode.

Proposal 4: UE detects LP-WUS and counts number of correct detected group IDs in connected or IDLE mode based on UE’s capability and report the results to TE in connected mode.

Time offsets

One issue is time offset:

- WF - Option 1: T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4

According to RRM requirements, the time error is defined as following:

Time error= Residual time error + time drift (frequency offset*320ms) T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4

We prefer to reuse it for demodulation test.

Proposal 5: RAN4 to use following for demodulation test:

Time error= Residual time error + time drift (frequency offset*320ms)

T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4

Whether to define single requirement for Type 1 and Type 2 receivers

One issue is whether to define single requirements for Type 1 and 2 receivers:

Whether to define single requirement for Type 1 and Type 2 receivers - Proposals - Option 1: RAN4 to study the performance for both receiver types (Type 1: OOK and Type 2: OFDM) and further discuss based on simulation results if one or two sets of requirements are to be defined. - Option 2: Do not define separate demodulation requirements for Type-1 and Type-2 LP-WUR capable UEs if those are not tied to Envelope vs Sequence Detection.

We don’t see the relationship between demodulation requirements and Type1/2 receiver capability.  Type1 and 2 receivers are differentiated according to power consumption, which has nothing to do with baseband processing. We don’t support defining such capability rules for demodulation requirements.

Proposal 6: Support option 2, don’t define any capability rules related to Type1/2 receivers for demodulation requirements.

Choice of M for ORFM receiver

One issue is M for OFDM receiver:

- Agreement: - M=1 and M=4 as a starting point - Interested companies can also bring results with M=2

We prefer to only consider M=4 to reduce the workload since this parameter doesn’t have baseband processing changes.

Proposal 7: Only consider M=4.

Calculation of timing error

One issue is timing error:

- Proposals - Option 1: Time error= Residual time error + time drift (frequency offset*320ms)

Support Option 1 since it is agreed in RRM assumption.

Proposal 8: Support Option 1: Time error= Residual time error + time drift (frequency offset*320ms)

Number of coded bits after rate-matching

One issue is number of coded bits after rate-matching:

- Agreement: - [12, 16] for 2 info bits - 32 for 5 info bits - Other options are not precluded

We support 12 bits for 2 info bits as starting point.

Proposal 9: Support 12 bits for 2 info bits as starting point.

- Conclusion

In this contribution we provide our views on performance requirements for LP-WUS, the proposals are:

Proposal 1: The test metric should be the number of correctly detected LP-WUS signal received by LP-WUR corresponding to the group ID belongs to the UE.

Proposal 2: Introduce a test mode that enables UE to directly report to the TE the number of detected LP-WUS signal received by LP-WUR corresponding to the group ID belongs to the UE.

Proposal 3: Not to introduce dedicate FAR test. For the MDR simulation alignment, set FAR to 1%.

Proposal 4: UE detects LP-WUS and counts number of correct detected group IDs in connected or IDLE mode based on UE’s capability and report the results to TE in connected mode.

Proposal 5: RAN4 to use following for demodulation test:

Time error= Residual time error + time drift (frequency offset*320ms)

T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4

Proposal 6: Support option 2, don’t define any capability rules related to Type1/2 receivers for demodulation requirements.

Proposal 7: Only consider M=4.

Proposal 8: Support Option 1: Time error= Residual time error + time drift (frequency offset*320ms)

Proposal 9: Support 12 bits for 2 info bits as starting point.

- Reference

[1] R4-2508626_WF for [115][332] NR_LPWUS_Demod. Vivo