3GPP TSG-RAN WG4 Meeting #116bis	R4-2513469

Prague, Czech Republic, 13-17 October 2025

Agenda item:	6.10.2.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance part of optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

According to the last meeting’s way forward, most issues have been allocated to be treated in the CR discussion. In this contribution, we provide our further views on the 3-seacher test cases.

- Discussion

L3 measurement delay by optimising CSSF outside gap in CA/DC

Test set for solution 3

Since Solution 3 reduces the measurement delay of SCells, the test set for Solution 3 should be designed uniformly to evaluate how effectively this enhancement improves the measurement process of SCells. For Solution 3, it is essential to let the UE measure and report the measurement results of three Cells simultaneously. Here is a possible test set for Solution 3:

Four cells are needed and one PCell, one neighbour Cell of PCC or one inter-RAT E-UTRAN Cell, and two deactivated SCells.

Proposal 1: To unify Solution 3 test cases, the test set of Solution is able to refer to four cells: one PCell, one neighbour Cell of PCC or one inter-RAT E-UTRAN Cell, and two deactivated SCells.

In our view, the test case will let the three cells, the neighbour Cell of PCC and two deactivated SCells, report A6 or B2 to trigger the measurement. The reporting delay for each of the three cells should match the measured reporting delay established in their single-cell configurations. This is to make sure the UE is treating these three cells’ measurements simultaneously.

- Conclusions

Proposal 1: To unify Solution 3 test cases, the test set of Solution is able to refer to four cells: one PCell, one neighbour Cell of PCC or one inter-RAT E-UTRAN Cell, and two deactivated SCells.

4. References

[1] R4-2512124 WF on RRM requirements for NR_RRM_Ph5_Part1