3GPP TSG-RAN WG4 Meeting #116bis	R4-2513482

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:    4.21.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 NES

Document for:	Discussion

- Introduction

For Rel-19 NES, RAN4 discuss the RRM impacts of OD-SSB with issues and agreement captured in [1]. In this paper, we provide our views on remaining issues.

- Discussion

OD-SSB

One maintenance issue is whether the OD-SSB MAC CE can trigger L3 measurement reporting [2].

Issue 1-4-4: OD-SSB based L3 reporting Agreement The discussion of the issue shall not impact the RAN4 core part completion of the WI.

RAN2 discussed the issue in previous meeting and achieved following agreements. Then RAN4 will make conclusion.

Agreement RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.

In scenario 2 (before SCell activation), it is beneficial to introduce OD-SSB MAC CE triggered measurement and reporting. The common understanding is gNB is expecting a timely report upon activating OD-SSB with fast measurement window, for instance, if the NW wants to check the signal quality of the deactivated SCell. From UE side, UE can trigger the measurement report if there is valid measurement report and then UE can stop fast measurement. It may be argued that the network can always configure a very low value in Event A1 to trigger serving cell measurement reporting, but this implementation may bring excessive measurement reporting (especially for case 2, in which the AO-SSB will also trigger the measurement reports).

The aforementioned approach of OD-SSB MAC CE triggering L3 measurement reporting is similar as introduced in Rel-18 RRM “report on SCell activation” (i.e., SCell Activation/Deactivation MAC CE can be used to trigger the UE to report available measurement results of the SCell), and the workload is manageable in RAN2 since framework can be reused.

Proposal 1: Support L3 report triggered by OD-SSB activation MAC CE that UE triggers report after measurement based on OD-SSB FMW.

Another issue is to check the condition(s) under which the OD-SSB based cell identification before deactivated SCell measurement can be skipped.

Issue 1-4-7: Cell identification Agreement - If UE has already measured OD-SSB for Tidentify_intra_without_index_ODSSB or Tidentify_intra_with_index_ODSSB and the OD-SSB deactivated period is within 5s and the SCell is detectable when OD-SSB is transmitted, then the OD-SSB is activated again and the SCell remain detectable with the same spatial reception parameter and same frequency, UE shall skip the OD-SSB identification component provided the timing to that cell has not changed more than 3200/2µ Tc. - Note: Option 2 can be discussed in maintenance stage.

Issue 1-4-7: Cell identification The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios. - Proposals - Option 1: - UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within 5s and then the subsequent OD-SSB is transmitted with the same spatial reception parameter. - Option 2: - Case 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band. - Case 2: Always-on SSB on the cell

From our perspective, the intention of cell identification of serving deactivated SCell is to acquire its coarse timing. When OD-SSB SCell and other active serving cell in the same band are contiguous, as they have strong correlation, coarse timing of the OD-SSB SCell can refer to PCell. In this sense, it seems reasonable to skip OD-SSB based cell identification for the deactivated serving SCell. This can be regarded as a kind of enhancement, and we are open whether to capture it in maintenance phase.

It shall be noted, for intra-frequency neighbor cell of the deactivated OD-SSB SCell (neighbor cell may be located in a different gNB), the cell identification can not be skipped and shall be performed based on measCycleSCell as legacy.

The motivation of case 2 is that time tracking has been maintained based on AO-SSB when OD-SSB is activated on the deactivated SCell, and then a FWM is started. However there is possibility that the coarse timing is lost if UE only perform cell detection at the start period of TPSS/SSS_sync.

In addition, as per the agreed CR in RAN4#116 meeting, after OD-SSB is activated, EMP is used for cell identification for the deactivated SCell, where is the union of OD-SSB and AO-SSB in case2. The union of OD-SSB and AO-SSB can fasten cell identification delay, we don’t think there is benefit to modify the above requirements.

Table 9.x.5.3-1: Time period for OD-SSB based PSS/SSS detection in FMW, deactivated SCell

| Frequency range | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| FR1 | ceil(M Note 2 x Kp) x max(Tlower Note 1, EMP) x CSSFintra |
| FR2 | ceil(M Note 2 x KFR x Kp x Klayer1_measurement) x max(Tlower Note 1, EMP) x CSSFintra |
| NOTE 1: When UE reports not supporting lower bound capability, the Tlower equals 10ms; Otherwise, Tlower equals 0ms. NOTE 2: When multiple frequency layers are configured, M equals 6 in FR1 and Mpss/sss_sync_w/o_gaps+1 in FR2. Otherwise, M equals 5 in FR1 and Mpss/sss_sync_w/o_gaps in FR2. |  |

For UE supporting On-demand SSB operation and when OD- SSB transmission is indicated, the EMP for OD-SSB based intra-frequency measurement are - the periodicity of OD-SSBs in the configured DL BWP if no first SSB transmission is configured, or - the periodicity of the union of OD-SSBs and first SSBs if first SSBs are transmitted and the OD-SSB activation is indicated.

Proposal 2: Regarding whether to skip cell identification,

-We are open to whether capture the case of skipping OD-SSB based cell identification for deactivated serving SCell when the OD-SSB SCell being measured is contiguous to an active serving cell in the same band. For intra-frequency neighbor cell of the deactivated OD-SSB SCell, the cell identification can not be skipped and shall be performed based on measCycleSCell as legacy.

-suggest not to skip cell identification even if AO-SSB is present.

OD-SIB1

In RAN4#116 meeting, two proposals (P2 and P3 shown as below) for OD-SIB1 were discussed, and the conclusion is to keep the previous agreement, and further discuss whether additional requirements are needed.

Issue 2-1-1: RRM requirements impacts for OD-SIB1 - Proposal 2: RAN4 to consider following change to cell reselection delay in idle mode, when UE performs cell reselection to an OD-SIB1 cell: Add the phrase “At intra-frequency cell re-selection, Tdetect,NR_Intra , Tmeasure,NR_Intra and Tevaluate,NR_Intra may be longer if the target Cell is a NES Cell supporting on-demand SIB1 transmission.” for intra-frequency NR cells in clause 4.2.2.3 of TS 38.133 and the same phrase in clause 5.1.2.3 for inactive mode. Add similar statements in clauses 4.2.2.4 and 5.1.2.4 for inter-frequency NR cells and in clauses 4.2.2.5 and 5.1.2.5 for inter-RAT E-UTRAN cells. - Proposal 3: RAN4 to consider the impact on RRM core requirements for one or more UL-WUS Msg1 retransmission attempts in the NES cell, if configured.

Agreement on Ad hoc (R4-2512135): - Keep current agreement and if companies think additional requirements are needed discuss in maintenance stage.

RAN4 agreements in R4-2508441 Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption may be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.

Regarding P2, UE is not required to read SIB1 of the neighbour cell when UE detects/measures the cell and evaluates cell reselection criteria. Therefore we don’t think Tdetect,NR , Tmeasure,NR and Tevaluate,NR shall be longer if the target cell is a NES Cell supporting on-demand SIB1 transmission.

Proposal 3: Keep the current requirements of Tdetect,NR, Tmeasure,NR and Tevaluate,NR, if the target cell is a NES Cell supporting on-demand SIB1 transmission.

Regarding P3, as per RAN1, the UL-WUS Msg1 retransmission would be performed in case UE does not receive corresponding RAR. In our understanding, UL-WUS msg1 retransmission has been included in TSI (see the agreed BigCR). The further optimization on priority is not needed.

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. If the target NR cell is an NES cell and UE is capable of on demand SIB1 acquisition, TSI-NR could be longer due to UL WUS transmission and RAR reception for on demand SIB1 acquisition of NES cell.

Proposal 4: Additional requirements of UL-WUS retransmission are not needed in NES cell.

- Conclusions

OD-SSB

Proposal 1: Support L3 report triggered by OD-SSB activation MAC CE that UE triggers report after measurement based on OD-SSB FMW.

Proposal 2: Regarding whether to skip cell identification,

-We are open to whether capture the case of skipping OD-SSB based cell identification for deactivated serving SCell when the OD-SSB SCell being measured is contiguous to an active serving cell in the same band. For intra-frequency neighbor cell of the deactivated OD-SSB SCell, the cell identification can not be skipped and shall be performed based on measCycleSCell as legacy.

-suggest not to skip cell identification even if AO-SSB is present.

OD-SIB1

Proposal 3: Keep the current requirements of Tdetect,NR, Tmeasure,NR and Tevaluate,NR, if the target cell is a NES Cell supporting on-demand SIB1 transmission.

Proposal 4: Additional requirements of UL-WUS retransmission are not needed in NES cell.

References

[1] R4-2504906 WF on RRM requirements for Netw_Energy_NR_enh_Part1

[2] R4-2512327 WF on RRM requirements for Netw_Energy_NR_enh_Part1 v1