3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513484

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.15.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance requirements for On-demand SSB SCell operation

Document for:	Discussion

- Introduction

For Rel-19 NES, the performance requirements discussion was initially discussed in RAN4#115 with progress captured in [1]. In this contribution, we provide our views on the performance impacts on relevant objectives.

- Discussion

For OD-SSB, the way forward in last meeting are summarized as follows:

Sub-issue 1: Test scope Issue 3-1-1: Scenario RAN4 to discuss whether the following test scenarios can be agreed. - RAN4 to define the OD-SSB based test case for both Case 1 and Case 2. - RAN4 to test OD-SSB based Scell activation for scenario 2 and 2a. - For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B. - For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell. - RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement. - RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission. - No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB. - RAN4 to verify the EMP requirement in case 2 Alt Time-C1. Sub-issue 2: Test case list Issue 3-1-3: Test case list RAN4 to discuss whether the following OD-SSB test cases can be agreed. - SCell activation in Case 1 - SCell activation in Case 2 Alt Time C-1 - OD-SSB based multiple SCell activation - OD-SSB based direct SCell Activation at SCell addition in Case 1 - OD-SSB based direct SCell Activation at SCell addition in Case 2 - OD-SSB based PUCCH SCell Activation in Case 1 - OD-SSB based PUCCH SCell Activation in Case 2 - Fast SCell Activation with OD-SSB; - OD-SSB based SCell Activation with the L3 reporting during activation - Intra-frequency measurement(deactivated SCell) in Case 1 - Intra-frequency measurement(deactivated SCell) in Case 2 - Intra-frequency measurement in Case 1 - Intra-frequency measurement in Case 2 - L1-RSRP measurement with OD-SSB parameter update(Scenario 3B) - Interruption at OD-SSB activation/deactivation - Interruptions during measurements on deactivated NR SCC Sub-issue 3: Test configuration Issue 3-1-4: General principle RAN4 to discuss whether the following principles to design the OD-SSB test case can be agreed. - Both FR1 and FR2 - At least test OD-SSB periodicities with 5ms, 20ms. FFS 10ms - Only test non-DRX mode - The test without SSB time index detection - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting - SMTC and measCycleSCell as 40ms/160ms

Regarding the Scenarios to be verified, we agree with most of the recommended WF. Regarding the whether to verify UE’s behaviour regarding T2, we think there is no need to verify any behaviour that UE is allowed to fallback to legacy measurement behaviour or when two consecutive OD-SSB are triggered within T2 limits. However, the behaviour that two consecutive OD-SSB triggering are spaced larger than T2 can be verified. In details, UE shall trigger FMW each time when OD-SSB is triggered.

Proposal 1: RAN4 to discuss whether to verify the requirements in the TC that OD-SSB are triggered multiple times in deactivated SCell measurement, and UE trigger FMW each time when each OD-SSB activation fulfils T2 conditions.

As per progress in core requirement discussion, the proposed test case lists are as follows:

Table 1. Test cases list for OD-SSB

| TC | Case#1/Case#2 | FR1/FR2 | Remarks |
| --- | --- | --- | --- |
| Deactivated SCell measurement | Case#1 | Both FR1 and FR2 |  |
|  | Case#2 | Both FR1 and FR2 |  |
| SCell activation based on OD-SSB | Case#1 | Both FR1 and FR2 | both known and unknown case |
|  | Case#2 | Both FR1 and FR2 | both known and unknown case |
| Direct SCell activation based on OD-SSB | Case#1 | Both FR1 and FR2 | both known and unknown case |
|  | Case#2 | Both FR1 and FR2 | both known and unknown case |
| PUCCH SCell activation based on OD-SSB | Case#1 | Both FR1 and FR2 | both known and unknown case |
|  | Case#2 | Both FR1 and FR2 | both known and unknown case |
| OD-SSB based SCell Activation with the L3 reporting during activation | Case#1 | Both FR1 and FR2 | Pending on Core discussion |
|  | Case#2 | Both FR1 and FR2 | Pending on Core discussion |
| Multiple OD-SSBs based deactivated SCells measurement | Case#1 | Both FR1 and FR2 | UE is configured with two OD-SSB SCells: In deactivated SCells measurement, OD-SSB are activated one by one. |
|  | Case#2 | Both FR1 and FR2 | UE is configured with two OD-SSB SCells: In deactivated SCells measurement, OD-SSB are activated one by one. |
| Multiple OD-SSBs based SCells activation | Case#1 | Both FR1 and FR2 |  |
|  | Case#2 | Both FR1 and FR2 |  |
| Note: For Case#2, RAN4 define TCs in Alt Time-C1 and can select some tests for Alt Time-C2. |  |  |  |

Regarding the details of SSB/SMTC configurations, to verify the new behaviour/requirements introduced in Rel-19 NES, it is proposed to defined periodicity of OD-SSB, SMTC and measCycleSCell as 20ms/40ms/160ms respectively.

Proposal 2: Configure the periodicity of OD-SSB, SMTC and measCycleSCell as 20ms/40ms/160ms respectively.

Proposal 3: Only define TC for non-DRX mode.

Proposal 4: For OD-SSB, RAN4 defined TC as table 1.

Proposal 5: For Case#2, RAN4 define TCs in Alt Time-C1 and can select some tests for Alt Time-C2.

- Conclusions

Proposal 1: RAN4 to discuss whether to verify the requirements in the TC that OD-SSB are triggered multiple times in deactivated SCell measurement, and UE trigger FMW each time when each OD-SSB activation fulfils T2 conditions.

Proposal 2: Configure the periodicity of OD-SSB, SMTC and measCycleSCell as 20ms/40ms/160ms respectively.

Proposal 3: Only define TC for non-DRX mode.

Proposal 4: For OD-SSB, RAN4 defined TC as table 1.

Proposal 5: For Case#2, RAN4 define TCs in Alt Time-C1 and can select some tests for Alt Time-C2.

References

[1] R4-2512327 WF on RRM requirements for Netw_Energy_NR_enh_Part1